3GPP TSG-RAN WG2 Meeting #132												R2-2508351

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	8.20.1

Source: 	Sharp, CHTTL, SES, Huawei, HiSilicon, Samsung

Title:  	Discussions on NTN Ku Band Capabilities

Document for: 	Discussion

# Introduction

In a RAN4-led WI on NTN Ku Band (WI code: NR_NTN_Ku_bands), RAN4 agreed to include two FR1-NTN bands (n248, n247) and two FR2-NTN bands (n509, n508) as the priority 1 Ku bands [1][2] to cover 10.7~14.5GHz frequency ranges, as follows:

### 5.2.2 Operating bands for FR1-NTN NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1. Table 5.2.2-1: NTN satellite bands in FR1-NTN NTN satellite operating band1; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 – 1626.5 MHz; 2483.5 – 2500 MHz; FDD. n252; 2000 – 2020 MHz; 2180 – 2200 MHz; FDD. n248; 14000 MHz - 14500 MHz; 10700 MHz – 127502,3 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 127502,3 MHz; FDD. NOTE 1: NTN satellite bands are numbered in descending order from n256. NOTE 2: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 3: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. ### 5.2.3 Operating bands for FR2-NTN NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1. Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n5121; 27500 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5112; 28350 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5103; 27500 MHz - 28350 MHz; 17300 MHz - 20200 MHz; FDD. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504,5 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504,5 MHz; FDD. NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT..

Due to the addition of those bands, a VSAT UE operating in the newly added FR1-NTN bands is required to report two capabilities, i.e., VSAT UE type in NTN (fixed or mobile) and beam steering type (electronic or mechanical). Those capabilities are captured in the RAN4 UE feature list [3] as R4 60-1 and 60-2, respectively.

In RAN2#131bis meeting, RAN2 discussed how to support those capabilities and made the following working assumption [4]:

WA: Introduce new Rel-19 capability bits applicable for both FR1-NTN Ku and FR2-NTN Ku (FFS if we use exactly the FR1-NTN Ku and FR2-NTN Ku terminology or if we find another alternative to further align to RAN4 terminology) Come back in the next meeting on the need of Rel-18 CRs to clarify the description for legacy UEs

This document is to finalize all relevant open issues to endorse/agree the (draft)CRs in RAN2#132.

# Discussion

## Confirmation of Working Assumption and CR Texts

RAN2 made the working assumption that RAN2 introduced new Rel-19 capability bits applicable for both FR1-NTN Ku band and FR2-NTN Ku band. The working assumption tries to follow the RAN4 feature list which asked no FR1-FR2 differentiation. The reason was FR1-NTN Ku band and FR2-NTN Ku band are on the same frequency band, so there is no reason to have different VSAT capabilities with setting to different values. Table 1 shows which UE capability is used to report the VSAT capabilities, based on the working assumption.

| Supported band | VSAT Capability to use |
| --- | --- |
| Support legacy FR1-NTN band | N/A |
| Support legacy FR2-NTN band | Rel-18 capability |
| Support only FR1-NTN Ku band (n247 and/or n248) | Rel-19 capability |
| Support only FR2-NTN Ku band (n508 and/or n509) | Rel-19 capability |
| Support both FR1-NTN Ku band and FR2-NTN Ku band | Rel-19 capability |

Table 1. VSAT capability reporting according to the working assumption.

From the capability signaling perspective, there is no confusion/misunderstanding foreseen. In our view, the working assumption can be confirmed.

Confirm WA: Introduce new Rel-19 capability bits applicable for both FR1-NTN Ku band and FR2-NTN Ku band.

Regarding how to clearly capture the difference between Rel-18 capability and Rel-19 capability, “Ku band” is not used in RAN2 specification and TS 38.101-5 RAN4 specification, so simply using “Ku band” is not desirable. We can consider using frequency or band number. It is expected that using band numbers may require future work if more new bands are introduced in the later releases. Especially, RAN4 may introduce additional bands for Ku band in Rel-20 or onward. The VSAT capability depends on hardware design (e.g. antenna size) related to operating frequency. Specifying range of frequencies would be the best way at this moment.

In VSAT capability field descriptions, frequency information is specified to clarify which capability is used.

The corresponding text proposal for TS 38.306 is as follows:

| ntn-VSAT-AntennaType-r18 Indicates whether a VSAT UE uses electronic or mechanical steering antenna in FR2-NTN bands above 14.5 GHz. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only |
| --- | --- | --- | --- | --- |
| ntn-VSAT-AntennaType-r19 Indicates whether a VSAT UE uses electronic or mechanical steering antenna in FR1-NTN and/or FR2-NTN bands between 10.7 GHz and 14.5 GHz as defined in TS 38.101-5 [34]. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | No |
| ntn-VSAT-MobilityType-r18 Indicates whether a VSAT UE is a mobile or fixed VSAT in FR2-NTN bands above 14.5 GHz. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only |
| ntn-VSAT-MobilityType-r19 Indicates whether a VSAT UE is a mobile or fixed VSAT in FR1-NTN and/or FR2-NTN bands between 10.7 GHz and 14.5 GHz as defined in TS 38.101-5 [34]. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | No |

The 38.306 draftCR [7] captures the same text changes, and the 38.331 draftCR [8] captures ASN.1 signaling.

## Necessity of Rel-18 CR

Although the supported frequency for a UE may be release-independent, Ku band is located far from both legacy FR1-NTN band and legacy FR2-NTN band. It is very difficult for legacy UEs to simply add Ku band due to the different frequency characteristics. Hence, we do not see the big necessity of the Rel-18 (draft)CRs for the Ku band capability. It would be better to have minimal corrections following the RAN4 request.

Only Rel-19 (draft) CRs are provided for Ku Band Capability.

## Clarification of FR1-NTN band in TS 38.306 and TS 38.331

Before the introduction of Ku band, FR1-NTN band was on the same band as FR1 band. For this reason and preference of minimal spec changes, TS 38.306 and TS 38.331 did not have any clarification on FR1-NTN band whereas FR2-NTN is captured. The RAN4 Ku band WI introduced new FR1-NTN bands on 10.7 GHz to 14.5 GHz. Now there is difference between FR1 and FR1-NTN. In RAN2#131bis meeting, two company CRs [5][6] proposed the clarifications to TS 38.306 and TS 38.331 specifications, for example:

< TS 38.306 >

This feature is applicable only for FR1, FR2-1, FR1-NTN and FR2-NTN band, otherwise it is absent.

< TS 38.331 >

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1/FR1-NTN: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

We see that now the clarification on FR1-NTN in TS 38.306 and TS 38.331 is necessary. It is not only for the difference between FR1-NTN and FR1 but also for consistency with FR2/FR2-NTN band.

Clarify FR1-NTN terminology in TS 38.306 and TS 38.331.

The corresponding 38.306 draftCR [7] and 38.331 CR [9] capture the corresponding changes.

## Enhanced RRM Requirements for FR1-NTN

As FR1-NTN Ku band is on the same band with FR2-NTN Ku band, the same requirement can apply. TS 38.133 [10] clarified that the requirements defined for FR2-NTN apply for VSAT operating in cells with FR1-NTN numerology as follows:

### 3.6.12 Applicability of requirements for Satellite Access The requirements for Satellite Access defined in clauses with suffix ‘C’ apply provided that UE indicates nonTerrestrialNetwork and is accessing a cell served by a Satellite Access Node (SAN). The requirements apply provided that serving and all neighbour satellites on the same layer are of same satellite type (LEO or GEO). Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT operating in cells with FR1-NTN numerology.

It has an impact to the UE capability spec. A company CR [5] proposed to capture a similar sentence in TS 38.306 as follows:

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes for FR2-NTN It is optional for UE to support enhanced RRM requirements for measurements for FDD FR2-NTN bands in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, other NTN measurement requirements (as specified in TS 38.133 [5], clause 4.2C.2 for RRC_IDLE and clause 5.1C.2 for RRC_INACTIVE) are applied. This capability applies to VSAT working in FR1-NTN as well.

We see that this clarification is aligned with the RAN4 specification, so it is needed.

Clarify that Enhanced RRM requirements in IDLE and INACTIVE modes for FR2-NTN apply to VSAT working in FR1-NTN numerology.

The 38.306 draftCR [7] captures the proposed change.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- Confirm WA: Introduce new Rel-19 capability bits applicable for both FR1-NTN Ku band and FR2-NTN Ku band.

- In VSAT capability field descriptions, frequency information is specified to clarify which capability is used.

- Only Rel-19 (draft) CRs are provided for Ku Band Capability.

- Clarify FR1-NTN terminology in TS 38.306 and TS 38.331.

- Clarify that Enhanced RRM requirements in IDLE and INACTIVE modes for FR2-NTN apply to VSAT working in FR1-NTN numerology.

# Reference

[1] R4-2510831	Big CR for 38.101-5 for the addition of Ku bands	Eutelsat Group

[2] 3GPP TS 38.101-5	NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements, v19.1.1

[3] R4-2511883	Rel-19 RAN4 UE feature list for NR (version 2)	CMCC

[4] R2-2507703	RAN2#131bis Report from Break-out session on NES, NR-NTN and IoT-NTN	Session Chair (ZTE Corporation)

[5] R2-2506934	(38.306 CR1356) Introduction of Ku band	Huawei, HiSilicon

[6] R2-2506933	(38.331 CR5492) Introduction of Ku band	Huawei, HiSilicon

[7] R2-2508352	(38.306 draftCR) Introduction of NTN Ku Band	Sharp, CHTTL, SES, Huawei, HiSilicon, Samsung

[8] R2-2508353	(38.331 draftCR) Introduction of NTN Ku Band	Sharp, CHTTL, SES, Huawei, HiSilicon, Samsung

[9] R2-2508247	(38.331 CR5492r1) Introduction of FR1-NTN terminology	Huawei, HiSilicon, Sharp

[10] 3GPP TS 38.133	Requirements for support of radio resource management, v19.2.0