3GPP TSG-RAN WG2 Meeting #132	R2-2508359

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial considerations on NTN design for 6G

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

RAN2#131bis discussed NTN for an hour, with the main topic being how to harmonize the TN and NTN design. Different understandings arose on how to harmonize, but general understanding seemed to be that the NTN should be considered in the initial design stages of 6G TN, unlike 5G where it required a lot of tweaking to address the NTN related challenges. Thus, the following agreement were met, urging companies to identify the learnings from 5G and how to apply them to day1 6G NTN integration in TN.

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

This document from the 3GPP TSG-RAN WG2 meeting discusses initial considerations for integrating non-terrestrial networks (NTN) into 6G design, emphasizing harmonization with terrestrial networks (TN), essential day-1 requirements like synchronization without GNSS, handling long propagation delays, mobility enhancements, and measurement frameworks, while proposing prioritization of FDD, common design features such as CB-PUSCH, and NTN-specific system information to support features like pre-compensation, discontinuous coverage, and time/location-based mobility.

# 2	Discussion

6G is assumed to define different device families each sharing a common set of minimum requirements. Different device families enable enough market opportunities for variety of devices with different form factors, hardware and processing capabilities, data rates and cost constraints now and in the future. 5G NTN added an additional device class VSAT, which could be stationary, nomadic or moving. These VSAT related use cases are relevant also for 6G and can be built as part of a TN device family for fixed wireless access (FWA), or customer premise equipment (CPE).

Observation 1: Integration of the NTN device class for VSAT should be harmonized as a part of FWA/CPE device class.

Proposal 1: NTN should be considered in all 6G device families.

## 2.1	Essential requirements for day-1

One of the largest differences between terrestrial and non-terrestrial networks is the fact that in non-terrestrial networks the network nodes move and not only the UEs. That makes that synchronisation in NTN essential for aligning the timing and frequency between UE and moving satellite nodes. Unlike terrestrial networks, NTNs must address significant doppler shifts, varying propagation delays, and frequent handovers/cell changes introduced by the high-speed movement of satellites.

In 5G, NTN is relying on GNSS, and the UE is aware of its own position. Using this information, and information about the ephemeris of the satellite, the UEs are able to pre-compensate both the propagation delay and the Doppler shift. This is used for instance for initial access, mobility procedures and PLMN selection. As GNSS can be spoofed, jammed or unavailable due to radio conditions, this makes the system vulnerable. Furthermore, the GNSS module also adds cost, power consumption and complexity to the UE. Therefore, the 6G NTN system should be able to function without the need for the UE to have GNSS capability. It is expected that RAN1 is studying this and will provide a synchronisation solution to this issue. Note that synchronisation must also support seamless mobility, including handovers between terrestrial and non-terrestrial networks (TN-NTN) and across multiple satellite orbits.

Observation 2: In order to ensure harmonization, RAN2’s design can assume at least that the UE is able to pre-compensate. Further considerations can be discussed after RAN1 progress.

However, although RAN1 studies this, RAN2 are the working groups considering handover and other items, which may use location such as conditional handover. As this does require a certain level of location accuracy, RAN2 should study the requirements as these are related to RAN2 procedures..

Proposal 2: RAN2 to study what location requirements are needed for location-based procedures.

Apart from the fact that the network nodes are moving in NTN networks, the physical distance between these network nodes and the users is another key difference between NTN and TN, as in TN the users are separated from the gNB by a few hundreds or thousands of meters, while in NTN the users might be hundreds to thousands of kilometres apart from the gNB. Therefore, the propagation delay in NTN is very long when compared to terrestrial networks. Even the propagation delay between neighbour cells may be different if those cells are from different satellites.  Finally, in TN, the network can take advantage of the near-far effect when configuring and triggering mobility to other network nodes. In NTN, all users are so distant from the network nodes that the received power from the gNB is very similar among them, making it harder to distinguish between users in the cell centre or at the cell edge only based on the radio measurements.

Observation 3: Another difference between TN and NTN is the distance between users and gNB of hundreds to thousands of kilometres, causing huge propagation delays compared to TN.

Due to the above characteristics, Rel-17 addressed first of all HARQ stalling caused by RTT times significantly exceeding what could be supported by a reasonable number of HARQ processes, at least for GEO satellites for which it would be in the hundredths. Secondly, as mentioned above, methods and procedures were added to account for the satellite moving i.e. relying on GNSS coupled with relevant information i.e. ephemeris information in an NTN specific SIB19. Such information also enabled the UE to execute handovers between both NTN satellites, cells, and even TN to NTN handovers. Lastly, a framework was added to account for additional complexity while performing measurements of moving nodes, and of nodes with different propagation delays.

Since the above limitations are still valid, the initial considerations for 5G NTN are a good starting point for 6G NTN as well.

Observation 4: 6G NTN base requirements may be based on the Rel-17 NR NTN requirements and learnings from 5G NTN topics

Proposal 3: RAN2 to study the following essential aspects for NTN for day-1;

- Addressing long and varying RTT times ,

- Synchronisation to moving satellite Nodes,

- Mobility enhancements, including TN-NTN and multi-orbit,

- Measurement framework addressing moving nodes and larger propagation delays

## 2.2	Common design functionalities with TN

As a part of the ongoing 6G requirements study in SA1, several emerging verticals are identified with stringent service continuity and mobility performance demands. Among these, Immersive Media Services for Advanced Air Mobility (AAM) which has requirements of 10 ms of maximum interruption for a media service that supports 60 fps and dynamic HO between TN and NTN networks, as well as Seamless Immersive Reality in Education which requires 50 ms end-to-end latency, are two use cases that impose particularly tight constraints on handover procedures.

The stringent requirements directly drive the need for a careful study on the handover mechanisms in 6G.  In NR NTN the concept of measurement window and timing were re-used to facilitate the measurement of NR NTN cell, albeit with some amendments addressing at least the issue of different propagation delay by satellites i.e. by enabling additional SMTC configurations in smtc4. On top of this, Rel-19 brought enhancements to the framework by significantly extending the measurement periodicity of up to 160 ms, although up to 640 ms was requested by satellite vendors in RAN1. This change was mainly to account for limited power and need for support of a vast (1024) number of cells on the satellite. 6G study mentions gapless measurement concept, which proposing companies considers to reduce interruption time of data. Even though the limited cell availability considered in Rel-19 may be a challenge for the design of the system, we still see many common aspects with the discussions of Energy Efficiency in 6G day 1. For example, the extension of the SSB periodicity, that is being discussed in RAN1 in the 6G study, or the fact that the cell is not available for data transmissions and receptions for a period of time are common between NTN and TN.

Observation 5: The fundamentals behind TN 5G measurement framework was reusable for NR NTN, although enhancements were made to accommodate for the additional propagation delay and different delays among neighbour cells.

Proposal 4: 6G NTN measurement framework shall be designed with 6G TN as baseline, but configuration should take the extended and varying propagation delay into account without increasing the complexity unproportionally. FFS whether limited availability of cells can be harmonized with TN energy savings as well.

Another learning from 5G NR is related to the duplexing mode. In TR 38.811, the length of a guard period to accommodate the roundtrip delays on LEO, MEO and GEO satellites has been evaluated, ranging between 14 ms for LEO satellites and 540 ms for GEO satellites. It was concluded that “such excessive guard time would lead to a very inefficient radio interface specially in GEO or even MEO based access. It may be acceptable in the case of LEO access systems”. Therefore, 5G NTN has only been specified for FDD carriers, while there is only one TDD variant for NTN, which was introduced in Rel-19 for NB-IoT, and significantly change the TDD frame design when compared to the LTE/NR TDD frame, for example, by having a 90ms frame structure, instead of a 10 ms frame.

The efficiency for a specific orbit depends on the duration of the DL/UL periods. Table 1 shows the calculated losses for different cases. An UL/DL period of 20ms, means that a 20 ms DL period is followed by a guard band corresponding to the minimum RTT plus the propagation delay difference, followed by an UL period of 20 ms. In this case, the losses are between 14-23% for 600 km orbit and 19-31% for a 1200 km orbit, as shown in Table 1. The UL/DL period has a larger impact than the propagation delay. The period should be much longer than the propagation delay to reduce the loss. However, longer UL/DL periods are clearly beneficial in terms of efficiency but come with the cost of a longer delay between UL and DL.

Table 1 Unused time resources (fraction) for different cases for a LEO at 600 and 1200 km for conventional TDD with different UL/DL periods

| Orbit | Coverage per satellite (diameter) | Elevation angle | UL/DL period = 5 ms | UL/DL period = 20 ms | UL/DL period = 160 ms |
| --- | --- | --- | --- | --- | --- |
| 600 km | 1500 km | 90 deg | 40% | 14% | 2% |
| 600 km | 1500 km | 30 deg | 54% | 23% | 4% |
| 1200 km | 1500 km | 90 deg | 49% | 19% | 3% |
| 1200 km | 1500 km | 30 deg | 64% | 31% | 5% |

Observation 6: NTN TDD requires longer guard periods between DL and UL than TN. Even in LEO constellations the efficiency of NTN TDD deployments will be very low when compared to the NTN FDD.

The primary focus on the NTN related studies should be on FDD operation, following the learnings of the 5G NR studies and the above mentioned low efficiency of TDD.

Proposal 5: FDD and HD-FDD should be prioritized for 6G NTN

Furthermore, in NR, the uplink transmission procedure involves a UE transmitting a Scheduling Request (SR), waiting for an initial uplink grant, transmitting a Buffer Status Report (BSR), and finally sending the first payload on the granted PUSCH resources. The SR/BSR procedure may bring 2 RTT latency for data scheduling hence impact the data transfer performance. During RAN2 #131-bis, CB-PUSCH has been proposed as a solution to reduce scheduling latency in the UP discussion where the data can be transmitted over CB-PUSCH resource. Due to the long RTT in NTN system, especially in GEO, the CB-PUSCH mechanism is even more useful for NTN than TN. Therefore, if CB-PUSCH is to be supported in TN and NTN for UE in RRC Connected state after the UE acquired the UL synchronization, we assume it should be part of the common design feature for both TN and NTN.

Proposal 6: Common design of CB-PUSCH should be considered for NTN and TN for UE in RRC Connected state (if both are to be supported).

## 2.3	NTN specific design considerations

Discontinuous coverage was introduced to IoT NTN to enable a low-cost deployment, but may also be relevant in case of ramping up deployment, since areas may exist which does not have TN coverage, and only occasional NTN coverage i.e. due to moving cells. IoT NTN used a separate SIB, as the validity timer was considered different for the information, which could be applied directly to 6G NTN.

Observation 7: NTN dedicated satellite information also enables discontinuous coverage to accommodate for deployment ramp-up.

Since the signal strength variation between the edge and the centre of an NTN cell is small, it is hard for the network to determine handover triggering based solely on the measurement results. Considering the above assumption, that the UE is aware of the cell’s coverage area, the UE is sufficiently aware of its own location, and that neighbour cell coverage area over time is available, conditional handover is applicable.

Similarly, assistance information provided to assist the UE in cell reselection or redirection may be beneficial for the UE, as it can bring power saving gains.

The above features were primarily resolved through providing the UE with enough information to be able to make autonomous decisions on pre-compensation, handover, measurements, and other procedures. Such information were provided using the NR SIB structure, although some changes were necessary w.r.t. when and how to signal these. Overall, we believe that the learning is that albeit the 6G TN system information framework may be harmonized with NTN, some considerations on the design is needed to accommodate for NTN.

Proposal 7: The following functionalities for NTN can use the harmonized system information design, but NTN specific system information content and procedures are needed;

- Pre-compensation information

- Discontinuous coverage

- Broadcast and service areas

- Considerations of RTT for various timers e.g. MAC

- Time/location based mobility features such as CHO, cell reselection, satellite switching, or measurement triggering

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Integration of the NTN device class for VSAT should be harmonized as a part of FWA/CPE device class.

Proposal 1: NTN should be considered in all 6G device families.

Observation 2: In order to ensure harmonization, RAN2’s design can assume at least that the UE is able to pre-compensate. Further considerations can be discussed after RAN1 progress.

Proposal 2: RAN2 to study what location requirements are needed for location-based procedures.

Observation 3: Another difference between TN and NTN is the distance between users and gNB of hundreds to thousands of kilometres, causing huge propagation delays compared to TN.

Observation 4: 6G NTN base requirements may be based on the Rel-17 NR NTN requirements and learnings from 5G NTN topics

Proposal 3: RAN2 to study the following essential aspects for NTN for day-1;

- Addressing long and varying RTT times ,

- Synchronisation to moving satellite Nodes,

- Mobility enhancements, including TN-NTN and multi-orbit,

- Measurement framework addressing moving nodes and larger propagation delays

Observation 5: The fundamentals behind TN 5G measurement framework was reusable for NR NTN, although enhancements were made to accommodate for the additional propagation delay and different delays among neighbour cells.

Proposal 4: 6G NTN measurement framework shall be designed with 6G TN as baseline, but configuration should take the extended and varying propagation delay into account without increasing the complexity unproportionally. FFS whether limited availability of cells can be harmonized with TN energy savings as well.

Observation 6: NTN TDD requires longer guard periods between DL and UL than TN. Even in LEO constellations the efficiency of NTN TDD deployments will be very low when compared to the NTN FDD.

Proposal 5: FDD and HD-FDD should be prioritized for 6G NTN

Proposal 6: Common design of CB-PUSCH should be considered for NTN and TN for UE in RRC Connected state (if both are to be supported).

Observation 7: NTN dedicated satellite information also enables discontinuous coverage to accommodate for deployment ramp-up.

Proposal 7: The following functionalities for NTN can use the harmonized system information design, but NTN specific system information content and procedures are needed;

- Pre-compensation information

- Discontinuous coverage

- Broadcast and service areas

- Considerations of RTT for various timers e.g. MAC

- Time/location based mobility features such as CHO, cell reselection, satellite switching, or measurement triggering