3GPP TSG-RAN WG2 Meeting #132	R2-2508360

Dallas, US: Nov. 17th-21st, 2025

Agenda item:	10.3.3.1 Data Transfer, Model Transfer, and AIML

Source:	Qualcomm Incorporated

Title:	Discussions on 6G AI/ML

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion and Decision

# Introduction

In the last RAN Plenary #108, RP-251881 was agreed to study a 6G Radio Access Technology. The aim is to deliver a “transformative leap” beyond 5G, focusing on a non-backward compatible system designed to meet diverse future use cases with enhanced performance, efficiency, and simplicity. The study prioritizes user experience, energy efficiency, and system extensibility, while addressing the realities of spectrum utilization and deployment.

In RAN2#131bis meeting [1], RAN2 agreed on the requirements for data collection at least for the AI/ML-related data, where requirements are agreed to apply for both data collection for the UE side (refereed as UE side data collection [1]) and data collection for the network side use cases (referred to as NW side data collection [1]).

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

As described in the agenda for RAN2#132 meeting [2], this contribution paper analyzes different types of data, their services/use case scenarios, requirements, endpoint pairs, size and frequency of reporting, and transfer, etc.

# Analysis of Different Types of Data in 6G

The status of data collection for model training in 5G is as follows:

- UE data collection for network side model training:

  - RAN2 discussed and standardized gNB-centric and OAM-centric (an immediate MDT-based data collection solution) for the network side model training for beam management [3]

- UE data collection for UE side model training:

  - SA2 approved a study item for 5G Rel-20 SP-250413:

    - KI#1 is about the transfer of data over UP for UE data collection in TR 23.700-04.

    - The SID will be concluded in Dec 2025.

  - SA5 approved a study item for 5G Rel-20 in SP-250867:

    - WT#1.1 includes study feasibility and potential requirements for data collection (e.g., UE-side and Network-side) to enable model training.

    - The SID will be concluded in Jun 2026.

- Dataset/parameter transfer from the network to the UE for two-sided model training:

  - SA2 has sent the reply LS (S2-2508104/SP-250900/R2-2506751) in Aug, 2025.

  - SA5 has sent the reply LS (S5-254083/SP-250905/R2-2506757) in Aug, 2025.

  - The RAN plenary immediately replied in RP-252966 in Sep, 2025 with a clarification.

  - The SA plenary discussed the incoming LS RP-252966 in Sep, 2025 about the Rel-20 scope update in SA2 and SA5

    - SA2 and SA5 will further discuss whether to enhance the Rel-20 SID for 5G to support this feature in Oct 2025.

Based on the above discussion, the data collection in 5G can be categorized as follows:

- “UE data for UE side use-cases (e.g., AI/ML model training at UE side)”,

- “Dataset/parameter transfer from network for UE-side use cases (e.g., UE-part AI/ML model training of two-sided models at the UE side)”, and

- “UE data for network use-cases (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization, etc)”.

## 2.1 UE Data Collection for UE Side Use Cases

Many of the agreements reached in the RAN2#131bis meeting [1] on requirements for data collection (at least for AI/ML model training at the UE side) are already considered requirements for UE-side data collection in 5G NR [4]. Based on the RAN Plenary request, SA2, SA5, and RAN2 are studying the UE-side data collection in Release 20 5G Advance.

Observation 1: Agreements in the RAN2#131bis meeting [1] on requirements for data collection (at least for   AI/ML model training at the UE side) are already considered requirements for UE-side data collection in 5G NR [4].

## 2.2 Dataset/parameter transfer from network for UE-side use cases

Note that in addition to UE data collection for AI/ML model training at the UE side, the dataset/parameter transfer from the network to the UE for UE side model training of UE-part AI/ML model of two-side models is under discussion in SA2, SA5, and RAN2 in Release 20 5G Advance.

Observation 2: In Release 20 5G advance, SA2, SA5, and RAN2 are discussing:

- Standards-based solution for data collection for the UE side model training, and

- Standards-based solution for dataset/parameter sharing from the network side to the UE side for UE side model training (of the UE-part model of the two-sided models).

Proposal 1: In 6G, the discussion on the following data collection topics is postponed until SA2, SA5, and RAN2 make further progress in Release 20 5G Advance:

- Data collection for UE side Use Cases (e.g., UE side AI/ML model training), and

- Dataset/parameter transfer from network to the UE for UE-side use cases (e.g., model training of the UE-part model of the two-sided models at the UE side)

## 2.3 UE Data Collection for NW Side Use Cases

UE data collection for the NW side use cases has been discussed since Rel-10, when MDT was introduced. Since then, new features under the SON/MDT umbrella have been standardized in 4G LTE and 5G NR. In Rel-19, gNB-centric data collection (enhancements to L3 measurement framework) and OAM-centric data collection (enhancements to immediate MDT framework) were standardized for the UE data collection for the network’s model training. RAN2 has been studying, analyzing, and standardizing the UE data collection for the network side use cases since Rel-10.

Irrespective of whether the data collection is for the UE side purpose or network-side purpose, the data collection solution(s)/framework should be developed with close collaboration of the relevant WGs (e.g., RAN1, RAN2, RAN3, SA2, SA5), RAN2 should start its study analyzing network use case requirements, endpoint pairs, size and frequency of reporting, transfer, etc. of diverse types of data (e.g., AI/ML related data, QoE, SON/MDT, etc).

Proposal 2: For the 6G data collection study, RAN2 starts its study with UE data collection for network use cases (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization), for example, its requirements, endpoint pairs, size and frequency of reporting, and transfer of the data.

While discussing 6G data collection for network use cases, its requirements, endpoint pairs, size and frequency of reporting, and transfer of the data, RAN2 should refer to different types of data collected in 5G NR. Table 1 illustrates different types of data standardized in 5G, and their use case, endpoint pairs, size and frequency of reporting, and transfer of the data.

Table 1: Different types of data in 5G NR

| Types of data | Use case | Producer | Consumer | Size and Frequency of Reporting | Transfer (UE to network) |
| --- | --- | --- | --- | --- | --- |
| L1 Data | Physical layer operations | UE | RAN | Periodic, network-controlled activation/deactivation (periodicity configurable by NW) | Using UCI |
| L2 Data | MAC Procedures | UE | RAN | Periodic, Trigger (periodicity configurable by NW) | Using MAC CE |
| L3 Data | Mobility, etc | UE | RAN | Periodic, Trigger | Using RRC |
| AI/ML Data | UE data collection for the network’s model training | UE | RAN or OAM | Logging at the UE (logging interval configurable by NW) | UEInformationRequest / UEInformationResponse over SRB6 |
| Logged MDT Data | Coverage and capacity optimization | UE | OAM (TCE) | Logging at the UE (logging interval configurable by NW) | UEInformationRequest / UEInformationResponse over SRB2 |
| Immediate MDT data | MNO observability and performance monitoring | UE | RAN, OAM (TCE) | Periodic, event-triggered (periodicity configurable by NW) | Over SRB1 |
| SON Data | Different SON reports for failure, near failure detection, and optimizations (e.g., MRO) | UE | RAN, OAM | Logging at the UE (logging interval configurable by NW) | UEInformationRequest / UEInformationResponse over SRB2 |
| QoE Data | UE application layer data for application and RAN optimizations | UE application layer | RAN, OAM (MCE) | Logging at the UE application / AS layer (configurable by NW) | Push-based mechanism over SRB4 |
| Positioning Data | Estimate the UE position to offer different MNO services | UE | LCS Client / AF | single-shot, periodic, event triggered. “Size” depends on the positioning method, but generally “small”. | NAS (SRB2) or TCP/IP |

Observation 3: For the 6G data collection study, RAN2 considers Table 1 as a reference for UE side data collection for the network use case (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization),, its requirements, endpoint pairs, size and frequency of reporting, and transfer of the data.

In 6G, for a type of data, a certain aspect of data collection is not expected to change. For example, for a data type, at least its use case, producer, consumer, size and frequency of reporting are not expected to change in 6G compared to 5G.

Proposal 3: In 6G, for a type of data (e.g., L1, L2, L3 measurements, AI/ML data, SON/MDT, QoE, etc.) (if agreed to be supported) for network use case (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization), at least the following are expected to remain the same as 5G (as illustrated in Table 1):

- Use case (or purpose) of the data collection,

- Producers and consumers of the data, and

- Size and frequency of the reporting of the collected data (e.g., configurable by network).

In the 5G cellular network, transfer of measurements/data was designed to meet the requirements of control plane procedures and network operations (please see Fig. 1). In 6G, for the transfer of collected data from the UE to the network for network use cases, a data transfer solution from the UE to the network should not create unnecessary delay, at least for data/measurements critical for proper control plane procedures and network operations. For example, L3 measurement / immediate MDT measurements should not be carried over a new plane (e.g., data plane), which may result in additional delay forwarding data from the new plane (e.g., data plane) to the control plane.

Fig. 1: Transfer of different types of measurements/data in 5G network

Note that immediate MDT / L3 measurements are used for critical decision-making over the control plane, such as handovers.  Therefore, the introduction of a new plane (e.g., data plane) to unify the transfer of the different types of UE data should be carefully evaluated, for example, its implications on existing plane control procedures and network operations.

Proposal 4: Measurement/data (e.g., L1, L2, L3 measurements, and immediate MDT measurements) critical for real-time control plane procedures and network operations should be carried over control plane to avoid any unnecessary delay.

On the other hand, the measurement/data not critical for control plane procedures can be offloaded from the control plane to ease access stratum (AS) memory requirements at the UE. For example, AI/ML data, logged MDT data, QoE data, and SON data should be carried over IP to relax UE requirements and efficient UE participation.

Proposal 5: New Measurement/data (e.g., AI/ML data, etc.) not critical for real-time control plane procedures and network operations can be carried over IP instead of the control plane to relax UE requirements and efficient UE participation.

# AI/ML Framework Considerations

In RAN2#131bis meeting [1], RAN2 agreed on the following:

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In 5G, the UE and RAN features were not designed to exploit the power of AI/ML (at least in the beginning). Features and procedures were designed in 5G for consistent UE behavior; however, this also results in restricting UE to take optimal decision. For example, upon beam failure detection and recovery in 5G, the UE performs the following:

- UE performs beam failure recovery first on the serving cell,

- If beam failure recovery fails, UE declares RLF,

- UE tries to perform reestablishment,

- If reestablishment fails, UE tries to perform reconnection.

In 6G, the AI/ML may be key enablers for different UE and RAN features. Therefore, UE and RAN features should be designed to exploit the power of AI/ML. For example, in the above example of beam failure detection and recovery, the UE and RAN features / procedures should be designed to enable UE to take optimal decisions based on available measurement predictions (e.g., perform recovery on the neighboring cells/beams without performing beam failure recovery at the serving cell if relevant configuration is available).

Proposal 6: UE and RAN features should be designed to exploit the power of AI/ML, i.e. normal UE operations should be designed to benefit from potential AI/ML deployment in future releases.

# 3 Conclusions

Observation 1: Agreements in the RAN2#131bis meeting [1] on requirements for data collection (at least for   AI/ML model training at the UE side) are already considered requirements for UE-side data collection in 5G NR [4].

Observation 2: In Release 20 5G advance, SA2, SA5, and RAN2 are discussing:

- Standards-based solution for data collection for the UE side model training, and

- Standards-based solution for dataset/parameter sharing from the network side to the UE side for UE side model training (of the UE-part model of the two-sided models).

Proposal 1: In 6G, the discussion on the following data collection topics is postponed until SA2, SA5, and RAN2 make further progress in Release 20 5G Advance:

- Data collection for UE side Use Cases (e.g., UE side AI/ML model training), and

- Dataset/parameter transfer from network to the UE for UE-side use cases (e.g., model training of the UE-part model of the two-sided models at the UE side)

Proposal 2: For the 6G data collection study, RAN2 starts its study with UE data collection for network use cases (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization), for example, its requirements, endpoint pairs, size and frequency of reporting, and transfer of the data.

Observation 3: For the 6G data collection study, RAN2 considers Table 1 as a reference for UE side data collection for the network use case (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization),, its requirements, endpoint pairs, size and frequency of reporting, and transfer of the data.

Proposal 3: In 6G, for a type of data (e.g., L1, L2, L3 measurements, AI/ML data, SON/MDT, QoE, etc.) (if agreed to be supported) for network use case (e.g., regular network operations, AI/ML model training at network side, coverage and capacity optimization (CCO), Mobility Robust Optimization (MRO), RACH optimization), at least the following are expected to remain the same as 5G (as illustrated in Table 1):

- Use case (or purpose) of the data collection,

- Producers and consumers of the data, and

- Size and frequency of the reporting of the collected data (e.g., configurable by network).

Proposal 4: Measurement/data (e.g., L1, L2, L3 measurements, and immediate MDT measurements) critical for real-time control plane procedures and network operations should be carried over control plane to avoid any unnecessary delay.

Proposal 5: New Measurement/data (e.g., AI/ML data, etc.) not critical for real-time control plane procedures and network operations can be carried over IP instead of the control plane to relax UE requirements and efficient UE participation.

Proposal 6: UE and RAN features should be designed to exploit the power of AI/ML, i.e. normal UE operations should be designed to benefit from potential AI/ML deployment in future releases.

# 4. References

[1] RAN2#131bis meeting report

[2] Agenda 3GPP TSG-RAN WG2 Meeting #132

[3] TS 38.331, Radio Resource Control (RRC); Protocol specification

[4] RP-242389, “LS on AIML data collection”, RAN, September 2024