3GPP TSG-RAN WG2 Meeting #132	R2-2508361

Dallas, USA, 17th – 21st of November 2025

Agenda item:	7.0.2.22

Source:	Nokia

Title:	On per-band UE capabilities for Rel-18 LTM

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and Decision

# 1	Introduction

This paper presents our view regarding the recent LS [1] that RAN WG1 has sent to RAN WG2 regarding the per-band UE capabilities for Release 18 LTM.

# 2	Discussion

During RAN1#122bis it was discussed how to interpret some of the Release 18 LTM capabilities that are signalled per band. As a result of this discussion, RAN1 has sent the LS to RAN2 [1] and has included the following action:

2. Actions: To RAN WG2 group ACTION: RAN1 respectfully asks RAN2 to capture the following in the description of ltm-BeamIndicationJointTCI-r18, ltm-BeamIndicationSeparateTCI-r18, ltm-MAC-CE-JointTCI-r18, and ltm-MAC-CE-SeparateTCI-r18: “The UE reports the same capability for all bands in one FR.”

The reason behind such action to RAN2 is that in the current TS 38.306 description of these four capabilities, listed in the LS by RAN1, it is stated that for the cross-band operation (e.g. inter-frequency LTM handover) the band that is indicated per each capability is the source band [2], as can be seen below.

| ltm-BeamIndicationSeparateTCI-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI-state indication for LTM procedure and indicating/activating a pair of UL/DL TCI-state in a cell switch command. This capability comprises the following parameters: - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of configured DL TCI state(s) per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of configured UL TCI state(s) per candidate cell. - qcl-Resource-r18 indicates the supported QCL source RS in the LTM TCI-state configuration. - maxNumberDL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate DL LTM TCI state(s) across candidate cells. The maximum number of configured separate DL LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberUL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate UL LTM TCI state(s) across candidate cells. The maximum number of configured separate UL LTM TCI state(s) across candidate cells is N*8, where N={1..64}. - maxNumberCells-r18indicates the maximum number of configured cells for separate DL/UL LTM TCI states A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ltm-BeamIndicationJointTCI-r18 Indicates whether the UE supports unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, indicating and activating a single joint LTM TCI state in a cell switch command. This capability comprises the following parameters: - maxNumberJointTCI-PerCell-r18 indicates the maximum number of configured joint LTM TCI state(s) per candidate cell - qcl-Resource-r18 indicates of the supported QCL source RS in the LTM TCI-state- configuration. - maxNumberJointTCI-AcrossCells-r18 indicates index N of the maximum number of configured joint DL LTM TCI state(s) across candidate cells. The maximum number of configured joint LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberCells-r18 indicates the maximum number of configured cells for joint LTM TCI state(s). A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |
| ltm-MAC-CE-SeparateTCI-r18 Indicates whether the UE supports MAC-CE activated DL/UL LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated DL TCI states per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated UL TCI states per candidate cell. - maxNumberDL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM DL TCI states across all candidate cells and serving cell DL TCI states across all serving cells. - maxNumberUL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM UL TCI states across all candidate cells and serving cell UL TCI states across all serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by unifiedSeparateTCI-r17. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |
| ltm-MAC-CE-JointTCI-r18 Indicates whether the UE supports MAC-CE activated joint LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated joint LTM TCI states configuration. - maxNumberJointTCI-PerCell-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states per candidate cell. - maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cell TCI states across serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. NOTE: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by of unifiedJointTCI-r17. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |

In RAN1’s opinion such source band based indication may not help the target cell while preparing the TCI state configuration for the UE, as it is the target cell (target band) that is more significant for such preparation.

Observation 1: According to RAN1, it does not make sense if the listed per-band LTM capabilities refer to the source band in case of cross-band operation, as it matters more what the UE supports in the target band.

Thus, RAN1 in [1] suggests a solution to this capability signalling inefficiency. Namely, it is proposed to specify that all frequency bands in this frequency range (FR) share the same value for these capabilities. By doing so, the target cell in the target band can assume the UE supports the same set of capabilities as signalled for the source band.

Observation 2: RAN1 suggests all frequency bands in a given frequency range (FR) should share the same value for these capabilities. Then the target cell in the target band can assume the UE supports the same set of capabilities as signalled for the source cell/band.

While this alignment could make sense, to reduce the diversity in how the same capability is reported for different bands, we also acknowledge that this imposes certain limitations on the UEs, especially considering these are Release 18 features. More specifically, the UEs might support these capabilities in some bands within given FR while not supporting them in other bands from the same FR. If RAN2 follows the suggestion made by RAN1, then the UE shall support these capabilities in a unified way in all or none of the bands in this FR.

Observation 3: If RAN2 follows the suggestion made by RAN1 in the recent LS, then the UE will need to support these capabilities in a unified way: either in all or none of the bands in any given FR.

Furthermore, when HO is prepared, source and target node exchange various information elements, such as  HandoverPreparationInformation. This can comprise a list of UE capabilities (e.g. UE-CapabilityRAT-ContainerList) per each RAT. It might not always be the case (depending on the actual UE capability enquiry), but in general the source gNB possesses a list of UEs capabilities for different bands within the FR. Thus, such list might be conveyed in HandoverPreparationInformation.

Observation 4: A list of UE’s capabilities for different bands within the FR might be conveyed to the target node in HandoverPreparationInformation.

Considering all of the arguments made above, we suggest to discuss in RAN2 whether the changes to TS 38.306 requested by RAN1 in [1] are feasible.

Proposal 1: RAN2 is asked to discuss if changes requested by RAN1 (all frequency bands within the FR share the same value for the following capabilities: ltm-MAC-CE-JointTCI-r18, ltm-MAC-CE-SeparateTCI-r18, ltm-BeamIndicationJointTCI-r18, ltm-BeamIndicationSeparateTCI-r18) are doable considering Release 18 LTM is frozen for ~2 years.

If RAN2 is happy to pursue the changes suggested in RAN1 LS [1], then the corresponding CR to [2] may look as shown in the Annex.

Proposal 2: If RAN2 finds it feasible to introduce the changes requested by RAN1, consider the TP in the Annex.

# 3 	Conclusion

In this paper we have briefly discussed per-band UE capabilities for Release 18 LTM, considering the recent LS from RAN1 [1]. The following observations and proposals have been made:

Observation 1: According to RAN1, it does not make sense if the listed per-band LTM capabilities refer to the source band in case of cross-band operation, as it matters more what the UE supports in the target band.

Observation 2: RAN1 suggests all frequency bands in a given frequency range (FR) should share the same value for these capabilities. Then the target cell in the target band can assume the UE supports the same set of capabilities as signalled for the source cell/band.

Observation 3: If RAN2 follows the suggestion made by RAN1 in the recent LS, then the UE will need to support these capabilities in a unified way: either in all or none of the bands in any given FR.

Observation 4: A list of UE’s capabilities for different bands within the FR might be conveyed to the target node in HandoverPreparationInformation.

Proposal 1: RAN2 is asked to discuss if changes requested by RAN1 (all frequency bands within the FR share the same value for the following capabilities: ltm-MAC-CE-JointTCI-r18, ltm-MAC-CE-SeparateTCI-r18, ltm-BeamIndicationJointTCI-r18, ltm-BeamIndicationSeparateTCI-r18) are doable considering Release 18 LTM is frozen for ~2 years.

Proposal 2: If RAN2 finds it feasible to introduce the changes requested by RAN1, consider the TP in the Annex.

# References

- R1-2508124 LS on per-band UE capabilities for LTM TSG RAN WG1 #122bis Prague, Czechia, Oct 13th – 17th, 2025

- 3GPP Technical Specification 38.306 User Equipment (UE) radio access capabilities (Release 18), V18.7.0 (2025-09)

# Annex – Change Request to TS 38.306

#### 4.2.7.2	BandNR parameters

| ltm-BeamIndicationJointTCI-r18 Indicates whether the UE supports unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, indicating and activating a single joint LTM TCI state in a cell switch command. This capability comprises the following parameters: - maxNumberJointTCI-PerCell-r18 indicates the maximum number of configured joint LTM TCI state(s) per candidate cell - qcl-Resource-r18 indicates of the supported QCL source RS in the LTM TCI-state- configuration. - maxNumberJointTCI-AcrossCells-r18 indicates index N of the maximum number of configured joint DL LTM TCI state(s) across candidate cells. The maximum number of configured joint LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberCells-r18 indicates the maximum number of configured cells for joint LTM TCI state(s). A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, this capability refers to the source band. UE indicates the value for this capability consistently across all bands within the FR. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ltm-BeamIndicationSeparateTCI-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI-state indication for LTM procedure and indicating/activating a pair of UL/DL TCI-state in a cell switch command. This capability comprises the following parameters: - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of configured DL TCI state(s) per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of configured UL TCI state(s) per candidate cell. - qcl-Resource-r18 indicates the supported QCL source RS in the LTM TCI-state configuration. - maxNumberDL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate DL LTM TCI state(s) across candidate cells. The maximum number of configured separate DL LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberUL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate UL LTM TCI state(s) across candidate cells. The maximum number of configured separate UL LTM TCI state(s) across candidate cells is N*8, where N={1..64}. - maxNumberCells-r18indicates the maximum number of configured cells for separate DL/UL LTM TCI states A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, this capability refers to the source band. UE indicates the value for this capability consistently across all bands within the FR. | Band | No | N/A | N/A |
| ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133 [5]. UE shall set the capability values for maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18 consistently for all bands. These capability values represent the maximum number across all the supported bands. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]. | Band | No | N/A | N/A |
| ltm-MAC-CE-JointTCI-r18 Indicates whether the UE supports MAC-CE activated joint LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated joint LTM TCI states configuration. - maxNumberJointTCI-PerCell-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states per candidate cell. - maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cell TCI states across serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. NOTE: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by of unifiedJointTCI-r17. For cross-band operation, this capability refers to the source band. UE indicates the value for this capability consistently across all bands within the FR. | Band | No | N/A | N/A |
| ltm-MAC-CE-SeparateTCI-r18 Indicates whether the UE supports MAC-CE activated DL/UL LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated DL TCI states per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated UL TCI states per candidate cell. - maxNumberDL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM DL TCI states across all candidate cells and serving cell DL TCI states across all serving cells. - maxNumberUL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM UL TCI states across all candidate cells and serving cell UL TCI states across all serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by unifiedSeparateTCI-r17. For cross-band operation, this capability refers to the source band. UE indicates the value for this capability consistently across all bands within the FR. | Band | No | N/A | N/A |