3GPP TSG-RAN WG2 Meeting #132	R2-2508373

# Dallas, United States, Nov. 17 – 21, 2025

Agenda item:	8.13.2

Source:	Apple

Title:	SUI procedure for MH relay - RIL [A501][A502]

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In the last RAN2 meeting (RAN2#131bis [1]), there are some RIL issues raised in [2] being discussed and closed. But the triggering and transmission of SUI procedure in the rapporteur CR [3] are still not entirely correct.

During the after-meeting ASN.1 review, [RIL A501] and [RIL A502] have been raised to address those issues. In this paper, we discuss these RIL issues and provide the corresponding TP to resolve them.

.

# 2 Discussion

### 2.1 A501: Usage of sl-PagingIdentityRemoteUEList

During the ASNRIL issue Z455 [2] has been discussed, and the following conclusion has been made:

- Introduce a paging ID list in SL-TxResourceReqL2U2N-Relay-r17 included in SUI message, used to report paging IDs of indirect child UEs at least for last relay UE, as in issue Z455. Refer to TP in R2-2506983. Z455 can be closed.

In the latest rapporteur RRC CR for Rel-19 Multi-hop SL relay, the paging ID list has been added as part of SidelinkUEInformaitonNR message, as shown below:

SL-TxResourceReqL2U2N-Relay-r17 ::= SEQUENCE { sl-DestinationIdentityL2U2N-r17 SL-DestinationIdentity-r16 OPTIONAL, sl-TxInterestedFreqListL2U2N-r17 SL-TxInterestedFreqList-r16, sl-TypeTxSyncListL2U2N-r17 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16, sl-LocalID-Request-r17 ENUMERATED {true} OPTIONAL, sl-PagingIdentityRemoteUE-r17 SL-PagingIdentityRemoteUE-r17 OPTIONAL, sl-CapabilityInformationSidelink-r17 OCTET STRING OPTIONAL, ..., [[ sl-PagingIdentityRemoteUEList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-PagingIdentityRemoteUE-r17 OPTIONAL ]] }

But the current SUI procedure text in 5.8.3.3 has included the L2 U2N relay UE to report its child UE’s paging ID to gNB, which is essentially only include a single Paging ID for the NW to helps monitor, when the RRC_CONNECTED relay UE may not be able to monitor and intercept paging for this child, as shown below:

#### 5.8.3.3 Actions related to transmission of SidelinkUEInformationNR message <text omitted> 3> if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE: 4> include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource: 5> set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission; 5> set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission; 5> set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission; 5> set sl-LocalID-Request to request local ID for L2 U2N Remote UE [RIL]:W500, SLRelay transiting to RRC_CONNECTED or in RRC_CONNECTED state; 5> set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3; 5> set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

For a Rel-19 U2N relay UE (except the first U2N relay), the SUI reporting may need to include the new Paging ID list parameter in its “sl-TxResourceReqL2U2N-Relay “. According to the newly added field description of this IE [3], this IE is only used for indirect children.

sl-PagingIdentityRemoteUEList

Contains a list of paging identity of indirectly connected child UEs

Therefore, we propose to add the procedure text to include this IE when UE wants NW to monitor paging for an indirect child under the SL-Destination.

Proposal 1 	L2 U2N Relay UE in RRC_CONENCTED state includes sl-PagingIdentityRemoteUEList in SUI if there at least one Paging ID from an indirect child under the reported SL Destination to be reported to gNB.

### 2.2 A502: UE type in SUI message of Intermediate relay

RAN2#131bis [1] has already agreed the following:

Z456 can be left to UE implementation to set the UE type and network implementation to identify which UE is an intermediate

We understand that the main outcome of the online discussion in RAN2#131bis [1] is that there is no need to introduce a new UE type for intermediate relay UE because NW may have some ways to identify the sender of SUI message is an intermediate relay UE. For example, when gNB receives the SUI message, the lower layer can indicate gNB that the RRC message is encapsulated in SRAP PDU, so that the gNB knows that the relay UE sending the SUI message is not directly connected to the gNB. So, it must be an Intermediate U2N Relay UE.

However, the “left to UE implementation to set the UE type” part does not solve the problem raised in Z456 but create more confusion.. It is very clear that the intermediate relay UE, whenever include any of its child UE as SL destinations, should not set its UE type as “remote”, and must set it as “relay”, which is consistent with the current RRC procedure and helps NW to understand the true intention of SUI message. This part needs to be clarified and cannot be “left to UE implementation”.

Observation 1	Intermediate Relay UE should only set UE type as “remote” when it reports any of its children in SUI.

Besides the “UE type” setting, we also need to understand how an intermediate relay UE set the values in the sl-TxResourceReqListCommRelay, which is essentially a list of sl-TxResourceReqL2U2N-Relay, each of which corresponds to a SL destination of the relay UE intends to communicate.

Because intermediate relay UE need reach SL destinations in both directions: upstream (parent) and downstream (children), We can see two candidate options for the intermediate relay UE to consider when sending SUI:

Option 1: Sending two separate SUI messages: the first SUI message with UE-type “remote” and only include upstream SL destination. And the second SUI message with UE-type set to “relay” and include all its children information.

Option 2: Sending a single SUI message with UE-type “relay” and include both upstream and downstream destinations in the same sl-TxResourceReqListCommRelay

In our view, Option 1 is incorrect for the following reasons:

- SUI message is an all-inclusive message which has all the necessary information for a UE’s “current” Sidelink operations, which may include U2N relay, U2U relay, basic sidelink, SL DRX, SL positioning. This means all the information related to SL features must be included in a single SUI transmission and any change of this will trigger a new SUI with “all” the information, as indicated in the first sentence in 5.8.3.3: “The UE shall set the contents of the SidelinkUEInformationNR message as follows and shall include all concerned information, irrespective of what triggered the procedure:”. So, this is not a design for delta updates of the UE information and gNB should not be burdened to remember a prior SUI message content. So, it is essentially wrong and eerie to left a single U2N relay destination in a separate SUI message and be reported to gNB separately.

- The current SUI procedure in 5.8.3.3 and ASN.1 structure does not allow a UE to be triggered to send two SUI messages with different UE types, so it is unclear how to modify 5.8.3.3 to enable Option 1 for an Intermediate relay UE.

- According to the triggering conditions listed in 5.8.3.2, the SUI message is triggered “if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL2U2N-Relay; or if the information carried by the sl-TxResourceReqL2U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message”. Literally speaking, if an intermediate relay UE adopts the “two-SUI” scheme (Option 1), then this highlighted condition is always true because the content of “sl-TxResourceReqL2U2N-Relay “ is different in every other SUI message, and the intermediate relay UE will constantly be triggered to generate SUI transmisisons. To fix this, more constraints and exceptions has to be added for this special L2 Multi-hop relay case in section 5.8.3.2.

Therefore, we think Option 1 has much more specification impact.

Observation 2	Allowing Intermediate Relay UE to put upstream SL destination (parent) and downstream SL destination(s) (children) in different SUI messages with different UE-type is against SUI message’s all-inclusive principle and would have significant spec impact in 5.8.3.2 and 5.8.3.3.

Based on the above analysis, we think only Option 2 is the right way for intermediate relay UE to report its SUI message, so we propose .

Proposal 2	When an Intermediate Relay UE intends to report any of its child UE in SUI message, the UE shall indicate “UE-type” as “relayUE” and include both upstream and downstream SL destinations in the sl-TxResourceReqListCommRelay.

Given the above P1/P2, we provided a TP to be adopted in RRC specification in Annex.

Proposal 3 	RAN2 adopts the TP in Annex to resolve A501 and A502.

# 3 Conclusion

In this contribution, we discuss the two ASN.1 issues related to SUI, and have the following observations:

Observation 1	Intermediate Relay UE should only set UE type as “remote” when it reports any of its children in SUI.

Observation 2	Allowing Intermediate Relay UE to put upstream SL destination (parent) and downstream SL destination(s) (children) in different SUI messages with different UE-type is against SUI message’s all-inclusive principle and would have significant spec impact in 5.8.3.2 and 5.8.3.3.

Them, we have the following proposals:

Proposal 1 	L2 U2N Relay UE in RRC_CONENCTED state includes sl-PagingIdentityRemoteUEList in SUI if there at least one Paging ID from an indirect child under the reported SL Destination to be reported to gNB.

Proposal 2	When an Intermediate Relay UE intends to report any of its child UE in SUI message, the UE shall indicate “UE-type” as “relayUE” and include both upstream and downstream SL destinations in the sl-TxResourceReqListCommRelay.

Proposal 3 	RAN2 adopts the TP in Annex to resolve A501 and A502.

# 4 References

[1] Chairman’s notes, RAN2#131bis

[2] R2-2506983, ZTE

[3] “R2-250XXXX Corrections to WI SLRelay post_R2_131bis_Rapp_01.docx” Rel-19 SL Relay RRC CR

# 5 Annex

First change

#### 5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

<text omitted>

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink single hop L2 U2N relay communication and the UE is acting as L2 U2N Relay UE or if SIB12 includes sl-L2U2N-MH-Relay and if configured by upper layers to transmit NR sidelink multi hop L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE transiting to RRC_CONNECTED or in RRC_CONNECTED state, if the UE identified by sl-DestinationIdentityL2U2N is a L2 U2N remote UE in need of local ID;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID of the direct child UE in case of multi-hop or the Paging UE ID of L2 U2N Remote UE in case of single-hop, if it is not released as in 5.8.9.8.3;

5>	set sl-PagingIdentityRemoteUEList to the one or more paging UE ID(s) of the indirect child UE connected to the child UE identified by sl-DestinationIdentityL2U2N, if not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink single hop L2 U2N relay communication and the UE has a selected L2 U2N Relay UE or if SIB12 includes sl-L2U2N-MH-Relay and if configured by upper layers to transmit NR sidelink multi hop L2 U2N relay communication and the UE has a selected L2 U2N Relay UE (except the case that the UE is an intermediate U2N Relay UE and has at least one connected child UE):

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to remoteUE;

3>	if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay communication:

4>	include sl-TxResourceReqL3U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L3 U2N relay communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-RLC-ModeIndicationList to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB(s) have been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink L3 U2N relay communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE if the UE is acting as NR sidelink L3 U2N Relay UE or to remoteUE otherwise;

<text omitted>

End of changes