3GPP TSG-RAN WG2 Meeting #132                                                                  R2-2508405

Dallas, USA, 17th-21st November 2025

Title: 	Discussion on RRC functionality and RRC states

Source: 	ZTE Corporation, Sanechips

Agenda item:	10.3.2.1

Document for:	Discussion and Decision

- Introduction

The study on 6G contains one CP related objective: Control Plane architecture (including RRC states) and protocol design. In last RAN2 meeting, we discussed the RRC states for 6GR and made the below agreements.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this document, we further discuss the 6G RRC functionalities and RRC states, and also provide our views on the necessity of RRC layer split.

- Discussion

  - RRC functionalities

RRC in 5G NR supports a number of functionalities, according to TS 38.331 section 4.4, those functions can be categorized into following types:

- Fundamental functions that are critical to basic operations in RAN. These functions are expected to be supported from beginning and the details should be studied during SI phase. Such as the broadcast of system information, paging, RRC connection management, access control, AS security handling, Mobility control, radio bearer management, radio configuration, spectrum management, QoS control, NAS signalling transfer and UE capability enquiry/transfer.

- Enhanced functions which are not critical to basic operation but can assist the network to achieve better performance, such as QoE, logged measurement report and logged measurement report for network side data collection.

- Advanced functions which are related to specific deployment or services: such as NPN, slicing, NCR, IAB, MBS, positioning, sidelink and sidelink relay.

For 6G RRC functionalities, we think the fundamental functions in 5G can be considered as a baseline. Among the detailed functions, the study of access control (access barring mechanism) requires inputs from SA/CT, and QoS management also involves SA/CT, for mobility control (including measurement management handled by RRC), we may need to discuss the lower-layer triggered mobility, but anyway RRC-based mobility should be kept in 6G

Proposal 1: 	At least the following RRC functionalities are supported for 6GR RRC:

- Broadcast of system information;

- Paging;

- RRC connection management;

- Access control (pending on SA/CT inputs);

- Mobility control (including measurement);

- Radio Bearer management (including SRB and DRB);

- Radio configuration;

- Spectrum management;

- QoS control (also involve SA/CT);

- Transfer of NAS signalling;

- UE capability enquiry/transfer.

  - Background: RRC states in 5G

5G supports 3 different RRC states summarized as in table below.

- RRC_CONNECTED: Fully active state with data transmission and network-controlled mobility

- RRC_INACTIVE:  Stored security context in RAN, RAN paging and UE controlled mobility. Small data transfer is enabled if SDT feature is supported

- RRC_IDLE: No active connection, UE monitors CN paging. UE controlled mobility is used

The overall state machine in NR is summarised in the table below:

|  | RRC_CONNECTED | RRC_INACTIVE | RRC_IDLE |
| --- | --- | --- | --- |
| State characteristics | - Fully active radio connection for user-plane and control-plane traffic | Connection suspended to reduce signaling and power consumption, but context retained for fast resumption | No active connection; UE reachable only via CN paging |
| UE Context (stored at UE) | - Full AS context - Security context (K_NR, COUNT values) - SRB/DRB configuration and QoS info- Serving cell and measurement config- Timers and DRX info | - AS context retained but suspended - Security context retained - DRB/ SRB parameters stored for quick resumption - RAN-based paging config and RAN area ID for location tracking | - Only NAS context retained - AS context released - Security context retained at NAS level |
| Network Context (stored at gNB / AMF) | - Full UE AS context in gNB (RLC, MAC, PDCP configs) - QoS flows & DRB mapping - Security context active- Scheduling state, HARQ buffers - Serving cell info, measurement config | - AS context stored in gNB-CU (suspended) - Security context retained - UE location tracked at RAN area level (RAN paging) | - Only NAS context stored at AMF - Security context retained at core level - No AS context in gNB - UE tracked via Tracking Area (CN paging) |
| Security Context | - Full RAN security context + NAS security context | - RAN security context is stored, NAS security context is also retained | - Only NAS security context is kept |
| Radio Bearers | SRB0/1/2, DRBs established | - SRB and DRB configurations retained (inactive), but RBs are suspended. - SRB1 and DRB can be resumed in case of SDT | None |
| Paging Mechanism | RAN and CN-based (depends on configuration) | RAN-based paging (RAN area ID) CN based paging is also used but only to recover from error cases (lost context) | CN paging (Tracking Area ID) |
| Mobility Handling | Network controlled mobility via handover | UE controlled mobility via cell selection/reselection and RAN-based paging | UE controlled mobility via cell selection/reselection and CN-based paging |
| Measurement Configuration | Active (configured by RRC) | Configuration Retained (measurements are suspended) | Released |
| User Plane Activity | Full data transmission over SRBs and DRBs | Possible only with SDT feature for small data (over SRB/DRB) | Not available |
| Transition Triggers | CONNECTED -> INACTIVE/IDLE: Network controlled (RRCRelease) INACTIVE/IDLE -> CONNECTED: Network controlled (paging) or UE triggered (MO traffic) | INACTIVE -> CONNECTED: paging/MO data triggered INACTIVE -> IDLE: Error case transition (e.g. due to lost context) | CONNECTED -> IDLE: Network controlled RRCRelease IDLE -> Connected: Paging/MO data triggered |
| Power Consumption | Highest | Medium | Lowest |
| Latency to Data Transmission | Minimum | Low (fast resume/SDT) | High (full RRC setup required including establishment of RAN security context) |

The 3-state RRC model in 5G as summarized above is well suited to enable optimal trade-off between latency for UP traffic and UE power consumption. Although all 3 states were fully specified as day 1 features in NR, only IDLE and CONNECTED states were commercialized. Although INACTIVE state has been specified in NR and made mandatory for the UE to support this, this was never commercialized dure to the following reasons:

- ENDC option did not support INACTIVE state. This meant that all the NSA networks never deployed this state.

- In case of standalone operation, lack of availability of user plane traffic in Rel-15 for INACTIVE state meant that the UEs seldom had chance to go to and/or stay in INACTIVE state. This is because the modern smart phones with multiple applications generating background traffic almost never allow sufficient inactive period to justify the UE to be sent to INACTIVE state

Observation 1: 	The RRC state modelling using 3 RRC states (CONNECTED, INACTIVE and IDLE) is well suited to enable a good tradeoff between UP latency and UE power consumption

Observation 2: 	RRC INACTIVE state was not commercialized because of lack of support for small data in INACTIVE state which means that there is rarely any chance to keep the UE in this state

  - RRC states and modelling in 6G

Like in 5G, fast transition from a power efficient state to full CONNECTED state is essential to meet the ITU requirements. IDLE state is at least needed to model the UE functionality before any RAN context is setup (e.g. during initial power ON etc). The main question then is regarding a state like INACTIVE state. Instead of directly discussing the RRC states for 6G, it is first better to understand the functionality needed at the UE and network side for different states. Specifically, we should enable states with UE controlled mobility (for power saving) and network-controlled mobility (for fast data transfer and seamless mobility). Then the question is whether 6G supports a state where data transmission is possible with UE controlled mobility. This is then similar to INACTIVE state with SDT in 5G. One of the main lessons learnt from 5G is that having more than one RRC state with UE controlled mobility and without UP data transfer is not really needed. Thus, if INACTIVE state is to be successful, it shall support small data transmission in an optimal way.

Proposal 2: 	In addition to the IDLE state, 6GR shall support an RRC state with UE controlled mobility (i.e. cell selection/reselection) with small data transmissions (i.e. INACTIVE state with SDT)

With multiple RRC states, RAN2 should discuss the triggers for state transitions. Paging and MO triggered transitions from low power RRC states (like INACTIVE/IDLE) to CONNECTED is an obvious thing to support. The transition from the CONNECTED state to IDLE/INACTIVE is fully network controlled in 5G and this should also be the case for 6G.

Proposal 3: 	The state transition from the network-controlled mobility state (e.g. CONNECTED) to a UE controlled mobility state (e.g. INACTIVE/IDLE) should only be possible via explicit network signalling (e.g. RRCRelease)

At the last RAN2 meeting companies proposed that the use of IDLE state should be minimized in 6G. This seems logical, especially if INACTIVE like state with SDT is supported as a day 1 feature. Then, if we assume that IDLE state is mainly used for initial connection setup and context establishment, then the INACTIVE state should be optimal for handling UEs with small data transmissions. Although SDT feature supports RACH and CG based SDT transmissions, the actual feature still may be more optimal in a few ways:

- RRC-less SDT: SDT in 5G requires RRCResume procedure to be initiated before small data transmission is performed. Since SDT was added on top of existing INACTIVE state in a later release in 5G, the choice of RRC based SDT (i.e. reusing the resume procedure to initiate SDT) was a choice made as a trade-off to minimize number of changes to then existing Resume procedure. However, such restrictions don’t apply when designing the solution from scratch. So, optimal transmission of small data in INACTIVE should support further simplification, especially to minimize the signalling overhead with the RRCResume procedure (e.g. using RRC-less transmissions).

- Improved Capacity for SDT: SDT in 5G is restricted to initial BWP. If the intention is to use INACTIVE state as the main RRC state to park the UEs in, the network capacity for the small data transmissions in this state should be dimensioned to be sufficiently large. Otherwise, the UEs will either be kept in CONNECTED or IDLE states and this will lead to similar situation as we have in 5G. Thus, in 6G when designing the INACTIVE state, the SDT transmissions should not be restricted to only initial BWP.

- SDT without Xn connectivity: In case of 5G, INACTIVE state relies on Xn connectivity for context fetch and for paging. This restriction means that INACTIVE state (and SDT) can only be supported if Xn connectivity exists between the RAN nodes. During the initial deployments operators may not have full Xn connectivity across all the gNBs. Thus, the INACTIVE state feature cannot be very useful in such initial deployments and the feature will not provide incremental benefits as the networks are upgraded. A better approach is to allow INACTIVE state and SDT transmissions even when there is no Xn connectivity. This requires the mechanisms to allow forwarding of RAN paging and to allow UE context fetch via NG-like interface. This will involve RAN3 to study the necessary mechanisms to support this work.

Proposal 4: 	In order to make INACTIVE state with SDT as the main state for UE power saving, RAN2 should enable the following features for this state:

  - Minimize the signalling overhead and latency for initiating SDT (e.g. using RRC-less SDT)

  - Improve system capacity for SDT by allowing SDT over non-initial BWP

  - Decouple INACTIVE/SDT features from requiring Xn connectivity as a prerequisite by enabling CN based context fetch/paging

  - Connection management

In 5G, different RRC procedures are applied for transition between different RRC states, those basic RRC procedures can be considered as the baseline for 6G. On top of them, RAN2 can further study the main content of each message and the whether any enhancement is needed.

Proposal 5: The following 5G RRC procedures are considered as a baseline for connection management in 6GR.

- RRC connection establishment (for transition from RRC_IDLE to RRC_CONNECTED);

- RRC connection resume (for transition from RRC_INACTIVE to RRC_CONNECTED);

- RRC connection reconfiguration;

- RRC connection re-establishment;

- RRC connection release (for transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE);

  - RRC layer design

In 5G CU-DU split scenario, the RRC layer is located at gNB-CU, but gNB-DU is responsible to generate the CellGroupConfig, so, whenever the DU generates the new configuration, the DU needs to transmit the configuration to the CU, let the CU to generate the whole RRCReconfiguration message, thus the DU generated configuration needs to be transmitted via DU-CU-DU-UE.

For 6G study, there was proposal that RRC layer can be split into RRC-high and RRC-low parts, the RRC-high remains in RAN-CU and RRC-low is located at RAN-DU. The main motivation is to enable the RAN-DU to directly send RRC message to the UE, so that the lower layer configuration latency can be reduced.

However, this is beneficial only if the configuration modification is triggered by DU. In 5G deployment, we observed that in most cases, such as normal RRC establishment, RRC reconfiguration due to service change (e.g. voice setup/release), the RRC reconfiguration procedures are triggered by CU, for which higher layer parameters (RRC/SDAP/PDCP) may also need update. Purely lower layer configuration modification triggered by DU happens rarely in the field.

Observation 3: 	In 5G deployment, the RRCReconfiguration procedure triggered by RAN-DU is rare.

On the other hand, if we allow DU to trigger RRC reconfiguration directly, then more cooperation between CU-DU will be needed. For instance, some coordination is needed to avoid CU and DU triggering RRC reconfiguration simultaneously. Thus, more discussion is needed whether/when to inform each other when the configuration is updated. Therefore, this may bring more complexity and the goal of latency reduction may not be achieved if coordination is required before DU sends the RRC signalling to UE.

Observation 4: 	Supporting RRC split (e.g. allow DU to directly send CellGroupConfig to the UE) may require more coordination between RAN-CU and RAN-DU.

In any case, whether to support CU-DU split will be determined in June 2026, so, before that, we think RAN2 should discuss RRC functionalities based on the assumption that only one RRC layer is supported. Note that this is separate from the discussion on lower layer security.

Proposal 6:	RAN2 to postpone the discussion on RRC layer split until RAN3 or RAN makes decision on the support of CU-DU split and the design decisions until then should be based on a single RRC layer.

- Conclusion

Based on the discussion above, we have the following observations and proposals:

Proposal 1: 	At least the following RRC functionalities are supported for 6GR RRC:

- Broadcast of system information;

- Paging;

- RRC connection management;

- Access control (pending on SA/CT inputs);

- Mobility control (including measurement);

- Radio Bearer management (including SRB and DRB);

- Radio configuration;

- Spectrum management;

- QoS control (also involve SA/CT);

- Transfer of NAS signalling;

- UE capability enquiry/transfer

Observation 1: 	The RRC state modelling using 3 RRC states (CONNECTED, INACTIVE and IDLE) is well suited to enable a good tradeoff between UP latency and UE power consumption

Observation 2: 	RRC INACTIVE state was not commercialized because of lack of support for small data in INACTIVE state which means that there is rarely any chance to keep the UE in this state

Proposal 2: 	In addition to the IDLE state, 6GR shall also support an RRC state with UE controlled mobility (i.e. cell selection/reselection) with small data transmissions

Proposal 3: 	The state transition from the network-controlled mobility state (e.g. CONNECTED) to a UE controlled mobility state (e.g. INACTIVE/IDLE) should only be possible via explicit network signalling (e.g. RRCRelease)

Proposal 4: 	In order to make INACTIVE state with SDT as the main state for UE power saving, RAN2 should enable the following features for this state:

- Minimize the signalling overhead and latency for initiating SDT (e.g. using RRC-less SDT)

- Improve system capacity for SDT by allowing SDT over non-initial BWP

- Enable incremental deployment benefits for INACTIVE state by decoupling INACTIVE/SDT features from requiring Xn connectivity by enabling CN based context fetch/paging (ask RAN3 to study this)

Proposal 5: 	The following 5G RRC procedures are considered as a baseline for connection management in 6GR.

- RRC connection establishment (for transition from RRC_IDLE to RRC_CONNECTED);

- RRC connection resume (for transition from RRC_INACTIVE to RRC_CONNECTED);

- RRC connection reconfiguration;

- RRC connection re-establishment;

- RRC connection release (for transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE);

Observation 3: 	In 5G deployment, the RRCReconfiguration procedure triggered by RAN-DU is rare.

Observation 4: 	Supporting RRC split (e.g. allow DU to directly send CellGroupConfig to the UE) may require more coordination between RAN-CU and RAN-DU.

Proposal 6:	RAN2 to postpone the discussion on RRC layer split until RAN3 or RAN makes decision on the support of CU-DU split and the design decisions until then should be based on a single RRC layer.