3GPP TSG-RAN WG2 Meeting #132										R2-2508408

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.3.1.3

Source: 	Sharp

Title:  	Scheduling and Retransmission Aspects

Document for: 	Discussion

Introduction

In RAN2_131bis meeting, RAN2 had following agreement on 6GR UP [1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This document discusses 6GR user plane scheduling and retransmission aspects.

Discussion

Unified Buffer Status Reporting

Notifying buffer status information of the UE to the network enables the network to provide moderate resources to the UE, which can avoid wasting of resource or delaying the data transmission. In NR, both BSR and DSR are used to indicate the buffer status of the UE. BSR MAC CE is used to report the total data volume buffered in UE whereas DSR MAC CE is used to report the delay status of the buffered data. There are some duplications between the two mechanisms, which result in signalling overhead or even inefficiency. This should be avoided when defining signalling mechanisms of such information delivery in 6G. 6GR should study a unified buffer status reporting for 6G which considers both total buffered data volume and delay status of the buffered data.

Observation 1: BSR and DSR have duplication which results in signalling overhead or even inefficiency.

Proposal 1: RAN2 to study a unified buffer status reporting including total buffered data volume and delay status of the buffered data.

Low-latency UL Transmissions

In the RAN2#131bis meeting, it was agreed to study potential scheduling enhancements to address the latency associated with SR, BSR, and DSR procedures.

In 5G NR, UL data transmission typically requires scheduling through UL grants. To request a UL grant, the UE reports buffer status to the network via BSR or DSR on PUSCH, allowing the network to schedule an appropriate UL grant. If no PUSCH resource is available, the UE transmits an SR on PUCCH and waits for the corresponding UL grant to send the BSR or DSR. This multi-step process introduces one or more round-trip delays, which can significantly limit performance for latency-sensitive traffic.

To mitigate such latency, 5G NR enhanced the configured grant (CG) mechanism, which allows the network to preconfigure periodic UL grants with fixed size so that the UE can transmit UL data/BSR/DSR without sending an SR. However, this approach is not efficient for aperiodic or variable traffic, where the network is unaware of the actual UL data arrival time and buffer status.

To enable the network to know the UE’s UL data status as soon as possible, we think one approach is to introduce contention-based UL scheduling for BSR or DSR. Specifically, by reserving a small pool of shared UL resources accessible by multiple UEs in a contention-based manner, UEs can transmit BSR or DSR immediately upon UL data arrival without the need for SR transmissions. This approach can reduce latency while maintaining reasonable resource efficiency through proper pool configuration.

Therefore, we propose that RAN2 study contention-based UL scheduling as a potential mechanism for low-latency BSR/DSR reporting in 6G.

Observation 2: Multi-step UL scheduling process (i.e., SR – BSR/DSR – UL resource) introduces one or more round-trip delays, which can significantly limit performance for latency-sensitive traffic

Proposal 2: RAN2 to study contention-based UL scheduling for low-latency BSR/DSR reporting in 6G.

Another approach to reduce latency caused by the multi-step process could be finer granularity of SR transmission which enables direct UL resource scheduling without requiring a separate BSR transmission. For example, by allowing SR to deliver slightly detailed information such as buffer status level or traffic type, the network may be able to immediately allocate sufficient and appropriate UL resource to the UE upon SR reception. This approach thus can shorten the overall UL scheduling latency.

Proposal 3: RAN2 to study finer granularity of SR to enable direct UL resource allocation without separate BSR transmission.

Unified ARQ/HARQ Retransmission

In 5G NR, error correction is managed by two protocols: i.e., HARQ in MAC and ARQ in RLC. HARQ provides fast retransmissions with forward error correction for rapid link-level error recovery, while RLC ARQ performs a slower but more comprehensive error correction and retransmission function, ensuring reliable data delivery in acknowledged mode. This dual retransmission approach is now considered as complexity and overhead that can impact system efficiency. For 6GR, an improved user plane protocol design should consider combining these error correction functionalities into a unified protocol stack. Such integration would reduce redundancy, lower latency, and improve overall performance efficiency while still maintaining the reliability and flexibility required by future 6G services.

One could argue that HARQ feedback may have risks of false alarm and misdetection. However, even in NR, the probability is very low. We can consider more reliable HARQ feedback or introducing a limitation of the long-term ARQ retransmission only for essential events.

Observation 3: ARQ/HARQ dual retransmission approach is now considered as complexity and overhead that can impact system efficiency.

Proposal 4: ARQ and HARQ are merged into a single unified retransmission.

Conclusion

Based on the discussion given above, RAN2 is requested to discuss agree the following proposals:

Proposal 1: RAN2 to study a unified buffer status reporting including total buffered data volume and delay status of the buffered data.

Proposal 2: RAN2 to study contention-based UL scheduling for low-latency BSR/DSR reporting in 6G.

Proposal 3: RAN2 to study finer granularity of SR to enable direct UL resource allocation without separate BSR transmission.

Proposal 4: ARQ and HARQ are merged into a single unified retransmission.

References

[1] RAN2_131bis Chair Note