3GPP TSG-RAN WG2 Meeting #132											R2-2508410

Dallas, United States, Nov. 17th-21st

Agenda item:		10.3.3.2

Title: 	Energy efficiency in 6G

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The following guideline has been given at RAN2#131bis on Network and UE Energy Efficiency [1]:

- Design goal:

Strive to develop solutions that consider network energy efficiency and UE power saving

- For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this paper, we share some consideration on the solutions helpful to UE and NW energy saving and targeting to have simplified and unified solution in 6G.

- Discussion

  - Energy saving in 5G

    - Features for UE power saving

UE in RRC_CONNECTED

C-DRX: UE will be configured with on-duration, DRX cycle, inactivity timer, and retransmission timer. UE will monitor PDCCH during active time, including on duration of the DRX cycle, the time UE is performing continuous reception while the inactivity timer has not expired, the time when the UE is performing continuous reception while waiting for a retransmission opportunity and skip PDCCH monitoring during non-active time.

Figure 1: DRX Cycle

Bandwidth Adaptation (BA): When BA is configured, the UE only has to monitor PDCCH on the one active BWP i.e. it does not have to monitor PDCCH on the entire DL frequency of the cell. A BWP inactivity timer (independent from the DRX inactivity-timer described above) is used to switch the active BWP to the default one.

DCI with CRC scrambled by PS-RNTI (DCP): UE may be indicated, when configured accordingly, whether it is required to monitor or not the PDCCH during the next occurrence of the on-duration by a DCP monitored on the active BWP. If the UE does not detect a DCP on the active BWP, it does not monitor the PDCCH during the next occurrence of the on-duration, unless it is explicitly configured to do so in that case. A UE can only be configured to monitor DCP when connected mode DRX is configured, and at occasion(s) at a configured offset before the on-duration.

LP-WUS in connected: A UE configured with DRX in RRC_CONNECTED can be configured with LP-WUS. LP-WUS is monitored outside active-time. If LP-WUS is detected, the UE shall start the on-duration timer or LP-WUS PDCCH monitoring timer to start PDCCH monitoring and enter active-time.

Dual DRX group: Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. Each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are on-duration and inactivity-timer.

Relaxing measurements for RLM/BFD: UE determines whether it is in low mobility state and/or whether its serving cell radio link quality is better than a threshold.

PDCCH skipping and Search space set group (SSSG) switching.

BWP switching: Adapting the DL maximum number of MIMO layers by BWP switching.

Cross-slot scheduling during active time.

UE in RRC_IDLE and RRC_INACTIVE

Relaxed neighbor cells RRM measurements: UE relax neighbor cells RRM measurements when it meets the criteria determining it is in low mobility and/or not at cell edge.

Further relaxed serving and neighbor cell measurements and Serving cell measurement offloading for LP-WUS UE: UE supporting LP-WUS are allowed to relax serving cell measurements on MR, further relax neighbor cell measurements on MR and/or offload serving cell measurements from MR to LR.

TRS in TRS occasions: The TRS in TRS occasions may allow UEs in RRC_IDLE/RRC_INACTIVE to sleep longer before waking-up for its paging occasion.

PEI: In order to reduce UE power consumption due to false paging alarms, the group of UEs monitoring the same PO can be further divided into multiple subgroups. With subgrouping, a UE shall monitor PDCCH in its PO for paging if the subgroup to which the UE belongs is paged as indicated via associated PEI.

LP-WUS in idle and inactive: The group of UEs monitoring the same PO can be further divided into multiple subgroups. With subgrouping, a UE shall monitor PDCCH in its PO for paging if the subgroup to which the UE belongs is paged as indicated via associated LP-WUS.

    - Features for network energy saving

DCI triggered CHO: UE may be notified via DCI to enable CHO conditions(s) configured with NES event indication.

    - Features beneficial for both NW and UE energy saving

Although the following features were introduced under network energy saving work item in 5G, we understand they are beneficial for both NW and UE energy saving as UE can skip some transmission and common signal/channel monitoring and get more time to sleep.

Cell DTX/DRX: To facilitate reducing gNB downlink transmission/uplink reception active time, UE can be configured with a periodic cell DTX/DRX pattern (i.e. active and non-active periods).

When cell DTX is configured and activated for the concerned cell, the UE may not monitor PDCCH in selected cases or does not monitor SPS occasions during cell DTX non-active duration. When cell DRX is configured and activated for the concerned cell, the UE does not transmit on CG resources or does not transmit a SR during cell DRX non-active duration. This feature is only applicable to UEs in RRC_CONNECTED and only for single TRP scenario in 5G.

SSB-less SCell: For an intra-band or inter-band CA SCell, a UE may obtain timing reference and AGC source from another serving cell in case the UE is not provided with SSB nor SMTC configuration for this SCell.

On-demand SSB SCell: On-demand SSB-based SCell operations are supported for UEs in RRC_CONNECTED configured with carrier aggregation (CA), applicable to both intra-band and inter-band CA configurations for FR1 and FR2 in non-shared spectrum. With RRC and MAC-CE to signal activation/deactivation state of OD-SSB transmissions, the same MAC-CE to update the transmission parameter of an activated OD-SSB after the SCell activation completion and implicit deactivation based on the number of OD-SSB bursts to be transmitted.

On-demand SIB1: Instead of always periodically transmitting SIB1, the gNB can provide on-demand SIB1, i.e., upon receiving an OD-SIB1 request from a UE supporting OD-SIB1.

Paging adaption in time domain: The value range for parameter N, which is the number of paging frames in one paging cycle, is extended to make it possible to have increased interval between PFs. The value range for Ns, which is the number of paging occasions within one paging frame, is increased to compensate the decrease in the number of PFs.

SSB adaption in time domain: SSB adaptation indicated via DCI.

PRACH adaption in time domain: PRACH adaption indicated via DCI.

Observation 1: Cell DTX/DRX, SSB-less SCell, On demand SIB1, on demand SSB SCell, Paging/SSB/PRACH adaption in time domain, although introduced under network energy saving work item in 5G, they are beneficial for both UE and NW energy saving as UE can skip some transmission and common signal/channel monitoring and reception and get more time to sleep.

  - Energy efficiency in 6G

b)	Energy efficiency and energy saving: both for network and device.

Energy efficiency and energy saving both for NW and UE has been identified as general objective for 6G RAN design [2]. In this section, we share understanding on what UE/NW power saving features can be considered and improved in 6G, focusing on solutions beneficial for both NW and UE energy saving as well as simplified and unified solution for the same purpose.

    - Solutions beneficial for both NW and UE energy saving

As identified in chapter 2.1.3, we understand Cell DTX/DRX, SSB-less SCell, On demand SIB1, on demand SSB SCell, Paging/SSB/PRACH adaption in time domain are beneficial for both UE and NW energy saving and similar solution can be considered in 6G with inheritance and improvement.

Cell DTX/DRX in all RRC states

For Cell DTX/DRX, with the active duration and cycle, network will be in power saving state during non-active time and skip the transmission and reception in DL and UL. But in 5G, the use of Cell DTX/DRX is limited to UEs in RRC_CONNECTED but for other common signal transmission and common information broadcasting, e.g. SSB, SIB and paging, to UE in RRC_IDLE and RRC_INACTIVE, there is still room for further energy efficiency improvement as these contributes a lot to the network energy consumption. The major reason for not having it supported in RRC_IDLE and RRC_INACTVE is the non-backward compatibility. For 6G, we are considering UE and NW energy efficiency from day 1, we understand Cell DTX/DRX in all RRC states should be studied with the common signal and channel transmission and UE specific transmission and reception considered.

Figure 2. Cell DTX/DRX in all RRC states

Proposal 1: Study Cell DTX/DRX in all RRC states, including impact on common signal and channel transmission and UE specific transmission and reception.

SSB-less Cell

SSB-less SCell has been supported in 5G for inter-frequency and intra-frequency case with timing reference and AGC source obtained from another cell but not allowed for other cells.

In 6G, we understand the SSB-less SCell can also be supported and we are considering to extend the usage to other cells, e.g. assisting cells in the multi-cell/carrier coordinated initial and system access procedure, for which the information required for access can be provided by anchor cell and the timing reference can also be obtained from the anchor cell, especially for co-located deployment.

Figure 3: Co-located deployment of anchor cell and assisting cell

For non-co-located deployment, whether it is possible to rely on the anchor cell to obtain timing reference or other ways to obtain timing reference for SSB-less assisting cell would require confirmation from RAN1.

Figure 4: Non-Co-located deployment of anchor cell and assisting cell

Proposal 2: Study SSB-less Cell for SCell in CA and assisting cell in multi-cell/carrier coordinated initial and system access procedure.

On-demand SSB

On-demand SSB-based SCell operations are supported in 5G NR for UE in RRC_CONNECTED configured with CA. And only network triggered on demand SSB activation and deactivation is supported.

In 6G, similarly as above for SSB-less Cell, on demand SSB can be used for SCell and assisting cell in the multi-cell/carrier coordinated initial and system access procedure and consider both NW triggering and UE triggering. And two cases can also be considered:

- Case1: SSB transmission start upon triggering, e.g. SCell in CA or assisting cell in multi-cell/carrier coordinated initial and system access

- Case 2: SSB transmission update upon triggering, applicable in any cell.

Figure 5: SSB transmission start or update upon triggering

Proposal 3: Study on demand SSB, considering both NW and UE trigger, and SSB transmission start or update upon triggering.

On-demand SIB and SIB-less Cell

As mentioned in [3], similar as the on demand SIB1 and on demand SI in 5G NR, we understand on demand system information mechanism should still be supported in 6G, which is helpful in reducing the energy consumption at both UE and network side.

For some cells, like the anchor cell in the multi-cell/carrier coordinated initial and system access procedure, its system information will include at least:

- The scheduling information of system information

- Information required for initial access for the anchor cell/carrier and assisting cells/carriers, considering both TN and NTN system and slicing.

- Cell reselection related information supporting reselection to another 6G cell or 5G cell, with slicing considered

- PWS notification and time information.

RAN2 need to consider which part of the above can be broadcast on demand and which part should be always broadcasting.

For other cells, like the assisting cells, since the access information can be provided by anchor cell, RAN2 need to consider whether it is possible to have the assisting cell to be SIB-less and support on demand switch from SIB-less cell to a normal cell, i.e. with SIB broadcasting.

Proposal 4: Study on demand SIB for both anchor cell and assisting cells in multi-cell/carrier coordinated initial and system access and SIB-less only for assisting cells.

Common signal and channel adaption

For SSB adaption, extended SSB periodicity should be considered in 6G, which will impact the SIB1 transmission, RACH occasion configuration and paging as they are closely associated with SSB transmission.

To co-exist with the Cell DTX, the SSB transmission should also be aligned with Cell DTX configuration to avoid SSB transmission and reception in non-active time.

Proposal 5: Study extended SSB periodicity and SSB transmission in Cell DTX, with impact on SIB1 transmission, RACH occasion configuration and paging considered.

    - Simplified and unified solution for the same purpose

DRX

We understand DRX has been quite helpful in UE power saving and would like to still have this in 6G.

DCP and LP-WUS in connected

DCP and LP-WUS has a lot in common and how they work has been shown below:

Figure 6: DCP and LP-WUS for UE in RRC_CONNECTED

Such functionality, dynamic indication on whether to monitor or not the PDCCH during the next occurrence of the on-duration in DRX, can still be useful in 6G but unified solution is expected.

Relaxed RRM measurements

We have defined a lot of rules for relaxed measurements on serving or neighbor cells, with a list of thresholds and criterion. However, all these falls into two categories: low mobility and not at cell edge or close to cell center.

PEI and LP-WUS in idle and inactive

PEI and LP-WUS have been introduced in 5G NR for early indication before paging message while the design in 5G has a lot in common, e.g. the grouping of UE for the early indication while the major difference is the DL signal to send this indication.

In 6G, for UE power saving purpose, we understand similar mechanism would still be there but we expect simplified and unified solution, e.g. to support only one single solution for paging early indication.

Proposal 6: Support DRX for UE power saving in 6G.

Proposal 7: Study unified solution for the following functionality:

- Dynamic indication on whether to monitor or not the PDCCH during the next occurrence of the on-duration in DRX

- Relaxed serving and/or neighbor cell measurements for UE with low mobility and/or close to cell center.

- Paging early indication

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Observation 1: Cell DTX/DRX, SSB-less SCell, On demand SIB1, on demand SSB SCell, Paging/SSB/PRACH adaption in time domain, although introduced under network energy saving work item in 5G, they are beneficial for both UE and NW energy saving as UE can skip some transmission and common signal/channel monitoring and reception and get more time to sleep.

Proposal 1: Study Cell DTX/DRX in all RRC states, including impact on common signal and channel transmission and UE specific transmission and reception.

Proposal 2: Study SSB-less Cell for SCell in CA and assisting cell in multi-cell/carrier coordinated initial and system access procedure.

Proposal 3: Study on demand SSB, considering both NW and UE trigger, and SSB transmission start or update upon triggering.

Proposal 4: Study on demand SIB for both anchor cell and assisting cells in multi-cell/carrier coordinated initial and system access and SIB-less only for assisting cells.

Proposal 5: Study extended SSB periodicity and SSB transmission in Cell DTX, with impact on SIB1 transmission, RACH occasion configuration and paging considered.

Proposal 6: Support DRX for UE power saving in 6G.

Proposal 7: Study unified solution for the following functionality:

- Dynamic indication on whether to monitor or not the PDCCH during the next occurrence of the on-duration in DRX.

- Relaxed serving and/or neighbor cell measurements for UE with low mobility and/or close to cell center.

- Paging early indication.

- Reference

[1] Draft_RAN2_131bis_MeetingReport_v1

[2] RP-251881 New SID: Study on 6G Radio

[3] R2-2508409_6G initial and system access