3GPP TSG-RAN WG2 #132	R2-2508421

Dallas, United States, 17th – 21st November 2025

Source:	NTT DOCOMO INC.

Title:	On system information security

Document for:	Discussion

Agenda Item:	10.3.3.3

- Introduction

In this contribution, we express our views on the idea of implementing security protection for system information, while also referring to the requirements established by the SA WGs.

- Discussion

  - Mobility-related information

In NR, system information is neither encrypted nor integrity protected, which means that an attacker can tamper with the content of system information broadcast by a gNB and relay it to the UE. Furthermore, by setting up a fake base station, it is even possible to broadcast false system information and have UEs receive it. Therefore, it may be beneficial to implement security protection for system information in 6G.

In our view, introducing integrity protection for certain types of information transmitted via system information would be effective. For example, if integrity protection is applied to system information related to neighboring cell measurements, it would prevent tampering with or fabricating such information by attackers. This, in turn, would help protect against attacks that attempt to disrupt network-controlled IDLE mode mobility of the UE.

Observation 1.	Introduction of security protection for system information prevents tampering and false base station attacks.

Observation 2.	Security protection of system information related to neighboring cell measurements is beneficial to keep IDLE mode mobilities under NW’s control.

Proposal 1.	RAN2 starts a study to introduce security protection for system information that conveys mobility-related data, to defend against attacks from false base stations.

Considering that the content of system information is basically broadcast, there may be no point in encrypting system information. On the other hand, integrity protection for system information will protect from tampering and false base station attack. Therefore, it seems straightforward to consider only integrity protection. But important point is that this decision should be made by SA3.

Proposal 2.	It is up to SA3 to discuss and decide what kind of security protection is introduced for system information.

  - PWS information

However, when introducing integrity protection for the Public Warning System (PWS), which is one of the types of information transmitted via system information in NR, there are several challenges that need to be addressed:

First, it is specified in TS 22.268 [1] that neither integrity protection for Warning Notification nor protection against false Warning Notification shall be presented, unless required otherwise by local regulations.

## 4.8 Security Requirements Security requirements are as follows: - PWS shall only broadcast Warning Notifications that come from an authenticated authorized source. The following requirements only apply when not roaming internationally: - When required by regional or national regulations, the integrity of the Warning Notification shall be protected. If no such regulatory requirement exists, there shall be no integrity protection of Warning Notifications, and all Warning Notifications shall be presented to the PWS application on the PWS-UE. - When required by regional or national regulations, the PWS shall protect against false Warning Notification messages. If no such regulatory requirement exists, there shall be no protection against false Warning Notifications, and all Warning Notifications shall be presented to the PWS application on the PWS-UE. Note 1: These requirements are subject to regulatory policies. NOTE 2: The authentication and authorisation of the source are outside the scope of 3GPP Specifications.

Observation 3.	Security protection of system information indicating PWS involves several challenges that need to be resolved: PWS requirements in TS 22.268 and unreachability of PWS via other operators’ base stations.

Second, if the security key used for integrity protection is provided to the UE through network authentication, the UE will not be able to decode PWS messages broadcast by base stations of operators with which it has not performed the authentication procedure.

For example, in NR, if a UE is a subscriber of MNO#1, it can naturally receive ETWS or CMAS through the system information broadcasted by MNO#1's gNB when within MNO#1's coverage area. However, even when the UE is outside MNO#1's coverage, if it is within the reception range of system information broadcasted by another MNO#2's gNB, it can still receive ETWS/CMAS even without a subscription to MNO#2.

On the other hand, if security protection such as integrity protection is introduced for system information used to deliver PWS in 6G, the UE will not be able to receive ETWS/CMAS broadcasted by MNO#2 unless it already possesses the security key for MNO#2's system information.

Due to this drawback, the reachability of PWS may decrease in areas not covered by certain operators, especially in local areas. Considering that Japan frequently experiences natural disasters such as earthquakes, tsunamis, and typhoons, and that many residents and travelers are present in these local areas, we oppose any approach that could reduce the probability of receiving PWS.

As a possible way to resolve this issue, one could consider pre-distributing the security keys of MNOs other than the one the UE is subscribed to. However, from a security standpoint, if these keys are temporary and subject to regular updates, the UE would frequently receive key update requests, resulting in poor efficiency and making this approach rather impractical.

Observation 4.	Regarding the system information used to deliver PWS, if the UE does not obtain the corresponding security key, it will be unable to decode the system information, which may lead to reduced reachability of PWS.

Proposal 3.	RAN2 studies security protection of system information considering requirements for PWS messages in TS 22.268 and without reducing reachability of PWS.

- Conclusions

Mobility-related information

Observation 1.	Introduction of security protection for system information prevents tampering and false base station attacks.

Observation 2.	Security protection of system information related to neighboring cell measurements is beneficial to keep IDLE mode mobilities under NW’s control.

Proposal 1.	RAN2 starts a study to introduce security protection for system information that conveys mobility-related data, to defend against attacks from false base stations.

Proposal 2.	It is up to SA3 to discuss and decide what kind of security protection is introduced for system information.

PWS information

Observation 3.	Security protection of system information indicating PWS involves several challenges that need to be resolved: PWS requirements in TS 22.268 and unreachability of PWS via other operators’ base stations.

Observation 4.	Regarding the system information used to deliver PWS, if the UE does not obtain the corresponding security key, it will be unable to decode the system information, which may lead to reduced reachability of PWS.

Proposal 3.	RAN2 studies security protection of system information considering requirements for PWS messages in TS 22.268 and without reducing reachability of PWS.

- References

[1]	TS 22.268, “Public Warning System (PWS) requirements,” V20.0.0.