3GPP TSG-RAN WG2 Meeting #132	R2-2508423

Dallas, USA, Nov. 17th-21st, 2025

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	(C003) Remaining RRC issue for NR NTN MBS

Document for	:	Discussion and Decision

Introduction

In this paper, we discuss some remaining issues related to RILs raised in ASN.1 review [1].

There were no agreement about issue C003 since it was postponed to the next meeting.

R2-2506868	[C003] Clarification of how the UE uses the service area information in SIB and USD	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 1: Add a NOTE “NOTE: If the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD.” in section 5.2.2.4.29 Actions upon reception of SIB27. Adopt the TP in Annex.

[C003] Postponed to the next meeting

R2-2506868	[C003] Clarification of how the UE uses the service area information in SIB and USD	CATT	discussion	Rel-19	NR_NTN_Ph3-Core	Late

Proposal 1: Add a NOTE “NOTE: If the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD.” in section 5.2.2.4.29 Actions upon reception of SIB27. Adopt the TP in Annex.

[C003] Postponed to the next meeting

Discussion

Remaining issue

(C003) How UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned.

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C003 | NTN | 1 | clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned | Yes, R2-250xxxxx | CATT (Da Wang) |  | v008 | ToDo |

[Description]: We prefer to clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned. Since the UE is not expect to receive MBS service outside the service area, it would be better for the UE to get a precise service area scope.

[Proposed Change]: Add the following NOTE to clarify the relationship between the Target Service Area in the USD and the ISA in SIBXX.

NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD

[Comments]:

[Rapp] Even thought it is unlikely that both ways of delivering the information may be configured simultaneously, RAN2 should further discuss which one is prioritized (if needed).

[Qualcomm] Which ever area UE wants to use should work pointing to (may be not exactly same) almost same region. Otherwise this should be error case.

====

The situation intended in this issue is as follows.

- The UE receives the service area coverage specified in the USD and the frequency information for the delivered services.

- The UE receives the ISA contained in the SIBXX.

- At this time, a mismatch occurs between the area coverage specified in the USD and the ISA coverage specified in the SIB.

In such a situation, if the UE is located inside the ISA, executing the action to establish the MRB is consistent with the agreement reached at the RAN2#127 meeting.

Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions. - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In TS26.502, the MBSF generates the USD, and this information is transmitted to the RAN via the Nmbstf. The RAN generates the MBS session information based on this information. That is, the information about the area intended by the content provider may not be simultaneous, but it can be said that alignment with the RAN is implicitly maintained. Therefore, the situation proposed in this issue may be a corner case.

Observation 1: Inconsistencies in area information between USD and SIBXX may be a corner case.

Information inconsistencies are likely to occur when either USD or SIBXX is received, which indicates either a failure in the NW-side functionality or a lack of alignment. Area information between USD and SIBXX has been synchronized at least once, after which SIBXX is delivered to the UE. Additionally, upon entering a cell providing MBS service, the basic operation of Rel19 UE is to confirm the association between the SIBXX ISA and the ISA ID in MCCH before initiating the MRB establishment procedure. So, the RRC procedures (e.g. MRB establishment procedure) should be performed according to the SIBXX ISA when inconsistencies occur between SIBXX and USD.

Observation 2: The area information between USD and SIBXX has been synchronized at least once. In addition, UE is to confirm the association between the SIBXX ISA and the ISA ID in MCCH before initiating the MRB establishment procedure.

Proposal 1: (C003) As the ISA is not aligned with the target service area in USD, the RRC procedures should be performed according to the SIBXX ISA.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1: Inconsistencies in area information between USD and SIBXX may be a corner case.

Observation 2: The area information between USD and SIBXX has been synchronized at least once. In addition, UE is to confirm the association between the SIBXX ISA and the ISA ID in MCCH before initiating the MRB establishment procedure.

Proposal 1: (C003) As the ISA is not aligned with the target service area in USD, the RRC procedures should be performed according to the SIBXX ISA.

Reference

- NTN Review file v022.docx