3GPP TSG-RAN2 Meeting #132	R2-2508436

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.4.3 Other issues

Source: 	Ericsson

Title:  	LP-WUS other issues (38306-1, 38304-1, 2 and 3)

Document for:	Discussion and Decision

# Introduction

The following topics are discussed in this contribution:

- 38306-1: LP-WUS capability outside Rel-19 paging capability container.

- 38304-1: Clarification for the common codepoint in TS 38.304.

- 38304-2: Clarification for RX level and cell quality value in multi-beam operations.

- 38304-3: Discuss how to capture the agreement in RAN1 LS R1-2508170.

# Discussion

## #38306-1: LP-WUS capability outside Rel-19 paging capability container

RAN2 agreed:

The UE capabilities for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message. Detailed signalling can be further discussed.

RAN2 needs to decide the details, i.e. option 1 or 2 [1]:

- Option 1: only indicating the supported band list for LP-WUS regardless of the receiver type and other capabilities;

- Option 2: indicating the supported band list for LP-WUS and the receiver type.

The LP-WUS capability includes a list of bands where the UE supports LP-WUS. Per band the UE can indicate to support OFDM and/or OOK based LP-WUS:

UE-RadioPagingInfo-r19 ::=            SEQUENCE {

lpwus-SupportedBandList-r19           SEQUENCE (SIZE (1..maxBands)) OF LPWUS-SupportedBandInfo-r19 	OPTIONAL,

…

LPWUS-SupportedBandInfo-r19 ::=      SEQUENCE {

supportedBandIndicator-r19           FreqBandIndicatorNR,

-- R1 62-1: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal

lpwus-OOK-r19                        ENUMERATED {supported}             									OPTIONAL,

-- R1 62-1a: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence

lpwus-OFDM-r19                       ENUMERATED {supported}             									OPTIONAL,

-- R1 62-1b: LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured

lpwus-LP-SS-r19                      ENUMERATED {supported}             									OPTIONAL,

minimumTimeGapOOK-r19                ENUMERATED {cap1, cap2, cap3}   									OPTIONAL,

minimumTimeGapOFDM-r19	            ENUMERATED {cap1, cap2, cap3}				    					OPTIONAL

}

When the UE supports OFDM based LP-WUS the UE may also indicate to support LP-SS based RRM measurements. The UE also indicates the minimum time gaps that it supports with OFDM and OOK-based LP-WUS:

The gNB that redirects an LP-WUS UE to a frequency where the UE supports LP-WUS does not take into account the time offset(s) configured on that frequency and the minimum time gaps supported by the UE.

The gNB that redirects an LP-WUS UE to a frequency where the UE supports LP-WUS does not take into account whether LP-SS is configured on the frequency and whether the UE supports OFDM with LP-SS based RRM measurements.

The LP-SS and minimumTimeGap UE capability are not needed for redirection. However there is not a lot of overhead when the LP-WUS capability is just copied outside the Paging capability container:

The UE signals the LP-WUS UE capability inside and outside the Rel-19 Paging container.

## #38304-1: Common codepoint in TS 38.304

The LP-WUS monitoring rules should include monitoring of the common codepoint:

If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.4C in TS 38.213 [4] or the common codepoint associated with all subgroups is detected, the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2 if PEI is supported and PEI configuration is provided in system information, which is up to UE implementation. If the UE does not detect an LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.4C in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1.

The common codepoint is added to the LP-WUS monitoring rules in TS 38.304.

## #38304-2: RX level and cell quality value in multi-beam operations

During RAN2#131 issue RRC-13 was discussed, i.e. whether beam selection is up to UE implementation or that configuration in SIB is used [2]:

Rapporteur understands that the below LR measurement based RX level and cell quality value:

-	Qrxlevmeas _lr= current measured cell RX level value of the serving cell based on LR (RSRP).

-	Qqualmeas _lr = current measured cell quality value of the serving cell based on LR (RSRQ).

should be derived by UE implementation in multi-beam operations, i.e. no additional RRC parameters like nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation for LR measurement is needed.

Companies are invited to provide comments on whether LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations.

Note that in TS 38.304 it was already specified that the LP-WUS beam selection is left to UE implementation e.g. whether the UE uses the strongest beam or the average of N beams:

The UE monitors one LO per DRX cycle. An LO is a set of LP-WUS monitoring occasions (LP-WUS MOs). In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation.

Then during the online discussion in RAN2#131 it was agreed:

LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.

And in TS 38.304 it has been captured:

In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation.

The sentence can be read that how the UE determines the Rx level (LR-RSRP) and cell quality (LR-RSRQ) from the LR measurements is up to UE implementation. We do not think that it is up to UE implementation how to measure LR-RSRP/LR-RSRQ. Furthermore LR-RSRP/LR-RSRQ measurements do not differ in case of multi-beam and single-beam operations. Unless it is be clarified what the intention of the sentence is we propose:

Remove “In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation” from 38.304.

## #38304-3: Discuss how to capture the agreement in RAN1 LS R1-2508170.

In RAN1#122bis, the following clarification was made regarding the LO determination based on lpwus-LoFrameOffsetList:

Agreement

The LO determination based on lpwus-LoFrameOffsetList is clarified as follows:

- If the number of POs associated with a LO is less than Ns (the number of POs per PF), the UE uses the (i_sNPOWO+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (i_sNPOWO+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList.

- For the case when both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured,

  - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay;

  - Otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay;

  - Otherwise, the UE monitors legacy PO.

- For the case when only one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is configured,

  - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay (whichever is configured) and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO;

  - Otherwise, the UE monitors legacy PO.

The LO monitoring rule can be summarized as:

- In case there is more than one candidate LO before the PO that the UE is able to monitor, then the UE monitors the LO closest to the PO, i.e. the NW sends one LP-WUS before the PO to wake up the UE.

The following clarification in TS 38.304 is proposed:

If single value is configured for lpwus-LoFrameOffsetList, and if the gap between the LO and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

If more than one values are configured for lpwus-LoFrameOffsetList, and if the gap between the LO associated with at least one offset and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the smallest offset value with which the gap between the LO and the corresponding PO is no less than the wake-up delay, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2. In case the UE is able to monitor more than one LO associated with its PO, then the UE monitors the LO closest to its PO.

Clarify in TS 38.304 that in case there is more than one candidate LO before the PO that the UE is able to monitor, then the UE monitors the LO closest to the PO

# Summary

RAN2 is kindly asked to discuss the following LP-WUS issues:

Proposal 1	The gNB that redirects an LP-WUS UE to a frequency where the UE supports LP-WUS does not take into account the time offset(s) configured on that frequency and the minimum time gaps supported by the UE.

Proposal 2	The gNB that redirects an LP-WUS UE to a frequency where the UE supports LP-WUS does not take into account whether LP-SS is configured on the frequency and whether the UE supports OFDM with LP-SS based RRM measurements.

Proposal 3	The UE signals the LP-WUS UE capability inside and outside the Rel-19 Paging container.

Proposal 4	The common codepoint is added to the LP-WUS monitoring rules in TS 38.304.

Proposal 5	Remove “In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation” from 38.304.

Proposal 6	Clarify in TS 38.304 that in case there is more than one candidate LO before the PO that the UE is able to monitor, then the UE monitors the LO closest to the PO

# References

- R2-2506965, [LPWUS-Cap-OI-1] Discussion on open issue for LP-WUS UE capabilities, Huawei, DISC, RAN2#131-bis

- R2-2505393, Discussion summary and list of RRC open issue for LP-WUS WUR, vivo, DISC, RAN2#131