3GPP TSG-RAN WG2 Meeting #132	R2-2508451

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.3.2.3

Source:	Apple

Title:	Views on 6G Paging and System Information and Initial Access

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to 6GR SID (RP-252912), the control plane design is one of the RAN2-led objectives, which includes paging, System information and initial access.

Paging and system information were discussed in RAN2#131bis meeting, and some preliminary agreements were reached.

This contribution gives our views on the following directions in 6G study:

- Paging

- System information

- Initial access.

# 6G Design

## Paging

RAN2 reached the following agreements on 6G paging design in RAN2#131bis meeting.

Agreements - Paging design should consider energy efficiency and simplicity for both UE and Network.

Here we will give our thoughts on 6G paging design from these aspects.

1. Network energy efficiency

In NR, uniform PO distribution is supported from NR Day 1 to ensure load balancing, but it’s not good for network energy saving.

<6G goal>

Intuitively, for network energy saving, the non-uniform PO distribution design is needed. This means concentrating all POs within the paging DRX cycle, keeping network in a sleep state for the remaining time. This is the most energy-efficient method.

Figure 1: Illustration of the benefits of non-uniform PO distribution for network energy saving

<5G NES situation>

In NR R19 NES WI, the non-uniform PO distribution direction concept was attempted to be introduced into 5G system. It’s the R19 NES paging adaptation feature. However, due to the backward compatibility issue, this feature can only bring the limited NES gain due to the following two restrictions:

- Restricion#1: always configuring two sets of paging resources

Due to backward compatibility, NW needs to always configure two sets of paging resource for legacy UE and new UEs respectively when 5G paging adaptation is enabled. Correspondingly, NW also needs to send two sets of paging messages for legacy UE and new UEs respectively. It significantly limits its NES gain.

6G paging adaptation should be targeted for day-1 feature and should not be bound by such legacy constraints, allowing for the exploration of cleaner and more energy-efficient techniques.

- Restricion#2: The 5G solution is not flexible (only extending N and Ns with one new value)

As illustrated in Figure 2, 5G paging adaptation studied two solutions to achieve non-uniform/bundled PF:

- Option a): bundle PFs by revising the paging formula in TS 38.304.

- Option b): concentrate all POs in one PF by extending the values of N and increasing POs per PF.

The conclusion is that both solutions can work but more companies had concern on option a) because it will lead to major specification impacts (i.e. introduce a new formula in TS 38.304). Thus, option b) was finally adopted. However, due to concern of backward compatibility, N is only extended to 32 and Ns is only extended to 8.

It is obvious that 5G paging adaptation solution can’t be directly reused in 6G because it is not flexible enough to handle 6G use cases. 6G paging adaptation should design a flexible solution to enable / disable non-uniform/bunded PF/PO distribution from day-1.

Figure 2: Two studied solutions of 5G paging adaptation to achieve non-uniform PO/PF

Observation 1: 5G paging adaptation solution has limited NES gain due to backward compatibility and it can’t flexibly enable PF bundling.

<6G direction>

For paging adaptation, solution Paging adaptation: design a flexible solution to enable / disable non-uniform/bunded PF/PO distribution from day-1.

Since the design of 6G Day 1 does not require consideration of backward compatibility, it is proposed that to support the non-uniformed PF/PO distribution design in the flexible way, to achieve the effect shown in Figure 1.

Observation 2: 6G paging study from the network energy efficiency perspective should consider the non-uniform PO distribution direction in flexible way as day-1 design.

2.  UE energy efficiency

From UE point of view, the following features of the NR paging design are introduced for UE implementation simplification and for UE power saving, which should be inherited and enhanced in 6G paging design.

In NR, to reduce UE power consumption due to false paging alarms, WUS and subgroup based paging mechanism was introduced. NR introduces two WUS based paging features.

- In R17, PEI based paging mechanism was introduced, where PEI was used as the WUS signal and the design is based on the PDCCH DCI.

- In R19, LP-WUS based paging mechanism was introduced, where LP-WUS was used as the WUS signal and the design is based on new signal.

In both features, the group of UE monitoring the same PO can be further divided into multiple subgroups. With subgrouping, UE is required to enable the legacy paging monitoring in its PO if the subgroup to which the UE belongs is paged as indicated via associated WUS signaling. The paging subgroup of one UE can be either allocated by CN via NAS registration procedure or calculated based on cell specific subgrouping configuration.

Because the paging frameworks based on PEI and LP-WUS are very similar and are mainly used for different UE device types (due to the different HW requirements), in 6G design, we should also consider the unified subgroup and WUS based paging mechanism as the 6G native paging design, and the study on WUS signal design can be left to RAN1.

Figure 3: Illustration of Subgroup and WUS based paging framework

Observation 3: 6G paging study from the UE power saving perspective should consider the unified WUS and subgroup-based paging framework.

3.  Simplicity from UE side

From reducing UE complexity perspective, UE is expected to monitor only one type of paging over one PO resource set.

In NR, RRC INACTIVE UE is required to monitor both CN initiated and RAN initiated UE paging message, each corresponding to a set of PO resource which calculated based on UE ID and the paging DRX cycle. The DRX cycle may be different for the two types of paging. In other words, NR INACTIVE UE needs to compute two sets of PO resources and monitor two types of paging over the two PO sets respectively. This brings the UE complexity.

In addition, if CN paging and RAN paging can be converged into one paging, it can simplify UE paging reception procedure, as the current UE operations based on CN paging and RAN paging are different.

In 6G design, UE we should ensure that only one PO set is used for UE to monitor the UE specific paging.

Observation 4: 6G paging study from UE complexity reduction perspective should consider that UE only supports one type of paging, and the paging transmission only uses one PO resource set.

In addition, the paging cause carried in the paging message can simplify the UE operation to decide whether to response the paging immediately, especially for MUSIM UE device.

Observation 5: 6G paging study from UE complexity reduction perspective should consider including more paging cause than 5G in the paging message.

4.  Simplicity from network side

In NR, when network initiates UE paging procedure, network will page the UE within the paging area. regarding the paging area, tracking area is the max paging area for CN paging, and RNA is the max paging area for RAN paging.

In order to reduce the paging signaling overhead in network side, paging area optimization is supported in network side, and network may attempt paging for multiple times and enlarge the paging area for the next time if NW doesnot receive the paging response for this time.

With the paging area optimization, some operators may still have the concern for the multiple frequency deployment scenario. In multiple frequency deployment, since UE may camp on any frequency, network has to transmit the paging over all the frequencies in that area, it may increase the network burden.

For the issue in multiple frequency deployment, it could be easily resolved by restricting the frequency at which UE is allowed to camp on. According to NR design, UE will always follow the network frequency priority to select the cell and always select the cell on the high-priority frequency if the quality is no problem. Therefore, network can reduce the number of CC for paging transmission by setting the CC that allow UEs to camp on.

Observation 6: 6G paging study from network complexity reduction perspective should consider restricting the number of frequencies that allow UE to camp on.

5. 6G paging summary

Based on the discussion in previous sub-sections, we propose:

Proposal 1: 6G paging study should support the following design:

- The non-uniform PO distribution direction in flexible way as day-1 feature

- Unified WUS and subgroup-based paging framework

- Only one PO resource set used for UE to monitor paging, and only one paging type to be supported

- More Paging cause included in paging message.

- Reduce the number of paging CC by limiting the number of CC that a UE can camp on.

## System Information

RAN2 reached the following agreements on 6G system information design in RAN2#131bis meeting.

Agreements - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. - NOTE: this is a starting list

Here we will give our thoughts on 6G paging design from these aspects.

In NR, system information is divided into minimum SI and other SI. Their main characteristics are shown in the table below.

From the perspective of power/energy saving and resource efficiency improvement, the most significant features of NR SI design is the introduction of two features: 1) on-demand broadcast and 2) area specific SI (or stored SI).

- By using on-demand broadcasting design, network does not need to transmit SI all the time when there is no UE need, which can save system resources and network energy.

- By using area specific SI (i.e. stored SI) design, UE does not need to reacquire SI every time it performs cell reselection, which can save UE power.

However, these two features cannot be applied to Minimum SI in NR, because Minimum SI provides information related to initial access, and this configuration will be different in each cell. And later from network energy saving perspective, the on-demand SIB1 is introduced in NES specific cell in R19 NES WI.

In 6G, considering the benefits of these two features, we should inherit the two features in 6G design and study the possibility of applying them to Minimum SI. Regarding the Minimum SI, MIB is transmitted on PBCH and the design depends on RAN1 study, so RAN2 study can first focus on SIB1.

- Regarding on-demand SIB1 broadcast direction, since NR already supports it in NES cells for NES UE in R19, supporting it in 6G cells is straightforward.  Supporting it from 6G Day1 ensures that all UEs will support it without any backward compatibility issues. This will also make the design much simpler.

- Regarding the area specific SIB1 direction, according to the configuration in the field deployment, it can be found that the configuration in SIB1 that are not related to the initial access is basically the same within a certain area.

As shown in Figure 4, if the SIB1 configuration is basically same and the difference part is only in the initial access configuration part, according to NR cell specific SIB1 design, UE is required to acquire SIB1 per cell reselection; but with area specific SIB1 concept, if the configuration related to the UE mobility is same, UE only SIB1 in the first cell#1 and can skip the SIB1 acquisition after the cell reselection to new cell in the same area,  thereby saving UE power and shortening cell reselection delay.

Figure 4: Illustration of the benefits of area specific SIB1 design (good for UE power)

- If the configuration in NR SIB1 can be further divided into a cross-cell part (e.g. cellAccessRelatedInfo cellSelectionInfo, si-SchedulingInfo) and a cell-specific part (e.g. RACH-ConfigCommon), the cross-cell part can be configured at the area level.

Through this SIB1 split, under the area specific SIB1 concept, we can also allow for differences in some parameters in the SIB1 configuration, especially the different configurations for the initial access.

For example, When UEs move within the area but do not trigger initial access, the SIB1 acquisition per cell reselection can be skipped, and UE can acquire the initial access configuration of the camping cell only when it intends to trigger the initial access.

Figure 5: Example of the UE operation on SIB1 acquisition with area based SIB1

Proposal 2: 6G SIB should support the following design and NR design can be the starting point:

- Area specific SIB design

- On-demand SIB design

- On-demand SIB1 design.

Proposal 3: 6G SIB1 study should consider the following aspect:

- Area specific SIB1 design

- Splitting SIB1 split into area specific configuration and cell specific configuration.

## Initial access

<Early indication>

Early indication is supported in NR initial access, and the purpose is to inform NW the triggering cause and the situation which may impact the transmission of the RACH procedure or be used for network access control.

Early indication is transmitted from to network in explicit or implicit way during the initial access, and can be categorized into two types:

- Type-1 indication: the implicit way through the Msg1/preamble transmission using the RACH partition design.

The type-1 indication supported in NR is summarized as below:

- Type-2 indication: the explicit way to carry it in the Msg3 transmission.

The Type-2 indication supported in NR is summarized as below:

The introduction of too many early indication places a great demand on the number of RACH partition and LCID, which makes the fragmentation of RACH resources and the consumption of LCIDs increasingly serious.

Observation 7: The early indication design in 5G resulted in severe RACH resource fragmentation and LCID consumption.

6G network is designed to support more services, features, and scenarios, so if this 5G early indication design is still used, the situation will be more serious than in 5G. Therefore, we need to reconsider this early ideation design principle.

Lessons learned from 5G dictate that we must at least ensure that 6G design reduces demand to the RACH partition and the consumption of the LCID.

Observation 8: The 6G early indication should avoid/reduce the RACH resource fragmentation and the consumption of LCIDs.

Based on 5G early indications, we can categorize them into different types and consider different design principles for 6G, as follows:

- Cat 1: For CCCH message size indication

In NR, we support 48-bit and 64-bit CCCH message sizes which is designed to be distinguished by LCID. Considering it can used together other early indications, the LCID number, it resulted in doubling the number of all Type-2 early indication.

    - In 6G, to avoid excessive LCID consumption by this design, we should avoid Cat-1 indication and enhance the MAC subheader and MAC subPDU format to indicate the CCCH message size.

- Cat 2: To indicate the features which doesnot affect the RACH procedure

    - In NR, this indication is used to indicate the features which may impact the network access control.

    - In 6G, the indication in this category can be avoided, because we can assume network can acquire the similar information from RRC message (e.g. cause value) to perform the access control.

- Cat 3: To indicate the features that affects the RACH procedure

    - In NR, the indication in this category mainly refers to the Msg1, Msg3, Msg4 repetition transmission, and this is impossible to completely avoid.

    - The indication in this category is mainly determined based on UE capability and UE channel condition. Considering the UE channel condition and UE capability will not change significantly during the RACH procedure, for repetition indication, we do not need to design the repetition indication for each message transmission separately.

    - In 6G, we should strive to reduce the number of the indication and merge the indications with similar functional conditions as much as possible.

Proposal 4: The 6G design of early indication should follow the following principles:

- Avoid/Reduce the dependencies on RACH partitions and LCID codepoints (in MAC subheader)

- Avoid using LCID to indicate the CCCH message size (if multiple CCCH size is supported).

<Access control>

Access control is performed during the initial access. And Figure 6 illustrates the NR access control framework in each step during the initial access.

Figure 6: NR access control framework

Since there are no problems with the current NR access control framework, and we can inherit it to 6G.

Proposal 5:  6G access control framework should reuse 5G design (in Figure 5) as a baseline.

Regarding the UAC design, whether AC and AI can provide traffic or service level, or richer information, depends on the SA discussion.

Observation 9: Whether 6G UAC can provide finer-grained AC/AI control depends on SA discussion.

Regarding the UAC design, whether AC and AI can provide traffic or service level, or richer information, depends on the SA discussion.

In addition, considering the actual deployment in the commercial network, the UAC function is basically disabled, which can lead to network congestion.

Therefore, in 6G network, if network still follows 5G policy and this UAC configuration remains optional or disabled, RAN2 is proposed to consider how to handle the network congestion avoidance in the absence of UAC.

Proposal 6:  RAN2 6G study should consider how to address network congestion issue if NW doesn’t provide UAC parameters.

# Conclusions

This contribution presents our views on 6G paging, system information and initial access. We have the following proposals:

<6G Paging>

Proposal 1: 6G paging study should support the following design:

- The non-uniform PO distribution direction in flexible way as day-1 feature

- Unified WUS and subgroup-based paging framework

- Only one PO resource set used for UE to monitor paging, and only one paging type to be supported

- More Paging cause included in paging message.

- Reduce the number of paging CC by limiting the number of CC that a UE can camp on.

<6G System Information>

Proposal 2: 6G SIB should support the following design and NR design can be the starting point:

- Area specific SIB design

- On-demand SIB design

- On-demand SIB1 design.

Proposal 3: 6G SIB1 study should consider the following aspect:

- Area specific SIB1 design

- Splitting SIB1 split into area specific configuration and cell specific configuration.

<6G Initial Access>

Observation 7: The early indication design in 5G resulted in severe RACH resource fragmentation and LCID consumption.

Observation 8: The 6G early indication should avoid/reduce the RACH resource fragmentation and the consumption of LCIDs.

Proposal 4: The 6G design of early indication should follow the following principles:

- Avoid/Reduce the dependencies on RACH partitions and LCID codepoints (in MAC subheader)

- Avoid using LCID to indicate the CCCH message size (if multiple CCCH size is supported).

Proposal 5:  6G access control framework should reuse 5G design (in Figure 5) as a baseline.

Observation 9: Whether 6G UAC can provide finer-grained AC/AI control depends on SA discussion.

Proposal 6:  RAN2 6G study should consider how to address network congestion issue if NW doesn’t provide UAC parameters.

# References

- RP-252912	Study on 6G Radio