# 3GPP TSG RAN WG2 Meeting #132      	                        R2-2508454
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	10.3.3.1

Source:	Apple

Title:	Discussion on 6G data collection and data transfer

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion and Decision

# 1 Introduction

The SID of 6G Radio (SID RP-251881) was agreed in RAN#108 [1]. The SID objective on 6G data collection can be found as follows:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

And the SID objective on 6G “data transfer” is:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Some initial agreements on 6G data collection and data transfer were made in RAN2#131b [2]. In this contribution, we continue to discuss the following aspects:

- Requirement of 6G data collection

- 6G AI/ML data collection framework

- 6G data transfer

# 2 Discussion

## 2.1 6G data collection

### 2.1.1: Requirement of 6G data collection

In RAN2#131b [2], the following agreements on 6G data collection were made:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

First, we think the definition of “standardized data” in the above agreement needs clarification from RAN2 point of view. Please note that in 5G AI/ML discussion, there was no formal definition of “standardized data” captured in TR 38.843 [3], which leads to some confusion in SA2 discussion. In our understanding, the “standardized data” is the data whose format/content and measurement requirement are explicitly defined in 3GPP specifications. We think RAN2 should confirm this understanding.

Proposal 1: RAN2 understand that the “standardized data” in 6G AI/ML data collection means the data whose format/content and measurement requirement are explicitly defined in 3GPP specifications.

Then, we think the followed discussion on visibility requirement (for both AI/ML and non-AI/ML data collection) should be deferred to RAN plenary, as it is a non-technical discussion. This is evident from the fact that no technical requirements for visibility have been presented / identified or agreed upon to date. Furthermore, our concerns on RAN2 discussion on visibility discussion are:

- As one lesson learnt in 5G, RAN2 discussion on visibility requirement was quite inefficient and caused a lot of confusion to other WGs because RAN2 doesn’t have expertise for data visibility requirement.

In 5G AI/ML discussion, RAN2 discussion on visibility requirement of UE-side data collection was quite inefficient in Rel-18 and Rel-19. This discussion took quite a long time in RAN2 (almost one year), but no clear conclusion could be made. Finally, such discussion was finalized in RAN plenary. Furthermore, the visibility requirement caused a lot of confusions to both SA2 and SA5. We think the root cause of the inefficient discussion is because RAN2 is not the correct WG to discuss data visibility which has no technical requirement presented / identified.

- It is not clear what specific data is collected for 6G sensing and 6G AI/ML at this stage.

As agreed in RAN2#131b [2], RAN2 will not discuss sensing until April 2026. Then, we doubt how RAN2 can conclude its visibility requirement without any technique discussion on 6G sensing in RAN2. Our constructive suggestion is that RANP plenary should first involve.

Observation 1: RAN2 is not the correct WG to continue discussion on visibility requirement because:

- As one lesson learnt in 5G, RAN2 discussion on visibility requirement was quite inefficient and caused a lot of confusion to other WGs because visibility requirement is non-technical and out of RAN2 scope.

- It is not clear what specific data is collected for 6G sensing and 6G AI/ML at this stage.

Thus, we propose:

Proposal 2: As one lesson learnt in 5G, RAN2 leaves the followed discussion on visibility requirement for both AI/ML and non-AI/ML data collection to RAN plenary.

### 2.1.2: 6G AI/ML data collection framework

In this section, we focus on 6G data collection framework only for AI/ML use case. 5G studied two kinds of data collection framework: NW-side data collection and UE-side data collection. We provide a summary on their specification status in 5G in Table 1, which is the starting point of 6G study.

|  |  | Solution description | Consumer of transferred data | Agreed use cases in 3GPP | 5G specification status |
| --- | --- | --- | --- | --- | --- |
| NW-side data collection |  | - Logging L1/L3 measurements in multiple times in AS buffer and report in one RRC message - UERAN via RRC | RAN | - AI/ML based BM - AI/ML based CSI compression (Rel-20) - AI/ML mobility (Rel-20) | - Framework completed in Rel-19. - Rel-20 is expected to add new logged measurement quantities (e.g. L3 RSRP for AI mobility) |
| UE-side data collection | Solution 1a | UEUE-side server via 3GPP transparent tunnel (e.g. WiFi) | UE-side server | Expected for all use cases | No need of 3GPP specification work (already widely used in industry) |
|  | Solution 1b | UEUE-side server via 3GPP non-transparent tunnel | UE-side server | Not discussed yet | Precluded in RAN plenary |
|  | Solution 2 | UECNUE-side server via 3GPP specified tunnel. FFS CP or UP solution | UE-side server and CN | Not discussed yet | Pending on SA2. Solution details are not clear. |
|  | Solution 3 | UEOAMUE-side server via 3GPP specified tunnel. FFS CP or UP solution | UE-side server and OAM | Not discussed yet | Pending on SA5. Solution details are not clear. |

Table 1 Summary of AI/ML data collection specification work in 5G

#### 2.1.2.1: 6G NW-side data collection

According to Table 1, we think 5G NW-side data collection solution (i.e. measurement logging) is already a general data collection framework for NW-side model because it is applicable to most studied AI/ML use cases.

Observation 2: 5G NW-side data collection solution (i.e. measurement logging) is already a general data collection framework for NW-side model because it is applicable to most studied AI/ML use cases.

Furthermore, we think there is no requirement to unify NW-side data collection with UE-side data collection due to the following reasonings:

- Their data consumers are different:

  - NW-side data collection: RAN is the only consumer of the data. Thus, it doesn’t make sense to share the data with CN.

  - UE-side data collection: UE-side server, OAM or CN can be the consumer of the data. Thus, it can be considered as part of unified 6G data transfer framework.

- Their requirements for transfer of large data volume are different:

  - NW-side data collection: As discussed in Rel-19, it doesn’t have such requirement because NW can split data collection task among multiple UEs

  - UE-side data collection: It is a major challenge because one particular UE’s complete dataset needs to transfer to NW.

Observation 3: NW-side data collection is not required to unify with UE-side data collection because:

- RAN is the only consumer of the data, and therefore it doesn’t make sense to share the data with CN.

- It doesn’t need to transfer large data volume as NW can split collection task among multiple UEs.

Thus, we think 5G logging measurement framework can be the starting point of 6G NW-side data collection. And 6G can further study its enhancement (e.g. support data collection across all RRC states).

Proposal 3: On 6G NW-side data collection:

- Data transfer is terminated in RAN as 5G and it is not required to unify with UE-side data collection.

- Take 5G logging measurement framework as starting point of 6G and study its enhancement.

#### 2.1.2.2: 6G UE-side data collection

On UE-side data collection, we can see that SA2 and SA5 are still working on 5G solution, which may be reused in 6G. Meanwhile, UE-side data collection may be part of 6G unified “data transfer” framework. Thus, we think 6G UE-side data collection has potential overlapping with “data transfer” framework in 6G study and 5G UE-side data collection. The duplicated efforts should be avoided. Thus, RAN2 should postpone the discussion of 6G UE-side data collection after their conclusion is clear.

Proposal 4: Due to potential overlapping with 6G “data transfer” and 5G UE-side data collection, postpone the RAN2 discussion on 6G UE-side data collection after their conclusion is clear.

## 2.2 6G “Data Transfer”

In RAN2#131b [2], RAN2 has agreed that:

Agreements Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP)

In this section, we discuss the typical use cases listed above (“sensing, aI/ml, son/mdt, QoE”) for the following data transfer scenarios with different termination points

- “Data” transport between UE and RAN node

- “Data” transport between UE and CN

Note that there could be potential “UE(Producer)  UE (consumer)” data transfer scenarios for “UE-UE” sensing mode, which is one of the six sensing modes included for 6G study as endorsed by RAN plenary [4]. But since this kind of inter-UE transfer requires the support of Sidelink communication, which is not under 6G Day1 study scope, we have omitted this scenario here. Also, “RAN RAN” and “RAN-CN” data transfer scenarios are also not discussed because they are not in RAN2 scope.

### 2.2.1: UE-RAN data transfer

Here, we analyse all the data transfer scenarios for relevant use cases which need to be terminated in RAN.

AI/ML

For AI/ML, it is still unclear whether there is a convincing justification for RAN to consume “data”, especially if we differentiate “data” as something not needed for Uu operation (as compared to “measurement”), but only for training and performance improvement. For example, in Rel-19 AI/ML for air interface work, the CSI report is transferred from UE to gNB for “performance monitoring” purpose for UE-sided model, but those are reported as L1 signalling, which is part of the UE’s mandatory requirements of PHY procedures, not some extra “data” transfer. This is also true for NW-sided model, when L1 CSI report is used for “inference” and “performance monitoring”.

If the data collected are solely for the training purpose for UE-sided model, we believe there may not need to be terminated by RAN (as it is hard to imagine a technical reason to train models in each and every gNB as opposed to training in a centralized cloud facility as it is done today), it can be terminated by CN or an OTT server instead. For NW-side data collection, the UE reporting data to RAN can be used for the training of NW-side model(s) and for performance monitoring. Note that this data may overlap with “measurement” configured by NW for UE to collect for baseline operation, or be something additional. If 5G AI/ML baseline is followed, then we can consider include this as part of 6G RRC protocol function.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Payload | Delay | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| NW-side data collection for training | UERAN | Depends | Depends on what “data” to be consumed in RAN | High | 6G RRC | UECN |
| performance monitoring for NW-side model | UERAN | Depends | Small | Medium (near real-time) | 6G RRC |  |

Table 2-1 UE-RAN data transfer Scenario for AI/ML use case

Sensing

For Sensing use case, we think there are two large categories depending on the nature of how sensing data is consumed:

- Sensing-assisted communication: in this category, sensing data is mainly used for “communication optimization’ use cases, e.g., sensing assisted beam management, sensing assisted mobility. For those cases, we think the sensing data reporting is somewhat “urgent” because NW may utilize those sensing data to make “better” decisions of Uu operation (e.g., whether/when to handover). From this perspective, RAN as data consumer is justified. But we also admit that the RAN node should not rely on sensing data for its Uu operation as not all UEs will support ISAC. An additional point to consider is that if sensing measurements are used to assist for communications, then they are not so different from regular RRM measurements used for Uu operation.

- Sensing (non-communication purpose): in this category, sensing data is used for “object” detection and such a detection has nothing to do with communication. For those cases, usually, there is no urgent timing requirements for sensing data collection, and 6G may also consider letting the sensing data report to a CN Network Function (NF) instead. Depending on how this sensing data is eventually utilized, this can also be divided as “safety use case” and “non-safety” use case. In general, the sensing data transfer for safety use case is more time-sensitive than non-safety use cases.

Regarding the six sensing modes endorsed by RAN plenary#109 [4]: we only focused on “TRP-UE DL bistatic” and “UE-TRP DL bi-static” modes, because those two modes may involve some data transfer terminated at RAN. For those sensing data, the payload size may be very large if “raw” sensing data is transported but could be small if the “producer” is supposed to pre-process sensing data first and only send filtered/processed sensing results to the consumer. It is also worth noting that “UE-TRP UL bi-static“ case is the only “UE as data consumer” case identified so far.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Payload | Latency | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| Sensing-assisted communication (e.g., beam management) | UERAN | Depends | Small (processed) or Large (raw data) | Low | 6G RRC or new protocol | N/A |
| TRP-UE DL in bistatic sensing for non-communication purpose (object detection) | UERAN | Depends | Small (processed) or Large (raw data) | Medium (for Safety use case) High (for non-safety case) | 6G RRC or new protocol | UECN |
| UE-TRP UL in bistatic sensing for non-communication purpose (object detection) | RANUE | Depends | Small (processed) or Large (raw data) | Medium (for Safety use case) High (for non-safety case) | new protocol | RANCNUE |

Table 2-2 UE-RAN data transfer Scenario for Sensing use case

SON/MDT

For the SON/MDT use case, the RRC protocol has been used in prior 3GPP solutions. We think it is fine to re-consider whether this transfer should be considered as part of “6G data transfer” framework and whether a new protocol can be considered or not.

It is worth noting that not all SON/MDT features involve UEs, and data communication under RAN3 domain are not considered in this table. Also, for immediate MDT, we understand that this simply reuses RRC measurements. There is no need to have a new “data transfer” design to duplicate this part, which would be quite inefficient.

For logged MDT, the UE basically stores some statistics and measurements in OOC or IDLE/INACTIVE RRC state, and then report this when it enters RRC_CONNECTED state. Such reported data is not directly related to the serving gNB, but to be used for overall network optimization. Similarly, for SON features, RLF report and RA reports are also essentially in the same nature. We think those data transfer scenarios, although being legacy RRC procedure, should be fine to be replaced with a UECN data transfer instead in 6G, if needed. At any rate, with MDT the final consumer of the data is not RAN, but TCE – which is a CN/OAM network entity.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Size | Latency | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| Immediate MDT | UERAN | Event-triggered or periodic reporting | Small | Low | 6G RRC | N/A |
| Logged MDT | UERAN | reporting upon entering CONNECTED | Small | High | 6G RRC or new protocol | UECN |
| SON (include RLF report, RA report, etc) | UERAN | Reporting upon entering CONNECTED | Small | High | 6G RRC or new protocol | UECN |

Table 2-3 UE-RAN data transfer Scenario for SON/MDT use case

QoE

To optimize user experience in application layer. 5G NR has introduced RAN visible QoE to enable RAN to optimize radio resources based on application layer service quality. However, in 5G, the UE needs both those QoE measurements to RAN (visible to RAN) and in a container (not visible to RAN). Such a duplicated transmission in Uu interface may be avoided by exposing RAN-visible QoE by CN, after reported by UE to CN, as those reporting are not latency-critical.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Payload | Delay | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| RAN-Visible QoE | UERAN | Depends | Small | High | 6G RRC or New Protocol | UECNRAN |

Table 2-4 UE-RAN data transfer Scenario for QoE use case

Based on the above analysis of the use cases for UE-RAN terminated data transfer, we have a few observations.

The first observation is that the definition of “data” is not very clear so far. For example, in NW-side data collection, the gNB data collected in Rel-19 AI/ML could be L1 or L3 measurement reports provided by UE in Uu interface. Then, gNB will include those as “data” to further transfer them to some sort of “data collection” servers. However, the transfer of those “data” in Uu interface do not need any new “data transfer” solution besides the existing measurement framework. In other words, the usage of “data” and “measurements” somehow overlaps and may cause confusion.

Observation 4: “Data” in certain use cases (e.g., AI/ML, MDT) overlapped with Uu RRC “measurements”.

It is better to have a clear distinction of what belongs to “data” and what does not belong to “data” in 6G data transfer discussion between UE and RAN node. First, it is evident that “user traffic” is not “data” as they are not even supposed to be visible to any NW entity. Then, regarding all the RAN-visible information, we think any RRC configurations for command/control provided by NW are not “data”. More importantly, any L1/L2/L3 UE reporting which is critical to Uu operation are not “data”, but “measurement”. If all of those are excluded, then any remaining “information” to be shared between UE and RAN should be labelled as “data”.

Proposal 5 	The scope of “data” for data transfer terminated between UE and RAN excludes the following: a) UE’s report of measurements which are needed for Uu operation (e.g., scheduling/handover); b) RAN node’s configuration/command/control information to UE; c) user traffic and any other UE data which is not supposed to be visible to RAN node.

Another observation from the above analysis is that some of the “data” are more relevant to RAN node, but some are not. For the data which is more relevant, it usually also requires real-time collection and short delay. But for the data which is less relevant, the delay requirement is much more relaxed. It is also valid to raise the question about whether the latter “data transfer” case can be equivalently achieved by a “UECN” data transfer instead.

For example, in sensing use cases, as the sensing measurements reported by UE to assist communication is very useful for RAN node to optimize Uu scheduled decisions, then it is desirable to require UE to report them as quick as possible to RAN directly. However, for other sensing results for object detection purpose, the reporting can have more time budget because this data can be either processed in RAN node or by a sensing function in CN.

Observation 5: In many cases where UE->RAN data transfer (using RRC) is used today, the final consumer of the data is not RAN. Therefore, it is feasible (and indeed more efficient) to replace certain use cases of UERAN data transfer with UECN data transfer.

Then, regarding the data transfer protocol to be used for UE-RAN data transfer, it is clear that IP protocol is not suitable because the protocol stack between UE and 6G RAN node is highly unlikely to include IP layer. If we take 5G protocol stack as a baseline, the IP connection would still be established between UE and UPF. Therefore, a non-IP protocol is to be considered.

Observation 6: IP-based approach is not suitable for UE-RAN data transfer.

Then, regarding the exact protocol to be used to support data transfer, there could be two choices:

- 6G RRC protocol

- New 6G UE-RAN data transport protocol

In regards of how to down-select those two options, we think RAN2 need consider how many use cases in the above Table 2-1 to Table 2-4 will eventually employ “UE-RAN” data transfer scheme as its 6G solution. As we explained earlier, when compared to 5G, it is possible that only sensing-assisted communication use case for sensing seems to be a new potential use case which need to have data transfer implementation localized/constrained between UE and RAN node, but all other new data transfer requirements could be either use legacy 5G CP solution or can be fulfilled with alternative termination points (e.g., UE-CN). Literally speaking, if there is only such a single new scenario to be considered for 6G data transfer terminated at RAN node, maybe it is worth evaluating whether 6G RRC (as enhanced on top of 5G NR RRC) is sufficient to support this. Of course, there are some legitimate concerns that RRC-based approach may be not enough to support “bulky data” and do not have flexible QoS support. Those issues can be further studied by RAN2 once the data transfer scenarios and their respective requirements for RAN-terminated data are finalized.

Proposal 6 	RAN2 consider to choose either 6G RRC or new non-IP protocol to support UE-RAN data transfer, after taking into account the following factors: a) which use cases must rely on data transfer terminated within 6G RAN (i.e., unable to be replaced by UE-CN data transfer scheme); b) whether there are new requirements (e.g., segmentation, QoS) to justify the new protocol.

### 2.2.2: UE-CN data transfer

Similar to the last section, we analyse all the data transfer scenarios for relevant use cases which occur between UE and a network function in 6G CN (Core Network).

AI/ML

For the UE-side model training, several options have been discussed in Rel-19. For the 6G study, while we still believe that non-3GPP approach is enough to support them between a UE and OTT server, it is logically doable by an alternative UE-CN approach if the data needs to be visible to a NF in CN.

However, for model transfer, there is still no justification to make UE-side model visible to NW. We included this “hypothetical” case just for the purpose of comparison.

For NW-side model training, it is possible for UE to send relevant data directly to CN instead of letting RAN node to collect them. This is for the use case where AI/ML is used for optimizing the use case rely on upper-layer data collection (e. g., AI Positioning). It is included in table below so that RAN2 can check whether those potential data transfer scenario(s) can benefit from a unified protocol solution or not.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Size | Delay | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| Model transfer (UE-side model or UE part in two-side model) | CNUE | Infrequent (day/week/month) | Large | High | IP-based, NAS-like | OTT server UE |
| data collection for training for NW-side models based on upper-layer data (e.g, positioning data reporting to LMF) | UECN | Depends | Depends | High | IP-based, NAS-like | UERANCN |
| data collection for training for UE-side models | UECN | Depends | Depends | High | IP-based, NAS-like | UE OTT Server |

Table 3-1 UE-CN data transfer Scenario for AI/ML use case

Sensing

As discussed earlier, if Sensing Management Function (SeMF) is determined to be a new NF in 6GC, it makes sense to let the SeMF play a central role in sensing data transfer. This means for the bi-static sensing modes (including both TRP-UE DL and UE-TRP UL), it makes sense for both UE and TRP (RAN node) to report the sensing related information to the sensing function first, and then let sensing function to process those data and then determine whether there is further need to let UE and RAN node to further consume some sensing data or sensing results for the relevant use case.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Size | Delay | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| TRP-UE DL in bistatic sensing for non-communication purpose (object detection) | UECN | Depends | Small (processed) or Large (raw data) | Medium (for Safety use case) High (for non-safety case) | IP-based, NAS-like | UERAN |
| UE-TRP UL in bistatic sensing for non-communication purpose (object detection) | CNUE | Depends | Small (processed) or Large (raw data) | Medium (for Safety use case) High (for non-safety case) | IP-based, NAS-like | RANUE |

Table 3-2 UE-CN data transfer Scenario for Sensing use case

SON/MDT

For SON/MDT, there is no UE-CN data transfer design in legacy 4G/5G. Nonetheless, it is not completely unreasonable to assume 6G provides a new opportunity to consider whether those RRC-based data transfer can be shifted to UE-CN terminated data transfer because there is neither strong latency requirements nor need for RAN node to consume those data “immediately”.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Size | Latency | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | UERAN | Upon entering CONNECED | Small | High | IP-based, NAS-like | UECN |
| SON (include RLF report, RA report, etc) | UERAN | Upon entering CONNECED | Small | High | IP-based, NAS-like | UECN |

Table 3-3 UE-CN data transfer Scenario for SON/MDT use case

QoE

The “QoE container” include in UE’s MeasurementReportAppLayer message (as part of 5G NR RRC protocol) is not visible to RAN node. Logically, this can be substituted with a UE-CN data transfer instead.

| Data Transfer Scenarios | Direction (producerconsumer) | Reporting Periodicity | Payload | Delay | Protocol candidates | Alternate termination point? |
| --- | --- | --- | --- | --- | --- | --- |
| QoE container data | UECN | Depends | Small or medium | High | IP-based or NAS-like | UERANCN |

Table 3-4 UE-RAN data transfer Scenario for QoE use case

Regarding the exact data transport protocol for “UE-CN” termination scenarios, we think this largely has two directions:

- Option 1: IP-based protocol approach, whether the “data” is routed by UPF to the final data function in CN.

- Option 2: NAS-like non-IP approach, where the UE has sent the data to CN via an interface between RAN node and a “DMF (Data Management function)” in CN directly. This would be similar to the NAS protocol between UE and AMF

At the current stage, we think RAN2 cannot down-select between the above two options, as how UE-CN data transfer is to be done also depends on the progress of SA2 Study on “Unified Data Framework” [5]. Therefore, RAN2 can only study the necessary AS layer support for both data transfer options and make some preliminary evaluations.

Proposal 7 	As a start point for data transfer between UE and CN, RAN2 study and evaluate necessary AS layer support for both IP-based and non-IP based (e. g., NAS-like) approaches, while any down-selection requires coordination with SA2 WG.

# 3 Conclusion

In this contribution, we share our views on 6G data collection and data transfer. Our observation and proposals are summarized in one page as follows:

Requirement of 6G data collection

Proposal 1: RAN2 understand that the “standardized data” in 6G AI/ML data collection means the data whose format/content and measurement requirement are explicitly defined in 3GPP specifications.

Observation 1: RAN2 is not the correct WG to continue discussion on visibility requirement because:

- As one lesson learnt in 5G, RAN2 discussion on visibility requirement was quite inefficient and caused a lot of confusion to other WGs because visibility requirement is non-technical and out of RAN2 scope.

- It is not clear what specific data is collected for 6G sensing and 6G AI/ML at this stage.

Proposal 2: As one lesson learnt in 5G, RAN2 leaves the followed discussion on visibility requirement for both AI/ML and non-AI/ML data collection to RAN plenary.

6G AI/ML data collection framework

Observation 2: 5G NW-side data collection solution (i.e. measurement logging) is already a general data collection framework for NW-side model because it is applicable to most studied AI/ML use cases.

Observation 3: NW-side data collection is not required to unify with UE-side data collection because:

- RAN is the only consumer of the data, and therefore it doesn’t make sense to share the data with CN.

- It doesn’t need to transfer large data volume as NW can split collection task among multiple UEs.

Proposal 3: On 6G NW-side data collection:

- Data transfer is terminated in RAN as 5G and it is not required to unify with UE-side data collection.

- Take 5G logging measurement framework as starting point of 6G and study its enhancement.

Proposal 4: Due to potential overlapping with 6G “data transfer” and 5G UE-side data collection, postpone the RAN2 discussion on 6G UE-side data collection after their conclusion is clear.

6G data transfer

Observation 4: “Data” in certain use cases (e.g., AI/ML, MDT) overlapped with Uu RRC “measurements”.

Proposal 5 	The scope of “data” for data transfer terminated between UE and RAN excludes the following: a) UE’s report of measurements which is critical to Uu operation (e.g., scheduling/handover); b) RAN node’s configuration/command/control information to UE; c) user traffic and any other UE data which is not supposed to be visible to RAN node.

Observation 5: In many cases where UE->RAN data transfer (using RRC) is used today, the final consumer of the data is not RAN. Therefore, it is feasible (and indeed more efficient) to replace certain use cases of UERAN data transfer with UECN data transfer.

Observation 6: IP-based approach is not suitable for UE-RAN data transfer.

Proposal 6 	RAN2 consider to choose either 6G RRC or new non-IP protocol to support UE-RAN data transfer, after taking into account the following factors: a) which use cases must rely on data transfer terminated within 6G RAN (i.e., unable to be replaced by UE-CN data transfer scheme); b) whether there are new requirements (e.g., segmentation, QoS) to justify the new protocol.

Proposal 7 	As a start point for data transfer between UE and CN, RAN2 study and evaluate necessary AS layer support for both IP-based and non-IP based (e. g., NAS-like) approaches, while any down-selection requires coordination with SA2 WG.

# 4 References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO.

[2] RAN2#131b, Chair Notes.

[3] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[4] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC

[5] SP-250488 Study on Architecture for 6G System