# 3GPP TSG RAN WG2 Meeting #132      	                           R2-2508456
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Remaining issues on Rel-19 NES

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 NES, and some essential corrections were discussed in RAN2#131b [2]. However, there are still three remaining issues. In this contribution, we discuss the following three identified open issues on Rel-19 NES:

- Issue 1: Whether/How to introduce OD-SSB specific ssb-ToMeasure

- Issue 2: Whether/How to support SSB adaptation for deactivated SCell

- Issue 3: Correction of 38.300

# 2 Discussion

## 2.1 Issue 1: Whether/How to introduce OD-SSB specific ssb-ToMeasure

This issue is described as follows:

- RRC issue 5: [C184] The needs of different ssb-ToMeasure configurations for OD-SSB R2-2506817 [C184]Impact of od-ssb-PositionsInBurst on ssb-ToMeasure CATT discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 1: A clarification that OD-SSB-PositionsInBurst is configured as a subset of ssb-ToMeasure within the associated MeasObjectNR should be added to the field description of the OD-SSB-PositionsInBurst. R2-2507334 Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 Ericsson discussion Rel-19 Netw_Energy_NR_enh-Core Late Proposal 6: RAN2 to agree that OD-SSB specific MO includes values for ssb-ToMeasure to match the OD-SSB activated by the MAC CE. - Apple and vivo think this is sufficient Discussion on the impact of od-ssb-PositionsInBurst on ssb-ToMeasure is postponed to next meeting

In legacy RRC specification, the IE ssb-ToMeasure is configured in SSB-ConfigMobility per MO. It is used to indicate the UE which beam(s) to perform L3 measurements (including both serving cell and neighbor cell L3 measurements) in SMTC window.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                OPTIONAL,   -- Cond SSBorAssociatedSSB2

ssbSubcarrierSpacing                SubcarrierSpacing                          OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                          OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                         OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

===================omit unrelated part=============

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

===================omit unrelated part=============

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }     OPTIONAL    -- Need M

]]

}

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. When the UE is within an altitude range indicated by altitudeRange, it ignores the ssb-ToMeasure (without suffix), and applies the corresponding ssb-ToMeasure-r18 if present, otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

Observation 1: In legacy RRC specification, the IE ssb-ToMeasure is configured in SSB-ConfigMobility per MO. It is used to indicate the UE which beam(s) to perform L3 measurements (including both serving cell and neighbor cell L3 measurements) in SMTC window.

For OD-SSB, RAN2 agreed to use OD-SSB specific SMTC for serving cell measurement and legacy SMTC for neighbor cell measurements [2]:

Agreements:

5.	[L201]: To align with R4 conclusion, R2 confirms the adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

6.	[L202]: To align with R4 conclusion, R2 confirms the adaptive SMTC usage for SSB adaptation does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy (can check if any further changes are needed or if this is already covered by the latest RRC CR)

Meanwhile, current NES RRC running CR has included the beam bitmap (i.e. od-ssb-PositionsInBurst-r19) in OD-SSB-Config-r19, which can be adapted via OD-SSB MAC-CE.

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19           OD-SSB-ConfigId-r19,

od-ssb-SFN-Offset-r19         INTEGER (0..15)                                               OPTIONAL, -- Need R

od-ssb-halfFrameIndex-r19     ENUMERATED {zero, one}                                      OPTIONAL, -- Need R

od-ssb-ActivationStatus-r19   ENUMERATED {activated}                                      OPTIONAL, -- Need S

od-ssb-Periodicity-r19        ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

od-ssb-PositionsInBurst-r19   CHOICE {

shortBitmap                   BIT STRING (SIZE (4)),

mediumBitmap                  BIT STRING (SIZE (8)),

longBitmap                    BIT STRING (SIZE (64))

}                                                                                        OPTIONAL, -- Cond ODssbAOssb

od-ssb-NrofBursts-r19         ENUMERATED {n5, n10, n15, n20, n25, n30, n40, n50, n75, n100, n150, n200} OPTIONAL, -- Cond MACCE

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

It means that the NW may dynamically switch to different set of SSB beams via OD-SSB MAC-CE.

Observation 2: According to NES RRC running CR, the beam bitmap of OD-SSB (i.e. od-ssb-PositionsInBurst-r19) is included in OD-SSB-Config-r19, which means that different sets of SSB beams can be dynamically adapted via OD-SSB MAC-CE.

Based on above analysis, after switching to different OD-SSB beam bitmap (od-ssb-PositionsInBurst-r19), NW may need to indicate the UE to measure different set of SSB beams. Thus, it makes sense to introduce OD-SSB specific ssb-ToMeasure. One may argue that od-ssb-PositionsInBurst-r19 can be reused for such purpose (so, OD-SSB specific ssb-ToMeasure is not needed). However, we think it is not correct. As illustrated in Figure.1, it is possible that the NW may transmit 8 beams for OD-SSB but only the first 4 (wide) beams are used for L3 measurement while the last 4 (narrow) beams are L1 measurement. Thus, introducing OD-SSB specific ssb-ToMeasure is necessary.

Figure.1 Illustration of the necessity of introducing OD-SSB specific ssb-ToMeasure

Observation 3: It makes sense to introduce OD-SSB specific ssb-ToMeasure because NW may need to indicate the UE to perform L3 measurement on different set of SSB beams after adapting OD-SSB beam bitmap (od-ssb-PositionsInBurst-r19). od-ssb-PositionsInBurst-r19 can’t be reused for such purpose.

Furthermore, as both OD-SSB beam bitmap and OD-SSB SMTC are configured per OD-SSB-Config-r19, OD-SSB specific ssb-ToMeasure should be also configured per OD-SSB-Config-r19, i.e. one OD-SSB-Config-r19 is 1:1 mapped to one pair of OD-SSB specific SMCT and ssb-ToMeasure. It can be implemented by introducing a new list of odssb_ToMeasure with 16 elements in MO. For example:

ODSSB-ToMeasureList-r19::=                 SEQUENCE (SIZE(1..16)) OF SSB-ToMeasure

Proposal 1: To allow NW to indicate the UE to measure different set of SSB beams after adapting OD-SSB beam bitmap, introduce the following new list of odssb_ToMeasure with 16 elements in MeasObjectNR (i.e. 1:1 mapping between one OD-SSB-Config-r19 and one pair of OD-SSB specific SMCT and ssb-ToMeasure):

ODSSB-ToMeasureList-r19::=                 SEQUENCE (SIZE(1..16)) OF SSB-ToMeasure

One clarification is that above new OD-SSB specific ssb-ToMeasure is only used for serving cell measurement of OD-SSB, same as OD-SSB specific SMTC. The UE still uses legacy ssb-ToMeasure for neighbor cell measurements.

Proposal 2: Same as OD-SSB specific SMTC, OD-SSB specific ssb-ToMeasure is only used for serving cell measurement on OD-SSB and legacy ssb-ToMeasure is used for neighbor cell measurements.

There was one proposal to clarify OD-SSB-PositionsInBurst is configured as a subset of ssb-ToMeasure. However, as illustrated in Figure.1, OD-SSB specific ssb-ToMeasure may be a subset of OD-SSB-PositionsInBurst. Furthermore, we don’t have such restriction for legacy ssb-ToMeasure. Thus, we propose to leave it to NW implementation as legacy.

Proposal 3: It is up to NW implementation how to configure OD-SSB specific ssb-ToMeasure and OD-SSB-PositionsInBurst, i.e. no need of restriction that OD-SSB-PositionsInBurst is a subset of ssb-ToMeasure.

Finally, one may intend to extend the OD-SSB specific ssb-ToMeasure to SSB adaptation due to their similarity. However, we understand that RAN1 didn’t agree to adapt ssb-PositionsInBurst in Rel-19 NES. It is also reflected in NES RRC running CR that ssb-PositionsInBurst is not included in Adapt-SSB-Config-r19.

Adapt-SSB-Config-r19 ::=                            SEQUENCE {

ssb-BurstPeriodicityList-r19   SEQUENCE (SIZE (1..2)) OF Adapt-SSB-BurstPeriodicity-r19  OPTIONAL,     -- Need N

posInDCI-ssbPeriodicityIndicationForScell-r19 INTEGER (1..maxDCI-2-9-Size-r18)            OPTIONAL      -- Need N

}

Adapt-SSB-BurstPeriodicity-r19 ::=  SEQUENCE {

ssb-Periodicity-r19           ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }  OPTIONAL,  -- Need S

ssb-Offset-r19                INTEGER (0..15)                                   OPTIONAL,  -- Need N

ssb-halfFrameIndex-r19        ENUMERATED { zero, one }                       OPTIONAL   -- Need N

}

Observation 4: RAN1 didn’t agree to adapt ssb-PositionsInBurst in Rel-19 NES. It is also reflected in NES RRC running CR that ssb-PositionsInBurst is not included in Adapt-SSB-Config-r19.

Thus, we propose:

Proposal 4: As RAN1 didn’t agree to adapt ssb-PositionsInBurst in Rel-19 NES, RAN2 confirm not to extend OD-SSB specific ssb-ToMeasure to SSB adaptation.

## 2.2 Issue 2: Whether/How to support SSB adaptation for deactivated SCell

This issue can be described as follows:

Proposal 5 [H130] is not pursued. [H130] is Rejected - RRC rapporteur suggests to continue discussion on whether there is a need for clarification in the meeting minutes and if there is a need to activate adaptation via RRC configuration. - HW understands that if this RIL is rejected we also assume that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells Can come back in the next meeting to check whether we can confirm the understanding that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells

First, please note that RAN4 has discussed this issue and agreed to not introduce requirements for DCI based SSB adaptation of deactivated SCell in RAN4#116 [3]:

Agreement (on Thursday):

RAN4 not to define requirement for DCI based SSB adaption for deactivated SCell measurement and SCell activation.

- Note: this RAN4 agreement does not intend to impact the discussion and decision in other WGs.

Observation 5: RAN4 has discussed whether to support SSB adaptation in deactivated SCell and agreed to not define requirements for DCI based SSB adaptation of deactivated SCell in RAN4#116.

From RAN2 perspective, we think the understanding “a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells” is conflicted with the highlighted part in TS 38.321 [4]:

TS 38.321 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

Our understanding is that the highlighted part means that the UE doesn’t apply the DCI for deactivated SCell, even if the UE can receive the DCI from another serving cell (e.g. cross-carrier DCI indication from PCell). We propose that RAN2 should confirm this understanding.

Proposal 5: On “not monitor the PDCCH for the SCell if the SCell is deactivated” from TS 38.321, RAN2 confirm that it means that the UE doesn’t apply the DCI for deactivated SCell, even if the UE can receive the DCI from another serving cell (e.g. cross-carrier DCI indication from PCell).

If RAN2 can agree Proposal 5, it means that DCI based SSB adaptation is not applicable to deactivated SCell. We think it makes sense, especially at the late stage of Rel-19. Otherwise, RAN2 and RAN4 will need further specification work. For example, RAN2 may need to discuss whether to introduce two new separate measCycleSCell for SSB adaptation in deactivated SCell (i.e. measCycleSCell is adapted to the changed SSB periodicity via DCI).

Observation 6: If DCI based SSB adaptation can be applicable to deactivated SCell, RAN2 and RAN4 will need further specification work. For example, RAN2 may need to discuss whether to introduce two new separate measCycleSCell to adapt to SSB adaptation in deactivated SCell (i.e. measCycleSCell is adapted to the changed SSB periodicity via DCI).

Based on above analysis, we propose that RAN2 conclude that DCI based SSB adaptation is not applicable to deactivated SCell. It can be captured in TS 38.300 and no need to introduce RRC restriction proposed in H130. Meanwhile, since RAN4 has discussed this issue, RAN2 conclusion should be sent to RAN4 and cc RAN1.

Proposal 6: RAN2 conclude that DCI based SSB adaptation is not applicable to deactivated SCell:

- Capture the RAN2 conclusion in TS 38.300 and no need to introduce RRC restriction proposed in H130.

- Send the RAN2 conclusion to inform RAN4 and cc RAN1.

Finally, the last remaining issue is whether to introduce direct SSB adaptation activation with RRC configuration. We think that the enhancement is not necessary because legacy direct SCell activation can achieve the same intention. In details, the legacy direct SCell activation can already provide SSB periodicity in the same RRC message for SCell activation (i.e. NW includes the required ssb-periodicityServingCell when sending RRCReconfiguration with sCellState-r16).

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                        OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                          OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                       OPTIONAL,   -- Cond SCellAddSync

===================omit unrelated part=============

}

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                             OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                      OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                     OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                  OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                        OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

}

===================omit unrelated part=============

Observation 7: It is not necessary to introduce direct SSB adaptation activation with RRC configuration because legacy direct SCell activation can achieve the same intention (i.e. NW includes the required ssb-periodicityServingCell when sending RRCReconfiguration with sCellState-r16).

Thus, we propose RAN2 to confirm the understanding.

Proposal 7: RAN2 conclude not to support direct SSB adaptation activation with RRC configuration because legacy direct SCell activation can achieve the same intention (i.e. NW includes the required ssb-periodicityServingCell when sending RRCReconfiguration with sCellState-r16).

## 2.3 Issue 3: Correction of 38.300

In latest Running CR of TS 38.300 [5], the following text on intra-frequency and inter-frequency measurements with OD-SSB is captured:

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. - SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the SSB frequency configured in the measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different. NOTE 2: For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells. NOTE 2a: If a UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above. NOTE 2b: The above measurement object associated with the serving cell refers to the serving cell measurement object for OD-SSB when OD-SSB is activated, otherwise it refers to the serving cell measurement object for SSB.

We think the NOTE 2b is not aligned with RAN2#131b agreement (i.e. when OD-SSB and AO-SSB are in same frequency, legacy MO is applied):

19.	RAN2 confirms that legacy servingCellMO is used for deriving serving cell measurement results in the following cases:

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured.

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

- if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and od-ssb transmission is not activated

Thus, we suggest to revise NOTE 2b to correctly align with the agreement.

Proposal 8: To align with the agreement that legacy MO is applied when OD-SSB and AO-SSB are in same frequency, make the following correction on NOTE 2b of Running CR of TS 38.300:

NOTE 2b:	The above measurement object associated with the serving cell refers to the serving cell measurement object for OD-SSB when SSB is absent and OD-SSB is activated, or when SSB and OD-SSB have different frequency. Ootherwise, it refers to the serving cell measurement object for SSB.

# 3 Conclusion

In this contribution, we discuss remaining issues on Rel-19 NES. Our observations are:

Observation 1: In legacy RRC specification, the IE ssb-ToMeasure is configured in SSB-ConfigMobility per MO. It is used to indicate the UE which beam(s) to perform L3 measurements (including both serving cell and neighbor cell L3 measurements) in SMTC window.

Observation 2: According to NES RRC running CR, the beam bitmap of OD-SSB (i.e. od-ssb-PositionsInBurst-r19) is included in OD-SSB-Config-r19, which means that different sets of SSB beams can be dynamically adapted via OD-SSB MAC-CE.

Observation 3: It makes sense to introduce OD-SSB specific ssb-ToMeasure because NW may need to indicate the UE to perform L3 measurement on different set of SSB beams after adapting OD-SSB beam bitmap (od-ssb-PositionsInBurst-r19). od-ssb-PositionsInBurst-r19 can’t be reused for such purpose.

Observation 4: RAN1 didn’t agree to adapt ssb-PositionsInBurst in Rel-19 NES. It is also reflected in NES RRC running CR that ssb-PositionsInBurst is not included in Adapt-SSB-Config-r19.

Observation 5: RAN4 has discussed whether to support SSB adaptation in deactivated SCell and agreed to not define requirements for DCI based SSB adaptation of deactivated SCell in RAN4#116.

Observation 6: If DCI based SSB adaptation can be applicable to deactivated SCell, RAN2 and RAN4 will need further specification work. For example, RAN2 may need to discuss whether to introduce two new separate measCycleSCell to adapt to SSB adaptation in deactivated SCell (i.e. measCycleSCell is adapted to the changed SSB periodicity via DCI).

Observation 7: It is not necessary to introduce direct SSB adaptation activation with RRC configuration because legacy direct SCell activation can achieve the same intention (i.e. NW includes the required ssb-periodicityServingCell when sending RRCReconfiguration with sCellState-r16).

Based on above observations, our proposals are:

Issue 1: Whether/How to introduce OD-SSB specific ssb-ToMeasure

Proposal 1: To allow NW to indicate the UE to measure different set of SSB beams after adapting OD-SSB beam bitmap, introduce the following new list of odssb_ToMeasure with 16 elements in MeasObjectNR (i.e. 1:1 mapping between one OD-SSB-Config-r19 and one pair of OD-SSB specific SMCT and ssb-ToMeasure):

ODSSB-ToMeasureList-r19::=                 SEQUENCE (SIZE(1..16)) OF SSB-ToMeasure

Proposal 2: Same as OD-SSB specific SMTC, OD-SSB specific ssb-ToMeasure is only used for serving cell measurement on OD-SSB and legacy ssb-ToMeasure is used for neighbor cell measurements.

Proposal 3: It is up to NW implementation how to configure OD-SSB specific ssb-ToMeasure and OD-SSB-PositionsInBurst, i.e. no need of restriction that OD-SSB-PositionsInBurst is a subset of ssb-ToMeasure.

Proposal 4: As RAN1 didn’t agree to adapt ssb-PositionsInBurst in Rel-19 NES, RAN2 confirm not to extend OD-SSB specific ssb-ToMeasure to SSB adaptation.

Issue 2: Whether/How to support SSB adaptation for deactivated SCell

Proposal 5: On “not monitor the PDCCH for the SCell if the SCell is deactivated” from TS 38.321, RAN2 confirm that it means that the UE doesn’t apply the DCI for deactivated SCell, even if the UE can receive the DCI from another serving cell (e.g. cross-carrier DCI indication from PCell).

Proposal 6: RAN2 conclude that DCI based SSB adaptation is not applicable to deactivated SCell:

- Capture the RAN2 conclusion in TS 38.300 and no need to introduce RRC restriction proposed in H130.

- Send the RAN2 conclusion to inform RAN4 and cc RAN1.

Proposal 7: RAN2 conclude not to support direct SSB adaptation activation with RRC configuration because legacy direct SCell activation can achieve the same intention (i.e. NW includes the required ssb-periodicityServingCell when sending RRCReconfiguration with sCellState-r16).

Issue 3: Correction of TS 38.300.

Proposal 8: To align with the agreement that legacy MO is applied when OD-SSB and AO-SSB are in same frequency, make the following correction on NOTE 2b of Running CR of TS 38.300:

NOTE 2b:	The above measurement object associated with the serving cell refers to the serving cell measurement object for OD-SSB when SSB is absent and OD-SSB is activated, or when SSB and OD-SSB have different frequency. Ootherwise, it refers to the serving cell measurement object for SSB.

# 4 References

[1] RAN2#131, Chair Notes.

[2] RAN2#131b, Chair Notes.

[3] RAN4#116, Chair Notes.

[4] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[5] R2-2507273, Network Energy Savings Enhancements miscellaneous stage-2 corrections, Huawei, HiSilicon.