# 3GPP TSG RAN WG2 Meeting #132      	                        R2-2508458
Dallas, US, Nov. 17 – 21, 2025

Agenda item:	8.1.3

Source:	Apple

Title:	Remaining issues on NW-side data collection (including Z007/ S052/C074)

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 AI/ML for air interface. And in RAN2#131b [2], some essential corrections were discussed. In this contribution, we discuss the following remaining issues related to NW-side data collection:

- Discussion on important RILs

  - Z007: Whether to further capture restriction of multiplexing SON/MDT report and AIML logged data

  - S052: Quantity configuration for Event triggered Logging of CSI measurements

  - C074: Duplicated release/discard UE behaviour

- Stage 2 issue: whether supporting SN configured RAN centric data collection

# 2 Discussion

## 2.1 Discussion on important RILs

### 2.1.1 Z007: Whether to further capture restriction of multiplexing SON/MDT report and AIML logged data

This RIL is described as follows:

[Description]: According to RAN2 agreements achieved in RAN2#131bis meeting - Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message. Up to the network to ensure that data is not requested at the same time But there is nowhere to capture this agreement in the RRC spec so that the data from SON/MDT and NW side data collection can be retrieved simultaneously from specification perspective In our understanding, it shall be captured in the field description of CSI-LogMeasReportConfig. [Proposed Change]: csi-LogMeasReportReq This field is used to indicate whether the UE shall report information about CSI radio measurements logged in RRC_CONNECTED for network-side data collection. This information element shall be absent if the logMeasReportReq is present.

CR Rapporteur provided the following response:

[Comments]: [WI CR rapporteur-v022]: We suggest to discuss this issue in a Tdoc. There were discussions in the past on whether there should be clear restrictions on the NW, by adding them in the field descriptions but some companies were not in favour of it. In the current specification, not allowing multiplexing is captured in the procedural text from clause 5.7.10.3 below, by using “else if”, which excludes the case of having both SON/MDT reports and AIML reports in the same UEInformationResponse message. It can be discussed further if this procedural text is sufficient or not to capture the RAN2 agreement. 1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else if csi-LogMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRBX; 2> discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: 2> submit the UEInformationResponse message to lower layers for transmission via SRB1.

And in RAN2#131b [2], this issue was discussed but postponed:

Agreements

10	[Z007] RIL Z007 to be resolved after RAN2#131bis.

As far as we know, there are two reasons why it was postponed:

- It is not sufficient to just restrict multiplexing NW-side collected data and SON/MDT data because UEInformationResponse may also use SRB1 to transmit other data (e.g. EMR).

- There was no consensus whether to capture the restriction in a NOTE of Section 5.7.10.3 or field description of CSI-LogMeasReportConfig.

Observation 1: Z007 was postponed due to the following 2 reasons:

- It is not sufficient to just restrict multiplexing NW-side collected data and SON/MDT data because UEInformationResponse may also use SRB1 to transmit other data (e.g. EMR).

- There was no consensus whether to capture the restriction in a NOTE of Section 5.7.10.3 or field description of CSI-LogMeasReportConfig.

For 1), we confirm the issue. According to the ASN.1 of UEInformationRequest, it includes the following different kinds of reporting:

- EMR (Early Measurement Reporting) related reporting via SRB1: as highlighted below.

- SON/MDT related reporting via SRB2: as highlighted below.

- Other reporting via SRB1: coarseLocation for NTN and flightPathInfoReq for UAV, as highlighted below.

UEInformationRequest-r16 ::=     SEQUENCE {

rrc-TransactionIdentifier        RRC-TransactionIdentifier,

criticalExtensions               CHOICE {

ueInformationRequest-r16         UEInformationRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

UEInformationRequest-r16-IEs ::= SEQUENCE {

idleModeMeasurementReq-r16       ENUMERATED{true}                         OPTIONAL, -- Need N

logMeasReportReq-r16             ENUMERATED {true}                        OPTIONAL, -- Need N

connEstFailReportReq-r16         ENUMERATED {true}                        OPTIONAL, -- Need N

ra-ReportReq-r16                 ENUMERATED {true}                        OPTIONAL, -- Need N

rlf-ReportReq-r16                ENUMERATED {true}                        OPTIONAL, -- Need N

mobilityHistoryReportReq-r16       ENUMERATED {true}                        OPTIONAL, -- Need N

lateNonCriticalExtension         OCTET STRING                             OPTIONAL,

nonCriticalExtension             UEInformationRequest-v1700-IEs           OPTIONAL

}

UEInformationRequest-v1700-IEs ::= SEQUENCE {

successHO-ReportReq-r17          ENUMERATED {true}                        OPTIONAL, -- Need N

coarseLocationRequest-r17        ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             UEInformationRequest-v1800-IEs           OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

flightPathInfoReq-r18            FlightPathInfoReportConfig-r18           OPTIONAL, -- Need N

successPSCell-ReportReq-r18      ENUMERATED {true}                        OPTIONAL, -- Need N

reselectionMeasurementReq-r18    ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             SEQUENCE {}                              OPTIONAL

}

Observation 2: UEInformationResponse can include the following different kinds of reporting:

- EMR (Early Measurement Reporting) related reporting via SRB1

- SON/MDT related reporting via SRB2

- Other reporting via SRB1: coarseLocation for NTN and flightPathInfoReq for UAV

As UEInformationRequest already includes separate indications for the different types of reporting, we think NW can ensure csi-LogMeasReportReq and other request indication(s) of reporting are not included in the same UEInformationRequest message.

Proposal 1 (RIL Z007): As UEInformationRequest already includes separate indications for the different types of reporting, NW can ensure csi-LogMeasReportReq are not included with any other request indication(s) in the same UEInformationRequest message.

For 2), we think both ways can work, but a NOTE of Section 5.7.10.3 is preferred because it looks more like a guidance for NW implementation rather than a mandatory NW behaviour. Furthermore, we noticed that the following NOTE has already been captured in Section 5.7.10.3:

NOTE: It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested in the same message.

Then, we think the simplest solution is to revise the NOTE to reflect Proposal 1. Thus, we propose:

Proposal 2 (RIL Z007): Revise the existing NOTE in procedure text of Section 5.7.10.3 to reflect Proposal 1 (TP in Appendix 1).

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested included with any other request indication(s) in the same UEInformationResponse message.

We also provide a TP in Appendix 1.

### 2.1.2 S052: Quantity configuration for Event triggered Logging of CSI measurements

The issue is:

[Description]: Eevent triggered CSI measurement logging can be performed based on L3 events. It is agreed to use CSI framework and not measurement framework and the event is captured as below in RRC CR. CSI-LoggedMeasurementEventTriggerConfig-r19 ::= SEQUENCE { threshold-r19 CHOICE { aboveThreshold-r19 MeasTriggerQuantity, belowThreshold-r19 MeasTriggerQuantity }, hysteresis Hysteresis, timeToTrigger TimeToTrigger, ... } Now it is not clear how does the UE get the quantity configuration for the L3 filtering for deriving L3 measurements for the event evaluation for logging CSI measurements.

Our understanding is that the logging events reuse the same event evaluation approach as measurement event A1/A2, i.e. the MO indicated by IE ServingCellMO is applied for L3 serving cell measurement. And please note that quantity configuration (e.g. L3 filter coefficient) is part of MO configuration.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                     OPTIONAL,   -- Cond SSBorAssociatedSSB2

ssbSubcarrierSpacing                SubcarrierSpacing                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                        OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                               OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

=====omit related part====

}

Thus, we think it is sufficient to capture the clarification on which MO to apply in the field description of CSI-LoggedMeasurementEventTriggerConfig-r19. For example:

csi-LoggedMeasurementEventTriggerConfig

This field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception. The L3 serving cell measurements performed on the measurement object indicated by servingCellMO is used to evaluate the entering and leaving condition.

Thus, we propose:

Proposal 3 (RIL S052): Same as L3 measurement event A1/A2, RAN2 confirm that the event evaluation of logging events uses L3 serving cell measurement via the MO indicated by servingCellMO. Capture the following clarification in the field description of CSI-LoggedMeasurementEventTriggerConfig-r19.

csi-LoggedMeasurementEventTriggerConfig

This field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception. The L3 serving cell measurements performed via the measurement object indicated by servingCellMO is used to evaluate the entering and leaving condition.

### 2.1.3 C074: Duplicate release/discard UE behaviour

The issue is:

[Description]: in case of MCG RLF, UE will initiate RRC re-establishment procedure, and the CSI logged measurement configuration will also be released due to “2> release spCellConfig, if configured;”, as it’s a part of spCellConfig. For the similar reason, it’s also unnecessary to add duplicate description of “release loggedDataCollectionAssistanceConfig,” and “discard the logged measurement entries”, since the same contents have been added in RRC re-establishment procedure. [Proposed Change]: update the procedural text as below: 3> else: 4> consider radio link failure to be detected for the MCG, i.e. MCG RLF; 4> discard any segments of segmented RRC messages stored according to 5.7.6.3;

We agree with this issue and proposed change because the concerned release behaviors are already covered in other sections of TS 38.331 (i.e. RRC re-establishment procedure, HO and RRC release). Thus, we propose to agree C074.

Proposal 4 (RIL C074): Suggest to PropAgree C074, i.e. remove the following duplicated UE behavior on releasing CSI logged measurement configuration:

4>	release CSI-LoggedMeasurementConfig, if configured;

4>	release loggedDataCollectionAssistanceConfig, if configured;

4>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

## 2.2 Stage 2 issue: whether supporting SN configured RAN centric data collection

In post-meeting email discussion on 38.300 running CR [3], there was discussion on whether to capture “For NR-DC, the configuration and reporting of NW sided data collection can only be via MN”, according to the following agreements made in RAN2#129b [4] and RAN2#130 [5].

For NR-DC, the configuration and reporting of NW sided data collection can only be via MN

RAN2 confirms that previous agreement excluded SN being configured for OAM centric data collection

However, there was different understandings that above agreements are only for OAM centric data collection. It is still not clear whether supporting SN configured RAN centric data collection.

We agree that the above agreements are intended for only OAM centric data collection because the discussion was related LS to SA5 and TS 37.320 spec impacts. However, we think it is better for RAN2 to make a conclusion on RAN centric data collection. In our view, it is problematic to support SN configured data collection:

- Technically, spec impact on inter-node message is needed to negotiate UE's L1 measurement logging capability between MN and SN. Otherwise, the total configured CSI-RS resource set number may be beyond the UE’s capability if MN and SN independently configures NW-side data collection for the UE. It is too late to pursue such specification impacts.

- It is not aligned with the OAM centric data collection.

Thus, we propose:

Proposal 5a (stage 2 issue): RAN2 conclude that SN can’t be configured for RAN centric data collection, same as OAM centric data collection.

Proposal 5b (stage 2 issue): Capture “For NR-DC, the configuration and reporting of both OAM centric and RAN centric NW sided data collection can only be via MN” in TS 38.300.

# 3 Conclusion

In this contribution, we discuss open issues on NW-side data collection. Our Observations:

Observation 1: Z007 was postponed due to the following 2 reasons:

- It is not sufficient to just restrict multiplexing NW-side collected data and SON/MDT data because UEInformationResponse may also use SRB1 to transmit other data (e.g. EMR).

- There was no consensus whether to capture the restriction in a NOTE of Section 5.7.10.3 or field description of CSI-LogMeasReportConfig.

Observation 2: UEInformationResponse can include the following different kinds of reporting:

- EMR (Early Measurement Reporting) related reporting via SRB1

- SON/MDT related reporting via SRB2

- Other reporting via SRB1: coarseLocation for NTN and flightPathInfoReq for UAV

Based on observations, we have the following proposals:

Important RIL issues

Proposal 1 (RIL Z007): As UEInformationRequest already includes separate indications for the different types of reporting, NW can ensure csi-LogMeasReportReq are not simultaneously included with any other request indication(s) in the same UEInformationRequest message.

Proposal 2 (RIL Z007): Revise the existing NOTE in procedure text of Section 5.7.10.3 to reflect Proposal 1 (TP in Appendix 1).

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested included with any other request indication(s) in the same UEInformationResponse message.

Proposal 3 (RIL S052): Same as L3 measurement event A1/A2, RAN2 confirm that the event evaluation of logging events uses L3 serving cell measurement via the MO indicated by servingCellMO. Capture the following clarification in the field description of CSI-LoggedMeasurementEventTriggerConfig-r19.

csi-LoggedMeasurementEventTriggerConfig

This field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception. The L3 serving cell measurements performed via the measurement object indicated by servingCellMO is used to evaluate the entering and leaving condition.

Proposal 4 (RIL C074): Suggest to PropAgree C074, i.e. remove the following duplicated UE behavior on releasing CSI logged measurement configuration:

4>	release CSI-LoggedMeasurementConfig, if configured;

4>	release loggedDataCollectionAssistanceConfig, if configured;

4>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

Stage 2 issues

Proposal 5a (stage 2 issue): RAN2 conclude that SN can’t be configured for RAN centric data collection, same as OAM centric data collection.

Proposal 5b (stage 2 issue): Capture “For NR-DC, the configuration and reporting of both OAM centric and RAN centric NW sided data collection can only be via MN” in TS 38.300.

# 4 References

[1] RAN2#131, Chair Notes.

[2] RAN2#131b, Chair Notes.

[3] R2-250xxxx, Correction on AI for NR Air interface feature in TS38.300, vivo.

[4] RAN2#129b, Chair Notes.

[5] RAN2#130, Chair Notes.

# Appendix 1

#### 5.7.10.3	Reception of the UEInformationRequest message

<Unrelated text Omitted>

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else if csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested included with any other request indication(s) in the same UEInformationResponse message.