3GPP TSG-WG2 Meeting #132	R2-2508472

Dallas, USA, November 17 – 21, 2025

Agenda Item:	10.3.1.1

Source:	Ericsson

Title:	User Plane Function Assessment in 6GR

Document for:	Discussion, Decision

# 1	Introduction

As 5G deployments mature and move beyond initial commercial implementations, in 6G it is critical to refine and optimize the user plane protocol stack architecture beyond 5G limitations to meet new demanding requirements on performance, energy efficiency, and flexibility.

The following agreements have been reached in RAN2#131bis for the user plane. In this contribution, we further discuss required functionalities of the 6G user plane protocol stack.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

# 2	Discussion

The current 5G user plane protocol stack comprises of the Physical (PHY), Medium Access Control (MAC), Radio Link Control (RLC), Packet Data Convergence Protocol (PDCP), and Service Data Adaptation Protocol (SDAP) layers. Even though it delivers 5G’s promised performance, its modular, layered design creates inefficiencies in processing, memory access, and energy use for resource-constrained devices.

6G development should prioritize holistic stack redesign over piecemeal evolution, examining cross-layer interactions and seeking a unified architecture that consolidate functionalities rather than layering multiple complex solutions that undermine each other’s effectiveness.

6G presents an opportunity to address the over-complexity of 5G through architectural modifications creating a leaner protocol stack that maintain functional integrity while improving efficiency. Elimination of dual connectivity (DC) opens new possibilities for protocol simplification, allowing for streamlined processing paths and reduced computational complexity.

## 2.1 	Functionalities in the user plane protocol stack

One aim of the 6G study item [1] is to aim for system’s simplicity by minimizing the specification of multiple options for the same functionality. RAN2 should thus strive to reduce overhead and redundant functionality within the user plane protocols and by also reconsider the functionality distribution among layers.

For example, 5G NR protocols are built to support dual connectivity (DC), and even standalone deployments cannot exploit the one-to-one mapping between RLC and PDCP packets for the case without DC. While in LTE one RLC protocol data unit (PDU) with its own SN concatenated multiple PDCP PDUs, the RLC PDU in 5G can only include at most one PDCP PDU. Still, both PDCP PDU and RLC PDU in 5G include their own SN, which unnecessarily adds overhead and processing.

Furthermore, in 5G, several inter-layer indications are specified to synchronize protocol states locally between entities in each transmitter and receiver sides. For example, in the transmitter side, RLC AM indicates ACKs to PDCP, PDCP indicates discards to RLC, and at the receiver side, RLC and PDCP internal states can lose synchronization, given their different individual reordering and status reporting mechanisms. Another point for simplification is the retransmission schemes separately specified in PDCP, for handover recovery, and RLC for ARQ.

Complexities in 5G’s protocol stack stem from separated state variables, packet queues, retransmission schemes and reordering mechanisms among protocol layers, affecting both transmitter and receiver side.

6G should thus study how to reduce such complexities in functional distribution and related inter-layer indications between RLC, PDCP and also SDAP.

As agreed in RAN2#131bis, RAN2 should first establish a view of essential UP protocol functionality for 6G. Therefore, with the help of the following table, we analyse existing functionalities in the UP layers of 5G and discuss their role towards 6G.

The discussed functionalities of PDCP and RLC are considered as today on a per radio bearer level, i.e. their configuration may differ between different radio bearers, to which we assume different QoS flows may be mapped on, thus allowing service differentiation.

| Layer in 5G | Functionality and description | Comment towards 6G (not precluding layer) |
| --- | --- | --- |
| SDAP | Mapping QoS flows to DRBs QoS flows as established by the CN and indicated with their QoS flow identifier (QFI) to RAN, can be mapped to same or different DRBs, subject to gNB’s mapping rules. | Essential basic functionality to enable 5G QoS framework. |
| SDAP | QFI marking An SDAP header is appended to the packets to identify their QoS flow association, i.e. needed for the case of having different QoS flows mapped to the same DRB. | Essential, e.g. to indicate UL QFI from RAN to CN, for CN operation specific to QFIs. |
| SDAP | Reflective QoS Enables the UE to monitor QoS flows to DRB mapping in DL and apply the same rule to UL. | Not needed, as not used in practical deployments. Instead QoS to DRB mapping RRC configuration is used. |
| PDCP | Data recovery and re-establishment PDCP serves as the handover anchor protocol, handles protocol reset behavior as part of PDCP re-establishment procedure, and allows continuity after handover. For data recovery, for AM DRBs, performs retransmissions of not yet confirmed to be received data.. | Essential to enable lossless handovers. Simplification potential in specification (retransmissions specified in data recovery as well as re-establishment procedures. |
| PDCP | PDCP status report Bitmap-based PDCP status reports optimize (reduce duplicates) required retransmissions after handover, i.e. provide additional information about reception status beside previously received RLC status reports. | Not needed, as this optimization is not used in practice. Redundancy with RLC status reporting. |
| PDCP | Header compression Allows compression of upper layer headers such as IP and above via IETF ROHC or Ethernet via EHC, or uplink data compression (UDC). | Needed e.g. for low-rate services in coverage limited scenarios. Relevant compression protocols in 6G timeframe to be discussed. |
| PDCP | Sequence number maintenance and window management A sequence number (SN) is assigned for each PDU i.e. each SDU to be transmitted. A transmit window as well as receive window are maintained based on COUNT values (HFN and SN). TX_NEXT defines the next COUNT (SN) to be assigned, and transmitter is assumed to not bring more SNs than the reordering window size in flight. The receiver maintains this reordering window based on lower edge RX_NEXT_DELIV, the highest not yet delivered COUNT still waited for. RX_NEXT (next after highest received SN) and RX_REORD (next after SN that triggered reorderingTimer) help in maintaining this. | Essential basic functionality. Redundancy between PDCP and RLC sequence numbers is FFS, see section below. |
| PDCP | Receiver in-order delivery (timer-based) Packets received from lower layer are buffered until their order according to the SNs is re-established and then delivered to higher layers in-order, or the timer reorderingTimer (started when an SN gap is started) expires. In this case delivery happens for all stored packets until RX_REORD and potential subsequent consecutive packets. RX_DELIV, RX_REORD are updated upon timer expiry. | Essential. In-order delivery required for high performance of loss-based congestion control algorithms in TCP/QUIC. Not necessarily on PDCP layer. Redundancy with RLC re-assembly timer. |
| PDCP | Receiver out-of-order deliver If configured, packets are immediately delivered to upper layers when received. Still, reorderingTimer with maintained state variables RX_DELIV and RX_REORD govern the lower window edge, i.e. until when packets are still accepted for reception (and delivery). | Needed for certain applications (e.g. time-critical) but has limitation in current form. Immediate out of order delivery too early considering e.g. HARQ RTTs, can consider finer configuration of when to start out of order delivery. Not necessarily on PDCP layer. |
| PDCP | Encryption/Decryption & integrity protection/verification PDCP payload is encrypted and PDCP headers and payload are integrity protected, using among others the SN as input to the security algorithm. On the receiver side, packets are decrypted, and integrity is verified. | Essential basic functionality Security algorithms may be improved, subject to SA3 discussions. |
| PDCP | DC split bearer For UL operation in dual connectivity (DC), the transmit algorithm how to split data among the two transmission paths is specified. | Not needed. |
| PDCP | Duplication For UL operation in DC as well as in CA (with one RLC bearer per carrier), packets can be configured to be duplicated over these transmission paths. | Useful only (in CA) for high-reliability services, but not necessarily needed on PDCP layer (as it has been specified for DC). Some redundancy with ARQ in RLC (for CA). |
| PDCP | Receiver duplicate discarding Duplicate received packets are discarded. | Essential to avoid exposing duplicates to higher layers, that may occur e.g. due to duplicate HARQ/RLC retransmissions. Not necessarily on PDCP layer. |
| PDCP | SDU discard (timer) For each SDU, a discard timer is started upon arrival and upon expiry the packet is discarded, the discard is also indicated to RLC so that not yet transmitted packets can still be discarded. Noteworthy, the timing functionality is also referred to by other features such as delay status reporting (DSR). | Unclear need to support discard on SDU level (before SN assignment) considering pre-processing and given PDCP SN gap report feature. Timer/timing functionality needed for other features (e.g. DSR). |
| PDCP | SN gap report In Rel-18, an indication from transmitter to receiver to inform about a discard has been introduced. Upon reception, the SN indicated is considered received, i.e. avoiding reordering delays. | Need depends on functional split (SN assignment per layer), e.g. needed in case of reused RLC/PDCP SNs. Then not necessarily on PDCP layer. Needed for fast queue management, see Section 2.3. |
| PDCP | PDU Set discard Extends the SDU discard timer functionality to PDU Sets. | Unclear need |
| PDCP | PSI-based SDU discard Extends the SDU discard timer functionality for PDU Sets to be selective acc. to their importance. | Unclear need |
| RLC | Acknowledged mode (AM) & Unacknowledged mode (UM) Are specified in different transmit and receiver algorithms with their own set of state variables and timers. In AM, polling, status reporting and retransmissions as part of automated repeat request (ARQ) are done. | Reliable retransmissions by AM/ARQ is essential basic functionality. Study not considering functionality as different modes. See discussion in section below. |
| RLC | Sequence number maintenance & window management RLC assigns its own PDU sequence numbers in AM to all packets, and in UM to segmented packets only. When it comes to transmit/receive window management, a window size is defined and based on: In AM, TX_Next_Ack is the lower window edge, moved upwards based on feedback from receiver regarding RX_Next (highest in-order delivered) and RX_Next_Highest_Status. Further state variables are defined in receiver to keep track of status reporting and reassembly handling. In UM, TX_Next is the upper window edge in transmitter, moved upwards based on new transmissions. In the receiver, state variables are defined for reassembly handling. | Separate SNs in RLC may not be needed since redundant with PDCP SNs in case of standalone operation. See discussion in section below. |
| RLC | Segmentation and reassembly (timer-based) RLC SDUs are segmented to fit transport blocks from underlying protocols. Segmentation indicator fields and segmentation offsets are included in RLC PDU headers. When retransmitting in AM, SDUs may also be re-segmented. In the receiver, in both AM and UM timers and state variables (RX_Next_Status_Trigger / RX_Timer_Trigger, RX_Next_ Reassembly, t-Reassembly) are used to keep track of segments still considered for reassembly or to be considered lost (UM) or for status reporting (AM). | Essential basic functionality to map packets to radio frames, i.e. to enable data transmission for low data rates and to efficiently utilize different transport blocks sizes. |
| RLC | Acknowledged mode: Polling and status reporting In AM, the reception status report is polled, i.e. a poll bit is included according to certain rules. The reception status report sent back to the transmitter includes information about highest in-order received SNs as well as packets or segments lost. Beside polling, the status report may also be triggered based on identified reception gaps (and related expiry of t-Reassembly timer). | Needed basic functionality to recover from residual losses of lower layer HARQ Can be complemented by lower layer feedback from HARQ, i.e. to speed up retransmissions. See our companion paper [4]. |
| RLC | Acknowledged mode: Retransmissions In AM, retransmissions are triggered for negatively acknowledged SDUs or SDU segments by the RLC status report. | Essential basic functionality to allow (L2) retransmissions enabling re-segmentation. Can study triggering retransmissions also in UM, if triggered by lower layer feedback from HARQ as discussed in our companion paper [4]. In particular if AM and UM are not considered separate operational modes. See discussion in section below. |
| RLC | Transmitter SDU discard When indicated from PDCP, a not yet transmitted SDU is to be discarded. | See comment on PDCP SDU discard (timer). |
| RLC | Receiver discard timer Rel-19 introduces a functionality to move the lower window edge (RX_Next) upwards in AM, even though packets were not yetreceived, i.e. SNs before are discarded. The reasoning was that higher layer PDCP receiver applies a similar method with reorderingTimer, while AM would have continued retransmitting without this Rel-19 feature. | Not needed, since redundant with PDCP reorderingTimer functionality. |
| RLC | Transmitter retransmissions limitations Rel-19 introduces a feature to limit AM retransmissions in case of SDU discards. | Not needed optimization. Unclear in conjunction usage of PDCP SN gap report. |

Features in 5G have been added throughout the releases to address new requirements, improve and fix issues. The baseline for 6G design, when it comes to discussion on required functionality, should thus be Rel-19 NR. As analysed above, some functionalities are identified as redundant and their consolidation should be considered in 6G.

Discuss redundant functionality across protocol layers and potential consolidation, with NR Rel-19 functionality as the baseline.

In order to progress the analysis of required functionalities in RAN2, we propose to capture above analysis (table) in the TR to find alignment among companies’ views.

Consider above analysis for upper UP functionalities for 6G design for further discussions in the TR.

An important aspect that should be addressed alongside the UP functionalities is the appropriate level or granularity at which they can be applied (e.g., bearer-level). This discussion can commence even before protocol layers are defined and functionalities are mapped to those layers. In 5G, for single connectivity the functionality is specified in RLC and PDCP at the DRB level; therefore, a logical starting point for 6G discussions is to consider the DRB level as well. This is further discussed in our companion paper [5] on the QoS framework.

Given the one-to-one mapping of PDCP entity and RLC entity within one DRB in standalone-operation, as well as the one-to-one mapping of PDCP PDUs and RLC SDUs, one simplification aspect for further study is the usage of a common sequence number (SN), i.e. reuse of the PDCP PDU SN in the RLC SDU. This could potentially decrease overhead, as well as reduce processing and memory burdens.

Study reuse of PDCP sequence number in RLC in 6G UP.

Another area where the specification could be streamlined is the rather static set of RLC operational modes.: E.g. Acknowledged Mode (AM) differs from Unacknowledged Mode (UM). RLC specifies their algorithms separately, e.g. at the receiver side, however, both undergo the same common PDCP receiver algorithm that eventually determines the behaviour how packets are delivered to higher layers. The algorithms could potentially be consolidated into a common one, where AM and UM modes would differ only by configuring support for polling, status reports, and retransmissions, but not in their algorithms. This also relates to considering HARQ feedback for faster triggering of RLC retransmissions (see companion paper [4]), which could be enabled in such a common framework of AM/UM.

Study common framework in 6G RLC for AM and UM operation.

## 2.2		Processing capabilities

As agreed, the study on 6G targets a UP design that should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layers.

From a processing efficiency point of view, it is preferable to process incoming packets only once, and upon arrival at the protocol stack, and thereafter store packets in memory until transmission over-the-air. This reduces the required processing in the time-critical scheduling loop, i.e. upon submission via the radio interface. Thus, protocol stacks and implementations supporting pre-processing essentially helps to reduce processing burden during the time-critical scheduling loop.

Pre-processing implementations may include at least PDCP SN assignment, encryption, integrity protection, and preparing of PDCP-, RLC- and MAC-sub headers.

Pre-processing capability on transmitter side is essential to achieve the 6G target of minimizing processing and memory transfers.

Packets may become available for pre-processing in L2 at different points in times, and potentially with different burst sizes. I.e. because most internet traffic is bursty and has unpredictable arrival times.  Then, grouping of packets to potentially optimize pre-processing by e.g. jointly applying encryption/integrity (“PDCP concatenation”), may thus be rather counterproductive. The interdependencies created by such grouping would e.g. lead to delays by waiting for packets for joint encryption and also decryption, as packets albeit grouped for encryption/decryption may experience different delays in L2 protocols, e.g. due to retransmissions. Furthermore, concatenation applied before transport block sizes are known would lead to additional segmentation and thus overhead when transport block sizes are eventually known and smaller than the concatenation group size. Most modern hardware accelerators (HWAs) are optimized to support the pre-processing operations for typical MTU sizes as supported on the internet with maximum efficiency, minimal latency and minimized idle times (in terms of clock cycles). Thus, per-packet pre-processing is not a processing bottleneck. On the contrary, additional steps like grouping would lead to idle times in the HWA, breaking the processing pipeline, leading to inefficient use of the HWA.

Grouping of packets for joint pre-processing leads to delays and cannot efficiently utilize hardware accelerators optimized for typical MTU sizes.

The essentiality for the pre-processing capability in the transmitter should be reflected in discussions of protocol functionalities, i.e. on the one side, functionalities that inhibit pre-processing should be avoided, and on the other side, functionality resulting from the need to pre-process should be considered. An example for the former would be features for SDU buffer before SN assignment (like the SDU timer-based discard ), and for the latter would be pre-assigning SNs and the SN gap report to indicate a potential gap to the receiver. In other words, when designing the 6G UP protocols to optimize performance over the air, RAN2 should consider the need for per-packet pre-processing in implementations in order to achieve the 6G targets of minimizing processing and memory transfers.

When studying protocol functionality in 6G, also consider implementations based on per-packet pre-processing on transmitter side.

## 2.3		Transmit buffer queue management

6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB). One important aspect is to keep delays due to buffering in the RAN transmitter side in reasonable bounds. With active queue management (AQM), the transmitter can control the size and latency of the transmit queue awaiting TX over the radio link. It aims for a small queue to keep the E2E latency low while storing sufficient data to fully utilize the underlying link and thereby maximize the throughput. AQM achieves this by issuing signals to the end-to-end (E2E) transport protocol such as TCP and Quick UDP Internet Connections (QUIC), e.g. indicated via explicit congestion notifications (ECN) [RFC3168][2] if supported, or via artificial dropping of packets, upon which the E2E transport protocol detects and reduces its transmit rate. AQM works most efficiently if these indications are triggered timely and reach the traffic source as fast as possible. In 5G this is not the case, as it conflicts with pre-processing and reordering behaviour. After pre-processing, AQM can no longer access the IP packets’ ECN fields to set the ECN congestion experienced (ECN-CE) bit, as they are already ciphered. Also, AQM packet dropping after pre-processing with SN assignment would lead to an SN gap noted by the PDCP receiver which starts its reordering timer. Only upon expiry of the reordering timer, subsequent packets would be delivered, meaning that the indication would be delayed. In 5G, this is partially addressed with the Rel-18 feature of PDCP SN gap reporting. However, such gap indication signaling in PDCP would need to undergo queueing in RLC which also comes with a delay.

Indications to manage queueing latency in RAN to transport layer are essential to reduce overall end-to-end latency and are subject to delays considering 5G’s need for pre-processing.

AQM should be an integral part of the 6G layer 2. Since at the transmitter side, one cannot access encrypted IP fields of packets in its queue, it has been discussed setting a “CE flag” in its own layer 2 header to mark the packet front-of-queue upon packet submission. When this indication is received, the receiver “copies” it into the IP header’s ECN-CE field after de-ciphering and thereby notifies the E2E protocols instantaneously. Hence, critical communication services could also be better supported. If the E2E path does not support ECN (sender indicates this by ECN bits 00), the AQM algorithm should rather drop a packet. To avoid latency spikes caused by reordering, the transmitter-side protocol should have means to inform its receiving peer that PDUs have been dropped and that the reordering function at the receiver shall deliver subsequent packets immediately. The PDCP SN drop indication introduced for 5G in 3GPP Rel-18 should thus become another integral part of a 6G layer 2 protocol.

To achieve lower end-to-end latencies for general services (e.g. eMBB), study support for faster queue management as an integral component of 6G RAN (e.g. based on queue indications).

## 2.4		Functionalities in MAC

In the following we discuss example UP functionalities specified in 5G in MAC, i.e. for user plane operation itself. In standalone operation, the functionalities are therein configured on a granularity per UE, and in certain cases per carrier. This contrasts with the discussed functionalities in RLC, PDCP which on DRB level may be configured service specific. In the following we discuss the specified MAC features and their need or redundancy in 6G one by one. In the following we discuss an excerpt of such MAC functionality.

| Layer in 5G | Functionality and description | Comment towards 6G (not precluding layer) |
| --- | --- | --- |
| MAC | Multiplexing/Demultiplexing Within a MAC PDU transmitted as a transport block (TB), multiple sub-PDUs carrying data from same logical channel (i.e. concatenation) or different logical channels, as well as MAC control elements (CEs), are multiplexed. Logical channel identifiers are utilized for this purpose. | Essential basic functionality |
| MAC | Scheduling information reporting The MAC layer supports scheduling by signaling support from the UE to the gNB. The MAC layer makes use of physical layer indications as well as MAC CEs for this purpose which are considered in MAC procedures. Examples are scheduling request, buffer status reporting, delay status reporting. | Essential basic functionality Room for improvement and simplification for BSR and DSR discussed in our companion paper [4]. |
| MAC | Error correction through HARQ; With hybrid automated repeat request (HARQ) transmission errors can be corrected in a fast and efficient way, i.e. based on physical layer feedback signaling. Though, full reliability cannot be achieved due to existing residual errors. | Essential basic functionality Room for improvement, discussed in our companion paper [4]. |
| MAC | Logical channel prioritization For UL multiplexing of different logical channels into granted UL transmission resources, the logical channel prioritization (LCP) procedure in the UE defines order and amount of data per logical channel to be transmitted. In Rel-18/19, delay criticality has been added as a decision basis. | Essential general functionality, but details for further study. Discussed in our companion paper [4]. |
| MAC | Intra-UE Priority handling between overlapping resources Rel-16 introduced mechanisms in MAC to prioritize transmissions of logical channels between overlapping resources in the UE. The main use-case in mind has been URLLC, e.g. being able to map URLLC data, if available, to certain uplink resources only. | Not needed. Less complex alternative ways exist handle prioritization scenarios. |
| MAC | Control elements With MAC control elements (CEs) downlink and uplink control information can be conveyed between UE and gNB. I.e. employing HARQ. Several MAC CEs have been defined throughout the releases. Some CEs are considered within MAC itself, while others relate to other protocol’s procedures. | Need depends on information carried. Study should include individual elements. Some MAC CEs may be unified or simplified. |

Consider above analysis of example division of MAC functionalities for 6G design for further discussion in the TR.

# 3	Conclusion

In this paper, we have reached these observations:

Observation 1	Complexities in 5G’s protocol stack stem from separated state variables, packet queues, retransmission schemes and reordering mechanisms among protocol layers, affecting both transmitter and receiver side.

Observation 2	Pre-processing implementations may include at least PDCP SN assignment, encryption, integrity protection, and preparing of PDCP-, RLC- and MAC-sub headers.

Observation 3	Pre-processing capability on transmitter side is essential to achieve the 6G target of minimizing processing and memory transfers.

Observation 4	Grouping of packets for joint pre-processing leads to delays and cannot efficiently utilize hardware accelerators optimized for typical MTU sizes.

Observation 5	Indications to manage queueing latency in RAN to transport layer are essential to reduce overall end-to-end latency and are subject to delays considering 5G’s need for pre-processing.

We propose:

Proposal 1	Discuss redundant functionality across protocol layers and potential consolidation, with NR Rel-19 functionality as the baseline.

Proposal 2	Consider above analysis for upper UP functionalities for 6G design for further discussions in the TR.

Proposal 3	Study reuse of PDCP sequence number in RLC in 6G UP.

Proposal 4	Study common framework in 6G RLC for AM and UM operation.

Proposal 5	When studying protocol functionality in 6G, also consider implementations based on per-packet pre-processing on transmitter side.

Proposal 6	To achieve lower end-to-end latencies for general services (e.g. eMBB), study support for faster queue management as an integral component of 6G RAN (e.g. based on queue indications).

Proposal 7	Consider above analysis of example division of MAC functionalities for 6G design for further discussion in the TR.

# 4	References

RP-251881, New SID: Study on 6G Radio, 3GPP RAN#108, Prague, Czech Republic, June 9-13, 2025 [online]

Internet Engineering Task Force (IETF), RFC 9330, “Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture”, December 2023

R2-2507074, Common User Plane and Control Plane Aspects for 6G, 3GPP RAN2#131bis, Prague, Czech Republic, October 13-17, 2025.

R2-2508344, Scheduling and retransmissions, Ericsson, 3GPP RAN2#132, Dallas, USA, November 17 – 21, 2025

R2-2508474, RAN Aspects for QoS framework and Service-Awareness, Ericsson, 3GPP RAN2#132, Dallas, USA, November 17 – 21, 2025