3GPP TSG-RAN WG2 Meeting #132	R2-2508478

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.11.2

Source:				Nokia, Charter Communications

Title:	UE Transmit Power Continuity during RO type Switch

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1. UE transmit power continuity during RO type switching

In RAN2#131, how to ensure UE transmit power continuity during RO type switch was discussed. As a result, the following proposal introduces a power offset to compensate for the differences in the preamble power ramping steps in legacy ROs and additional ROs:

Proposal: For the RO type fallback between legacy RO and additional RO, a power offset given by the difference between the two quantities of preamble power ramping steps is added.

Note that RAN2 agreed to this proposal even though from RAN1’s perspective, there was no consensus on the need for any power offset to compensate the power difference when a SBFD-aware UE switches from additional ROs to legacy ROs, or vice-versa:

Conclusion (RAN1#121) For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

Observation 1: RAN2 agreed the introduction of a power offset to compensate the power ramping differences between additional and legacy ROs even though RAN1 concluded not to support such enhancement.

The recently agreed power offset is trying to solve the following issue as shown in Figure 1. Note the differences in the power ramping steps configured for the additional and the legacy ROs.

Figure 1. RO switch from additional RO to legacy RO without power ramping compensation. Same preambleTargetReceivedPower is configured for additional and legacy ROs.

Following the same principle as for the 2-step RACH to 4-step RACH switch, it was proposed to introduce the following power offset compensation:

By using this approach, the UE power continuity is ensured given that the preamble received target power is configured to the same value for both the additional ROs and legacy ROs, as shown in Figure 2:

Figure 2. RO switch from additional RO to legacy RO with power ramping compensation. Same preambleTargetReceivedPower is configured for additional and legacy ROs.

However, in our view, the current proposal is not completely solving the UE transmit power continuity issue when switching from additional RO to legacy RO or vice versa. The reason is that the agreed proposal only compensates for the potential differences in the power ramping step but doesn’t take into account the potential differences in the preamble received target power. In Configuration 2, the UE will receive 2 independent RACH-ConfigCommon configurations, which among others, contain different preambleTargetReceivedPower and/or different powerRampingStep. Thus, the power continuity cannot be achieved by applying the RAN2#131 compensation method as shown in Figure 3.

Figure 3. RO switch from additional RO to legacy RO with power ramping compensation. Different preambleTargetReceivedPower is configured for additional and legacy ROs.

Observation 2: With Configuration 2, UE receives two separate RACH-ConfigCommon settings. As a result, the current method does not completely solve the UE transmit power continuity issue when switching between additional RO to legacy RO or vice versa as it doesn’t take into account the potential differences in the preamble received power.

In other words, the UE transmit power continuity is not ensured in all scenarios. Note that the preambleTargetReceivedPower IE can take values from {-202..-60} dBm according to TS 38.331. This can lead to a large power difference between ROs, which is not acceptable from UE vendors point of view as per previous RAN2 meeting’s discussion.

The specific values of preamble received target power configured for each RO type is highly dependent on the SBFD specific interferences experienced in the network. In turn, the interference levels depend on several factors, e.g., type of scenario (e.g., urban macro or dense urban), inter-site distance, gNB transmit power, gNB self-interference capabilities, current load in the network, etc. As a general configuration principle, the network is expected to configure higher preamble target received power for the additional ROs to account for the differences in coverage conditions between additional ROs and legacy ROs.

Observation 3: The configured preamble received target power of additional ROs is expected to be higher than in legacy ROS to compensate for different coverage conditions due to the presence of SBFD interference.

In the context of 2-step to 4-step RACH fallback, RAN2 introduced a power compensation offset that accounts for the differences in power ramping steps. However, the difference in preamble received power target remains uncompensated, even though 2-step and 4-step preamble received target power are configured independently too. In our view, RAN2 did not consider the preamble target received power compensation in the past because network is expected to configure same value for both RACH types. Note that random access occasions only occur within UL slots and therefore coverage and interference conditions are the same. Differently, with the introduction of the SBFD feature, ROs are found in SBFD symbols and non-SBFD symbols. Due to the presence of gNB self-interference, inter-sector CLI and inter-cell gNB-to-gNB CLI, a sensible network configuration shall configure a higher preamble received target power for the SBFD ROs.

Observation 4: During the standardization of 2-step to 4-step RACH fallback, the compensation for different preamble received target power was not considered given that network is expected to configure same value for both RACH mechanisms.

Given the arguments above, we think it is needed to find a new way of compensating for both the differences in power ramping offset, and preamble received target power. To do so, we propose updating the current POWER_OFFSET_RO_TYPE variable defines in TS 38.321 such that it also accounts for potential differences in the preamble received target power. The TP below captures the new POWER_OFFSET_RO_TYPE definition. Our proposal works as well in scenarios where the preamble received target power are the same. If network configures received target power is the same for SBFD and legacy ROs, the proposed offset will only compensate for the power ramping differences, if any.

Proposal 1: Support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

TP : The power offset adjustment for UE transmit power continuity between SBFD RO to legacy RO (or vice versa) accounts for power ramping and preamble received target power differences and it can be determined as follows:

Section 5.1.4 2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 1st-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to preambleReceivedTargetPower (configured for second PRACH occasion) - preambleReceivedTargetPower (configured for first PRACH occasion) + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 2nd-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to preambleReceivedTargetPower (configured for first PRACH occasion) - preambleReceivedTargetPower (configured for second PRACH occasion) + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).. Note: Similar changes in 5.1.5 as well.

# 2. Conclusion

Observation 1: RAN2 agreed the introduction of a power offset to compensate the power ramping differences between additional and legacy ROs even though RAN1 concluded not to support such enhancement.

Observation 2: With Configuration 2, UE receives two separate RACH-ConfigCommon settings. As a result, the current method does not completely solve the UE transmit power continuity issue when switching between additional RO to legacy RO or vice versa as it doesn’t take into account the potential differences in the preamble received power.

Observation 3: The configured preamble received target power of additional ROs is expected to be higher than in legacy ROS to compensate for different coverage conditions due to the presence of SBFD interference.

Observation 4: During the standardization of 2-step to 4-step RACH fallback, the compensation for different preamble received target power was not considered given that network is expected to configure same value for both RACH mechanisms.

Proposal 1: To support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.