3GPP TSG RAN WG2 Meeting #132	R2-2508479

Dallas, USA     17-21 November 2025

Agenda Item:	9.3.2 RRM measurement prediction

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Measurement gap relaxation

Document for:	Discussion & Decision

# Introduction

The WID [1] , lists, among others, the following as objective for RAN2:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we provide our views on support of RRM measurement prediction and measurement event prediction for UE sided models and the UE-support for the NW-sided models.

# UE-sided model

In TR 38.744 [2], RAN2 studied various options for reducing measurement, and several patterns of observation and prediction combination, and reported the following:

      - For inter-frequency prediction, in addition to reducing UE’s measurement efforts, the UE throughput can also be increased if measurement gap configuration can be avoided or relaxed.

      - Different settings of MRRT and different measurement pattern result in different prediction accuracy.

      - Increasing MRRT results in decreasing prediction accuracy.

In this section, we provide our views on specification support for enabling the UE-sided model.

## Flexible measurement gap configuration

Reduction in measurement gap compared to legacy is one of the major advantages of using AI/ML models for RRM and event predictions. At the same time, if the model monitoring results in deviation, it may be beneficial to quickly switch from AI/ML based prediction to legacy framework. The ability to switch from AI/ML based measurement and event reporting to legacy framework becomes sometimes necessary, because the AI/ML based mobility is being supported from Rel. 20, whereas a stable framework for legacy mechanism exists.

A UE may achieve different accuracy for different settings for UE-sided model inference. There may be different costs (in terms of accuracy and resource consumption) associated with the desired accuracy.

To reap the benefits of AI/ML based prediction, while keeping the legacy mechanism readily available, it is proposed to configure the measurement gaps according to measurement and reporting granularity required according to legacy methods, and enable a lower level signalling to switch between the AI/ML based approach and legacy approach. In Rel. 19, there is already a possibility to skip measurement gaps with DCI signalling.

RAN2 shall consider a MAC-CE to skip certain measurement gap, or change different MRRT/ skipping pattern settings.

## MRRT and accuracy reporting

Model accuracy and prediction reliability must be consistently evaluated within the LCM framework. Since different combination of MRRT and different measurement pattern results in different prediction accuracy, RAN2 needs to consider the possibility that a UE may be capable of different performance under different circumstances. The UE may be able to achieve higher accuracy with lower MRRT and vice versa. Similarly, the UE may have different implementation, that achieve different accuracy under different resource consumption, such as power.

A UE may achieve different accuracy for different settings for UE-sided model inference. There may be different costs (in terms of accuracy and resource consumption) associated with the desired accuracy.

RAN2 shall discuss how to enable the UE to report parameters that affect the performance and costs to the network, so that the network can balance between accuracy and costs.

# NW-sided model

Provided that NW-sided model performs layer 3 (L3) beam-level prediction, and RRM measurements, the NW needs to receive a reference value (i.e., ground truth) against which the NW sided model monitors its performance. This is necessary to enable the network to perform the life cycle management of the network-sided model.

The RRM measurements and L3 beam level predictions may be available at the UE side, either through actual measurements (and filtering) or through AI/ML-based predictions generated by the UE-sided models.

The following options would be feasible for obtaining ground truth labels:

- Ground truth labels for RRM and beam predictions are based on legacy measurement and reporting framework.

- Ground truth labels for RRM and beam predictions are based on the prediction generated by UE-sided model.

- Ground truth labels for RRM and beam predictions are provided upon an explicit request from the network, with an explicit indication what type of UE-report it requires (such as ‘based on actual measurement’, ‘based on UE-sided model’ etc)

As Rel. 20 is the first release providing AI/ML-based mobility support, the UE-sided prediction of measurements is not as mature as the legacy RRM measurement framework. To mitigate the risk that the prediction bias (or model error prediction, prediction inaccuracy) of UE-sided model propagates to the NW-sided LCM, it could be beneficial to utilize the actual measurements and reports from legacy methods as ground truth labels for data collection.

For LCM of network sided model, UE needs to provide reference RRM values and event prediction (ground truth labels).

Using UE-sided model outputs to train NW-sided model may introduce prediction-bias.

RAN2 shall discuss whether the measurement reports provided by the UE for LCM of NW-sided nodes are based on

- Actual measurement and filtering (legacy)

- UE-sided model

- Network indicated preference (either actual or UE-sided UE-selected)

# Conclusions

In this contribution, we have made the following observations:

Observation 1 A UE may achieve different accuracy for different settings for UE-sided model inference. There may be different costs (in terms of accuracy and resource consumption) associated with the desired accuracy.

Observation 2 A UE may achieve different accuracy for different settings for UE-sided model inference. There may be different costs (in terms of accuracy and resource consumption) associated with the desired accuracy.

Observation 3 For LCM of network sided model, UE needs to provide reference RRM values and event prediction (ground truth labels).

Observation 4 Using UE-sided model outputs to train NW-sided model may introduce prediction-bias.

Based on above observations, we propose the following:

Proposal 1 RAN2 shall consider a MAC-CE to skip certain measurement gap, or change different MRRT/ skipping pattern settings.

Proposal 2 RAN2 shall discuss how to enable the UE to report parameters that affect the performance and costs to the network, so that the network can balance between accuracy and costs.

Proposal 3 RAN2 shall discuss whether the measurement reports provided by the UE for LCM of NW-sided nodes are based on

# References

| [1] | RP-252899, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," 3GPP, Beijing, China, 2025. |
| --- | --- |
| [2] | TR-38744, "TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR;," 3GPP, Sofia-Antipolis, 2025-09. |