3GPP TSG-RAN WG2 Meeting #132	R2-2508485

# Dallas, Texas, USA, 17 – 21 November 2025

Agenda item:		8.11.2

Source:	Xiaomi

Title:	Discussion on SBFD MAC open issues

Document for:		Discussion and Decision

# Introduction

After RAN2#131bis meeting, email discussion “[Post131bis][212][SBFD] CR for TS 38.321 (Samsung)” was held to further collect MAC open issues [1]. In this contribution, we discuss SBFD MAC open issues.

# Discussion

## [MAC-1] Simultaneous Fallbacks of RO Type and Msg1 Repetition Number

The potential issue is related to the case that when the conditions for RO type switching and for the Msg1 repetition number fallback are met at the same time, the further higher Msg1 repetition number would be chosen after the same or the next higher Msg1 repetition number is selected due to RO type switching, as below:

2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 1st-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; [--Omitted--] 4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 2nd-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; [--Omitted--] 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; [--Omitted--]

Firstly, the issue might be rare. Although both preambleTransMaxRO-Type and preambleTransMax-Msg1-Repetition can be configured to same set of values {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}, there is no requirement that those two switching thresholds should be configured with the same value.

Secondly, it is not clear how serious the system is impacted if the issue is not addressed. Our understanding is that even if further higher Msg1 repetition number is used, there is still no critical issue as the only impact is that UE potentially uses more Msg1 repetitions and the time for random access is increased.

In summary, there is no need to address the issue that RO type switching and Msg1 repetition number fallback are triggered simultaneously.

Proposal 1: [MAC-1] No need to address the issue that RO type switching and Msg1 repetition number fallback are triggered simultaneously.

# Conclusion

In this contribution, we discuss SBFD MAC open issues, and propose the following:

Proposal 1: [MAC-1] No need to address the issue that RO type switching and Msg1 repetition number fallback are triggered simultaneously.

# References

[1] R2-2508163, Samsung, "Summary of Rel-19 SBFD MAC open issues for maintenance"