3GPP TSG-RAN WG2 Meeting #132                               R2-2508494

Dallas, USA, Nov. 17th-21st

Source:	China Telecom

Title:	Discussion on Topology 2

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- 1.Introduction

Based on the progress of the last RAN2 meeting, relevant agreements regarding the aspects of A-IoT topology 2 have been reached, as detailed in [1] below.

- Agreements - 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information - 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - 3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB. Continue discussion next meeting on how resource allocation looks like.

In this contribution, we will further discuss the remaining issues related to D2T2 for Ambient IoT.

- 2.Discussion

In Topology 2, upon receiving an A-IoT service request from the CN, the gNB shall provide A-IoT information to the UE reader via RRC dedicated signaling. Similar to Topology 1, a service request type indication field (such as the transaction ID in Topology 1) is required; this enables the UE reader to properly schedule transmissions over the A-IoT interface. Furthermore, the Paging ID may carry A-IoT identification information, which is essential for completing the A-IoT service request. Therefore, both types of information should be included in the RRC dedicated signaling.

Proposal 1: The RRC signaling through which the gNB provides A-IoT information may include service request type indication information and Paging ID information.

According to the agreement of RAN2#131 bis meeting, the UE reader may provide assistance information related to A-IoT transmissions to assist the gNB with resource allocation. It is for further study what specific information would be useful to provide to the gNB. To facilitate more reasonable resource allocation by the base station, device connection status information is necessary, including the number of currently connected devices, the maximum number of connectable devices, and so on. In addition, the UE reader may feed back signal strength indication information during communications with A-IoT devices to the gNB, which can then adjust its resource allocation strategy based on this information.

Proposal 2: The UE reader may provide device connection status information and signal strength indication information to the gNB to assist with resource allocation.

Regarding the validity of allocated A-IoT radio resource for A-IoT radio interface, two options were identified during the Study Item (SI) phase:

- Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

As agreed in the last meeting, the gNB shall provide A-IoT resource information to the UE reader via RRC dedicated signaling. Throughout the procedure of handling different service requests, the gNB has full visibility of the service completion progress. Explicit network-controlled validity of A-IoT resources is more efficient than time-based validity, as it is challenging for the gNB to configure an appropriate time period. The time required to complete various A-IoT services is affected by multiple factors (e.g., the power level of A-IoT devices) that are beyond the gNB’s control. Therefore, we propose that only Option 1 needs to be supported in the Release 20 phase.

Proposal 3: RAN2 shall adopt Option 1 for A-IoT radio resource validity, i.e., the allocated radio resources remain valid until explicitly released by the network.

For the issue related to UE reader temporary out-of-connection, WID has restricted to include RLF and handover (HO), with no other specific enhancements. Regarding the Uu RLF handling of UE reader, the UE reader can follow the legacy behaviour to initiate RRC re-establishment procedure and suspend the transmission of all RBs in the Uu interface. For the A-IoT air interface, the ongoing A-IoT service shall be suspended as well. After the re-establishment procedure, the UE reader can follow the configuration of the network to resume/re-initiate/stop the A-IoT service.

As for HO, it is quite likely that the potential HO of the UE reader is under the control of the gNB, since there is no clear need to support the coexistence of A-IoT and UE triggered HO such as CHO. Thus, a simple way might be that the UE reader suspends the A-IoT service during the HO. After HO to the target cell, the UE reader can just follow the configuration of the network, which is similar to the above RLF case.

Proposal 4: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 5: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure. After HO to the target cell, the UE reader follows the configuration of the network.

- 2.Conclusion

Based on the above analysis, the following proposals are given for Ambient IoT Topology 2.

Proposal 1: The RRC signaling through which the gNB provides A-IoT information may include service request type indication information and Paging ID information.

Proposal 2: The UE reader may provide device connection status information and signal strength indication information to the gNB to assist with resource allocation.

Proposal 3: RAN2 shall adopt Option 1 for A-IoT radio resource validity, i.e., the allocated radio resources remain valid until explicitly released by the network.

Proposal 4: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 5: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure. After HO to the target cell, the UE reader follows the configuration of the network.

# References

- RAN2#131bis Chairman Notes.