3GPP TSG-RAN WG2 Meeting #132	R2-2508499

Dallas, USA,  Nov. 17th - 21st

Source: 	China Telecom

Title: 	Consideration of NB-IoT voice over GEO

Agenda Item:	9.7.2

Document for:	Discussion and decision

- Introduction

After RAN2#131bis meeting, the agreements of NB-IoT-NTN to support IMS voice call over GSO are shown as follows:

Agreements regarding the reply LS to SA2:

1.	Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope

2.	Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

3.	Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

4.	Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet

5.	We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

In this contribution, we provide our consideration for this WI.

- Discussion

Based on the latest discussion, majority of companies prefer the UP solution. For UP solution, separated DRBs for signalling and voice packets need to be established due to different transport requirements for these two types of packets. Voice packets require consistent jitter and delay handling and may benefit from prioritization or special treatment in satellite links.

Current specifications already support the establishment of up to two DRBs for NB-IoT, so this solution is not expected to introduce significant additional changes. Moreover, the UP solution inherently supports SPS based on RRC reconfiguration messages, which is essential for periodic small-packet transmissions such as VoIP, effectively reducing latency and control signalling overhead.

Observation 1: UP solution inherently supports SPS of voice.

The IMS voice call service is composed of talk spurt and silent period. Thus, the voice packets are in different sizes. To handle the voice packet with different sizes, we think the enhancement for SPS is needed to adapt this characteristics. Two SPS configurations could be used for talk and silent voice packets. UE identify the action talk or silence to choose different SPS resources. If the packet size varies in a large range, the dynamic scheduling could be used as baseline. However, a longer delay involves for the link transmission of GEO. We think it should be evaluated by SA2 for whether dynamic scheduling meets the QoS requirements.

Proposal 1: Two SPS configuration could be used for the voice packets are in different sizes.

Proposal 2: Dynamic scheduling needs the evaluation of SA2 for the QoS requirements.

With ROHC applied, the overhead of the UP solution can be reduced to 7 bytes, while the CP solution still requires 15 bytes even after optimization. For ultra-narrowband voice with a bitrate of only 1.2 kbps, lower overhead results in a higher proportion of effective data (63.2% for UP vs. 44.4% for CP). This directly translates to greater system capacity potential and contributes to improved voice quality. Thus, we think ROHC is more useful for UP solution.

Proposal 3: UP solution needs to consider the ROHC for overhead reduction.

- Conclusion

This contribution discussed the UP solution of NB-IoT voice over GEO. Proposals and observations are summarized as follows:

Observation 1: UP solution inherently supports SPS of voice.

Proposal 1: Two SPS configuration could be used for the voice packets are in different sizes.

Proposal 2: Dynamic scheduling needs the evaluation of SA2 for the QoS requirements.

Proposal 3: UP solution needs to consider the ROHC for overhead reduction.