3GPP TSG-RAN WG2 Meeting #132	R2-2508500

Dalla, USA, Nov. 17th-21st

Agenda item:	10.3.2.1

Source: 	China Telecom

Title:	Discussion on RRC Modelling and connection management in 6GR

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

- Introduction

In RAN #108 meeting, the New SID for “Study on 6G Radio” was approved [1] aiming to develop 6GR to meet a broad range of use cases, and requirements as defined during the RAN requirements study. In RAN2#131bis meeting, 6G SI was discussed for the first time and some high level agreements on control plane aspects for 6GR were reached.

In this contribution, we would like to share our opinions on RRC Modelling and connection management in 6GR, including cell management and RRC state management in 6GR.

- Discussion

Cell management in 6GR

Spectrum serves as the fundamental resource for wireless communication. Drawing from the deployment experiences of 5G spectrum, the mid-low frequency bands will continue to be the coverage foundation of 6G network. However, operators’ spectrums in the mid-low frequency bands usually have limited bandwidth, and due to spectrum re-farming, these spectrums become more and more scattered with narrower bandwidth available for 6G. Thus, it is even more difficult for operators to find continuous large bandwidth resources in 6G. To meet the capacity, data rates, coverage demands of 6G network, it is meaningful to study improved spectrum utilization and operations taking into account diverse spectrum allocations, as one of the objectives in the SID in 6G.

In last RAN2 meeting, Spectrum aggregation in 6GR was discussed and the following agreements were reached.

Agreements on 6GR Spectrum aggregation in RAN2#131bis:

1. 	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Spectrum aggregation of multiple pieces of spectrum

Regarding the spectrum aggregation of multiple pieces of spectrum, CA is considered as a beneficial feature in previous generations, and TR 38.914 requires the 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

However, the fragmented spectrums are not utilized well based on legacy CA framework, there are many drawbacks and limitations for the CA or SUL based solutions.

- The management of multiple cells based on legacy CA framework leads to the signaling overhead, delay, low spectrum utilization efficiency on account of SCell addition/release, and inflexible carrier selection before entering RRC_CONNECTED state. When more small bandwidth carriers especially considering the multiple fragmented spectrums in the mid-low frequency band are aggregated as independent PCell and SCells, handover due to UE mobility, load balance etc. requires further addition/release/activation/deactivation of SCells before/during/after the HO procedure, which causes much signaling overhead and delay. Moreover, since SCells are enabled in RRC_CONNECTED state, the RRC_IDLE/ RRC_INACTIVE state UE can only initiate random access on the selected UL/DL carrier associated with the determined PCell by cell searching procedure. This means that before entering the RRC_CONNECTED state, the available UL/DL carrier is determined only from PCell, which is not beneficial for load balancing and random access performance.

- SUL operation has the limitation on supporting only one available carrier in UL of a single cell at one time, which could not meet the demands to aggregate more than two carriers for the fragmented spectrum. When more than two carriers are required to support some high data rates services such as immersive multimedia, it must be combined with CA.

Therefore, new multi-carrier operation and cell management technologies could be studied to overcome the limitations and utilize the spectrum effectively and efficiently in 6G. From our perspective, potential solutions on multi-carrier operation and cell management in 6GR are discussed as follows.

- A single “Super cell” with multiple carriers

In order to utilize the fragmented spectrums more efficiently, one potential solution is to enable multiple UL/DL carriers configured and managed in one serving cell, which could be treated as one single cell and called a “Super cell”. These multiple carriers within a “Super cell” can be intra-band, and/or inter-band. Multiple Super cells are supported to be aggregated based on CA, for example when the carriers associated to different Super cells belong to different frequency ranges. Fig. 1 is an illustration for the association between multiple carriers and cells.

Fig. 1 Schematic diagram for a single “Super cell” with multiple carriers

As multiple carriers are regarded belonging to one Super cell, characteristics to be further discussed to overcome the lessons from NR spectrum utilization and aggregation framework are:

- Random access carrier selection to reduce the congestion and increase the successful rate: The multiple carriers of a Super cell are visible by a UE in RRC_IDLE/ RRC_INACTIVE (if supported) state, so that the UE can select a more suitable UL/DL carrier for random access procedure within multiple carriers of the super cell, rather than accessing from only UL/DL carrier associated with PCell as the NR legacy behaviour.

- No handover procedure across carriers in a Super cell and fast carrier activation/deactivation to reduce the latency and signaling overhead when the camping carrier is switched to an intra cell carrier: Intra cell carrier switching can replace conventional inter cell handover, eliminating the need for SCell addition/release/activation/deactivation. This is not only seamless mobility but also has the same benefits for load balancing.

- Carriers spectrum resource utilization in a Super cell as a large bandwidth resource in a single carrier, that can be flexibly and efficiently scheduled with performance improvement: A single TB mapping to multiple carriers in a Super cell can be scheduled, which brings potential benefits including overhead reduction and higher channel coding gain.

As analysed above, it is worth to study a new framework supporting a single “Super cell” with multiple carries for cell management in 6GR, not only to utilize the spectrum effectively and efficiently, but also to further reduce latency and system overhead.

Proposal 1: Regarding the cell management in 6GR, a new framework supporting a single “Super cell” with multiple carries could be studied for more efficient spectrum utilization and further reduction of the latency and the system overhead.

Flexible UL DL carrier association

Regarding how to use UL and DL spectrum more efficiently, decoupling of UL and DL carriers and enable Flexible UL DL carrier association could be considered in 6GR. In this sense, UL and DL carriers could be selected from respective spectrum resource pools based on NW configuration/indication according to practical application scenarios, which can achieve variable numbers and flexible associated frequencies of UL DL carriers. The support of at least a DL carrier in one FDD band associated with a UL carrier in another FDD band, and a UL/DL carrier in one FDD band associated with the carrier in another TDD band can be further discussed, as illustrated in Fig. 2.

Fig. 2 Flexible FDD-FDD, FDD-TDD UL DL carrier association

The flexible association that UL carriers associate flexibly with same/different number of DL carriers could be applied to the below scenarios and the benefits include:

- Adapt to unbalanced UL DL traffic load: This allows the system to flexibly adjust the utilized number of UL DL carriers adapting to UL or DL dominated services. In scenarios requiring substantial UL data transfer, more UL carriers can be allocated than DL, which could reduce the reliance on maintaining the same number of DL carries and save power as well, and vice versa. Fig 3. shows an example of carrier usage in this scenario.

(b) UL dominated services

- DL dominated services

Fig. 3 Example for flexible UL DL carrier association according to different UL DL traffic load

- Guarantee both capacity and coverage with fewer involved carriers: For instance, for services with higher DL capacity demands, higher frequency DL carrier with adequate bandwidth can be combined with wide coverage lower frequency UL carrier to ensure the requirements of DL capacity and UL coverage.

Thus, we suggest

Proposal 2: The flexible association between UL and DL carriers could be studied and supported for the cell management framework in 6GR.

Proposal 3: The support of at least a DL carrier in one FDD band associated with a UL carrier in another FDD band, and a UL/DL carrier in one FDD band associated with the carrier in another TDD band can be considered.

RRC state management in 6GR

In NR, there are three RRC states including RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED. Regarding RRC_INACTIVE state, it is introduced mainly for UE power saving as well as access latency reduction. For 6GR, it is also very important to consider the UE power saving and access latency reduction requirements. An intermediate state between RRC_IDLE and RRC_CONNECTED is still needed in 6GR.

In last RAN2 meeting, the following high level agreements were reached on RRC state management in 6GR.

Agreements on 6GR RRC state in RAN2#131bis:

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

As agreed in last meeting, whether to support Inactive mode or connected sub-state is open for now. Considering the lessons from NR, the RRC_INACTIVE state in 5G is not widely commercialized. The potential concerns maybe the actual benefit or performance gains brought by RRC_INACTIVE state. In order to introduce an intermediate state that is worthwhile to be used in commercial 6G NW, further enhancement shall be considered to avoid unnecessary state transition and signaling overhead, and to further improve balance of performance between UE power consumption and access latency.

In our opinion both directions (enhancement on inactive mode or introducing connected sub-mode) could be further studied and there is no hurry to exclude any one at the beginning of 6G SI. It is better to discuss the functionality to be supported at first.

Regarding UE based mobility, there are different potential solutions such as cell selection/reselection for enhanced Inactive mode or cell switch based on preconfigured conditions and/or RRM measurement for connected sub-mode. Regarding latency reduction, there are several aspects could be considered, such as UL access latency reduction through enhanced RA or configured grant for fast UL transmission, etc.. Besides, Measurement related latency such as early measurement could be considered as well. Regarding energy efficiency and signalling reduction, paging or wake-up simplifications/enhancements could be considered.

Proposal 4: Necessary enhancement on the intermediate RRC state (enhanced Inactive mode or connected sub-state) in 6GR shall be considered to have more performance gains on UE power saving and access latency reduction.

- Conclusion

In this contribution, we discuss control plane aspects in 6GR including cell management, RRC state management, system information and paging. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding the cell management in 6GR, a new framework supporting a single “Super cell” with multiple carries could be studied for more efficient spectrum utilization and further reduction of the latency and the system overhead.

Proposal 2: The flexible association between UL and DL carriers could be studied and supported for the cell management framework in 6GR.

Proposal 3: The support of at least a DL carrier in one FDD band associated with a UL carrier in another FDD band, and a UL/DL carrier in one FDD band associated with the carrier in another TDD band can be considered.

Proposal 4: Necessary enhancement on the intermediate RRC state (enhanced Inactive mode or connected sub-state) in 6GR shall be considered to have more performance gains on UE power saving and access latency reduction.

- Reference

RP-251881	Study Item: Study on 6G Radio