3GPP TSG-RAN WG2 Meeting #132	R2-2508540

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.2.1

Source:	Sony

Title:	Discussion on 6GR UE capability

Document for:	Discussion

# 1 Introduction

At RAN2#131bis, the following agreements were made on UE capability [1].

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this paper, we identify some issues to be addressed in 6G UE capabilities study and propose the way forward on how to proceed, especially on dynamic capability and RACS.

# 2 Discussion

Small size of UE capabilities is always preferred to avoid signalling overhead but on the other hand, large size allows UE capability flexibility. The right balance is needed between both extremes and we propose RAN2 to study UE capability size.

In 5G a UE capability update framework, based on UE Assistance Information, is defined. The UE Assistance Information may be sent by the UE when it prefers an adjustment of the parameters from the NW. This can be based on overheating, adjustment of DRX timing etc.

The UE capability framework in 5G is based on the UE Capability is sent at registration when the NW requests it, then it is stored in the UE Context in RAN and/or the Core Network. Thereby it is available in all RRC states and is moved between the nodes in the network when the UE moves. The problem with this solution is that the UE Capability is big, which makes it difficult to send from the UE to the NW at initial registration. This may in certain cases also delay the time until the UE is registered to the NW. Furthermore, the capabilities are filtered before sending them to the gNB and this filter is based on RAT type, supported frequency bands etc. When the UE moves to another cell the filtering is not forwarded to the new cell, thereby the new cell does not know what is missing and might need to request the capability again.

This needs to be solved in 6G since the capabilities will increase further as indicated above. Parts of the UE Capabilities may not be relevant for the actual network. Therefore, possible solutions of the capability framework should be studied. One proposal is to send a minimum set of capabilities during registration and more detailed capabilities in different areas when the UE or NW triggers more information on need basis.

Proposal 1	RAN2 to study the size of UE capabilities and how to define a UE capability framework which is more flexible and requires less signalling e.g. where only a smaller (minimum) set or of capabilities are sent initially.

It was agreed in RAN2#131bis meeting that to identify any use cases to justify dynamic capability change in RRC_CONNECTED mode. And we believe that due to the introduction of new features in 6GR, e.g., AI/ML, UE power saving, a UE may switch their preferences that are related to these new features in a more dynamic manner. For example, UE may switch between different AI/ML models according to its real-time data traffic or battery level, or a UE may switch to a different power saving plan according to its on-going service requirements. The characteristics of this preferences/information are as follows: 1) They are UE’s initiative. 2) They are potentially dynamic. Without this information being available to the network, the network may not be aware of UE’s preference/changing situation, which will lead to the degradation of user experience.

Once any of the UE side capability has changed, UEs should send the corresponding changes to the network and network should send confirmation/response according to these changes, so that UEs are aware about the network’s reactions to the capability changes.

Proposal 2	RAN2 to address potentially more dynamic changes of UE’s capability/preference, due to the introduction of new features and study dynamic capability.

RACS is an identity of a set of capabilities which maps to a set of capabilities which is stored in the NW. The first time a UE registers to the NW, it either sends the set of capabilities to the NW which responds with a RACS or the NW fetches the capabilities, corresponding to the RACS received from the UE or from a database.

RACS as defined today does not make the capabilities more dynamic as one RACS identity reflects a full set of capabilities. If something changes, a new set of capabilities needs to be identified with a new RACS identity. It does not limit the storage of the capabilities in the NW either, but it is used to reduce the signaling from the UE to the NW.

What could make the capabilities smaller and more dynamic is to separate the capabilities in smaller chunks with separate RACS identities, which could be updated separately. If the filtering of the capabilities is stored in the UE Context, the NW can request for the capabilities which were not requested before, the UE can also use the same signalling to potentially send an update of its capabilities based on a dynamic change of the UE capabilities.

Proposal 3	RAN2 to study whether the RACS framework can be improved to support a more dynamic capability handling between the UE and the gNB as well as within the network.

# 3 Conclusion

In this contribution, we propose:

Proposal 1	RAN2 to study the size of UE capabilities and how to define a UE capability framework which is more flexible and requires less signalling e.g. where only a smaller (minimum) set or of capabilities are sent initially.

Proposal 2	RAN2 to address potentially more dynamic changes of UE’s capability/preference, due to the introduction of new features and study dynamic capability.

Proposal 3	RAN2 to study whether the RACS framework can be improved to support a more dynamic capability handling between the UE and the gNB as well as within the network.

# 4 References

- Minutes RAN2#131bis, Prague