3GPP TSG-RAN WG2 Meeting #132	R2-2508542

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.3.1.1

Source:	Sony

Title:	Discussion on 6GR user plane protocol aspects

Document for:	Discussion

# 1 Introduction

At RAN2#131bis, the following agreements were made on user plane [1].

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this paper, we share our views on user plane protocol stack, handling of ACK type of traffic, and header format enhancement.

# 2 Discussion

## 2.1 User plane protocol stack

LTE user plane protocol stack has PDCP, RLC, and MAC sublayers. These sublayers were also present in NR protocol stack with the addition of SDAP protocol stack. Some detailed changes were introduced to each protocol layer while migrating from LTE to NR. We think that 6GR should reuse NR 5G UP and rel19 could be the baseline. Enhancements to each protocol layer based on the services need to be supported for 6G can be studied.

Apart from the protocol stack, QoS handling, number of DRBs and Logical channels should also be studied on top of NR. For example, RAN2 should study defining a realistic number of DRBs. Do we need to reserve LCID space for a number of DRBs which are never used etc. To begin with, NR rel-19 user plane should be the baseline.

Proposal 1	NR rel-19 user plane protocol stack should be the baseline for 6GR. RAN2 should study enhancements related to each protocol sublayer and other user plane topics depending on the type of services to be supported for 6G.

## 2.2 Handling of ACK type of traffic

6G will support different services but one aspect is the amount of traffic generated by ACK type of messages. This traffic can be generated at different layers like e.g. application-level TCP ACK or RLC-AM ACK or even PDCP status report. In order to reduce or optimise the ACK traffic, we see that there are two possible solutions on high level:

- No ack or ack correlation from other layer(s)

- Fake ack generation

For No ack or ack correlation from other layer, Asymmetric RLC configuration was discussed during rel-19. gNB will provide UL grant and UE will perform uplink transmission related to this UL grant. If gNB did not receive uplink RLC packet corresponding to an UL grant, then gNB can correlate between layers and ask for retransmission in HARQ layer. If cross layer ack correlation can work out then RLC-AM mode for uplink is not required and RLC-UM mode can be configured for uplink. For DL, RLC-AM is needed as it may be difficult to predict the reception at the UE.

For fake ack generation, instead of transmitting e.g. TCP ACK, a PDCP or RLC SN status report can be used over the air interface and a fake TCP ACK may be generated between PDCP and application layer for both UL and DL. For DL, UE PDCP layer generates ACK/NACK based on PDCP or RLC or HARQ status. For uplink, gNB may generate and send it to UPF like entity.

Proposal 2	 RAN2 to study handling of ACK type traffic using cross layer coordination.

## 2.3 Adaptive header format

NR user plane is of SDAP, PDCP, RLC and MAC sublayers where each layer performs its functions, with specified e.g., PDU format, PDU header, MAC subheader. Take the RLC AMD PDU header as an example, it contains a D/C, a P, a SI, and a SN. It provides necessary control information regarding the RLC PDU while on the other hand, it introduces control signalling overhead over the air interface. Due to the changing characteristics of the radio link quality as well as on-going service requirements, together with newly introduced features, RAN2 should study the need for a more adaptive format by considering the trade-off between effective user plane PDU delivery and control signalling overhead reduction.

It was agreed in RAN2#131bis that UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. On the other hand, considering the diversity of UEs with different capabilities to be supported in 6G, for some of the UEs with advanced capability on hardware processing as well as larger memory capacity, e.g., FWA or XR devices, network may provide an option for them of a more flexible protocol processing scheme.

One example of a flexible protocol processing could be on SN field of RLC/PDCP header. In legacy NR, both RLC and PDCP in L2 protocol will perform SN assignments and there is one-to-one correspondence between RLC and PDCP SDU. To allocate SN in both RLC and PDCP layer will not only increase the processing complexity, but also reduce the radio resource efficiency. Therefore, studying how to allocate SN in a more flexible and efficient way is necessary.

Proposal 3	RAN2 to study the need for adaptive user plane PDU format by considering the trade-off between effective user plane PDU delivery and control signalling overhead reduction. The study can start from a flexible and efficient SN allocation scheme.

# 3 Conclusion

In this contribution, we propose RAN2 to discuss and agree to the following proposals:

Proposal 1	NR rel-19 user plane protocol stack should be the baseline for 6GR. RAN2 should study enhancements related to each protocol sublayer and other user plane topics depending on the type of services to be supported for 6G.

Proposal 2 	RAN2 to study handling of ACK type traffic using cross layer coordination.

Proposal 3	RAN2 to study the need for adaptive user plane PDU format by considering the trade-off between effective user plane PDU delivery and control signalling overhead reduction. The study can start from a flexible and efficient SN allocation scheme.

# 4 References

- Minutes RAN2#131bis, Prague.