3GPP TSG-RAN WG2 Meeting #132	R2-2508547

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.3.3.1

Source:	Sony

Title:	Views on 6GR data transfer and AIML

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 a study item was agreed, Study on 6G Radio [1] and updated at RANP#109 [2], with the following objective related to data collection and AI/ML.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

At RAN2#131bis, the following agreements about AI/ML and data transfer were made [3].

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

For next meeting companies can consider at least the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

  - For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting and total data size

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this contribution, we would like to provide our views on 6GR common user plane and control plane in terms of possible enhancements of data transfer and AIML framework.

# 2 Discussion

## 2.1 General LCM and data collection framework

The AI/ML functions in 5G-Advanced rely on offline training, where models are trained on large datasets in the cloud and then deployed to the network. While useful, this approach has its limitations in more advanced 6G use cases, such as Integrated Sensing and Communication (ISAC), AI agents and extreme mobility. To unlock the full potential of 6G to fulfill a continuous, real-time learning loop where AI models/agents (at the UE and gNB) exchange data to refine their models, online training framework (e.g. reinforcement learning) should be supported, where models adapt to the live network environment. Different from offline training, online training processes data incrementally from distributed data sources. The size of data in a single report could be much less than that of offline training, but the frequency of reporting will be higher. For example, online learning for proactive resource allocation may update intermediate gradients at scheduling intervals. Therefore, a data transfer mechanism should be designed considering this data characteristics of high frequency and small size.

Both the UP based and CP based solutions for data collection have been studied in 5G-Advanced. A new SRBx with low priority for AI/ML data collection is agreed in Rel-19. This new SRB can satisfy the less time-sensitive data collection for offline training but may not satisfy the requirements for online training in 6G. 5G DRBs are tailored for the QoS needs of the user’s application. The 5G QoS framework is designed to differentiate between applications like "video," "voice," and "internet traffic." However, AL/ML training data has unique characteristics compared to traditional user plane data. The legacy user plane solution lacks the granularity to understand and prioritize different types of AI/ML data, such as distinguishing between raw sensor data, processed features, or model update gradients. A DRB is "data-agnostic", i.e. it treats all data within it the same. It cannot intelligently peek inside the data pipeline to perform in-network processing or aggregation. It doesn't understand the content or purpose of the data it carries. For AI/ML use cases such as AI agents, metadata (e.g., data source, timestamp, sensor type, feature vector, model gradients) is crucial for training, but 5G treats it as just another part of the payload.

The design of 6G data plane can be semantic-aware: The data plane can be able to understand the meaning and intent of the data it carries. For example, it can distinguish between a raw image, a feature vector extracted from that image, or a model weight update, and then apply different policies for routing, caching, or prioritization. As billions of devices report data, the sheer volume of MDT information can be overwhelming for a network. The data transmission based on the control plane is not suitable for transmitting large amounts of data, leading to inefficient data transmission. The legacy measurement data collection is periodic or event-triggered, which may not satisfy the requirements of on-demand data retrieval of AI agents. We propose to study a new data collection framework for AI/ML functions considering the uniqueness of AI/ML traffic. Therefore, in addition to UP-based and CP-based data collection solutions, it is also suggested to evaluate whether a dedicated AI/ML data collection solution is beneficial.

Proposal 1	RAN 2 to study life cycle management to support online training.

Proposal 2	RAN2 to study whether an application-aware/data type-aware AI/ML data collection solution is needed except for UP-based and CP-based solutions.

## 2.2 Types of AI/ML related data to be considered

Some new 6G cases (e.g. AI agent service, AI task offloading) require coordination of two-side models. Except for transferring L1 measurement data collected for training, other assistance information may also be needed for efficient coordination. For example, due to data distribution mismatch of training data and inference data, a model needs to be retrained. Instead of retraining and transferring the entire model, NW-side and UE-side can exchange the trainable parts of standardized model or the statistical distribution characteristics of latent representation, or a quantization codebook as assistance information.

6G systems may operate in highly dynamic environments (e.g., moving users, changing channel conditions etc.), where data distributions evolve over time. Traditional offline trained models, once deployed, remain static until manually retrained, making them less suitable for real-time 6G services. Online training allows models to continuously adapt to data distribution drift. If the data distribution drift of one side can be known by the other side of model, it would be beneficial to improve the performance and efficiency of two-sided model.

Proposal 3	Support transfer of assistance information (e.g. quantization codebook, data distribution drift) in data transfer design to efficiently manage two-sided models.

# 3 Conclusion

In this contribution we have made the following proposals related to data transfer and AIML.

Proposal 1	RAN 2 to study life cycle management to support online training.

Proposal 2	RAN2 to study whether an application-aware/data type-aware AI/ML data collection solution is needed except for UP-based and CP-based solutions.

Proposal 3	Support transfer of assistance information (e.g. quantization codebook, data distribution drift) in data transfer design to efficiently manage two-sided models.

# 4 References

- RP-251881. New WID: Study on 6G Radio

- RP-252912. Revised SID: Study on 6G Radio

- R2_131bis_Chairnotes, Prague.