3GPP TSG-RAN WG2 Meeting #132	R2-2508551

Dallas, USA, 17th-21st Nov, 2025

Title: 	LP-WUS RRC Issues (PO-to-LO association configuration, H050/E043)

Source: 	Huawei, HiSilicon

Agenda item:	8.4.2

Document Type:	Discussion

# 1. Introduction

We discuss the following LP-WUS issues in the contribution:

- Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less? [2]

- UAI RIL H050/E043

# 2. Discussion

## 2.1 On paging PO-to-LO association configuration [2]

As the reply LS to [1], SA2#171 meeting agreed the LS on the allocation of CN assigned subgroup ID for LP-WUS, with the following contents [2].

1. Overall Description: SA2 thanks RAN3 for the LS (S2-2508191/R3-255941) on allocation of CN assigned subgroup ID for LP-WUS. The LS highlights the following: # Overall description Regarding the range of CN assigned subgroup ID for LP-WUS, RAN3 made the following agreements: - The range of CN assigned subgroup ID in NG/Xn/F1 message is 0.. 30. This agreement means that when the AMF allocates CN assigned subgroup ID for the UE, it should consider the least range of subgroup ID available over NG-RAN cells corresponding to the UE registration area. RAN3 would like to provide following background information: according to the TS 38.213, for CN assigned subgroup ID range, dependent on the per-cell PO (Paging Occasion) to LO (LP-WUS Occasion) association: - in case PO-to-LO association is 4, subgroup ID #0 to #6 can be used;. - in case PO-to-LO association is 2, subgroup ID #0 to #14 can be used; - in case PO-to-LO association is 1, subgroup ID #0 to #30 can be used; RAN3 would like to ask SA2 to take the above into consideration, and discuss the potential specification impact, if any. # Actions To SA2: ACTION: RAN3 would like to ask SA2 to take the above into consideration, and discuss potential specification impact, if any. SA2 discussed the issue and would like to receive some further information from RAN3 and RAN2: a) What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose. b) Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less? c) Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213. SA2 would appreciate replies to the above questions. 2. Actions: To RAN3, RAN2 ACTION: SA2 kindly asks RAN3 to provide answers for a, c) and RAN2 to provide answers for b).

Given the reply LS is talking about the per-cell PO (Paging Occasion) to LO (LP-WUS Occasion) association, below provides the specification details for reference.

- In TS 38.331 DownlinkConfigCommonSIB ::= SEQUENCE { <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> lpwus-MoNumPerLo-r19 ENUMERATED {n1, n2, n3, n4}, lpwus-PoNumPerLo-r19 ENUMERATED {po1, po2, po4} OPTIONAL, -- Need R lpwus-PoNumPerLo Indicates the number of PO(s) associated with one LO for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). Value po1 means 1 PO associated with one LO, value po2 means 2 POs associated with one LO, and so on.. - In TS 38.213: ## 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> - A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association.

It can be observed that the PO to LO association could take 1, 2 or 4 broadcasted in SIB1 as a per Cell information. This means that the cell can broadcast its own association, while UEs monitoring its own PO could monitor the associated LO accordingly.  Also as indicated in the [1], the CN subgroup ID can take the following values dependent on the PO-to-LO association.

- in case PO-to-LO association is 4, subgroup ID #0 to #6 can be used;

- in case PO-to-LO association is 2, subgroup ID #0 to #14 can be used;

- in case PO-to-LO association is 1, subgroup ID #0 to #30 can be used;

- Observation 1: The PO-to-LO association taking 1, 2, 4 is broadcasted as per cell information, which will lead to different the CN subgroup ID value range.

- For question a) on the typical area that a PO-to-LO association configuration, as discussed above this is per cell broadcast information. However, this association cannot be configured on a cell level basis. One example is that if the UE moves from a cell with PO-to-LO association as one to a cell where the association is four, then the CN subgroup ID larger than 6 cannot be used, which leads to defective LP-WUS paging. Hence a reasonable assumption is that the configuration should be pre-configured in a large area. This configuration could be up to operator's policy or up to implementation, for example, at least at the tracking area list level.

- For the question b) above, as discussed above, it is not expected that the paging PO-to-LO association configuration will be changed often. Even if it happens, the PO-to-LO association configuration should be changed for all cells within the preconfigured area.

- Proposal 1: From RAN2 point of view, it’s not expected that the paging PO-to-LO association configuration will be changed often. Even if it happens, the PO-to-LO association configuration should be changed for all cells within the preconfigured area.

## 2.2 UAI RIL H050/E043

In last RAN2 meeting, the RIL H050/E043 was discussed but no conclusion was reached.

Based on the analysis in our contribution [3], the NW only knows the measurement results from MR based on existing RRC measurement report. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference of LR. Only relying on MR measurement results for LP-WUS configuration for CONNECTED UEs may lead to LP-WUS missing and data loss.

To avoid the worse LP-WUS performance in this case, it is beneficial to assist network for proper configuration since the UE is aware of the situation of LR. The UE can inform the network to stop using LP-WUS, or indicate whether the LP-WUS can be used again if it finds that the LR is recovered. For example, the UE is monitoring LP-WUS and measures LP-SS/SSB by LR, once it finds that the LR link quality is not good enough, the UE switches to MR and sends assistance information to the NW to disable the LP-WUS.

One of the controversial issues in last meeting is the condition for UAI reporting. One way is up to UE implementation without additional trigger conditions/thresholds, but there is concern for performance from network vendor. As alternative, the trigger threshold(s) (e.g., above the configured threshold(s)) can be defined if there are no additional RAN1/4 impacts since the WI is complete.

Proposal 2: UE can send UAI to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again. The condition for UAI transmission can be discussed:

- Option 1: up to UE implementation, without additional trigger conditions/thresholds;

- Option 2: defining trigger threshold(s) (e.g., above the configured threshold(s)) without additional RAN1/4 impacts.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Proposal 1: From RAN2 point of view, it’s not expected that the paging PO-to-LO association configuration will be changed often. Even if it happens, the PO-to-LO association configuration should be changed for all cells within the preconfigured area.

Proposal 2: UE can send UAI to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again. The condition for UAI transmission can be discussed:

- Option 1: up to UE implementation, without additional trigger conditions/thresholds;

- Option 2: defining trigger threshold(s) (e.g., above the configured threshold(s)) without additional RAN1/4 impacts.

# 4. Reference

- R3-255941 LS on allocation of CN assigned subgroup ID for LP-WUS, RAN3.

- S2-2509834 Reply LS on allocation of CN assigned subgroup ID for LP-WUS, SA2.

- R2-2507331, [H050][H053][H054][H055] Discussion on LP-WUS RILs, Huawei, HiSilicon.