3GPP TSG-RAN WG2 Meeting #132	R2-2508552

Dallas, USA, Nov 17th – Nov 21st, 2025

Agenda item:	9.3.2

Source:	Qualcomm Incorporated

Title:	RRM measurement prediction for UE-side models

WID/SID:	NR_AIML_Mob – Release 20

Document for:	Discussion and Decision

# Introduction

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

- Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In the WID document for Rel-20 AI/ML for Mobility [1], for RRM measurement prediction for UE-side models, the following are the stated objectives:

In this contribution, we discuss the following aspects of RRM measurement prediction for UE-side models:

- UE capabilities,

- Prediction configuration and reporting,

- Applicability.

# UE capabilities

In this section, we discuss the UE capabilities for the various use cases in RRM measurement prediction: intra-frequency, temporal domain prediction Case A and Case B, and inter-frequency prediction.

## Intra-frequency, temporal domain prediction Case A and Case B

Proposal 1: The following are the minimum set of UE capabilities for intra-frequency, temporal domain prediction Case A and Case B:

- The maximum length of prediction window (PW), for which the UE can support RSRP measurement prediction.

- The list of frequencies (or ranges of frequencies) for which the UE can support RSRP measurement prediction (the list or the range can be provided per FR).

- The total maximum number of cells (serving and neighbor) for which the UE can support RSRP measurement prediction.

## Inter-frequency prediction

Proposal 2: The following are the basic UE capabilities for inter-frequency prediction:

- For each target (predicted) frequency, the range or list of input frequencies the UE needs to measure.

- Per frequency, the maximum number of neighbor cells the UE can support RSRP measurement prediction (the neighbor cells are all collocated with the serving cell).

- The total maximum number of cells (serving and neighbor) for which the UE can support RSRP measurement prediction.

1) expresses the fact that the measured frequency and the predicted frequency cannot be too far apart, otherwise the UE cannot predict, i.e., UE can support inter-frequency prediction up to a certain maximum difference of frequencies.

Measured

Predicted

Measured

Predicted

Figure 1: Prediction of collocated cells in a frequency based on measurements of cells in other frequencies

# Prediction configuration and reporting

## Measurement configuration

The following proposal holds for all the use cases: intra-frequency, temporal domain prediction Cases A and B, and inter-frequency prediction.

Proposal 3: For cell measurement prediction, for each serving cell and neighbor cell configured to be predicted, the network configures the UE with the required measurement objects based on the UE prediction capabilities.

For RRM prediction, UE-sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details.

The following agreement was reached in RAN2#129-bis [2].

For event-triggered reporting of predicted and/or actual RRM measurements, we don’t think any additional reporting is needed than those associated with Measurement Event prediction, which can contain RRM measurement predictions. Unnecessary reporting involves additional UE processing and complexity and signaling overhead.

Proposal 4: RRM prediction reporting can be configured with existing events (A1-A6) and with the predicted events (pA1-pA6) that will be defined. No additional events are needed.

## Intra-frequency, temporal domain prediction Case B

1.	For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

The following agreement was reached in RAN2#131 [3].

In what follows, we clarify the network SSB transmission pattern configuration.

Observation 1: For a provided SSB transmission pattern configuration for a cell, it is up to UE implementation whether it can achieve additional measurement reduction due to measurement prediction, while satisfying RAN4 requirements on prediction accuracy and measurements.

Proposal 5: The network configures its SSB transmission pattern based on its constraints such as existing legacy UEs and network energy savings considerations. For temporal domain Case B prediction, the UE skip measurement pattern is left to the UE implementation.

# Applicability

2	Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

The concept of an Associated ID was introduced and specified in Rel-18/Rel-19 AI/ML for Beam Management. This topic was discussed in the Rel-19 AI/ML for Mobility SI, and the following agreement was reached in RAN2#130.

We think that many of the aspects agreed for Rel-18/Rel-19 AI/ML for Beam Management can be carried over for AI/ML for Mobility.

While the Rel-18/Rel-19 AI/ML for Beam Management focused on predictions per cell, the AI/ML for mobility focuses on mobility across cells and thus a different approach should be taken when it comes to associated IDs.

There have been two main approaches used by companies for AI/ML for mobility:

- Per cell prediction: Where prediction was done per cell and that’s the base for indirect event prediction.

- Per cluster prediction: Where prediction was done per group of cells, and that’s the basis for direct event prediction.

Since the associated ID of the AI/ML for beam management can still be used for predictions per cell, the associated ID of AI/ML for mobility should describe the relative deployment of cells in the area compared to each other.

Proposal 6: While the network can only configure the UE with an associated ID per cell, the network implementation should allocate the same associated ID to the largest number of contiguous cells. Additionally, if the network does not provide an associated ID for AI/ML for mobility, it is up to UE implementation to determine the applicability of the prediction configuration.

Proposal 7: The following are a few agreements from Rel-19 AI/ML for beam management WI that apply for AI/ML for mobility.

1. No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

2. Applicability reporting is added in RRCResumeComplete for prediction configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

3. RRCReconfigurationComplete shall include applicability/inapplicability status for:

a) All prediction configurations included in the immediately preceding RRCReconfiguration message.

b) Any previously configured prediction configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

4. Applicability reporting via UAI is always enabled.

5. During mobility, the target gNB can check the applicability by including target cell prediction configurations in the handover command message, i.e., RRCReconfiguration, and the UE can respond with RRCReconfigurationComplete.

A few remarks regarding the proposal above:

Regarding 1), since UE is expected to release all prediction configurations for AI/ML for Mobility upon RRC re-establishment, there arises no need to report applicability in RRCReestablishmentComplete.

Regarding 5), it is assumed that UE can process the target cell prediction configurations in the RRC reconfiguration message without additional RRC processing delay, since otherwise there is an impact on the handover timeline.

# Conclusions

Proposal 1: The following are the minimum set of UE capabilities for intra-frequency, temporal domain prediction Case A and Case B:

- The maximum length of prediction window (PW), for which the UE can support RSRP measurement prediction.

- The list of frequencies (or ranges of frequencies) for which the UE can support RSRP measurement prediction (the list or the range can be provided per FR).

- The total maximum number of cells (serving and neighbor) for which the UE can support RSRP measurement prediction.

Proposal 2: The following are the basic UE capabilities for inter-frequency prediction:

- For each target (predicted) frequency, the range or list of input frequencies the UE needs to measure.

- Per frequency, the maximum number of neighbor cells the UE can support RSRP measurement prediction (the neighbor cells are all collocated with the serving cell).

- The total maximum number of cells (serving and neighbor) for which the UE can support RSRP measurement prediction.

Proposal 3: For cell measurement prediction, for each serving cell and neighbor cell configured to be predicted, the network configures the UE with a set of beams and RSs to measure based on the UE prediction capabilities.

Proposal 4: RRM prediction reporting can be configured with existing events (A1-A6) and with the predicted events (pA1-pA6) that will be defined. No additional events are needed.

Observation 1: For a provided SSB transmission pattern configuration for a cell, it is up to UE implementation whether it can achieve additional measurement reduction due to measurement prediction, while satisfying RAN4 requirements on prediction accuracy and measurements.

Proposal 5: The network configures its SSB transmission pattern based on its constraints such as existing legacy UEs and network energy savings considerations. For temporal domain Case B prediction, the UE skip measurement pattern is left to the UE implementation.

Proposal 6: While the network can only configure the UE with an associated ID per cell, the network implementation should allocate the same associated ID to the largest number of contiguous cells. Additionally, if the network does not provide an associated ID for AI/ML for mobility, it is up to UE implementation to determine the applicability of the prediction configuration.

Proposal 7: The following are a few agreements from Rel-19 AI/ML for beam management WI that apply for AI/ML for mobility.

1. No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

2. Applicability reporting is added in RRCResumeComplete for prediction configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

3. RRCReconfigurationComplete shall include applicability/inapplicability status for:

a) All prediction configurations included in the immediately preceding RRCReconfiguration message.

b) Any previously configured prediction configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

4. Applicability reporting via UAI is always enabled.

5. During mobility, the target gNB can check the applicability by including target cell prediction configurations in the handover command message, i.e., RRCReconfiguration, and the UE can respond with RRCReconfigurationComplete.

# References

[1] RP-252899, WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Oppo, Interdigital).

[2] Chair notes, RAN2#129-bis meeting.

[3] Chair notes, RAN2#131 meeting.