3GPP RAN WG2 Meeting #132	R2-2508558

Dallas, USA, 17th-21st Nov., 2025

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon

Title:	Summary of A-IoT MAC open issues (outcome of [POST131bis][017][AIoT] MAC CR)

Document for: Discussion and Decision

# Introduction

The following document includes a list of open issues and the suggested resolutions.

# Contact information

Please provide your contact information.

| Company | Name | Email |
| --- | --- | --- |
| Lenovo | Jing HAN | hanjing8@lenovo.com |
| NEC | Zonghui XIE | xie_zonghui@nec.cn |
| ZTE | Ying HUANG | huang.ying11@zte.com.cn |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| CATT | Lei Wang | wanglei24@cictmobile.com |

# Remaining open issues for specification 38.391

## List of the open issues and type of issue

After RAN2#131bis meeting discussion, most of the open issues listed in R2-2507030 have been addressed, only 2 issues are left:

- Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field (Issue 3-7)

  - Status: the following agreements have been achieved, further discussion is expected to address the FFS point which may also relate to SA3/CT1 reply LS.

  - Agreements:

    - RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

    - For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). For integrity failure, for now RAN2 assumes that there is no AS response to the reader.

    - Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

- Paging ID length impact from new SA2 LS in S2-2507793 (Issue 1-3)

  - Status: Wait for SA2 conclusion before finalizing length ID field size

The above 2 issues (issue 3-7, issue 1-3) can be further discussed based on companies’ contributions.

# Other open issues if identified

## Table: Collection of remaining open issues

### Companies are invited to describe any other identified open issues not currently included within this document. Please note for the editorial suggestions/minor issues, as per chairlady’s guidance, companies are expected to give editorial inputs to the rapporteurs via email. No need to repeat those editorial issues here.

| Company | Other identified open issues? (please describe) |
| --- | --- |
| Lenovo | CT1 agreed an LS (C1-256624) to RAN2, which is LS on handling of inventory and command collision, that CT1 observed a new attack scenario (false paging) for parallel paging and ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens. RAN2 may need to discuss whether to consider this issue in R19 or R20, and what is the conclusion from RAN2 point of view. Rapp1: Thanks for the comments. I agree that RAN2 needs to have some discussion and reply LS to CT1. I will include this in the issue list. New issue 6-1: Whether there is RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision. |
| NEC | The timing between CFA R2D and responding D2R message is discussed in RAN1. During the discussion, time consumption for authentication is not considered. As a consequence, the timing between CFA R2D and responding D2R message (is about 200us for some chip duration based on TS38.291) is too small for authentication, that there may be no upper layer data, i.e., inventory report NAS message, generated by upper layer at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message. However, even the delayed inventory report message is generated later, it will not be retransmitted to reader based on current RAN2 MAC procedure. In such case, D2R Upper Layer Data Transfer message retransmission due to delayed upper layer data available for initial transmission should be supported for CFA inventory. The retransmission could be scheduled by R2D Upper Layer Data Transfer message. The following TP could be considered to solve this issue. 5.4.3 R2D message reception Upon reception of an R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall: 1> if the device has a stored AS ID and the R2D Upper Layer Data Transfer message is addressed to the device (i.e., the value of AS ID field is identical to the stored AS ID): 2> if the Choice Indication field indicates that the Data SDU field is included (i.e., CI field set to 1): 3> forward the upper layer data SDU in the Data SDU field to upper layers; 3> initiate the following D2R message transmission, as specified in clause 5.4.2; 2> else if the Choice Indication field indicates that the Received Data Size field is included (i.e., CI field set to 0): 3> if the Received Data Size field is set to 0: 4> initiate the D2R message transmission procedure as specified in clause 5.4.2; 3> else: 4> initiate the D2R segmentation procedure using this information as specified in clause 5.4.4; 1> else if the device has no stored AS ID, and if CFA procedure has been performed in the current procedure: 2> if the Choice Indication field indicates that the Data SDU field is included: 3> set AS ID to the value indicated by the AS ID field and store the AS ID; 3> forward the upper layer data SDU in the Data SDU field to upper layers; 3> initiate the following D2R message transmission, as specified in clause 5.4.2. 2> else if the Choice Indication field indicates that the Received Data Size field is included and the Received Data Size field is set to 0: 3> set AS ID to the value indicated by the AS ID field and store the AS ID; 3> perform the D2R message transmission procedure as specified in clause 5.4.2.. Rapp3: In the previous RAN2 discussion, the no data available case with 0 SDU becomes a more generic solution to cover any potential case with no NAS response. In this case, including the proposed change seems to be no harm, considering the change is only to procedure text and is quite straightforward. To save online discussion time, the rapporteur would like to suggest to agree the intention, but if there is other views we could have more discussion the meeting. New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message. |
| ZTE | For the Paging ID Type indication in paging message, the status captured in chairnote is “wait for SA2 and discuss in November”, which means this issue needs to be discussed in next meeting. And in last SA2 meeting, there is no conclusion regarding the paging ID type indication. Based on TS 33.369, it is specified clearly that NG-RAN includes the T-ID handling in the paging message, wherein the T-ID handling information is received from AIOTF. And the T-ID handling information indicates whether the T-ID is concealed type or stored type, and whether the stored T-ID type shall be updated with or without a command. TS 33.369: 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory ...... - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. Besides, based on the agareed SA2 CR S2-2509652, different behavious are defined for different types of paging IDs (i.e. filter information, temporary id, permanent ID), as copied in the below. So paging ID type needs to be informed to device so that device can differentiate the type of paging ID. 9. Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300 [5] and TS 38.391 [11]. The RAN Reader(s) broadcast the paging message that includes the AIoT Device Identification Information and the security parameters. If the received AIoT Identification Information contains: - Filtering Information, the AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8. - AIoT Device Temporary Identifier, the AIoT Device determines whether it matches the AIoT Identification Information, as described TS 33.369 [9]. - AIoT Device Permanent Identifier, the AIoT Device determines whether it matches the AIoT Identification Information by comparing it with the stored AIoT Device Permanent Identifier. So it would be good to add “paging ID type indication in paging message” as an open issue to be discussed in next meeting. [Futurewei]: We have a question for ZTE - other than enabling the message format optimization that you propose in the next entry, from functionality PoV, do we have to handle this type indication at the A-IoT MAC layer? Why can’t we do it in the A-IoT NAS layer? Will the MAC layer treat the different types differently, e.g., in term of forwarding or not forwarding it to the NAS layer? Thanks. From device complexity PoV, the MAC processing would be more streamlined if the device just needs to: 1) determine whether the Paging ID field is present or not (so as to determine whether the MAC layer alone can determine that the device is being selected); and 2) if the Paging ID field is present, forward the content in the Paging ID field (with the assistance of length information) to the upper layer for further processing. [ZTE] Thanks for your reply. Actually the type indication doesn’t need to be processed at A-IoT MAC layer. In our view, even if type indication is included along with paging ID, the MAC layer just deliver it the upper layer along with the paging ID and the security parameter. And indeed it is also feasible to indicate the type in NAS layer. However, as we can see, currently all NAS information (i.e. paging ID, security parameter) in paging message is visible at A-IoT MAC layer, i.e. not encapsulated as upper layer data SDU. So no matter whether the type is indicated in NAS or AS layer, there is no much difference to the device complexity. The motivation of our proposal is that it seems the type needs to be included in AS layer by NG RAN node in the paging message based on the SA3 TS. And we are no sure whether SA2 and CT4 would update the format of device permanent ID and filter information for this. Another concern is that we have an overal guidance from the Chairlady that “After December, NBC changes should be avoided as much as possible similar to NR MAC process.”. But we have no strong view and are ok to do it in NAS layer if companies think it would be more appropriate. [CATT]: thanks for the good discussion from Futurewei and ZTE. On this topic, my understanding is SA2 had sent an LS (refer to S2-2509655) to CT4 (CC RAN2). As per my SA2 colleague, the intention of this LS hopes CT4 to update the device ID format so that these ID types can be differentiated by the NAS format. So from ran2 perspective, we can further wait for CT4 progress. Rapp3: Thanks to ZTE for reminding us of this, and thanks to others for the good discussion. Indeed, RAN2 would better to conclude this aspect in Nov meeting to avoid potential NBC changes in future. From technical perspective, we have the same understanding as Futurewei and CATT. According to SA2 newly agreed CR embedded in LS S2-2509655, the AIoT Identification Information is supposed to an umbrella upper layer ID which also covers Temp ID. In this case, there is no need to involve AS layer to further hander Temp ID other than just forwarding this AIoT Identification Information to upper layer. Companies are encouraged to check with SA2 and CT4 colleagues, and in Nov meeting we can try to confirm whether there is RAN2 impact or not when treating the SA2 LS. New issue 6-3: Whether there is AS impact to support Temp ID, e.g., including upper layer ID type in paging message. Or, assume CT4 will address this based on SA2 LS S2-2509655. |
| ZTE | In current AIOT MAC running CR, the paging ID length field shall be included if paging ID Paging ID field is present. - Paging ID Length: This field is optionally present, as indicated by Paging ID Presence Indication field. If present, the length of the field is 8 bits. This field indicates the length of the Paging ID field in unit of bits when Paging ID field is present. However, if the paging ID is device temporary ID, the paging ID length field seems not needed, if the length of device temporary ID is fixed. And based on the agreed CT4 CR C4-254398, the length of device temporary ID is 32 characters as copied in the below: Table 6.1.6.3.2-1: Simple data types Type Name; Type Definition; Description; Applicability. AfId; string; Identifier used to identify the AF.. Tid; string; pattern: "^[A-Fa-f0-9]{32}$". In our view, it would be benefical to allow that Paging ID Length field is absent when Paging ID field is present (e.g., when device temporary ID is included as paging ID), to avoid unnecessary overhead (i.e. around 10bits can be reduced if device permanent ID is extended as indicated by SA2 ). So we suggest to update the field description of Paging ID Length field as below, or add an open issue on the presence of Paging ID Length field: - Paging ID Length: This field is optionally present when the Paging ID is not AIoT Device Temporary Identifier, as indicated by Paging ID Presence Indication field. If present, the length of the field is 8 bits. This field indicates the length of the Paging ID field in unit of bits when Paging ID field is present. Rapp3: This is tightly related to last comment, i.e., whether AS needs to be aware of Temp ID. Based on SA2 TS, it appears the CN just send the AIoT Identification Information to the BS reader, in which case the reader behavior is same no matter it’s Temp ID or other IDs. |

# Conclusions

During this email discussion, companies identified new issue 6-1/6-2/6-3 which may require more discussion. Together with the left issues 3-7/1-3 from last meeting, now we have the following 5 issues to be discussed in Nov meeting. Companies are invited to inputs views in the companies’ contributions.

- Issue 3-7: Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field.

- Issue 1-3: Paging ID length impact from new SA2 LS in S2-2507793.

- New issue 6-1: Whether there is RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision.

- New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

- New issue 6-3: Whether there is AS impact to support Temp ID, e.g., including upper layer ID type in paging message. Or, assume CT4 will address this based on SA2 LS S2-2509655.