3GPP TSG-RAN WG2 Meeting #132	R2-2508560

Dallas, US, 17th –21st Nov., 2025

Agenda Item:	9.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT T2 discussion

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, RAN2 made some progress on A-IoT T2 design and achieved the following agreements.

- Excerpts from RAN2#131bis agreements on Rel-20 A-IoT - gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information - gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - the reader schedules the AIoT interface transmissions within the resources allocated by the gNB. Continue discussion next meeting on how resource allocation looks like.

In this contribution, we further discuss the following issues:

- A-IoT resource allocation,

- Information of AIOT procedure/messages over NR Uu,

- UE reader selection,

- UE reader authorization.

# 2	Discussion

## 2.1	A-IoT resource allocation

### 2.2.1	Resource allocation in normal connected state

In the last meeting, companies proposed some detailed points for A-IoT resource allocation/validity. However, during online discussion, especially for out of connection case, it seems companies have different assumptions on how the resource looks like and what would be the basic resource allocation procedure even for normal connected state. Therefore, we would like to first clarify the basic procedure of resource allocation for normal connected state.

Figure 2.1-1	A-IoT resource is redundant if configured timer value is larger than practical usage.

In general, radio resource is defined by frequency domain and time domain. For A-IoT interface, the frequency domain resource is likely to have a fixed location (e.g., configured several RBs or frequency bandwidth), which would be easily implement with detailed signalling, then we would like focus on time domain resource first, including how determine the resource is invalid and how to configure the time domain resource.

As shown in the above figure, there are 4 time points to determine when to use the allocated A-IoT radio resource to perform a service (taking inventory service as the example):

- Time point #1: when the radio resource is configured/activated by BS

- Time point #2: when the service starts, i.e., upon UE reader receiving service request-like info

- Time point #3: when the service stops/completes, i.e., upon UE reader receiving service stop/complete-like info

- Time point #4: when the radio resource is released/deactivated by BS explicitly by signalling or a timer

And there are two options how the resource looks like:

- Option A: one-shot time domain resource. In our understanding, one-shot resource allocation can be seen as one kind of continuous resource allocation with boundaries of resource validity.

- Option B: periodical resource with configured patten which is similar to the NR configured grant

From the figure, we can see that regardless of continuous/one-shot/periodical resource, the resource is to be used by the UE reader after both of time point #1 and #2, i.e., the resource is configured and the service is initiated, and the resource cannot be used after either resource is released (time point#3) or service is completed (time point#4). However, the ideal case would be the resource is released (time point#3) just upon or shortly after the time of service complete (time point#4). Otherwise, if the time point #4 is earlier than time point #3, the resource is not sufficient to complete the service, while if the time point #4 is later than time point #3, the resource after time point#3 is redundant and to be wasted.

Observation 1a:	If the resources are stopped before the service is complete, the resources are not sufficient to complete the service which harms the performance. And if the resources are stopped after the service is complete, there will be a waste of the radio resources. Therefore, accurate time duration is really the blocking point.

Regarding time point #4, the timer-based solution is a quite hot topic in previous discussion. However, we observed that if a timer is to be used, it’s difficult to predict how long the service is going to be between the UE reader and devices, because this is quite related to reader implementation, e.g., AO configuration, unicast msg2 or common msg2, retransmission or re-access. If insufficient resources are allocated, more service requests from UE reader will be needed. To avoid much more times of resource requests from UE reader, UE should provide information, during the A-IoT operations, to assist gNB allocate sufficient A-IoT radio resources to complete the following A-IoT operations. For example, the number of CBRA failures or Msg3 transmission failures is needed to determine how many resources are needed for the subsequent re-access, whether there are remaining devices executing ongoing commands (e.g., delayed commands), etc.

Observation 1b:	The timer cannot function independently, as it also relies on UE assistant information when the timer is configured shorter than the required service completion time.

On the other hand, it should be an easier solution to let network explicit release the resource, since in RRC based architecture, the service request and service complete messages on NG interface are visible to BS, by which the BS can know when the service starts or stops. For example, the service completion can be decided by UE reader or upon reception of session release indication. Referring to A-IoT NGAP procedure in TS 38.413, there are two mechanisms to release A-IoT session:

- (Triggered by NG-RAN) The gNB reader can request the A-IoT CN node to release the A-IoT session when the service is completed. So, the UE reader can send session release request the gNB and then the gNB transfers this request to CN. Then the UE reader can release the ongoing A-IoT resources.

- (Triggered by A-IoT CN) The A-IoT CN node can terminate A-IoT sessions to send AIOT SESSION RELEASE COMMAND to the gNB reader. In this case, the gNB can send end indication to UE reader to release the ongoing A-IoT resources when the session is released.

Proposal 1a:	Timer based A-IoT radio resource allocation is not supported.

Proposal 1b:	The baseline solution for releasing the A-IoT radio resources should be based on explicit release indication from BS.

In the last meeting, assistance information for resource allocation was discussed.

RAN2#131bis agreement 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation.

We only observed the assistance information is useful when timer-based solution is adopted. And if the resource is allocated and released explicitly by the network, there is no need of the UE assistance information.

Observation 2:	No other UE assistance information is needed apart from the required time duration if timer-based solution is adopted by RAN2.

As for the periodic resource allocation, the procedure might be interrupted in the middle, and not able to seamless resume from the breakpoint, due to the strict timing relationship between a R2D transmission and its corresponding D2R transmission, or device draining power. This introduces additional complexity to UE reader to avoid such issues.

Observation 3:	The periodic A-IoT resource allocation method introduces additional complexity to UE reader implementation (e.g., schedule the A-IoT transmission) considering the timing relationship defined in A-IoT interface.

Proposal 2:	The baseline A-IoT resource allocation method should be the continuous time domain resource allocation, i.e., the UE reader performs A-IoT operations using the continuous resource until the A-IoT service is complete or the resources are released.

### 2.2.2	Resource validity when temporary out of connection

According to Rel-20 RAN WID, when UE reader moves to a new cell, the A-IoT radio resource allocated by the last serving cell is not valid, in order to avoid interference between Uu and A-IoT interface.

Rel-20 RAN WID (RP-251885) The work is subject to the following constraints: … RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. … RAN2#127bis agreements: The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

Observation 4a:	As agreed, the A-IoT radio resources are valid only in serving cell. Using these resources in another cell may cause interference.

During a HO or RLF procedure, for simplification, the baseline should be that a UE reader stops the ongoing A-IoT service/procedure. Then, the new serving cell, i.e., the target cell after RRC re-establishment (due to HO or RLF), decides whether the A-IoT radio resources can be used. The network determines whether the suspended A-IoT service can continue.

Observation 4b:	Whether the same A-IoT service can continue in target cell after HO/new cell after re-establishment is decided by the target cell/new cell.

Proposal 3a:	To simplify UE reader behaviour, the UE reader does not transmit/receive new messages via A-IoT interface during HO/RLF initiated reestablishment, and how to handle this A-IoT service will be based on instructions from the target cell after HO/the new cell after reestablishment.

If the target cell/new cell is also performing an A-IoT service, it can allow the UE reader to continue performing an A-IoT service, i.e., re-start a new A-IoT procedure. However, if this UE reader wants to continue its ongoing A-IoT service in the target cell after HO/RLF, there needs an additional transfer of context (e.g., paging ID, correlation ID) between the last serving and new BSs. This will make the procedure more complex, e.g., which context needs to be transferred, and what the procedure is if the context transfer fails.

Observation 5:	If the UE reader’s ongoing A-IoT service continues in the target/new cell after HO/RLF, an additional context transfer is needed between the last serving cell and the target/new cell, making the procedure more complex.

If UE reader’s ongoing service does not continue when HO/re-establishment is triggered, the UE reader does not need to maintain the context.

Proposal 3b:	As a baseline, the UE reader releases the A-IoT service related configuration/context upon HO/RRC re-establishment.

## 2.2	Information of AIOT procedure/messages over NR Uu

### 2.2.1	General Uu RRC design

Regarding how to include the essential A-IoT service-related information in Uu RRC messages, there are several options:

- Option 1: Define new RRC messages corresponding each NGAP message. For instance, one new RRC message for inventory request, one new RRC message for inventory report, and so on. In this way, at least 7 new RRC message are required.

- Option 2: Introduce two new RRC messages, one for UL and the other for DL, and IE level signalling can be defined for each NGAP message. For instance, inventory request related info will be included in a IE which is contained in the DL RRC message, and inventory results will be included in another IE which is contained in UL RRC message.

The two options have no essential difference, and option 2 will consume less RRC message, so we slightly prefer option2.

Proposal 4a:	For topology-2, introduce two new Uu RRC messages, one for UL and the other for DL, in total between UE reader and BS, and inside the RRC message, each IE-level signalling is corresponding each NGAP message.

Then whether there should be a new SRB to transfer the A-IoT RRC messages has been raised as well. We feel no strong motivation to have a new SRB, so we prefer to reuse the current SRB, e.g., SRB2 or SRB4.

Proposal 4b:	Reuse the existing SRB to carry the new A-IoT-related RRC messages.

### 2.2.2	Inventory Information over NR Uu

Based on the discussion in the last meeting, RAN2 need to identify what information is essential to be included in the NR Uu transmission, based on NGAP AIoT information from CN:

RAN2#131bis agreement 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information

Actually, the similar work is also needed to identify the essential AIOT NGAP information from gNB to CN, i.e., UL direction is also needed while the conclusions included DL only.

Observation 6:	When studying new information to be introduced in the RRC messages for T2, RAN2 uses the AIOTF Related IEs defined in NGAP (UL/DL) as a baseline.

For inventory procedure, the AIOT operations and involved corresponding IEs are defined in TS 38.413:

Figure 8.20.1.2-1: Inventory Request, successful operation. Figure 8.20.1.3-1: Inventory Request, unsuccessful operation.

According to clause 9.2.19 in TS 38.413, the information involved in the above messages is list as follows:

- INVENTORY REQUEST:

  - Message Type;

  - AIOTF Identifier;

  - A-IoT Correlation Identifier;

  - Inventory Request Transfer:

    - A-IoT Correlation Identifier;

    - A-IoT Device Identification Requested: CHOICE Device Identification for Paging among Single Device, Group Devices, and All Devices);

    - Requested Service Area Information: Requested Reader List (Global gNB ID, Reader Index) and Requested A-IoT Area List (A-IoT Area ID);

    - Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices, Time Interval;

    - Follow on Command Indication.

- INVENTORY RESPONSE:

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - Inventory Response Transfer: A-IoT Correlation Identifier, Criticality Diagnostics

  - Criticality Diagnostics

- INVENTORY FAILURE:

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - Inventory Failure Transfer: A-IoT Correlation Identifier, Cause, Criticality Diagnostics

  - Cause

  - Criticality Diagnostics

- INVENTORY REPORT

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - Inventory Report Transfer:

    - A-IoT Correlation Identifier

    - Global gNB ID

    - Reader Report List: A-IoT NAS PDU, RAN A-IoT Device NGAP ID

    - Inventory Complete Indication

As shown above, we have highlighted the information that are undoubtedly necessary to transmit between BS and UE reader. For instance, the A-IoT Device Identification Requested is needed to generate A-IoT paging by UE reader. Inventory Assistance Information and Follow on Command Indication are needed to assist to allocate more efficient resources and decide whether there is follow-on procedure after inventory completion.

Proposal 5:	After gNB receives INVENTORY REQUEST from CN, the following information is sent to the UE reader (selected from R19 NGAP INVENTORY REQUEST message):

Inventory Assistance Information (including Expected D2R Message Size, Approximate Number of Target A-IoT Devices, Time Interval),

Follow on Command Indication,

A-IoT Device Identification Info (including, the indication of Single Device/Group Devices/All Devices, and Paging ID if provided),

Security parameters if present.

During/after an inventory procedure initiated by an Inventory Request message, UE reader should transfer the inventory result, i.e., A-IoT NAS PDU in AIOT Msg3, to gNB over Uu RRC message. In addition, Inventory Complete Indication should also be included in Uu RRC message which is used to inform service completion to NW.

Proposal 6:	In response to A-IoT Inventory Request-relate information, the following information is sent to gNB by UE reader (selected from R19 NGAP INVENTORY REPORT message):

Inventory result(s), i.e., A-IoT NAS PDU in Msg3,

Inventory Complete Indication if needed.

As shown in the list, some information that are only used in the core network or on network interfaces have been grayed out, and those information do not need to be provided to the UE reader over the air interface. For instance, NGAP INVENTORY REQUEST additionally includes Requested Service Area Information. This information is used to indicate the requested inventory area information to gNB. In T2, if the gNB sends Inventory Request to one UE, that means this UE has been selected by NW, and UE does not need this information. So, Requested Service Area Information is not needed in Uu message.

In Topology 1, gNB reader can send either Inventory Response or Inventory Failure message. This is because the CN is not aware of RAN situation, and the BS usually can reject some CN request according to BS conditions, e.g., available radio resource, Uu conditions. However, in Topology 2, the resource to be used by the UE reader is assigned by BS which can also know the service requirement from the NGAP AIOT messages. Therefore, BS can determine whether to accept or reject the inventory request for both of itself and the UE reader, and UE reader can just follow the decision of the BS. This is usually what we do in Uu interface, for instance, the sidelink Relay UE or NCR node also just follow the configuration from the BS. Once the UE indicates it can perform a specific role, it means the UE is willing to undertake such services. Hence, there is no need to copy the inventory response/failure like procedure to Uu, which only bring signaling overhead/latency for the unnecessary UE-NW coordination. On the other hand, we also notice as a UE, the power consumption would be more critical than a BS reader. Therefore, the UE may want to stop A-IoT service when the battery of the UE becomes low. In this case, we should follow the current power saving mechanism, e.g., by UAI. We can further evaluate whether the current UAI signaling needs to be enhanced.

Proposal 7:	The UE reader should follow the BS configuration and perform the corresponding inventory procedure in A-IoT interface, meaning that there is no need to introduce Inventory Response/Failure Tranfer-like IE to Uu RRC message.

Finally, we have left some information that requires discussion. For instance, in all the NGAP Inventory related messages, Message Type, AIOTF Identifier, and A-IoT Correlation Identifier are included.

- Message type is used to uniquely identify the message being sent, and in NR RRC transfer, a message type is also needed to identify the A-IoT RRC message being sent, e.g., Inventory Request or Command Request.

- AIOTF Identifier, which uniquely identifies the AIOTF, to enable gNB to identify which AIOTF sends request or the corresponding feedback is routed to which AIOTF. The gNB can manage the mapping between UE and the connected AIOTF, so AIOTF ID is not needed in the DL/UL Uu message.

- A-IoT Correlation Identifier uniquely identifies an A-IoT session within an AIOTF, which is not needed by UE reader. If Correlation ID is used to generate transaction ID, the gNB can first generate transaction ID based on the received Correlation ID and then transmit this transaction ID to UE reader. So, Correlation ID is not essential in the DL/UL Uu message, and transaction ID can replace the Correlation ID. The gNB can manage the mapping between A-IoT Correlation ID and transaction ID.

Furthermore, for mapping the device to the NGAP transfer, e.g., associated with the follow-on command request/response, RAN A-IoT Device NGAP ID is included. Between UE reader and gNB, the RAN A-IoT Device NGAP ID can be replaced by other shorter device-related ID in Uu interface. For example, AS ID is unique in one UE reader coverage, and gNB can manage the mapping between AS ID in Uu and Device NGAP ID. So, AS ID can be used to identify device in NR Uu, e.g., A-IoT RRC Inventory Report message and follow-on A-IoT RRC Command request/response message, which has similar function to RAN A-IoT Device NGAP ID.

Proposal 8:	The transaction ID, which is used in the A-IoT interface between UE Reader and devices, is allocated by the gNB.

Proposal 9:	The AS ID, which is assigned by the UE reader to a specific device over the A-IoT interface, can be used to identify the device over the Uu interface between the UE reader and gNB, whose purpose is similar to the “RAN A-IoT Device NGAP ID”.

### 2.2.2	Command Information over NR Uu

For command procedure, the AIOT operations and involved corresponding IEs are defined in TS 38.413:

Figure 8.20.3.2-1: Command Request, successful operation. Figure 8.20.3.3-1: Command Request, unsuccessful operation.

According to clause 9.2.19 in TS 38.413, the information involved in the above messages is list as follows:

- COMMAND REQUEST:

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - RAN A-IoT Device NGAP ID

  - Command Request Transfer:

    - A-IoT Correlation Identifier

    - RAN A-IoT Device NGAP ID

    - A-IoT NAS PDU

    - Command Assistance Information: Estimate of Expected D2R Message Size

- COMMAND RESPONSE:

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - RAN A-IoT Device NGAP ID

  - Command Response Transfer:

    - A-IoT Correlation Identifier

    - RAN A-IoT Device NGAP ID

    - A-IoT NAS PDU

    - Criticality Diagnostics

  - Criticality Diagnostics

- COMMAND FAILURE:

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - RAN A-IoT Device NGAP ID

  - Command Failure Transfer

    - A-IoT Correlation Identifier

    - RAN A-IoT Device NGAP ID

    - Cause

    - Criticality Diagnostics

  - Cause

  - Criticality Diagnostics

As analysed in 2.3.1, similar to Inventory information over NR Uu, AIOTF Identifier is not needed in A-IoT RRC Command Request/Response message. Transaction ID allocated by gNB is used in Uu message, which maps to the A-IoT Correlation ID in NGAP interface. So, A-IoT Correlation ID is not needed in A-IoT RRC Command Request/Response message. AS ID is used in Uu message, which maps to the RAN A-IoT Device NGAP ID. So, RAN A-IoT Device NGAP ID is not needed in A-IoT RRC Command Request/Response message.

As for the upper layer data in COMMAND REQUEST and COMMAND RESPONSE, it can be directly carried through a container in Uu message.

Proposal 10:	After gNB receives COMMAND REQUEST from the CN, the following information is sent to UE reader (selected from R19 NGAP COMMAND REQUEST message):

Upper-layer data, i.e., A-IoT NAS PDU of command request

Command Assistance Information

Proposal 11:	In response to A-IoT Command Request-relate information, the UE reader sends either the upper layer data received from a device to the gNB or a command failure indication over Uu RRC message.

### 2.2.3	Session Release Information over NR Uu

For session release procedure, the AIOT operations and involved corresponding IEs are defined in TS 38.413:

Figure 8.20.4.2-1: A-IoT Session Release, successful operation. Figure 8.20.5.2-1: A-IoT Session Release Request

According to clause 9.2.19 in TS 38.413, the information involved in the above messages is list as follows:

- AIOT SESSION RELEASE COMMAND

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - AIOT Session Release Command Transfer: A-IoT Correlation Identifier, Cause.

- AIOT SESSION RELEASE COMPLETE

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - AIOT Session Release Complete Transfer: A-IoT Correlation Identifier, Criticality Diagnostics.

  - Criticality Diagnostics

- AIOT SESSION RELEASE REQUEST

  - Message Type

  - AIOTF Identifier

  - A-IoT Correlation Identifier

  - AIOT Session Release Request Transfer: A-IoT Correlation Identifier, Cause.

Proposal 12:	Include A-IoT RRC session release command/complete/request indication in NR Uu RRC message, which is for session release purpose. No other detailed information is identified to be useful from NGAP session release related information.

## 2.3	UE reader selection

In Rel-19 discussion, there are two options for UE reader selection: by RAN and by CN. According to the RAN WID and SA2 SID, we understand SA2 and RAN3 will decide coordinately which node to be responsible for the UE reader selection. But if UE reader is selected by RAN (after conclusions from RAN3 and SA2), we think the criteria can be left to implementation without impacting the Uu spec.

RAN2#128 agreement: - From RAN2 perspective, at least for RRC based architecture, there may be radio related reasons for gNB to be involved in reader selection. - Whether the gNB or CN selects the reader, and/or what information is shared between gNB and CN is up to RAN3/SA2

Proposal 13:	How to select a UE reader by the RAN (if agreed by RAN3 and SA2) can be left to network implementation. No Uu impact is foreseen.

## 2.4	UE reader authorization

Similar to NCR/IAB features, before a UE becoming a reader, it needs to be authorized by CN. And usually, only some specific AMF can support such new A-IoT authorized functionality. Therefore, the UE needs to indicate whether it can act as a reader in Msg5 together with the NAS SDU when performing registration and authorization, in order to let RAN correctly route the NAS SDU for this UE to a CN node (e.g., AMF) which can support the A-IoT related operations.

Proposal 14:	To ensure the UE reader's NAS message is routed to the correct CN node, the UE reader needs to include a reader indication in NR Uu Msg5.

As for the cell which does not support T2, the UE does not need to include the reader indication in Msg5. Considering whether supporting T2 or not is cell-specific information, it is better to include such information in SIB than a unicast message such as Msg4.

Proposal 15:	Introduce a SIB indication to indicate whether the cell supports T2 or not.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1a:	If the resources are stopped before the service is complete, the resources are not sufficient to complete the service which harms the performance. And if the resources are stopped after the service is complete, there will be a waste of the radio resources. Therefore, accurate time duration is really the blocking point.

Observation 1b:	The timer cannot function independently, as it also relies on UE assistance information when the timer is configured shorter than the required service completion time.

Observation 2:	No other UE assistance information is needed apart from the required time duration if timer-based solution is adopted by RAN2.

Observation 3:	The periodic A-IoT resource allocation method introduces additional complexity to UE reader implementation (e.g., schedule the A-IoT transmission) considering the timing relationship defined in A-IoT interface.

Observation 4a:	As agreed, the A-IoT radio resources are valid only in serving cell. Using these resources in another cell may cause interference.

Observation 4b:	Whether the same A-IoT service can continue in target cell after HO/new cell after re-establishment is decided by the target cell/new cell.

Observation 5:	If the UE reader’s ongoing A-IoT service continues in the target/new cell after HO/RLF, an additional context transfer is needed between the last serving cell and the target/new cell, making the procedure more complex.

Observation 6:	When studying new information to be introduced in the RRC messages for T2, RAN2 uses the AIOTF Related IEs defined in NGAP (UL/DL) as a baseline.

Resource allocation in normal connected state

Proposal 1a:	Timer based A-IoT radio resource allocation is not supported.

Proposal 1b:	The baseline solution for releasing the A-IoT radio resources should be based on explicit release indications from BS.

Proposal 2:	The baseline A-IoT resource allocation method should be the continuous time domain resource allocation, i.e., the UE reader performs A-IoT operations using the continuous resource until the A-IoT service is complete or the resources are released.

Resource validity when temporary out of connection

Proposal 3a:	To simplify UE reader behaviour, the UE reader does not transmit/receive new messages via the A-IoT interface during HO/RLF initiated reestablishment, and how to handle this A-IoT service will be based on instructions from the target cell after HO/the new cell after reestablishment.

Proposal 3b:	As a baseline, the UE reader releases the A-IoT service related configuration/context upon HO/RRC re-establishment.

General Uu RRC design

Proposal 4a:	For topology-2, introduce two new Uu RRC messages, one for UL and the other for DL, in total between UE reader and BS. And inside the RRC messages, IE-level signalling is corresponding to each NGAP message.

Proposal 4b:	Reuse the existing SRB to carry the new A-IoT-related RRC messages.

Inventory Information over NR Uu

Proposal 5:	After gNB receives INVENTORY REQUEST from CN, the following information is sent to the UE reader (selected from R19 NGAP INVENTORY REQUEST message):

Inventory Assistance Information (including Expected D2R Message Size, Approximate Number of Target A-IoT Devices, Time Interval),

Follow on Command Indication,

A-IoT Device Identification Info (including, the indication of Single Device/Group Devices/All Devices, and Paging ID if provided),

Security parameters if present.

Proposal 6:	In response to A-IoT Inventory Request-relate information, the following information is sent to gNB by UE reader (selected from R19 NGAP INVENTORY REPORT message):

Inventory result(s), i.e., A-IoT NAS PDU in Msg3,

Inventory Complete Indication if needed.

Proposal 7:	The UE reader should follow the BS configuration and perform the corresponding inventory procedure in the A-IoT interface, meaning that there is no need to introduce Inventory Response/Failure Tranfer-like IE to Uu RRC message.

Proposal 8:	The transaction ID, which is used in the A-IoT interface between UE Reader and devices, is allocated by the gNB.

Proposal 9:	The AS ID assigned, which is assigned by the UE reader to a specific device over the A-IoT interface, can be used to identify the device over the Uu interface between the UE reader and gNB, whose purpose is similar to the “RAN A-IoT Device NGAP ID”.

Command Information over NR Uu

Proposal 10:	After gNB receives the COMMAND REQUEST from the CN, the following information is sent to UE reader (selected from R19 NGAP COMMAND REQUEST message):

Upper-layer data, i.e., A-IoT NAS PDU of command request

Command Assistance Information

Proposal 11:	In response to A-IoT Command Request-relate information, the UE reader sends either the upper layer data received from a device to the gNB or a command failure indication over Uu RRC message.

Session Release Information over NR Uu

Proposal 12:	Include A-IoT RRC session release command/complete/request indication in the NR Uu RRC message, which is for session release purpose. No other detailed information is identified to be useful from NGAP session release related information.

UE reader selection

Proposal 13:	How to select a UE reader by the RAN (if agreed by RAN3 and SA2) can be left to network implementation. No Uu impact is foreseen.

UE reader authorization

Proposal 14:	To ensure the UE reader's NAS message is routed to the correct CN node, the UE reader needs to include a reader indication in NR Uu Msg5.

Proposal 15:	Introduce a SIB indication to indicate whether the cell supports T2 or not.