3GPP TSG-RAN WG2 Meeting #132	R2-2508567

Dallas, USA, 17th – 21st November 2025

Agenda item:		10.3.1.3

Source:		Lenovo

Title:		Discussion on UL scheduling and Retransmission protocols

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss further User Plane aspects for the 6G design. In particular, the focus is on UL scheduling latency reduction mechanism, potential improvements for the HARQ and ARQ protocol, as well as uplink scheduling information reporting.

# Discussion

In RAN2#131bis meeting RAN2 had the first 6G meeting, where initial views of companies on the scope of the areas to investigate for 6G have been exchange. Following agreements on the User Plane aspect have been reached:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

For 6G one area worth exploring more is for example the consolidation of the various data recovery mechanisms supported in 5G on different protocol layers, e.g. HARQ on PHY/MAC as well as ARQ on RLC layer, a data recovery mechanism on PDCP as well as higher layer, e.g. TCP layer, retransmission mechanisms. It will be beneficial to investigate whether some simplification or streamlining of the recovery mechanisms supported on the AS can be done. For example, by increasing the reliability of the HARQ protocol and also considering the additional reliability offered by AI/ML expected to be available from day one the recovery mechanism on RLC and PDCP can be simplified or even merged into one recovery procedure.

##### Data recovery mechanism

5G uses the following L2 protocol layer to enable error recovery: MAC, RLC and PDCP each of them targeting a different retransmission mechanism in terms of latency and reliability. 
MAC/PHY which employs a HARQ protocol enables fast retransmissions. Due to a wrong link adaption the selected MCS may not be suitable for the channel conditions which will lead to residual HARQ errors. The main reason for residual HARQ errors is furthermore errors of the HARQ feedback signalling, e.g. ACK->NACK errors, 
NACK->ACK errors on PUCCH or PDCCH misdetection and DTX-> ACK errors. To recover from such errors RLC ARQ can be used, e.g RLC AM. 
On PDCP layer retransmissions of lost data is supported at certain events like handover, e.g. when HARQ and RLC are reset.

Enhancing the reliability of HARQ feedback and optimizing the cross-layer interaction between HARQ and ARQ mechanisms can enable a simplified and faster recovery of residual HARQ errors. By improving HARQ feedback robustness, the need for complex retransmission handling at higher layers (e.g., RLC ARQ) can be reduced, allowing for a more efficient and lightweight protocol design.

When considering improvements to the reliability of the physical uplink control channel (PUCCH), which carries Uplink Control Information (UCI) such as HARQ feedback, further study by RAN1 would be required. Nevertheless, additional approaches may be considered to enhance HARQ feedback reliability and thereby reduce the occurrence of undetected HARQ errors. 
One potential approach is to transmit HARQ feedback information through the MAC layer, i.e., within a MAC Transport Block on the PUSCH, rather than exclusively on the L1 control channel (PUCCH). Leveraging the data channel for HARQ feedback transmission can offer improved robustness. However, the additional delay incurred by a PUSCH transmission and potential HARQ retransmission needs to be further considered.

Proposal 1: RAN2 to study mechanisms to improve the HARQ feedback reliability in order to reduce HARQ residual errors.

In certain cases, it may be more beneficial to trigger an RLC retransmission and update the link adaptation parameters (e.g., MCS) for the retransmission, or to schedule the retransmission on a different cell, rather than continuing with HARQ retransmissions. This may be advantageous when the gain achieved by an updated link adaptation outweighs the potential HARQ soft combining gain.

To further reduce the RLC recovery delay, the HARQ protocol at the transmitter side could inform the corresponding RLC transmitting entity of HARQ failures. In such cases, the RLC layer would not need to rely on gap detection at the receiver side and the subsequent transmission of RLC Status Reports. This cross-layer interaction could enable faster recovery from transmission failures and improve the overall latency performance.

It should be noted that the RLC ARQ mechanism specified in 5G NR provides a relatively slow data recovery procedure, which may not be suitable for latency-critical services requiring at the same time high reliability. In the downlink, the gNB could locally trigger RLC retransmissions based on the HARQ feedback, thus avoiding additional signaling and feedback delays.

### Proposal 2: RAN2 to study mechanisms for locally triggered ARQ retransmissions at the transmitter side based on HARQ status and feedback.

For uplink transmissions, the network may instruct the UE to trigger RLC retransmission(s) for the RLC PDU(s) included in a given transport block (TB). Specifically, the network could indicate whether the UE should perform an HARQ retransmission of a TB associated with a particular HARQ process, or alternatively, trigger RLC retransmissions of the RLC PDUs contained in that TB.

In cases where the network is unable to correctly decode a TB — even after several HARQ retransmissions — it may be preferable for the network to command the UE to initiate RLC retransmissions of the PDUs contained within the TB, instead of requesting further HARQ retransmissions. When the link adaptation (e.g., MCS selection) does not perform well under the current channel conditions, or when a carrier is significantly affected by inter-cell interference, it may be more efficient to abort ongoing HARQ retransmissions and instead trigger RLC retransmissions. Such retransmissions could be scheduled using a different MCS, different transport block size, and/or on a different carrier or cell.

Enabling RLC retransmissions triggered directly at the PHY/MAC level would significantly reduce recovery delay compared to the conventional approach, which relies on RLC status reporting to identify and request retransmission of missing PDUs.

Proposal 3: RAN2 to study mechanisms enabling faster RLC retransmission triggered based on HARQ status or feedback, including locally triggered ARQ retransmissions at the transmitter side and explicit network indications to initiate local NACK procedures.

Another aspect to be considered is the enablement of a faster advancement of the RLC transmitting window in order to avoid window stalling. In 5G NR, the RLC Tx window advancement relies on the reception of RLC Status Reports (SR) from the receiving side. The legacy RLC Status Report provides a comprehensive view of the reception state across the RLC window, including both acknowledgments for successfully received PDUs as well as indications of missing PDUs requiring retransmission.

A limitation of the current RLC Status PDU format, however, is that among the PDUs acknowledged, all remaining PDUs within the window are implicitly negatively acknowledged. To prevent premature NACK feedback, the t-Reassembly timer is employed, allowing lower-layer retransmissions (e.g. HARQ) to complete. While this mechanism avoids unnecessary retransmission triggers, it also delays the reporting of positive acknowledgments. Consequently, the advancement of the RLC Tx window is held back, potentially resulting in transmission window stalling.

##### LCP enhancements

The currently specified Logical Channel Prioritization (LCP) procedure in 5G NR is quite in-flexible: all PDUs belonging to a given logical channel (LCH) / radio bearer are treated uniformly from a QoS and scheduling perspective because the LCH priority is statically configured. In practice, packets multiplexed within the same LCH can have very different contributions to user experience and their relative importance may change over time (for example due to delay evolution or application-layer significance). XR-related enhancements introduced priority adaptation based on delay status (e.g. elevating an LCH’s priority when delay-critical data is pending). However, these enhancements operate only at the inter-LCH level: when an LCH’s priority is elevated, all packets of that LCH are elevated, regardless of whether an individual packet is delay-critical or not. In other words, intra-LCH prioritization is not supported.

Further, current UL resource allocation and prioritization do not consider other packet-level factors that affect user experience, such as:

- Per-packet delay urgency (time-varying deadlines within an LCH);

- Per-packet importance (relative contribution to application-level QoE, which may change dynamically);

- Inter-packet dependencies (e.g. coding/decoding or rendering dependencies between packets within or across LCHs).

These limitations can cause suboptimal scheduling and transport block formation: delay-critical or high-value packets may be delayed behind lower-value packets from the same LCH, or resources may be allocated inefficiently because static LCH priorities do not reflect packet-level urgency.

In 6G RAN2 should study a more flexible LCP and transport block generation framework that supports a fine-grained, e.g. per-packet, prioritization and time-variant priority adaptation. The framework should LCP procedure to adjust the prioritization to the changing significance of a data packet from the perspective of user experience thereby improving resource efficiency and end-user experience.

Proposal 4: RAN2 to discuss enhancements for the Uplink prioritization /LCP procedure compared to NR LCP, e.g. adjust to the changing significance of a data packet from the perspective of user experience

##### UL grants/ UL scheduling latency

At RAN2#131bis, it was agreed that dynamic grants and configured grants will be supported in 6G. The concept of Semi-Persistent Scheduling (SPS) was first introduced in LTE, primarily to reduce PDCCH overhead, which at the time was a key limiting factor for VoIP capacity. This concept was later adopted and extended in 5G NR through the introduction of SPS and Configured Grant (CG) mechanisms. Given that VoIP and similar low-latency, small-payload services are also expected to remain important in 6G, configured grants and SPS are considered essential scheduling tools to efficiently support such traffic types.

In addition to reducing control channel overhead, SPS/CG mechanisms inherently enable lower uplink scheduling latency, which is particularly beneficial for latency-sensitive services. In 5G NR, two types of configured grants are defined:

- Type 1, where the UE transmits according to preconfigured periodic resources, and

- Type 2, where the network can explicitly activate or deactivate a configured grant configuration via a PDCCH command.

The Type 2 configuration provides the network with greater flexibility to adapt uplink resource allocation to the actual data arrival patterns at the UE. In Release 18, the CG-PUSCH procedure was further enhanced to better support XR (Extended Reality) services. These enhancements include:

- support for multiple CG-PUSCH transmission occasions within a single period of a CG configuration, and

- support for indication of unused CG-PUSCH occasions through UCI multiplexed with a CG-PUSCH transmission, thereby improving resource efficiency and uplink capacity.

We think SPS/configured grants will also play an important role in 6G and hence should be supported. Further enhancements can be considered in order to better support specific new data traffic characteristics or to support contention-based uplink transmission as further explained below.

Proposal 5: Support configured grants in 6G. Further enhancements should be considered, e.g. to support contention-based UL transmission or better support certain traffic types.

Low latency is critical for the perceived Quality of experience of a service by the user. The dynamic UL scheduling procedure specified for 5G NR has an inherent delay due to the multi-step approach, e.g. SR followed by BSR and subsequent UL grants tailored to the reported buffer status. So far, two approaches can be mainly used to mitigate this latency:

- usage of configured UL grants

- configuration of a small periodicity for D-SR on PUCCH.

The configuration of frequent SR occasions enables a reduced time for requesting UL resource, but on the other hand PUCCH resources are capacity limited. CG UL grants are suitable for semi-periodic traffic, with constant data sizes. Since the MCS is selected for all the CG-UL grant occasions, the scheduler typically selects a conservative MCS in order to cater for changing channel conditions. This inherently reduces the resource efficiency.

Both solutions have limitations: CG-UL lacks adaptability for bursty or variable traffic, while frequent SR occasions are not scalable due to PUCCH capacity constraints.

For 6G, RAN2 should study mechanisms to further reduce UL scheduling latency. One candidate is to enable contention-based UL transmissions, where limited UL resources are available for immediate use without requiring a dedicated grant. Such a mechanism could allow, for example, the immediate transmission of buffer status or delay status information, thereby reducing UL latency compared to the procedures in 5G NR. Further study/analysis needs to be undertaken in order to ensure that the collision probability is sufficiently low. By restricting the data which is eligible for transmission on contention-based UL resources like e.g. BSR/DSR or delay-critical data on the contention-based uplink resources, which doesn’t happen very frequently the collision probability can be for example limited. In our understanding a contention-based PUSCH could be seen as a special type of CG UL grant.

Also, an AI/ML-based or AI/ML assisted uplink resource allocation procedure should be studied for 6G. For example, knowledge about the upcoming data traffic and its statistics in advance, such as time of occurrence, volume, and QoS of data, can be beneficial in scheduling decisions and support services that require low-latency transmissions. Predictions at the network-side using some assistance (UE status information) from the UE can be applicable for a less volatile scenario. However, the performance is likely to be limited by the uplink data knowledge available at the NW-side. This may lead to signaling of additional information from the UE to the NW (compared to the legacy uplink reporting, such as SR/BSR) to accomplish prediction outcomes with high accuracy. In contrast, necessary measures for uplink scheduling decisions can be performed closer to the edge, i.e., at the UE-side, which can be reported to the network well in advance by exploiting the knowledge and experience of the device.

Proposal 6: RAN2 to study mechanisms for 6G aiming to the reduce the UL scheduling latency, e.g. support of contention-based UL transmissions for certain data (e.g. uplink control information) or AI-based resource allocation procedure.

##### UL status reporting

Uplink (UL) status information is a key input for the gNB scheduler to perform efficient and adaptive resource allocation. In 5G NR (up to Release 19), multiple independent UL scheduling assistance information reporting procedures are specified. While each procedure serves a specific purpose, their independence may lead to situations where the gNB does not have a complete or timely view of the UE’s uplink status, as these reports are often triggered by different events or at different time instances.

For example, in Release 18, the Delay Status Reporting (DSR) procedure was introduced to support XR services as a reporting mechanism independent of the existing Buffer Status Reporting (BSR) procedure. This decision was appropriate for 5G, given that the BSR procedure had already been standardized and matured over multiple releases, and integrating DSR into the BSR framework would have added unnecessary complexity at that stage of specification development.

However, for 6G, the opportunity exists to design a unified uplink scheduling assistance framework from the beginning. Such an approach could provide a comprehensive and consistent view of the UE’s uplink transmission status to the gNB scheduler. In particular, a unified UL status reporting procedure could include not only the buffer occupancy information (as in BSR) but also delay-related information associated with the data awaiting transmission. The main motivation for such a design is to ensure that the scheduler always has access to a complete and up-to-date picture of the UE’s uplink status, enabling more accurate and efficient scheduling decisions. Furthermore, it could be considered whether, in addition to delay information, the UL status reporting could also include information on the application-layer importance or priority of the pending data. This would allow the scheduler to further optimize resource allocation optimizing the perceived Quality of Experience (QoE). It could be further considered to also report UE power headroom information and uplink Timing alignment (TA) information within a common UL status reporting procedure.

Proposal 7: RAN2 to study a UL status reporting procedure in 6G which includes information on the data/buffer status as well as the delay status of the data and potential further information, e.g. importance of the data.

# Conclusion

In this contribution, we discuss User Plane (UP) aspects of the 6G Radio Protocol Architecture. We have the following observations and proposals:

Proposal 1: RAN2 to study mechanisms to improve the HARQ feedback reliability in order to reduce HARQ residual errors.

### Proposal 2: RAN2 to study mechanisms for locally triggered ARQ retransmissions at the transmitter side based on HARQ status and feedback.

Proposal 3: RAN2 to study mechanisms enabling faster RLC retransmission triggered based on HARQ status or feedback, including locally triggered ARQ retransmissions at the transmitter side and explicit network indications to initiate local NACK procedures.

Proposal 4: RAN2 to discuss enhancements for the Uplink prioritization /LCP procedure compared to NR LCP, e.g. adjust to the changing significance of a data packet from the perspective of user experience

Proposal 5: Support configured grants in 6G. Further enhancements should be considered, e.g. to support contention-based UL transmission or better support certain traffic types.

Proposal 6: RAN2 to study mechanisms for 6G aiming to the reduce the UL scheduling latency, e.g. support of contention-based UL transmissions for certain data (e.g. uplink control information) or AI-based resource allocation procedure.

Proposal 7: RAN2 to study a UL status reporting procedure in 6G which includes information on the data/buffer status as well as the delay status of the data and potential further information, e.g. importance of the data.

# References