3GPP TSG-RAN WG2 Meeting #132 														     R2-2508572

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		10.3.3.3

Source:	Samsung

Title:	Considerations for Security and SON/MDT in 6GR

Document for:		Discussion

# 1. Introduction

In RAN#108, a study item on the next generation of non-backward compatible Radio Access Network (RAN), i.e., 6G Radio (6GR), was approved [1]. Based on the SID, this contribution provides a summary of Samsung’s views on Access Stratum security aspects and SON/MDT.

# 2. Discussion

## 2.1 Access Stratum Security

Security has been a major USP for 3GPP technologies from the beginning. With each generation, 3GPP has introduced considerable enhancements to security mechanisms. For instance, 5G NR (New Radio) Access Stratum (AS) security introduced significant advancements over 4G LTE (Long Term Evolution), including mandatory support of integrity protection for user data, stronger cryptographic algorithms, encryption of the International Mobile Subscriber Identity (IMSI) and usage of Temporary Mobile Subscriber Identity (TMSI), secure initial signalling messages and mutual authentication between the UE and network.

However, the security threats and vulnerabilities also have emerged over the time with the increasing complexity of networks, higher data rates, and emerging threats like quantum computing attacks. It is imperative that 6GR provides solutions to maintain trust, reliability, and resilience in the 6G ecosystem.  Two major threats that need to be addressed in 6GR are related to the protection of lower layer signalling (MAC CE) and the broadcasted system information message.

RAN2 discussed the protection of lower layer signaling in RAN2#131-bis and has send an LS to SA3 for further details. RAN2 can wait for the response/answers from SA3 on the same, and discuss the security of broadcasted system information message in this meeting.

Broadcasted System Information (SI) protection

Figure 4: Security concerns with false base station in 5G

Present day wireless communication systems are vulnerable to potential threats from false (fake) base stations, where the false base station is a device that impersonates as a cellular base station to intercept and manipulate communications towards mobile devices. The false base station could trick the mobile devices in the vicinity into connecting to it by emitting a stronger signal as the cellular base station or it may completely block the connectivity for these mobile devices to the genuine base station. Specifically, the various security threats include user tracking, data theft or fraud by intercepting unencrypted data or authentication codes, Denial-of–Service (DoS) attacks, downgrade attacks by forcing device to connect to less secure network protoocols and performance degradation (e.g. draining battery by repeated search for connection). The recent incident of a security breach with KT operator in South Korea (Aug 2025) involving 4 fake base stations resulted into micropayments fraud and has proven the worst fears.

More particularly, the broadcast messages such as MIB (Master Information Block) and SIBs (System Information Blocks) which carry vital system parameters that govern the initial access and connectivity to the network are in transmitted in plain text and could be easily tempered with by the fake base station. SA3 has previously studied on these security concerns and also worked out the solutions, which are based on the digital signing the system information using non-Post Quantum Cryptography (PQC) or PQC asymmetric cryptography algorithm [TR 33.809]. Accordingly, mobile devices can verify and validate the authenticity of the cell and take appropriate actions in terms of pursuing the connection or reselecting to another cell. Incidentally, RAN2 and SA3 had a liaison exchange on this topic [R2-2206976/8985] during 5G standardization phase (Release 18 time). But it did not progress much for 5G, primarily due to the concerns about complexity in modifying existing system information design in the middle of a generation.

A fake base station may tamper the system information parameters to force the UE to reselect to an older generation cell, even to a 2G/3G cell. Then, it will be able to override any of the advanced security features available in 6G.

Observation 1: In 5G NR, system information broadcast is prone to potential threats from False Base Stations.

As the 6G is offering an entirely new and fresh opportunity to consider the security aspects without any conerns of backward compatibility, we understand RAN2 needs to seriously engage in this study in coordination with SA3. The broader issues for RAN2 to enable protection of system information includes a careful evaluation of the solutions and their potential impacts to RRC and related performance and signalling aspects.

Figure 5: Protection of system information

Proposal 1: RAN2 to study enhanced AS layer security to protect broadcast system information message and mitigate threat from false base stations.

## 2.2 SON/MDT for 6G

3gpp introduced SON/MDT in Rel-10 for LTE and Rel-16 for NR. The feature was helpful for operators in the real field to reduce the costly manual parameter tuning. During the deployment of a new RAT, the amount of manual tests and parameter tuning are usually quite high since it is not very much practically possible to simulate the real world RF environment or the multi-vendor interoperability. It is also a common observation that the deployment of a new RAT takes some time to stabilise since coverage, mobility performance, throughput etc. will vary based on hardware, geography etc.

In other words, unless an automated solution is introduced in the early release, the CAPEX/OPEX for deploying 6G will be higher due to these manual operations. In TR 38.914 [3], the study of 6G RAN requirements, CAPEX/OPEX reduction is a major consideration. We also observe a strong need from operators to support SON/MDT features from 6G Day1. As far as we know, some fundamental functions related to self-configuration and self-optimization have been widely used by operators and have proven the usefulness since 4G.So it makes sense to introduce at least the fundamental functionalities of SON/MDT from the beginning of 6G.

Observation 2: Introducing SON/MDT from the beginning of 6G helps in reducing the operator’s cost for the introduction of 6G in their networks.

There is also another dimension to this problem. Designing optimisation along with the normal operation of the system will lead to a more robust system. For instance, in NR, the network may configure the UE for measurements for different purposes, such as for load balancing, addition/release/activation/deactivation of SCells, addition/release/mobility/activation/deactivation of SCG. Eventhough these may not be relevant to the Mobility Robustness Optimisations, UE logs them in SON reports such as RLF report or SHR, as the UE does not have a means to distinguish between them. By designing the measurements considering the optimisations also as a factor, from the begining, we can avoid these issues.

Another aspect is the interworking with AI. 6G will be an AI native system from the begining, but it also needs to work without AI, at least as a fallback. The network use cases of AI/ML typically requires data reporting from the UE. RAN2 has already agreed to study a standardised data collection framework considering AI/ML, SON/MDT etc. Considering SON/MDT from the first release helps in designing a streamlined system.

Observation 3: Considering SON/MDT along with measurement framework and AI/ML helps in designing a streamlined and robust system

Based on the observation 2 and observation 3, we propose to study the SON/MDT for 6G in the initial release.

Proposal 2: RAN2 to study the basic functions of SON/MDT for the first release of 6G.

# 3. Conclusion

Based on the discussion in this paper, the following observations and proposals are derived:

Observation 1: In 5G NR, system information broadcast is prone to potential threats from False Base Stations.

Observation 2: Introducing SON/MDT from the beginning of 6G helps in reducing the operator’s cost for the introduction of 6G in their networks.

Observation 3: Considering SON/MDT along with measurement framework and AI/ML helps in designing a streamlined and robust system

Proposal 1: RAN2 to study enhanced AS layer security to protect broadcast system information message and mitigate threat from false base stations.

Proposal 2: RAN2 to study the basic functions of SON/MDT for the first release of 6G.

# 4. Reference

- RP-251881	New SID: Study on 6G Radio		NTT DOCOMO	RAN#108

- RAN2#131-bis Char notes

- TR 38.914 V0.2.0 Study on 6G Scenarios and Requirements