3GPP TSG-RAN WG2 Meeting #132	R2-2508577

Dallas, USA, 17 - 21 November 2025

Agenda Item:	10.4

Source:	Huawei, HiSilicon

Title:	Discussion on 6GR mobility designs

Document for:	Discussion and Decision

# 1	Introduction

6GR intends to enable new business opportunities, reduce total cost of ownership (TCO) and improve user experience. The 6GR vision targets a true generational leap by making AI and sensing native capabilities of the air interface, and committing to energy-efficient design. In addition, 6GR design shall provide a framework supporting both new and legacy frequency bands, diverse device types, TN and NTN, while not being constrained by backward compatibility. The 6GR air interface should be crafted to deliver not only the above design target, but also drive the industry through 2030 and beyond.

In RAN2#131bis, 6GR mobility was discussed for the first time and the following agreement was reached:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this contribution, the study and analysis of essential components of 6GR mobility and measurement based on the agreed requirements above are presented.

# 2	Discussion on 6GR mobility designs

## 2.1	6GR mobility – Handover procedure

### 2.1.1	6GR inter-RAT mobility

As per the 6G SID [1], 6GR should support mobility in all RRC states as well as migration from 5G NR to 6GR and interworking and mobility between 5G NR and 6GR. This means that RAN2 needs to study intra-RAT mobility scenarios and inter-RAT mobility scenarios. Intra-RAT mobility scenarios further comprise intra-cell mobility scenarios (which may be handled through beam management procedures) and inter-cell mobility scenarios (which may be handled through mobility procedures). Early stages of 6GR deployment will likely take place as isolated 6G radio coverage areas while 5G NR is deployed, mature and provides continuous coverage, therefore RAN2 needs to study mobility scenarios where UEs move from 6GR to 5G NR as that will be crucial to guarantee the user experience during the early stages of 6GR deployment. However, during the 6GR SI RAN2 should study intra-RAT mobility scenarios as a priority and study inter-RAT mobility scenarios as a second priority.

Proposal 1:	RAN2 should study intra-RAT handover and inter-RAT handover in 6GR. RAN2 should start with intra-RAT handover, followed by inter-RAT handover (e.g., 6G to 5G handover).

### 2.1.2	6GR mobility – Unified handover: essential and useful components

Based on the analysis from our previous contribution [2], we showed that the main bottlenecks contributing to latency/interruption time during the mobility procedure are the TuncertaintyPUSCH and the TprocessingUE terms.

TuncertaintyPUSCH is timing uncertainty related to the transmission of the PUSCH carrying the RRC reconfiguration complete message, which may include the PRACH timing uncertainty TuncertaintyPRACH if the mobility is RACH-based (subject to the type of scenario).

While a RACH-less handover procedure incurs less latency, 6GR still needs a RACH-based handover procedure for the UE to maintain UL synchronization if the network doesn’t have another way to obtain the TA. TprocessingUE is the timing delay related to Tx/Rx beam switching and RF re-tuning. Given the agreement to study 6GR mobility on the basis of the requirements to minimize interruption and minimize throughput degradation, RAN2 should study and identify components which satisfy these requirements, whereas components which don’t help to satisfy these requirements shouldn’t be retained.

Observation 1:	UE processing and interruption uncertainty related to PRACH are the main bottlenecks that contribute to latency/interruption during mobility procedures.

- During 5G NR, many mobility solutions were introduced over the course of multiple releases and each solution had its advantages and drawbacks. For 6GR mobility, RAN2 should carefully study what components are essential to mobility and weigh their advantages and drawbacks in accordance with the requirements that were agreed in RAN2#131b. The result of this analysis should naturally lead towards the “unified” 6GR mobility procedure. In the remainder of this section, we will study and analyse various components and discuss their essentiality for the mobility procedure.

- Early DL synchronization is an essential component as UEs need to acquire DL frame timing in order to receive other physical layer reference signals and channels from the serving base-station. For the reception of physical layer reference signals and channels from neighbouring base-stations, the UE may assume the DL frame timing of these base-stations to be the same of the serving base-station or the UE may simply receive the SS/PBCH block transmitted these neighbouring base-stations based on the information provided in the candidate cell configurations. Early DL synchronization is low-cost in terms of network implementation complexity as the network doesn’t need to transmit any additional reference signal in order for UEs to detecting and measuring such signals. Early DL synchronization is also feasible without the use of measurement gaps, which are a major pain-point for both the UE (due to the interruption and throughput degradation it causes) and the network (due to the restrictions in scheduling and configuration it causes).

Observation 2:	Early DL synchronization towards candidate target cells can reduce interruption during mobility and has limited UE and network impact.

- UL synchronization is an essential component as UEs need to acquire the Timing Advance (TA) in order to communicate with the target cell. Early UL synchronization was introduced in LTM with the intention to determine the TA for a candidate target cell while the handover decision is not made yet, thus avoiding the need for a RA procedure during the handover execution and reducing the interruption time. and allow the candidate target base-station to calculate the TA and provide it to the source.. This is called "early UL synchronisation". While early UL synchronization can reduce the handover interruption time, it may consume PRACH resources from candidate target cells to which handover will not actually be executed and the transmission of the PRACH towards the target base-station may cause interruption in the connectivity of the UE with the source.

Observation 3:	Early UL synchronization can reduce handover interruption but it may waste resources (e.g., UL synchronization to cells to which handover is not triggered) and cause interruption.

PRACH-related latency is one of the main bottlenecks in the mobility procedure. In 5G NR, there are 64 RA sequences so that, in a RA occasion, it is not possible to allocate resources for contention-free RA (CFRA) to more than 64 UEs, and a higher periodicity of RA occasions will take more resources. Consequently, to allocate RA resources for contention-free access to a UE, the network may need to reduce the periodicity of the RA occasions that the UE may use, thus increasing the RA latency. In low load conditions, it may be possible to use contention-based RA, so that all occasions could be used by all UEs, but as soon as the collision probability increases due to more RA procedures, UEs would suffer larger latency.

If 6GR can support a larger number of RA sequences, it would be possible for more UEs performing handover to use all RA occasions with a dedicated RA preamble, thus reducing latency and avoiding collisions. We should make use of UE-dedicated RA resources during mobility procedures so that these resources can be used by the network to serve the same UE across multiple candidate cells and for an extended period. Such UE-dedicated RA resources would allow us to reduce the RA latency and also allow multiple base-stations to serve this UE in seamless manner from the UE’s perspective. Using UE-dedicated resources will also be instrumental to simplify UE management by reducing signalling overhead and power consumption that comes from executing state transition at the UE side (e.g., from idle to connected).

Observation 4:	If 6GR can support more RA preambles in one RA occasion, the handover latency can be reduced through CFRA-based UL access.

- Early RRC decoding is a feature that was introduced with LTM. LTM relies on early RRC decoding to reduce the interruption time. UEs may signal their capability for fast RRC processing through the fastProcessingConfig parameter. The main benefit of this feature is to decode the candidate cell pre-configuration and perform ASN.1 validity and compliance check ahead of the reception of the LTM cell switch command. However, this solution does not address the root cause of the problem: which is the inherent complexity of the RRC signalling in 5G NR. With each release, 5G NR kept introducing more and more IEs in RRC, leading to a prohibitively complex signalling structure. This must be avoided for 6GR and RAN2 should study how to simplify the RRC signalling structure to reduce decoding complexity. If RRC messages have low decoding complexity, then the overall system will work better and there will be no need for early RRC decoding.

- Observation 5:	If RAN2 can optimize RRC signalling to reduce decoding complexity, early decoding may not be needed.

- Pre-configuration of candidate cells includes the configuration of resources for early DL/UL sync and the UE configuration in the candidate target cell, which the UE applies upon reception of the MAC CE cell switch command. This reduces the size of the handover command, so it reduces the chances that the UE fails to receive it in degrading radio conditions. However, candidate cell pre-configuration adds significant complexity network for the network. One drawback is that candidate cells needs to reserve UL resources for PRACH and PUSCH to allow UEs to send the PRACH (for TA calculation on the network side) and send the PUSCH (to send the RRC reconfiguration complete message and formally complete the LTM procedure) and to reserve resources for the configuration in the target cell.

Observation 6:	Providing a candidate cell configuration to the UE requires the candidate cell to reserve radio resources for the UE, for initial access and for operation in the candidate cell.

- Another drawback of using candidate cell pre-configurations is that it adds signalling overhead on the network. If the network experiences significant load changes, then the network may need to perform radio resource re-allocation in order to tackle these load changes. This would require the network to keep track of the pre-configurations that it provided to each connected UE and apply corresponding configuration changes matching with the resource re-allocation. Therefore, the usage of candidate cell pre-configurations at the UE adds signalling overhead on the network as well as implementation complexity on the network and it should not be counted as an essential component for mobility.

Observation 7:	Candidate cells may need to modify the configurations already provided to UEs not in the cell in some cases (e.g., because of load change).

- Early CSI acquisition is a feature that was introduced with LTM in Rel-19. LTM relies on early CSI acquisition for obtaining CSI reports from the UE in order for the network to take appropriate scheduling decisions (i.e., TBS selection, MCS selection and PRB allocation) for PDSCH transmissions towards that UE. The main reasons why there is throughput degradation during mobility procedures are: 1) the interruption caused by the mobility procedure, 2) the time spent in getting the UE’s CSI reports on the right resources. UEs are configured with multiple CSI-RSs so that the UE can perform channel measurements and report those measurements back to the network. This is especially important in beam-based scenarios in which determining the right Tx/Rx beam as early as possible is essential for CSI reporting as {RI/PMI/CQI} are conditioned on the Tx/Rx beam. If there is beam mis-alignment on the UE side, then the UE would spend computational resources deriving a CSI report which wouldn’t be useful for the network as far as minimizing throughput degradation is concerned. Any latency or delays in identifying the right resources will contribute towards throughput degradation and negatively impact the user experience. Therefore, early CSI acquisition based on identifying the right Tx/Rx beam is essential in order to minimize throughput degradation during mobility.

Observation 8:	Early CSI acquisition is useful to minimize throughput degradation after handover. It is essential to identify the right Tx/Rx beam before CSI acquisition.

- Subsequent mobility is a feature that was introduced with SCPAC in Rel-17 and LTM in Rel-18. It .is aimed to allow preparing once multiple candidate cell configurations and then switching the UE multiple times between those configurations. When this feature was discussed, it was expected to reduce RRC signalling i.e., the network does not need to update the configurations. However, when the UE moves from one candidate cell to another one, it may continue to move further away from the previous cell and not back to it, while the source cell still needs to keep the previous configuration. For similar reasons to what we explained regarding candidate cell pre-configurations, the retention of candidate cell configurations significantly increases network complexity as all of the resources configured in the candidate cell configuration need to be constrained across multiple cells. The gain of subsequent mobility would primarily be seen in situations where the UE would be and remain in the coverage area of two or more candidate cells, which only occurs in rather limited areas and could also be addressed by multi-TRP multi-cell transmission. Therefore, the usage of subsequent mobility adds significant network implementation complexity while delivering limited gains.

Observation 9:	Subsequent mobility without need of reconfiguration increases network complexity for limited gain/scenario.

- The conventional (non-LTM) reconfiguration procedure allows delta signalling i.e., to include only differences between the source and the target configuration. It also allows the network to indicate to the UE L2 actions per protocol layer and per entity, including release and addition if more appropriate, taking into account the source and the target configuration. To support of subsequent mobility, a new reconfiguration procedure had to be specified, with partial release and replacement of the UE configuration and additional mechanisms to control L2 actions, with much more limited flexibility as compared with the conventional procedure. This also greatly increases the risk of reconfiguration failures, and, even before any deployment, major flaws were realized in the new procedure.

- For 6G, it should be possible to take advantage of short interruption time and avoid throughput degradation while using the same delta signalling like any reconfiguration procedure, without the increased complexity of a new reconfiguration procedure introduced only for the support of subsequent mobility.

Observation 10:	Support of subsequent mobility vastly increases UE complexity as it requires a specific RRC reconfiguration procedure, and does not allow delta configuration.

- Based on the above analysis, we make the following proposal.

Proposal 2:	In accordance with the agreed requirements, RAN2 studies at least:

To minimize interruption time: early DL sync, CFRA-based handover and early UL sync (without interruption),

To minimize throughput degradation: early CSI acquisition,

To balance complexity and benefits: one-time mobility (i.e., without subsequent mobility), using the basic reconfiguration procedure (which supports delta signalling).

Connected to the issue of throughput degradation, another factor that negatively impacts the UE experience is the delay in Layer 2 data transfer and data loss during the mobility procedure. In the mobility procedure, even the Layer 1 link has been established, the Layer 2 data transfer does not begin immediately due to delays in transmitting the status report and processing at Layer 2. For example, in NR, once the UE has successfully accessed the target cell, the target cell waits for the PDCP status report before deciding which data to transmit and processing it to avoid redundant transmission, leading to extra latency. If the PDCP status report is large, possibly when the UE receives a packet with a relatively high sequence number but is missing packets with a much lower sequence number, there may be additional delay due to the segmentation of the PDCP status report. This scenario is illustrated in the figure below:

Figure 1: An example of late L2 data transfer in handover procedure

In addition, Layer 2 protocol reset or re-establishment is performed, resulting in the flushing of buffered data at the source base-station, leading to potential data loss for UM services. While UM services can tolerate some data loss, minimizing it is crucial for a better user experience, which is why the HARQ mechanism is used for UM. In 6GR mobility, RAN2 should study how to accelerate the Layer 2 data transfer and minimize data loss to improve the user experience during handover.

Proposal 3:	To ensure service continuity, RAN2 should study L2 operations during handover to improve L2 continuity, taking into account both complexity and benefits when exploring potential solutions.

### 2.1.3	6GR mobility - Conditional handover and network resource reservation

Robustness is another requirement for the design of the 6GR mobility procedure. To ensure the robustness of mobility procedures throughout the UE lifetime within a 6GR network, it is necessary to design 6GR mobility procedures such that the handover failure rate is as close as possible to zero. Robustness can be quantified using the number of Handover Failures (HOFs) that occur throughout the UE lifetime.

Both the CHO and LTM procedures mitigate the risk of this vulnerability by sending the RRC configuration of the target base-station(s) in advance. In the case of CHO, the UE initiates the handover when execution conditions, based on measurement results, are met. In the case of LTM, the source cell sends a small cell switch command upon receiving a measurement report from the UE, without the need for interaction with the target cell since the handover is already prepared. Both of these procedures have the advantage that UEs have the necessary target cell configuration information in advance.

As mentioned earlier, using candidate pre-configurations has drawbacks. Both CHO and LTM rely on candidate pre-configurations, which results in implementation complexity for the network side due to resource reservation issues. Pre-configuring candidate cells at the UE imposes significant constraints on the network, requiring the serving cell to update candidate cell pre-configurations for neighbouring cells during load change. This also affects resource allocation in the uplink as neighbour candidate cells may need to reserve PRACH and/or PUSCH resources, impacting the Initial Access procedure for UEs entering the system. RAN2 should carefully examine these resource reservation issues for CHO and LTM.

Proposal 4:	For 6GR conditional handover and candidate pre-configurations, RAN2 should study network resource reservation issues.

## 2.2	6GR mobility – Unified Measurement framework

Measurement and reporting for 6GR mobility was discussed during RAN2#131bis. However, there was no consensus on what to study and on how to unify measurement and reporting. RAN2 should start by addressing the issues identified in 5G NR and carefully study and analyse how to solve these problems and avoid repeating the same mistakes for 6GR.

In 5G NR Rel-19, RAN1 introduced UE-initiated/Event-driven beam Reporting (UEIBR) to reduce measurement reporting overhead in intra-cell and inter-cell beam management and enable fast beam switching. For UEIBR, RAN1 introduced beam-based events that are similar to already existing cell-based events. UEIBR introduced Event-1, defined as “Quality of the current beam is worse than a certain threshold.”, which corresponds to Event-A2, defined as “Serving becomes worse than absolute threshold”. UEIBR also introduced Event-2, defined as “Quality of at least one new beam becomes a threshold value better than the current beam”, which corresponds to Event-A3, defined as “Neighbour becomes amount of offset better than PCell/PSCell”.

In 5G NR Rel-19, RAN2 introduced enhancements to LTM. In particular, RAN2 introduced reported triggered by L1 beam-level measurements of the LTM candidates and/or of the serving cell. UEs are configured with an LTM-CSI-ReportConfig with the parameters for performing event-triggered L1 measurement and reporting. Examples of beam-level events that are defined include: event LTM2, event LTM3, event LTM4 and event LTM5. Event LTM2 is defined as “beam of serving cell becomes worse than threshold”, which is the same like Event-1 from UEIBR. Event LTM3 is defined as “beam of candidate cell becomes offset better than beam of serving cell”, overlapping with Event-2 from UEIBR.

When comparing the reporting introduced by RAN1 for UEIBR and the reporting introduced by RAN2 for L1 measurements for LTM, we see many similarities:

Figure 2: L1 measurement reporting for UEIBR (left) and LTM (right)

Firstly, both UEIBR and LTM reports based on L1 measurements using RSRP measurements. Secondly, both reports include resource indicators: UEIBR reports include either the CSI-RS resource indicator (CRI) or the SS/PBCH block resource indicator (SSBRI), while LTM reports include a reference signaling resource index (RSRI) field which can be either an SSBRI or a CRI. Thirdly, the UEIBR report includes the L1-RSRP measurement result of the first beam (referred to as L1-RSRP #1) and the differential L1-RSRP measurement results of the remaining beams (referred to as differential L1-RSRP #i) while the LTM report also includes the L1-RSRP measurement result of the first beam (referred to as RSRP1) and the differential L1-RSRP measurement results of the remaining beams (referred to as DIFFRSRPi).

While mobility and beam management have evolved separately, it is clear that these functions are now converging, if not overlapping. 6GR should explore ways to streamline beam-based event measurement, triggering and reporting to prevent redundant features across RAN working groups, with unnecessary specification efforts. A unified approach to measurement and reporting implies a single framework that integrates all components of 6GR mobility.

Observation 11:	As 5G NR evolved, mobility and beam management started to overlap in terms of measurement and reporting functionalities.

For 6GR, RAN2 should study how to unify the measurement and reporting configurations for L1 and L3 measurements, as well as harmonizing cell-based reporting and beam-based reporting. We can see that from the similarities in the definition of events between cell-based event reporting and beam-based event reporting. Both types of measurements are derived from the same physical layer measurement samples. The difference between cell-based measurements and beam-based measurements lies in the beam-level consolidation process, which can be performed directly within the network with minimal added complexity.

The difference between L1 measurements and L3 measurements lies the L3 filtering process, which utilizes exponential smoothing. The purpose of L3 filtering is to reduce the noise from L1 measurements, which may fluctuate due to factors like fast fading, and to generate a “smooth” input for mobility event triggers. This filtering helps preventing ping-pong effects. This network can implement L3 filtering directly with minimal added complexity using L1 measurement reports from the UE, allowing the network to customize smoothing factors for optimal mobility decisions.

Therefore, RAN2 should adopt a measurement and reporting framework for 6GR mobility in which the UE transmits L1 measurement results to the network. The network can then handle all necessary consolidation, averaging, and filtering to make informed mobility decisions. This approach will ensure consistency in mobility decisions and enable better adjustment of handover control parameters for a seamless user experience during mobility.

Observation 12:	The network can use L1 measurement results to derive cell/beam-level measurements and L3-filtered measurements effectively.

RAN1 started working on 6GR at RAN1#121. The main topics studied include performance evaluation assumptions, waveform and frame structure, channel coding and modulation, and energy efficiency. In addition, discussions on initial access and MIMO will start in RAN1 starting from RAN1#124. RRM measurements are expected to be discussed under the initial access topic, while Beam Management is expected to be discussed under the MIMO topic.

The study and analysis of measurements and reporting for 6GR mobility requires a deep knowledge of the physical layer reference signals, in order to determine the measurement procedure, periodicities, triggers and contents to be reported. The physical layer reference signals, measurement periodicities and report content will be studied and developed by RAN1. The procedure, triggers and reporting format will be studied and developed by RAN2. Some aspects may be studied jointly by RAN1 and RAN2. However, RAN2 should wait at least until RAN1 has discussed initial access, MIMO and their respective reference signals before analysing measurement and reporting for 6GR mobility.

Observation 13:	RRM measurement and reporting discussions for 6GR mobility require knowledge of physical layer reference signals in 6GR, which RAN1 needs to provide to RAN2.

3GPP should learn from the approach used in 5G NR, where different working groups independently specified similar features simultaneously, and instead have these working groups collaborate to prevent duplication. RAN2 should closely collaborate with RAN1 to develop a cohesive measurement and reporting framework for 6GR mobility. For example, when unifying measurements and configurations for L1 and L3 measurements, RAN1 should be involved early to grasp the unification principles clearly.

Proposal 5:	RAN2 should collaborate closely with RAN1 to develop a unified solution for measurement and configuration that is suitable for L1 and L3 measurements for 6GR mobility. It is important to involve RAN1 early to ensure a thorough understanding of this principle.

## 2.3	6GR mobility – Energy efficiency in connected state

One of the driving forces for 6GR is TCO reduction wherein network energy consumption reduction is an important aspect. 6GR, committing to energy-efficient design, should take energy saving as a fundamental design principle in Day 1, for both network and UE, as have been agreed in the newly approved SI [1].

To achieve and develop meaningful solutions for UE and network energy saving, we should study how to design 6GR mobility and beam management procedures such that they consider energy efficient procedures. It was also agreed to study “Energy efficiency for both UE and NW” in last RAN2 meeting conclusion for mobility.

One technical direction we should study and investigate is that UEs perform mobility measurements in a more energy efficient manner. As discussed in our companion contribution [3] on "RRC state: requirements and functions discussion", the connected state should also include the time where the UE has no data to transfer and low UE power consumption is expected, so that it is possible to keep the UE in connected state for prolonged times and avoid state transition latency to resume data transmission. This means that connected state measurements should also be designed to minimize the UE power consumption.

As UEs in connected state go to sleep during “no-data” periods, when these UEs move from the coverage area of one cell to that of another cell: they may experience mobility events. Based on the current 5G NR mobility framework, this triggers a mobility event report where the UE’s RRC layer will generate the measurement report associated with the mobility event. This measurement report will then be sent to the lower layers for transmission in the UL. Given our assumption that the UE went to sleep for a prolonged period and that it has no data to transfer, we can assume that such a UE would not have a valid configured grant to make an UL transmission at the time the measurement report is received. This situation is shown in the figure below:

Figure 3: Illustration of UE-based energy efficient mobility in connected state

As the UE’s MAC layer receives the measurement report from the RRC layer, the MAC layer will trigger the RA procedure in order to request an UL grant in order to transmit the measurement result. Triggering the RA procedure effectively wakes the UE up and negates any power saving from that point onwards. Therefore, the current mobility reporting framework is designed to send reports any time a mobility event is triggered. This situation is unfavourable to the UE and leads to large UE power consumption, instead the framework should allow the UE to maximize power saving during “no-data” periods.

Observation 14:	The current mobility reporting framework in connected state may not meet the agreed energy efficiency requirements for 6GR mobility. Less measurement reporting is needed when the UE has no data.

In RAN1#122b, RAN1 agreed to study and evaluate network energy saving techniques based on long periodicities of synchronization signals (20ms and above) and in particular their impact on UE performance and user experience. RRM, mobility and beam management are among the areas listed for study by RAN1. Longer SSB periodicities may affect the timeliness of RRM measurements. Timeliness of RRM measurements will have a direct impact on mobility function performance, particularly in the form of late handover commands, which may result in handover failures.

For 6GR mobility, RAN2 should study a UE-based mobility function in connected state which would allow the UE to perform mobility while following the energy efficiency requirement. If the UE is sleeping and has no data to transfer, even if a mobility event is triggered then the UE can perform cell switch without notifying the network as long as the UE is within a certain area of the network. This area of the network may include a limited number of cells. This would allow the UE to stay in its sleep state where its power consumption is low even while the UE is moving within that area.

Proposal 6:	RAN2 should study a UE-based mobility function in the connected state for energy efficiency. E.g., the UE can switch cells within a certain area without notifying the network, based on its RRM measurements, when the UE is in a no-data period.

## 2.4	6GR mobility – Energy efficiency in idle state

For 6GR, RAN2 should investigate ways to minimize UE power consumption for cell reselection. Idle is the state of UEs upon powering on of UEs in a long-term deep sleep status. More detailed discussions regarding the 6GR idle state can be found in our companion contribution [3]. In beam-based deployments, UEs may need to perform multiple measurements for the same cell in different spatial or angular directions. This may happen in terrestrial deployments (where the UE may perform measurements in the horizontal domain) and in non-terrestrial deployments (where the UE may perform measurements in the vertical domain). Depending on various factors such as the radio conditions experienced by the UE, the type of network deployment, and the frequency band used for measurements, the measurement burden on the UE may be significant and greatly impact its power consumption. In 6GR we should study how to reduce the measurement burden of the UE so that the UE can achieve meaningful energy saving even in idle during cell selection/reselection measurements.

For Network Energy Saving, it would be beneficial to use longer SS/PBCH block transmission periodicity. Using long periodicity for SS/PBCH block transmission for cell selection/reselection measurements may have a limited impact on the UE when looking specifically at the functions of cell selection/reselection. This is because the measurement periodicity in idle may be much longer when compared with the measurement periodicity for mobility in connected state. For monitoring System Information and Paging, monitoring PDCCH transmissions requires UEs to acquire fine synchronization, this usually means that UEs must wake up earlier to acquire this fine synchronization. A UE may need to receive the corresponding SS/PBCH block two to three times to acquire fine synchronization. If the UE is in deep sleep prior to monitoring such PDCCHs (for SI or Paging), then the UE would need to wake up ahead of time to detect and measure two to three SS/PBCH blocks located before the PDCCH monitoring occasion. This is shown in the figure below:

Figure 4: SI and Paging process in 5G NR

In RAN1#122b, RAN1 agreed to study and evaluate network energy saving techniques based on long periodicities of synchronization signals (20ms and above) and in particular their impact on UE performance and user experience in the context of Initial Access. As part of this agreement: RRM, mobility and beam management are among the areas listed for study by RAN1. The RAN1 agreement from RAN1#122b is shown below for reference:

Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Unclustered provisioning with cell-common signaling, UE impact: - Cell search complexity and latency, including frequency search latency, - UE Power consumption, - Sync signal detection, coverage and tracking performance, - RRM, mobility, - Beam management, - Other properties are not precluded, - Improvements to address identified impact, e.g., - Additional sync signal needs, - Adaptation of sync signal transmission periodicity, - Sparser synch raster.

Observation 15:	RAN1 has started to investigate issues related to energy efficient RRM/mobility in idle state.

For 6GR, RAN2 should closely follow RAN1’s progress on energy efficiency and study mobility mechanisms which are more energy efficient. Such energy efficient mechanisms would allow UEs to stay in deep sleep for as long as possible while operations related to cell selection/reselection and camping on a cell take up the least amount of effort from the UE in order to maximize its energy savings.

Proposal 7: 	RAN2 should follow RAN1 progress on idle mobility measurement framework and study energy efficient cell selection/reselection.

# 3	Conclusion

We have discussed 6G mobility and made the following observations and proposals:

Observation 1:	UE processing and interruption uncertainty related to PRACH are the main bottlenecks that contribute to latency/interruption during mobility procedures.

Observation 2:	Early DL synchronization towards candidate target cells can reduce interruption during mobility and has limited UE and network impact.

Observation 3:	Early UL synchronization can reduce handover interruption but it may waste resources (e.g., UL synchronization to cells to which handover is not triggered) and cause interruption.

Observation 4:	If 6GR can support more RA preambles in one RA occasion, the handover latency can be reduced through CFRA-based UL access.

Observation 5:	If RAN2 can optimize RRC signalling to reduce decoding complexity, early decoding may not be needed.

Observation 6:	Providing a candidate cell configuration to the UE requires the candidate cell to reserve radio resources for the UE, for initial access and for operation in the candidate cell.

Observation 7:	Candidate cells may need to modify the configurations already provided to UEs not in the cell in some cases (e.g., because of load change).

Observation 8:	Early CSI acquisition is useful to minimize throughput degradation after handover. It is essential to identify the right Tx/Rx beam before CSI acquisition.

Observation 9:	Subsequent mobility without need of reconfiguration increases network complexity for limited gain/scenario.

Observation 10:	Support of subsequent mobility vastly increases UE complexity as it requires a specific RRC reconfiguration procedure, and does not allow delta configuration.

Observation 11:	As 5G NR evolved, mobility and beam management started to overlap in terms of measurement and reporting functionalities.

Observation 12:	The network can use L1 measurement results to derive cell/beam-level measurements and L3-filtered measurements effectively.

Observation 13:	RRM measurement and reporting discussions for 6GR mobility require knowledge of physical layer reference signals in 6GR, which RAN1 needs to provide to RAN2.

Observation 14:	The current mobility reporting framework in connected state may not meet the agreed energy efficiency requirements for 6GR mobility. Less measurement reporting is needed when the UE has no data.

Observation 15:	RAN1 has started to investigate issues related to energy efficient RRM/mobility in idle state.

Inter-RAT mobility

Proposal 1:	RAN2 should study intra-RAT handover and inter-RAT handover in 6GR. RAN2 should start with intra-RAT handover, followed by inter-RAT handover (e.g., 6G to 5G handover).

Unified handover corresponding to requirements

Proposal 2:	In accordance with the agreed requirements, RAN2 studies at least:

- To minimize interruption time: early DL sync, CFRA-based handover and early UL sync (without interruption),

- To minimize throughput degradation: early CSI acquisition,

- To balance complexity and benefits: one-time mobility (i.e., without subsequent mobility), using the basic reconfiguration procedure (which supports delta signalling).

Proposal 3:	To ensure service continuity, RAN2 should study L2 operations during handover to improve L2 continuity, considering both complexity and benefits when exploring potential solutions.

CHO and resource reservation issues

Proposal 4:	For 6GR conditional handover and candidate pre-configurations, RAN2 should study network resource reservation issues.

Unified measurement

Proposal 5:	RAN2 should collaborate closely with RAN1 to develop a unified solution for measurement and configuration that is suitable for L1 and L3 measurements for 6GR mobility. It is important to involve RAN1 early to ensure a thorough understanding of this principle.

Energy efficient mobility

Proposal 6:	RAN2 should study a UE-based mobility function in the connected state for energy efficiency. E.g., the UE can switch cells within a certain area without notifying the network, based on its RRM measurements, when the UE is in a no-data period.

Proposal 7: 	RAN2 should follow RAN1 progress on idle mobility measurement framework and study energy efficient cell selection/reselection.

# 4	References

- RP-251881, New SID: Study on 6G Radio;

- R2-2506858, “Discussion on 6G mobility designs”, Huawei, HiSilicon;

- R2-2508562, RRC state: requirements and functions discussion, Huawei, HiSilicon.