3GPP TSG-RAN WG2 Meeting #132                                                                           R2-2508594

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.3.1.1

Source:	HONOR

Title:	Discussion on Functionality for User Plane

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were achieved:

1. RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements. 2. UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. 3. 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB) 4. Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework. 5. Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant. 6. Study need for scheduling enhancements to address the SR/BSR/DSR latency 7. Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In this document, the baseline functions and the aspects requiring further study for User Plane are given.

- Discussion

  - Essential functions of user plane

In the 5G NR, the several UE types (e.g. Redcap/eRedcap) and different services (EMBB/URLLC/MMTC/XR/IAB/sidelink) are supported. However, the UE types and some services are considered in the later release. In the 6G Day 1, the diverse UE types and use cases will be supported, which have different bandwidth or different delay requirements. For example, the device types contain one or multiple eMBB (or 6G IoT) device types and other device types, e.g. XR/immersive experiences, FWA, vehicle UEs (VUEs), wearables/RedCap, sensing (as standalone or complementary), NTN (standalone or complementary), AI agents, collaborative robots, etc. Therefore, different protocol layers with different functions should be designed to make their configuration more flexible and adaptable during deployment. This session gives an overview functionality of each protocol layer in the current 5G system and analyses motivation whether they are needed or not for the 6GR.

Table.1 Overview functionalities which are needed for the 6GR

| Functions in 5G | Analysis | 6G |
| --- | --- | --- |
| QoS flow mapping | Include “Mapping between a QoS flow and a data radio bearer” and “Marking QoS flow ID (QFI) in both DL and UL packets”. 6G systems will support more services with different QoS requirements, it is needed that identify the QoS flow and mapping the QFI of data packets to an appropriate DRB. | support |
| Maintenance of SNs | In the PDCP and RLC, this serves as the foundation for reordering, duplicate discarding, ciphering, integrity protection, reassembly and retransmission. | support |
| Duplicate discarding/detecting | Duplicate reception may occur due to handover, duplication or HARQ. Identifying and discarding duplicate SDUs based on the PDCP SN is an important function to ensure data accuracy and prevent resource waste. | support |
| Compression and decompression | The compression algorithms (e.g., ROHC, EHC and UDC) serve as enhanced features for specific scenarios or services. And 6G will still support services such as small-packet traffic and vertical industries, etc. | support and deploy as needed. |
| Ciphering and deciphering; Integrity protection and integrity verification | In 5G NR, both are mandatory for control plane data, but optional for user plane data. Some services such as app traffic (e.g., eMBMS) may choose to operate without ciphering and integrity protection. Similarly, in 6G, there will still be traffics that do not require these security features. | support and deploy as needed. |
| Timer-based SDU discard | In order to clear the buffer in time and meet the overall transmission delay, the PDCP support the timer based SDU discard. Meanwhile, in the AI model transmission (delay-tolerant), the discard timer may be not necessary, the 6G can consider whether the timer based SDU discard is a mandatory function for the PDCP. A similar RLC discard function shall be optionally enabled based on the PDCP discard function. | Need discussion |
| Duplication | It is designed as enhancements for 5G URLLC services to improve reliability. Similarly, for 6GR, this function can be configured for specialized services with high reliability requirements. | configured for specialized services. |
| For split bearers and DAPS bearer, routing | If UE need to simultaneously transmit data to non-single cell in 6G, then these features can be considered. | Not consider in Day1 |
| Re-ordering, in-ordering and out-of-ordering delivery | The 6G should support various services, such as services that require high reliability but are not sensitive to latency (e.g., file downloads, emails), or services that are extremely sensitive to latency but can tolerate a small amount of packet loss (e.g., voice and video). | support |
| Segmentation, re-segmentation and reassembly of SDU | To achieve the adaptation of data PDU to transmission resources, the 6G shall support segmentation/reassembly of SDU. | support |
| ARQ | Relying on MAC HARQ retransmission is insufficient to meet the packet loss rate requirement of 10⁻⁵ for high-reliability services. Therefore, the ARQ is a necessary mechanism to ensure service reliability. | support |

Proposal 1: The functions supported in the Table.1 can be inherited to 6GR as baseline, e.g. QoS flow mapping, (de)compression, security handling, duplicate discard, (in/out of) ordering delivery, (re)segmentation, reassembly and ARQ.

Proposal 2: 6GR study the possible protocol stack and mandatory/optional function set for each layer.

2.2 Further studies for user plane

6G will support new services such as immersive communication and AI traffic, which place higher demands on spectrum efficiency and latency. In particular, some existing 5G technologies may reduce spectrum efficiency and increase latency, for example, in the following aspects:

- Transmit the unnecessary packets,

- Header redundancy,

- Handling without distinction for every packet in one DRB.

The next part focus on the above aspects and give the analysis.

2.2.1 Transmit the unnecessary packets

For the transmitting side, PDCP support timer-based discard and send the discard indication to RLC for the discarded SDU. RLC will discard the SDU only when the corresponding SDU is not transmitted to lower layer, which cause the outdated data is still transmitted in the air interface. In the Rel-19 XR, a solution was given as the transmitting RLC entity can stop transmitting outdated data and receiving RLC entity discard the data based on discard timer. This significantly reduces the transmission of unnecessary data.

For the receiving side, the PDCP can decide which SDU is outdated based on reordering timer. If  configured with AM mode, the receiving RLC entity will require the transmitting RLC entity to retransmit the SDU which is not received. There may be an unalignment that receiving RLC entity will request retransmission of the outdated SDU in PDCP which is not needed for PDCP entity.

According to the above analysis, 6G data transmission and reception should avoid retransmitting data that is no longer needed.

Proposal 3: 6G data transmission and reception should avoid (re)transmitting unnecessary data.

2.2.2 Header redundancy

In 5G NR, some mechanisms may cause header redundancy. Since there are higher transmission efficiency requirements, 6G should study reducing header redundancy.

- Security handling in PDCP

Currently, the security handling calculates and adds MAC-I information in header for each PDCP SDU. This per-SDU security mechanism is inefficient, especially for small data packets. To address these issues, concatenation could be considered. By combining multiple packets into a single unit, the number of required L2 headers and MAC-I fields can be reduced, thereby improving transmission efficiency and reducing L2 header overhead.

- One-to-one mapping between PDCP SDU to RLC SDU

PDCP and RLC associate SN with data packets and maintain the transmission/reception status of packets based on this information. This results in some L2 header redundancy. Header combination in PDCP and RLC may maintain packet status based on the same SN to reduce L2 header overhead.

Proposal 4: 6G should reduce header redundancy, e.g. header combination in RLC and PDCP, packet concatenation in PDCP etc.

2.2.3 Handling without distinction for every packet in one DRB

The XR discussion initially focused on the requirement of data types, such as video, audio, images and motion. Based on these data types, service requirements and several characteristics of XR transmission were outlined, including dependencies and varying priorities between different data, integrity requirements for data forming a data set, and multimodal data transmission.

Since AI scenarios still involve the transmission of audio, videos, and text, some XR packet characteristics remain applicable, such as data priority, packet dependencies, and multimodal data transmission. However, for AI data, some companies [2][3] outlined additional characteristics, such as tolerance for AI tokens, somewhat similar to the FEC encoding discussed in the XR. In short, regardless of previous differences between AI data and XR data, this all boils down to new 6G requirements: for the same service, different data packets may exhibit inconsistent characteristics, such as priority.

The above means the different packets in the same QoS flow require vary requirements. 6G should consider the data characteristics for transmission and reception to adapt to changes in the network environment.

Proposal 5: 6G data transmission and reception allow the different data handling in the same DRB based on the data characteristics.

- Conclusions

In this contribution, we discussed user plane and made the following proposals:

Proposal 1: The functions supported in the Table.1 can be inherited to 6GR as baseline, e.g. QoS flow mapping, (de)compression, security handling, duplicate discard, (in/out of) ordering delivery, (re)segmentation, reassembly and ARQ.

Proposal 2: 6GR study the possible protocol stack and mandatory/optional function set for each layer.

Proposal 3: 6G data transmission and reception should avoid (re)transmitting unnecessary data.

Proposal 4: 6G should reduce header redundancy, e.g. header combination in RLC and PDCP, packet concatenation in PDCP etc.

Proposal 5: 6G data transmission and reception allow the different data handling in the same DRB based on the data characteristics.

- References

- Report of 131bis meeting

- R2-2506940  Discussion on 6GR user plane functions and architecture Huawei, HiSilicon

- R2-2506850  Discussion on the 6G user plane features OPPO