3GPP TSG-RAN WG2 Meeting #132                                                                          R2-2508595

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	10.3.1.2

Source:	HONOR

Title:	Discussion on QoS and Service-awareness

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, the following agreements were agreed for UP [1]:

Agreements on UP - RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements. - UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. - 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB) - Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework. - Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant. - Study need for scheduling enhancements to address the SR/BSR/DSR latency. - Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we discuss the QoS and Service-awareness.

- Discussion

2.1 Applications/Service-awareness

SA has studied on 6G use cases and service requirements, and the following traffic types were given to study [2]:

- Integrated Sensing and Communication.

- AI Communication.

- Immersive Communication.

- Massive Communication.

- Ubiquitous Connectivity.

Immersive Communication extends the enhanced Mobile Broadband (eMBB) of 5G.  It requires to support for mixed traffic of video, audio, haptic and other environment data in a reliable and synchronous manner, combining low latency and high data rates. For example, the XR service studied in the later stage of 5G belongs to one type of immersive communication.

Massive Communication extends the enhanced mMTC of 5G, massive communication provides the service for massive number of devices or sensors, e.g., massive Internet of Things (MIoT), LPWA etc.

Ubiquitous connectivity is used to address presently uncovered or scarcely covered areas, particularly rural, remote and sparsely populated areas, and indoor connectivity.

According to the current agenda, sensing and AI for network will be studied in common plane. But the network for AI (e.g. AI mobile traffic) is one type of traffic data, which needs to be studied in user plane.

Proposal 1: RAN2 studies the following traffic types raised in SA in user plane:

- Immersive Communication.

- Massive Communication.

- Ubiquitous connectivity.

- Network for AI.

For the initial discussion of new 6G traffic, XR service in Immersive Communication can be as a starting point for the following study. In general, XR services include video, audio, pose/control and tactile data, and have the following characteristics:

- Non-integer periodicity.

- Delay sensitive.

- Jitter.

- Multi-modal service.

- PDU set transmission.

Non-integer periodicity and PDU set transmission are introduced by video frames. Delay sensitive and multi-modal service are for all data type. Jitter captures the deviation of the data arrival time. Meanwhile, the PDU set QoS parameters are introduced in [3]:

- PDU Set Delay Budget (PSDB): as defined in TS 23.501 [3], upper bound for the duration between the reception time of the first PDU (at the UPF for DL, at the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE in DL, at the UPF in UL). A QoS Flow is associated with only one PSDB, and when available, it applies to both DL and UL and supersedes the PDB of the QoS flow. - PDU Set Error Rate (PSER): as defined in TS 23.501 [3], upper bound for a rate of non-congestion related PDU Set losses between RAN and the UE. A QoS Flow is associated with only one PSER, and when available, it applies to both DL and UL and supersedes the PER of the QoS flow. NOTE 1: In this release, a PDU set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully. - PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3].

For AI mobile services, the types of data include images, text and audio. At the same time, video is also evolving. So, AI services usually also have the characteristics of XR services. However, text in AI is typically transmitted in units of a few characters, resulting in relatively small packets. Therefore, new services (e.g. immersive communications, AI mobile traffic) supported by 6G exhibit the following characteristics:

- Non-integer periodicity,

- Delay sensitive,

- Jitter,

- Multi-modal service,

- PDU set transmission,

- Small packet.

Proposal 2: Study the following traffic characteristics in 6GR for immersive communications and AI mobile traffic:

- Non-integer periodicity,

- Delay sensitive,

- Jitter,

- Multi-modal service,

- PDU set transmission,

- Small packet.

2.2 QoS related

In the previous framework, QoS was provided and managed by SA. In the overall 5G and 4G data scheduling, QoS is very helpful for the RAN side scheduling strategy. Especially after the introduction of XR, AI and sensing data, these services require higher spectrum utilization and lower latency. However, network-side scheduling strategies cannot always satisfy the transmission of all data for these services. Therefore, a more flexible QoS framework is needed. If the QoS endpoint is at SA, this will introduce significant latency. Conversely, RAN-level QoS can offer more flexibility and lower latency. It is believed that the RAN-level QoS serves the SA QoS framework and is a more granular framework than SA QoS framework. It can be dynamically adjusted according to the network environment and provide dynamic   service quality according to the load. Therefore, the RAN-level QoS can be achieved in the following ways:

- Study finer QoS parameters in RAN,

- Dynamic service quality based on lowest QoS requirement.

The finer QoS parameters are similar as PSER and PSDB, RAN-side can dynamically adapt transmission policy and scheduling policy based on the finer QoS parameters to perform differentiated data handling. The SA will provide the finer QoS parameters to RAN.

For another aspect, SA can provide the lowest QoS requirement and RAN offer the QoS based the current channel quality. If the channel quality changes, the network can adjust the scheduling policy, e.g. LCP and HARQ etc.

Proposal 3: Study the RAN-level QoS: (1) Study finer QoS parameters in RAN, (2) Provide dynamic service quality based on lowest QoS requirement.

In SA2#171, the views of companies on the 6G QoS framework are summarized as follows [4]:

Items that have consensus: Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience. Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g., traffic pattern, dynamic QoS requirements and awareness in the application/UE of what can be provided by the network, e.g., maximum bitrate. NOTE 1: Applications typically consist of an Application Client in the UE and application server in the DN.

Dynamic QoS requirements require that the RAN further sense real-time changes in application/service traffic attributes, such as variations in throughput, latency, and reliability. Such requirements may arise during different phases of the same service or apply to data packets of differing importance within the same service. For example, in immersive communications, normal scene rendering generally requires high bandwidth (~30–50 Mbps) and moderate latency (~20–40 ms). However, when a user triggers a critical action, the network must provide extremely low latency (<10 ms) and ultra-high reliability within a short time window (e.g., the next 50–100 ms) to prevent visual tearing and ensure seamless interaction. Once this critical state ends, service requirements revert to normal levels.

Observation 1: The RAN supports awareness of dynamic traffic characteristics—such as periodicity/burstiness, maximum burst size, high throughput, and low latency—to enable more efficient resource allocation and scheduling.

The dynamic QoS behavior reflects a finer granularity of service demands over time, and contrasts with the static QoS in 5G, which relies on fixed 5QI levels. Once a QoS flow is established, parameters such as scheduling policy, priority, and guaranteed bit rate (GBR) typically remain unchanged for the duration of the session. Although modifications are supported, the process is complex and lacks a "lightweight" mechanism. To support QoS at a finer time granularity, 6G should study fast adaptation of transmission configurations at both the UE and RAN.

Proposal 4: 6G study fast adaptation of transmission configurations at both UE and RAN to accommodate dynamic QoS requirements.

- Conclusions

In this contribution, we discussed user plane and made the following proposals:

Proposal 1: RAN2 studies the following traffic types raised in SA in user plane:

- Immersive Communication.

- Massive Communication.

- Ubiquitous connectivity.

- Network for AI.

Proposal 2: Study the following traffic characteristics in 6GR for immersive communications and AI mobile traffic:

- Non-integer periodicity,

- Delay sensitive,

- Jitter,

- Multi-modal service,

- PDU set transmission,

- Small packet.

Proposal 3: Study the RAN-level QoS: (1) Study finer QoS parameters in RAN, (2) Provide dynamic service quality based on lowest QoS requirement.

Observation 1: The RAN supports awareness of dynamic traffic characteristics—such as periodicity/burstiness, maximum burst size, high throughput, and low latency—to enable more efficient resource allocation and scheduling.

Proposal 4: 6G study fast adaptation of transmission configurations at both UE and RAN to accommodate dynamic QoS requirements.

- References

- R2_131bis_ChairNotes_25-10-17_13-00

- TR 22.870 Study on 6G Use Cases and Service Requirements

- TS38.300 v19.0.0

- S2-2509800