3GPP TSG-RAN WG2 Meeting #132                                                                             R2-2508596

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	10.3.1.3

Source:	HONOR

Title:	Discussion on Scheduling and Retransmission

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, the following agreements were agreed for UP [1]:

Agreements on UP - RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements. - UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. - 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB) - Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework. - Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant. - Study need for scheduling enhancements to address the SR/BSR/DSR latency. - Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we discuss the scheduling and retransmission.

- Discussion

2.1 Configured-grant enhancements

In 5G NR systems, uplink scheduling comprises two primary mechanisms: Dynamic Grant (DG) and Configured Grant (CG).

- Dynamic Grant: When a UE has uplink data to transmit, it initiates the process by sending a Scheduling Request (SR) or a Buffer Status Report (BSR) to the gNB. Upon receiving the request, the gNB dynamically allocates PUSCH resources via Downlink Control Information (DCI) transmitted over the PDCCH.

- Configured Grant: The network pre-allocates periodic uplink resources to the UE through RRC signaling (Type 1 CG, statically configured) or via PDCCH-based activation/deactivation (Type 2 CG). With CG, the UE can transmit data directly at predefined intervals without prior SR or BSR transmission, thereby reducing signaling overhead and scheduling delay.

Although CG has advantages in reducing latency and signaling overhead, there are still some limitations. As dedicated CG resources are allocated per UE, any unused resources are hard to reclaim despite Multi-Pusch CG was introduced in later release of 5G. CG is primarily designed for periodic traffic patterns, which limits its flexibility. To overcome these limitations, we support to study enhanced Configured Grant scheme in 6G from RAN2 perspective to provide a high resource utilization rate and flexible uplink access solution.

Proposal 1: RAN2 study enhanced Configured Grant scheme in 6G for uplink scheduling.

2.2 L2 ARQ-HARQ

For the UL transmission, there is no explicit network ACK/NACK for HARQ in 5G. When the HARQ is for new transmission, the UE has no knowledge of whether the pervious TB of this HARQ is successful. If the receiving side (network) considers the enhancement of ARQ based on HARQ, e.g. by SR, the UE do not need to detect whether the HARQ transmission is failure. If the transmission side (UE) considers the enhancement of ARQ based on HARQ, the UE needs to know whether the HARQ transmission is failure. Therefore, if the L2-ARQ based on HARQ is supported for UL in transmission side, the network needs to instruct the UE whether the HARQ transmission is successful.

For the DL transmission, the UE will indicate the HARQ feedback to network. However, the same issue as in the UL transmission is needed to be considered, whether to enhance this in receiving side or the transmission side.

Proposal 2: RAN2 studies whether to perform L2 ARQ based on HARQ failure at the transmitting side or the receiving side.

Proposal 3: If UL ARQ based on HARQ failure is supported in UE, the network needs to notify whether the UL HARQ transmission is successful.

In general, a single HARQ process transfers one or two TB(s), and each TB carries multiple packets from multiple LCHs. To achieve ARQ based on HARQ, it may be necessary to maintain a corresponding relationship between each RLC SDU and the TB. However, it could drastically increase the processing complexity of L2 layer. Therefore, it needs to strive for a method that reduces L2 operations while allowing the RLC layer to detect HARQ transmission failures.

Proposal 4: The solutions for L2 ARQ based on HARQ should consider the L2 layer complexity while allowing the RLC layer to detect HARQ transmission failures.

- Conclusions

In this contribution, we discussed user plane and made the following proposals:

Proposal 1: RAN2 study enhanced Configured Grant scheme in 6G for uplink scheduling.

Proposal 2: RAN2 studies whether to perform L2 ARQ based on HARQ failure at the transmitting side or the receiving side.

Proposal 3: If UL ARQ based on HARQ failure is supported in UE, the network needs to notify whether the UL HARQ transmission is successful.

Proposal 4: The solutions for L2 ARQ based on HARQ should consider the L2 layer complexity while allowing the RLC layer to detect HARQ transmission failures.

- References

- RP-252912