3GPP TSG-RAN WG2 #132	TDoc R2-2508615

Dallas, Texas, USA, Nov. 17th-21st

Agenda Item:	10.3.1.3 Scheduling, retransmissions and uplink scheduling information

Source:	Ericsson

Title:	UL HARQ- and CSI feedback in 6G

Document for:	Discussion, Decision

# 1	Introduction

Control messages for the Uu interface were defined by various (sub-) working groups in TSG RAN. UE and gNB exchange those messages via different “channels”. An NR UE sends uplink control information either through PDCP (SRBs), in MAC Sub-PDUs (MAC CEs) or on PUCCH (L1 UCI).

In this contribution we investigate why LTE mapped UL HARQ feedback (FB) and CSI reports onto PUCCH and we explain why this became increasingly complicated in NR. We present simulation results which show that mapping HARQ FB and CSI reports into MAC Sub-PDUs would improve the end-to-end performance and system capacity.

# 2	Discussion

## 2.1	NR’s UCI on L1

4G and 5G UEs send their UL HARQ-FB and CSI reports on a tailor-made physical channel (PUCCH).

To save signalling overhead, the HARQ-FB reports comprise only of a set of ACK/NACK bits without explicit process IDs. UE and network derive the HARQ process IDs from the previously scheduled HARQ processes, from DAI fields in the DL grants and from the PUCCH’s transmissions time.

This design was considered feasible, fast and resource efficient in HSDAP (FDD only, no CA, no MIMO) and still practical in LTE Rel-8.

Figure 1: L1 UCI with a single FDD carrier

But the PUCCH signalling became increasingly complex in later LTE releases (carrier aggregation, TDD) and in NR (more CA, mixed numerologies, code-block groups, …). The effort for book-keeping, inter-carrier coordination and resolving race conditions (e.g. in case of lost DL grants or ACK/NACK errors) increases substantially with the number of carriers and numerologies.

Figure 2: L1 UCI with multi-numerology CA

The allocation of PUCCH resources (PRI), and the setting of K1-values (time from assignment to UCI) and DAI values (downlink assignment index) must be coordinated across the DL grants that the gNB sends for different carriers. This would be complex but feasible if a single scheduler thread could take all scheduling decisions for all UEs in all carriers sequentially. However, with more carriers of various characteristics, scheduling is typically distributed across several parallel threads or processes. In such a setup the coordination of K1- and DAI values and PUCCH resources across carriers puts tight requirements on the inter-thread or inter-process communication. And such inter-process coordination translates into processing overhead and additional latency. Furthermore, if the UE did not detect all DCIs and hence derived a different PUCCH format than the gNB expected, this would lead to HARQ feedback errors.

NR’s HARQ feedback for carrier aggregation requires tight coordination among involved schedulers which is very challenging in practical implementation.

Similarly, the channel state information (CSI) evolved from a simple “CQI+rank” in LTE to a diverse set of CSI report types of various sizes and with different processing complexity. The latter restricts when the network may schedule those reports and regular data. For example, if the gNB requests an aperiodic CSI report, the UE needs 72 symbols (almost 6 slots) on a 30 kHz carrier to generate the report. Generating a pure PUSCH HARQ transmission takes only ~2 slots.

The selection and coordination of PUCCH formats becomes even more complicated when different UCI types (HARQ FB and CSI reports) must be multiplexed into the same slot. For this purpose, NR offers standardized multiplexing rules for all combinations of different UCI and resource types. These also led to complex timeline requirements when scheduling PUCCH and PUSCH resources. And the introduction of new functionality such as prioritization rules for eURLLC, often requires a complete re-design of those complex rules.

NR also allows multiplexing UCI with UL data. In this case the UE encodes UCI and data separately, maps the encoded UCI bits into a subset of the PUSCH resource elements (REs) and fills the remaining REs with the encoded data bits. In other cases, the UE punctures PUSCH data symbols and sends instead HARQ feedback bits therein. Both cases lead to decoding errors when the UE did not detect all DCIs and hence derived a different UCI format than the gNB expected. To avoid that gNB and UE derive a different number of PUSCH resources for HARQ feedback, the gNB must send the DAI also in the UL grant.

The above-mentioned processing constraints restrict UL scheduling if UL data coincides with a requested CSI report. In such a case the gNB cannot send another UL data grant in a subsequent PDCCH occasion for an earlier UL slot – UL grants shall not overtake each other, which is illustrated in the Figure below.

Figure 3: Scheduling restrictions due to CSI processing

And finally, the encoded UCI steals PUSCH resource elements from the encoded transport block (so-called rate matching) which increases the HARQ BLER especially when the UE is in challenging radio conditions.

More details and additional examples of the problems encountered with NR’s UCI and PUCCH can be found in [2], section 9.

Multiplexing HARQ feedback and CSI reports into PUCCH or PUSCH is complex and error-prone due to multiplexing rules and timeline requirements.

The complexity of HARQ-FB and CSI reports and the way they are transmitted in NR became one of the biggest pain points for network and UE implementation. The complexity hinders the swift implementation of new carrier aggregation variants (more carriers; different constellations of FDD/TDD carriers) or advanced CSI types.

The complexity of HARQ-FB and CSI reports and the way they are transmitted in NR became one of the biggest pain points for network and UE implementation.

A new generation is a rare opportunity to try to avoid complexity that piled up in previous generations. Of course, complex solutions may be justified if they offer additional benefits that justify their implementation and testing cost.

Therefore, RAN2 should study whether the complexity of NR’s solution for sending UL HARQ- and CSI feedback on PUCCH is justified by performance benefits compared to simpler schemes. Since RAN2 is responsible for the Uu user plane protocol stack and for its impact on end-to-end performance, we request RAN2 to perform this analysis.

Evaluate how different transmission schemes for UL HARQ- and CSI feedback affect the end-to-end performance for various applications. Choose a transmission scheme that offers the best performance/complexity trade-off.

## 2.2	6G HARQ- and CSI feedback in MAC Sub-PDUs

NR- and LTE UEs multiplex feedback such as BSR and PHR into the MAC PDU and transmit them in the PUSCH transport block which is protected by CRC and HARQ.

MAC offers a generic framework of multiplexing control information and user data.

Hence, we investigated whether it is feasible to multiplex 6G’s HARQ FB and CSI reports into MAC PDUs rather than onto a tailor-made physical channel. In the following we discuss some high-level principles and provide simulation results of such an approach.

In accordance with the discussion in the previous section, 6G’s UCI reports should contain explicit references to HARQ processes and CSI report configurations so that the gNB can interpret them irrespective of the time at which the UE sent them. This would relieve the gNB from DAI coordination and from choosing UCI- and PUCCH formats. And it would leave more freedom to the UE when generating and multiplexing reports. Explicit identities and self-contained reports are of course particularly important if 6G UEs send HARQ- and CSI reports in the payload of UL HARQ processes.

The above-mentioned scheduling restrictions in case of CSI types that require long processing delays would vanish. The gNB would just schedule a dynamic PUSCH grant when it expects HARQ FB or CSI reports and the UE multiplexes the prepared reports like other MAC sub-PDUs. We expect that a UE could, while processing a CSI measurement, continue sending UL data in intermediate UL slots to avoid the scheduling restriction depicted in Figure 3.

Self-contained HARQ-FB and CSI-formats would be larger than NR’s optimized UCI formats due to explicitly signalled IDs, MAC Sub-Headers and CRCs. Especially compared to NR’s small PUCCH formats the link budget of a PUSCH transmission would be noticeably worse. However, NR’s coverage is in practice not determined by the link budget of the PUCCH but rather by PUSCH. In other words, if an NR UE cannot transmit UL user data, TCP ACKs and RRC messages at a reasonable data rate, it does not help if it could still transmit PUCCH.

Explicit IDs for HARQ processes and CSI reports would make the reports self-contained and avoid timing restrictions, inter-carrier dependencies and race conditions that were observed with NR’s L1 UCI.

The PUSCH’s in-built CRC protection and UL HARQ would resolve NACKóACK errors that made LTE’s and NR’s DL HARQ unreliable. Also, CSI transmissions would naturally become robust and additional error handling would no longer be required. Of course, UL HARQ retransmissions would delay the multiplexed HARQ feedback and CSI reports. The performance impact of those needs to be investigated (see later sections).

The PUSCH’s in-built CRC protection and UL HARQ would improve robustness and avoid race condition at the expense of larger jitter.

## 2.3	System simulations

We carried out system simulations with realistic channel-, protocol- and traffic-models to assess the performance impact of multiplexing self-contained UL HARQ- and CSI reports into MAC sub-headers. We compare this with NR’s tailor-made PUCCH and UCI formats.

### 2.3.1	Simulation setup

We simulated a single carrier FDD scenario in which NR’s “UCI on PUCCH” is likely to perform well (see discussion above). With TDD and/or with many carriers the relative overhead due to explicit references would be less significant.

| Parameter | Value |
| --- | --- |
| Propagation | 3GPP SCM Urban Macro, 80% indoor |
| Inter-site Distance | 500 m, 9 cells |
| Carrier | 1.8 GHz, 20 MHz bandwidth, FDD |
| BS Antenna | 3GPP 36.814, 4 antenna elements |
| UE antenna | Isotropic, 2 elements |
| Power | BS=160 W, UE=0.25 W |
| Noise factor | BS=2.3 dB, UE=11 dB |
| RAN Protocols | MAC, RLC AM, PDCP |
| Users and Traffic |  |
| Part I: | File download, 5 MB file (TCP Cubic) |
| Part II: | Web page download (including HTTPS, TLS, DNS, multiple page objects, TCP Cubic). 2MB divided in 20 objects Realtime uplink video (2KB, 60FPS) (UDP/RTP) Small file upload (10 KB) (TCP Cubic) |
| Transport Network Delay | 5 ms, one-way |

We compare 5 different configurations in the following plots:

nr = UCI (CSI, SR, HARQ-ACK) on PUCCH and PUSCH as in NR

  - SR periodicity is 5 ms

The following 3 schemes are the same as ‘nr’, except …

  - csi-only = UE sends CSI reports as MAC CE in PUSCH payload

  - fb-only = UE sends UL HARQ feedback as MAC CE in PUSCH payload

  - cb-only = SR on PUCCH replaced by BSR on contention-based PUSCH

    - CB-PUSCH resource: Every uplink slot, 2 RBs, 8 DMRS

nopucch = Above 3 schemes together (PUCCH-less operation)

It should be noted that “cb-only” and “nopucch” are shown here only for completeness. The details about our proposals to send BSRs on a contention-based PUSCH (instead of SR on PUCCH) can be found in our companion paper [1].

### 2.3.2	File download (5 Mbyte)

Figure 4 shows the file download performance (file size divided by the transfer delay) for increasing system load. In this example all UEs request a 5 MB file which the server transfers using TCP. Hence, the transfer time is not only determined by the users’ downlink radio conditions but also by protocol effects such as the TCP slow start.

The NR baseline (UL HARQ FB and CSI reports sent on UCI/PUCCH) performs worst at all load levels! Multiplexing only the CSI reports into the MAC PDUs has only a small positive performance impact. If the UE sends only the UL HARQ feedback in MAC PDUs, the performance benefit is more significant (~10%).

Figure 4: File download performance vs. system load

The gain may not seem overwhelming at a first glance. But one should bear in mind that the primary purpose of sending self-contained HARQ- and CSI feedback in UL MAC PDUs was to simplify UE and NW implementation substantially. We expected a performance loss but see instead a noticeable gain!

Multiplexing UL HARQ- and CSI feedback into MAC PDUs improves the file download performance compared to NR’s complex “L1 UCI on PUCCH” scheme.

This unexpected result is caused by a positive free-ride effect: When the NW sends UL grants that are supposed to carry expected CSI reports or HARQ feedback for previously scheduled DL HARQ processes, the UE can often fit additional UL data or at least a BSR into the same UL grant. This effect occurs often since most data transfers are bi-directional (TCP/QUIC ACKs, …) and it accelerates the UL scheduling noticeably. This free-ride effect also helps to keep the PDCCH utilization reasonably low.

The network sends PUSCH grants when it expects UL HARQ- or CSI feedback. In many cases the UE can piggyback UL data and/or BSRs into the same PUSCH transport block. This free-ride effect accelerates data transfers and avoids excessive control signalling overhead when HARQ- and CSI feedback is multiplexed into MAC PDUs.

### 2.3.3	Traffic mix

The following plots compare the performance of the same 5 configurations (nr, csi-only, fb-only, cb-only, nopucch) for a mix of three different traffic types. As in the previous section, the e2e performance improves when we map HARQ- and CSI feedback into MAC PDUs. It can be observed that the performance gain is smaller for uplink-heavy applications. Realistic downlink web-page transfers (many objects of various sizes via several TCP connections) benefit most. The performance can be enhanced further if the UE sends also BSRs on a CB-PUSCH rather than SRs on PUCCH (nopucch).

Figure 5: Performance vs. system load for a traffic mix
(different UEs running different services in the same network)

Figure 6 shows the UL resource utilization (left) and the PDCCH utilization (right) for the same traffic mix.

The total UL resource utilization decreases noticeably when UE send CSI reports, HARQ-FB and BSRs in the PUSCH payload (see fb-only and nopucch). In this case, the gNB does not need to pre-book PUCCH resources at the edges of the carrier and allocate them instead dynamically for PUSCH transmissions. It should be noted that the plot is overly optimistic for the NR baseline since the simulator releases a dedicated SR allocation immediately when a UE’s file transfer ends. In reality, the gNB does not know whether more data is going to arrive and hence keeps the PUCCH SR allocation until terminating the RRC connection. A more realistic assessment for that case can be found in [1].

The overall UL resource utilization decreases noticeably when 6G UEs send their UL HARQ- and CSI feedback in the PUSCH payload.

Since the gNB must send UL grants to enquire expected HARQ- and CSI-feedback, the PDCCH utilization increases slightly when UEs send CSI- and HARQ feedback in the PUSCH payload (see fb-only, cb-only, nopucch). Due to the above-mentioned free-ride effect the increase remains however reasonably low. Even at high system load the PDCCH was not highly loaded (max. 4 CCEs, i.e., 4x6=24 PRBs and hence ~22% of the 106 PRBs). One should also note that even a 10% increase in PDCCH resources would correspond to only ~1-2% of the total DL resources (PDSCH+PDCCH). This has marginal effect on the e2e performance compared to the positive protocol effects described above.

The PDCCH resource utilization increases slightly when 6G UEs send their UL HARQ- and CSI feedback in the PUSCH payload. But since this increase is just a small fraction of the overall DL resources, the positive protocol effects over-compensate this effect (as the performance results revealed).

Furthermore, in these simulations the UEs sent RLC status reports in uplink. If 6G UEs send their HARQ feedback in the PUSCH payload one could consider omitting explicit RLC feedback and thereby save additional resources especially on PDCCH.

Figure 6: UL Resource utilization (left) and PDCCH load (right) for a traffic mix.
(different UEs running different services in the same network)

### 2.3.4	Uplink Coverage

To investigate the impact on uplink coverage we simulate voice traffic. It is a low-data-rate service for which the relative overhead of uplink control signalling could be significant. Hence, the performance of this service could potentially suffer when HARQ FB is larger due to explicit HARQ IDs and if it is mapped into the MAC PDU rather than onto a tailor-made physical channel.

Figure 7: File download performance vs. system load

The figure above shows the tail latency of downlink- and uplink voice traffic. UEs that receive downlink voice traffic generate and depend on uplink control signalling. Hence, if sending HARQ FB inside the MAC PDU would have negative performance impact (due to UCI having worse coverage if sent in the PUSCH payload compared to PUCCH), it would be visible primarily in the left plot. And while one can notice a small latency increase for large inter-site distances, the latency increase for uplink voice traffic increases much earlier for increasing ISDs (for both feedback schemes). In other words, the coverage area for voice traffic is determined by the UL data channel and not by the uplink control signalling.

Even for low-data-rate traffic the coverage is limited by the link budget of the uplink data channel capacity. The better link budget of NR’s PUCCH does not translate into better service coverage.

## 2.4	Summary

As discussed above, the current UCI formats and their transmission in a tailor-made physical channel (PUCCH) is one of the primary complexity pain points of 5G. This could be justified if the complexity would result in superior performance or system capacity for realistic use cases.

Most of the complexity would vanish if 6G UEs would multiplex their UL HARQ feedback and CSI reports into the MAC PDU. While one might have expected that such simplification decreases performance, system capacity or coverage, we observed the opposite in system simulations. NR’s PUCCH/UCI and its highly optimized (and hence complex) HARQ- and CSI codebooks offer great link budgets and coding efficiency. However, by carrying HARQ- and CSI feedback in the MAC PDU one can achieve advantageous protocol effects (see “free-ride” above) which over-compensate those benefits. That means, it is possible to achieve both, a 6G Uu protocol stack that is simpler and performs better.

More complicated solutions and optimizations should only be introduced if they proof substantial performance/capacity/coverage benefits that justify their cost/complexity.

As baseline, 6G UEs send uplink HARQ feedback and CSI reports in the PUSCH payload (MAC Sub-PDUs).

We think it is valuable to capture simulations and similar evaluations in the technical report to explain why RAN2 took the decisions.

Capture the performance results and the corresponding analysis in the 6G TR (detailed wording to be discussed).

# Conclusion

In the previous sections we made the following observations:

Observation 1	NR’s HARQ feedback for carrier aggregation requires tight coordination among involved schedulers which is very challenging in practical implementation.

Observation 2	Multiplexing HARQ feedback and CSI reports into PUCCH or PUSCH is complex and error-prone due to multiplexing rules and timeline requirements.

Observation 3	The complexity of HARQ-FB and CSI reports and the way they are transmitted in NR became one of the biggest pain points for network and UE implementation.

Observation 4	MAC offers a generic framework of multiplexing control information and user data.

Observation 5	Explicit IDs for HARQ processes and CSI reports would make the reports self-contained and avoid timing restrictions, inter-carrier dependencies and race conditions that were observed with NR’s L1 UCI.

Observation 6	The PUSCH’s in-built CRC protection and UL HARQ would improve robustness and avoid race condition at the expense of larger jitter.

Observation 7	Multiplexing UL HARQ- and CSI feedback into MAC PDUs improves the file download performance compared to NR’s complex “L1 UCI on PUCCH” scheme.

Observation 8	The network sends PUSCH grants when it expects UL HARQ- or CSI feedback. In many cases the UE can piggyback UL data and/or BSRs into the same PUSCH transport block. This free-ride effect accelerates data transfers and avoids excessive control signalling overhead when HARQ- and CSI feedback is multiplexed into MAC PDUs.

Observation 9	The overall UL resource utilization decreases noticeably when 6G UEs send their UL HARQ- and CSI feedback in the PUSCH payload.

Observation 10	The PDCCH resource utilization increases slightly when 6G UEs send their UL HARQ- and CSI feedback in the PUSCH payload. But since this increase is just a small fraction of the overall DL resources, the positive protocol effects over-compensate this effect (as the performance results revealed).

Observation 11	Even for low-data-rate traffic the coverage is limited by the link budget of the uplink data channel capacity. The better link budget of NR’s PUCCH does not translate into better service coverage.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Evaluate how different transmission schemes for UL HARQ- and CSI feedback affect the end-to-end performance for various applications. Choose a transmission scheme that offers the best performance/complexity trade-off.

Proposal 2	As baseline, 6G UEs send uplink HARQ feedback and CSI reports in the PUSCH payload (MAC Sub-PDUs).

Proposal 3	Capture the performance results and the corresponding analysis in the 6G TR (detailed wording to be discussed).

# References

R2-2508344, Scheduling and retransmissions, Ericsson, RAN2-132, Dallas, USA, Nov. 17th-21st

R1-2506841, Overview of the 6G air interface, Ericsson, RAN1-122bis, Prague, Czech Republic, October 13 – 17, 2025