3GPP TSG RAN WG2 Meeting #132                                                         R2-2508625

# Dallas, USA, November 17th-21st, 2025

Agenda item:	10.2.2

Source:	Ofinno

Title:	NTN Integration in 6G

Document for:	Discussion and decision

# Introduction

This document discusses the key issues for 6G NTN based on the agreed 6G SID [1].

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

# Discussion

The integration of Non-Terrestrial Networks (NTN) is a fundamental aspect of achieving ubiquitous connectivity in 6G. Building upon the advancements in 5G NR NTN, 6G still faces key operational challenges: long propagation delay, high connection setup latency, and increased UE power consumption compared to the Terrestrial Network (TN). This contribution discusses how to solve some of the challenges for NTN-TN integration in 6G.

## NTN SDT

The basic purpose of the NTN deployment is to support zero coverage hole worldwide, so the use case of the NTN service includes UEs in rural areas and/or ocean areas. In these cases, low-rate, sporadic, and small amount of data transmissions are assumed. In 5G NR, Small Data Transmission (SDT) was introduced to transmit short data by the UEs in RRC_INACTIVE state. A similar feature seems beneficial for energy-efficient and low-latency of NTN services. By using/extending SDT kind of feature in NTN, the UEs in RRC_INACTIVE can transmit data without resuming the RRC connection which may result in additional delays which for NTN, they can be much longer than the ones in TN environment. Considering integration of NTN and TN, RAN2 would also need to discuss how to handle the UE context handling/sharing/fetching for SDT between TN and NTN base stations (e.g., TN gNB on the ground and satellites gNB-on board).

Proposal 1: To discuss how to improve energy-efficient and low-latency small data transmission in NTN; for example, by considering to re-use (or apply) SDT kind of feature (defined in TN) for NTN.

## Time/location-based mobility

In 5G NTN, time/location-based CHO were introduced based on that the movement of the satellite and its cell coverage on the ground is predictable by the network. Moreover, as the difference between a signal strength at cell center and the one at cell edge is small within an NTN cell coverage, it is difficult for the network to determine when to trigger the handover based on the RSRP measurement report from the UE or to determine RSRP-based thresholds for the CHO. In summary, the time/location-based CHO can be the baseline for connected state mobility in NTN.

For idle/inactive mode mobility, the time/location-based measurement initiation brings energy saving benefit for the UEs (for example, in terms of neighbor cell measurements for the cell reselection). Therefore, it seems helpful to also re-use it in 6G NTN.

Proposal 2: Time/location-based UE mobility (e.g., CHO, measurement initiation) is the baseline for 6G NTN.

## IoT NTN

In IoT-NTN, Store-and-Forward (S&F) mode operation was introduced to serve the IoT devices in the area where the feeder link is not available. IoT devices using delay tolerant data (e.g., hourly/daily report) can be transmitted to the satellites operating in S&F mode. It can be useful for the UEs in remote locations (e.g., the ocean or desert). To achieve worldwide zero coverage hole by NTN, S&F mode operation can be considered in 6G NTN for all types of UE devices including normal UEs, not only IoT devices.

Proposal 3: Store-and-forward operation is considered in 6G NTN for all types of NTN UEs.

## GNSS Independent NTN Operation

In NR-NTN discussion since Release 17, it was assumed that the UEs are capable of GNSS information with enough accuracy. In Release 20, RAN1 is studying the GNSS resilient NTN operation [2] to support UEs with GNSS information temporarily unavailable or available but degraded (which could lead to a reduction in GNSS position accuracy). Many features introduced in NR-NTN are based on location information of the UE. For example, location-based CHO execution condition (i.e., condEvent D1/D2) is associated with a distance between location of the UE and a reference location of the serving/target satellite.

As discussed in Section 2.1 above, one of the key reason why such location-based mobility mechanism was introduced is that the RSRP-based CHO is not sufficient in NTN. If the GNSS information is not available, new mechanism not relying on the RSRP may be needed to handle the UE mobility in NTN environment.

Proposal 4: RAN2 needs to discuss how to handle mobility of UEs with GNSS information temporarily unavailable or available but degraded.

# Conclusion

The contribution discusses aspects related to 6G NTN. The following observations and proposals are made:

Proposal 1: To discuss how to improve energy-efficient and low-latency small data transmission in NTN; for example, by considering to re-use (or apply) SDT kind of feature (defined in TN) for NTN.

Proposal 2: Time/location-based UE mobility (e.g., CHO, measurement initiation) is the baseline for 6G NTN.

Proposal 3: Store-and-forward operation is considered in 6G NTN for all types of NTN UEs.

Proposal 4: RAN2 is required to discuss how to handle mobility of UEs with GNSS information temporarily unavailable or available but degraded.

# Reference

- RP-252912, “Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

- RP-251933, “Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation”, Thales