3GPP TSG RAN WG2 Meeting #132                                                                   R2-2508634

# Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	10.3.3.2

Source:	Ofinno

Title:	6G Energy saving framework

Document for:	Discussion and decision

# Introduction

RAN2#131bis meeting has made the following agreements for energy saving:

Agreements on Energy Saving For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

Based on the previous agreements, this contribution provides our views on the 6G energy saving.

# Discussion

## UE Power Saving Schemes in 5G

According to [1][2], UE power saving mechanisms aim to reduce battery consumption by adapting UE behavior based on traffic patterns and network configurations. The table below summarizes 5G key UE power saving schemes.

| 5G key UE power saving schemes |  |
| --- | --- |
| Discontinuous Reception (DRX) | This is a fundamental power-saving feature where the UE does not have to continuously monitor the PDCCH. DRX is defined by parameters like DRX on-duration timer, DRX inactivity-timer, and DRX cycle. The total time the UE monitors PDCCH is called Active Time. DRX can be configured in two DRX groups for different sets of Serving Cells. |
| Bandwidth Adaptation (BA) | The UE can be configured with one or more BWPs, which are subsets of the total cell bandwidth. Only one BWP is activated at a time, allowing the UE to operate on a narrower bandwidth and save power, especially during periods of low activity. Switching between BWPs is triggered by RRC signaling, DCI, an BWP inactivity timer, or RA initiation. |
| DCI with CRC scrambled by PS-RNTI (DCP) | To further reduce power consumption during DRX, the UE can be configured to monitor a DCP before its DRX on-duration. If the DCP indicates a specific value (or the UE does not detect the DCP), the UE can skip monitoring the PDCCH during the next on-duration. The DCP is only configured on the PCell and/or PSCell. |
| Cross-Slot Scheduling (CSS) | The cross-slot scheduling allows the gNB to schedule PDSCH or PUSCH in a slot later than the slot carrying the scheduling DCI. This provides the UE with "micro-sleep" opportunities between PDCCH decoding and data reception/transmission, thus saving UE power. |
| SCell Activation/Deactivation | In Carrier Aggregation (CA), Secondary Cells (SCells) can be activated or deactivated via a MAC CE. When an SCell is deactivated, the UE does not need to monitor its PDCCH/PDSCH or perform CQI measurements for the SCell. |
| Dormant BWP | The primary goal of a dormant BWP (one of the UE's dedicated BWPs) is to allow for quick SCell activation while enabling UE power saving. When the active BWP of an activated SCell is the dormant BWP, the UE alters its behavior to conserve power. Specifically, the UE stops monitoring the PDCCH and ceases transmissions on the PDSCH, PUCCH, and SRS on that SCell. |
| Adaptation of MIMO Layers | UE power saving can be achieved by adapting the maximum number of downlink MIMO layers, for example, by switching to a BWP configured with fewer layers. |
| UE Assistance Information (UAI) | The UE can send assistance information to the network indicating its preference for certain power-saving configurations, such as changes to DRX parameters, a reduced number of component carriers, or fewer MIMO layers. It can also indicate overheating issues. |
| Paging DRX | In RRC_IDLE and RRC_INACTIVE, the UE monitors for paging messages only during specific Paging Occasions (POs) within a DRX cycle, allowing it to sleep at other times. |
| Extended DRX (eDRX) | The eDRX feature allows for much longer DRX cycles, up to nearly three hours (10485.76 seconds), significantly reducing power consumption for UEs with infrequent data traffic. |
| Paging Early Indication (PEI) | To reduce power consumed by monitoring paging for other UEs, a group of UEs sharing a PO can be divided into subgroups. A UE only needs to monitor its PO if its subgroup is indicated in a PEI broadcast before the PO. |
| Radio Resource Management (RRM) Measurement Relaxation | UEs in RRC_IDLE and RRC_INACTIVE can relax RRM measurements of neighboring cells when they meet certain criteria, such as low mobility or not being at the cell edge. This relaxation is also available for RLM/BFD in RRC_CONNECTED state. |
| Tracking Reference Signal (TRS) | The network can provide configurations for TRS in SIB17. This may allow UEs in RRC_IDLE/RRC_INACTIVE to sleep longer before waking up for their paging occasion. |
| Low-Power Wake-Up Signal and Receiver (LP-WUS/WUR) | This involves a secondary, LP-WUR that continuously or discontinuously monitors for a specific LP-WUS. This allows the UE's main radio to remain in an "ultra-deep sleep" state until woken up by the LP-WUS, offering substantial power savings compared to legacy DRX. |

Limitations in 5G UE power saving

- Fragmented feature introduction limited effectiveness: Features like DCP, PEI, and LP-WUS were introduced incrementally across different releases, leading to lack of holistic coordination. Backward compatibility limited flexibility in optimizing signaling and reference signals.

- Complexity and signaling overhead: Multiple timers (e.g., DRX timers, BWP timers, SCell timers) created tuning challenges. Frequent switching between Active Time/BWPs/SCells could consume more energy than saved if not optimized.

The incremental introduction of power-saving features such as DCP, PEI, and LP-WUS in 5G resulted in fragmented operation and limited coordination across domains.

The coexistence of multiple timers and independent control mechanisms (e.g., DRX, BWP, SCell) increased configuration complexity and signaling overhead. Without unified management, frequent transitions between active states and component carriers could negate intended energy savings, leading to inefficient UE and network power utilization.

Study on 6G UE power saving

In 5G, UE energy efficiency evolved through a series of incremental mechanisms—such as DRX, eDRX, dormant BWP, and LP-WUS, which are introduced across multiple releases (Rel-15 to Rel-19)—each optimized for specific scenarios. This fragmented approach led to complex configuration dependencies, inconsistent operation across UE types, overlapping functions, and increased signaling overhead. To address this, 6G should study a unified power saving framework where UE energy efficiency is a foundational design element, not an afterthought. The goal is to establish a common set of states and procedures that all UEs, regardless of category or service type, can utilize as a baseline.

Study aspects:

- Study and define a minimum set of power saving states (e.g., Active, Light Sleep, Deep Sleep). A key first step in 6G power efficiency design is to define a standardized hierarchy of power states that apply to all UE types. For example:

  - Active: UE is fully operational; all radios and baseband components are active.

  - Light Sleep: Partial deactivation of components (e.g., selective receiver chains, inactive secondary carriers). UE can resume operation with minimal delay.

  - Deep Sleep: Most RF, baseband, and even DRAM components are powered down, relying on a low-power receiver (e.g., LP-WUR) for wake-up detection.

- Study and specify signaling procedures for power saving state entry, maintenance, and exit, including network-controlled and UE-initiated transitions.

  - A unified signaling framework is essential to manage transitions between active and low-power states without the redundancy seen in 5G NR (e.g., DRX + LP-WUS double signaling). This includes both network-controlled and UE-initiated procedures.

  - Define a signaling (e.g., NW command/UE request) primitives to indicate power saving state transitions.

  - Study how autonomous UE decisions (e.g., self-entry into light/deep sleep state) can be reported asynchronously to the network with minimal signaling.

### Expected Benefits:

- Establishes a coherent baseline architecture for power saving in 6G from the first release. Provides a unified foundation for scalable extensions in later releases.

- Simplifies network operation and UE implementation through consistent state definitions and signaling.

RAN2 to study and define a minimum set of UE power saving state (e.g., Active, Light sleep, and Deep sleep) applicable across all UE categories. The study should consider the definition//functionalities of each power saving state.

RAN2 to study and specify a signaling and/or event-based framework for UE power saving state transition (e.g., entry, maintenance, and exit), covering both network-controlled and UE-initiated transitions.

### Network Energy Saving Schemes in 5G

According to [3], network energy saving schemes focus on reducing the operational expenses of the network by, for example, switching off cell capacity boosters when not needed. The table below summarizes 5G key Network Energy Saving (NES) schemes:

| 5G key NES schemes |  |
| --- | --- |
| Cell DTX/DRX | The network can configure a periodic pattern of active and non-active periods for a cell. During non-active periods, the gNB reduces its downlink transmission and uplink reception activities, saving energy. UEs are informed of this pattern and adapt their monitoring and transmission behavior accordingly. A dedicated RNTI, cellDTRX-RNTI, is used for L1 signaling related to this feature. |
| On-Demand SSB (OD-SSB) On-Demand SIB1 (OD-SIB1) | Instead of broadcasting SSBs periodically, the gNB could transmit them only when needed, such as upon a UE wake-up trigger or initial access request. This would allow cells in low-traffic or dormant states to suppress unnecessary SSB transmissions, saving network energy. Similar to OD-SSB, OD-SIB1 could be provided only when a UE requests it, after detecting synchronization. This reduces periodic broadcast overhead. |
| Conditional Handover (CHO) for NES | A UE can be configured with CHO to move away from a cell that is entering an energy-saving state (e.g., activating cell DTX/DRX). |
| Camping restrictions for NES | A cell entering an energy-saving mode can restrict access to only UEs capable of handling that mode, using standard barring mechanisms for other UEs. |
| SSB-less SCell | To save energy from broadcasting Synchronization Signal Blocks (SSB), an SCell can be configured without an SSB. The UE can obtain timing and synchronization from another serving cell. |

Limitations in 6G network energy saving

- Cell DTX/DRX integration gaps: Cell DTX/DRX provided substantial BS-side energy saving, but coordination with UE DRX was limited. Inconsistent support across UEs created interoperability issues. It means that high potential BS-side energy saving was not fully realized, and coordination was largely left to network implementation rather than clear specification.

While cell DTX/DRX achieved notable network-side energy savings, the lack of standardized coordination with UE DRX limited overall system efficiency. Inconsistent UE support and reliance on vendor-specific implementations prevented full realization of joint energy-saving potential, highlighting the need for clearer 6G specifications enabling synchronized UE–network power saving operation.

Study on 6G joint UE/NW energy saving

In 5G NR, energy saving was addressed independently at the UE and network sides. UE energy saving mainly used DRX, DCP, and LP-WUS to reduce receiver activity. NES relied on mechanisms such as cell DTX/DRX, on-demand SSB/SIB1, and PRACH adaptation. However, these mechanisms were specified separately and often conflicted in operation. For example: A UE entering DRX while the cell enters DTX may lead to synchronization loss or delayed access. Network-side energy saving may require frequent reactivation, increasing UE power consumption. Therefore, 6G aims to jointly design UE and network power saving mechanisms from Day-1 to ensure energy efficiency is achieved cooperatively rather than competitively.

Study aspects: RAN 2 should study whether network and UE sleep modes (e.g., Cell DTX/DRX and UE DRX) can be jointly designed as part of a common 6G power saving model, avoiding fragmented approaches from 5G. For example, RAN2 could consider joint cycle alignment to better align UE DRX operation with network Cell DTX/DRX cycles:

- Define signaling for mutual DRX awareness (e.g., UE reporting preferred DRX cycle; gNB broadcasting Cell DTX phase).

- Introduce co-scheduled DRX/DTX windows for downlink and uplink.

- Investigate latency-optimized DRX entry/exit when cell DTX is active.

Expected Benefits:

- Reduced overlap of inactive/active mismatches between UE and network.

- Improved cell energy saving without causing UE reactivation or paging failures.

- Lower control plane overhead by aligning sleep timers.

RAN2 to study a joint UE/NW energy saving framework that integrates UE DRX and Cell DTX/DRX (e.g., synchronized cycle, active time, sleep time, etc.).

# Conclusion

Based on the analysis above, we have the following observations and proposals:

- The incremental introduction of power-saving features such as DCP, PEI, and LP-WUS in 5G resulted in fragmented operation and limited coordination across domains.

The coexistence of multiple timers and independent control mechanisms (e.g., DRX, BWP, SCell) increased configuration complexity and signaling overhead. Without unified management, frequent transitions between active states and component carriers could negate intended energy savings, leading to inefficient UE and network power utilization.

Proposal 1	RAN2 to study and define a minimum set of UE power saving state (e.g., Active, Light sleep, and Deep sleep) applicable across all UE categories. The study should consider the definition/functionalities of each power saving state.

Proposal 2	RAN2 to study and specify a signaling and/or event-based framework for UE power saving state transition (e.g., entry, maintenance, and exit), covering both network-controlled and UE-initiated transitions.

While cell DTX/DRX achieved notable network-side energy savings, the lack of standardized coordination with UE DRX limited overall system efficiency. Inconsistent UE support and reliance on vendor-specific implementations prevented full realization of joint energy-saving potential, highlighting the need for clearer 6G specifications enabling synchronized UE–network power saving operation.

Proposal 3	RAN2 to study a joint UE/NW energy saving framework that integrates UE DRX and Cell DTX/DRX (e.g., synchronized cycle, active time, sleep time, etc.).

# Reference

3GPP TR 38.840: “Study on User Equipment (UE) power saving in NR.”

3GPP TR 38.869: “Study on low-power wake up signal and receiver for NR.”

3GPP TR 38.864: “Study on network energy savings for NR.”