3GPP TSG-RAN WG2 Meeting #132	R2-2508649

Dallas, USA, November 17 – 21, 2025

Agenda Item:	10.3.2.2
Source:	Toyota ITC
Title:	Robust RRC Signaling Using Constraint ASN.1 Subtypes

Document for:	Discussion and Decision

# Introduction

The last RAN2 meeting discussed the potential for developing more practical delta signaling mechanisms for 6G Radio Resource Control (RRC) compared to previous generations, with a focus on implementation robustness and simplicity rather than specification convenience. Contribution [1] highlighted a significant issue: delta signaling introduces ambiguities in RRC signaling that can create security vulnerabilities. It also emphasized the substantial effort required by User Equipment (UE) vendors to build devices resilient to exploitation of these ambiguities.

Beyond security risks and implementation challenges, these ambiguities can adversely affect also critical applications. For instance, automotive applications rely heavily on the radio interface to support safety features, and signaling ambiguities could compromise road safety.

This contribution aims to propose a solution for more robust RRC signaling by leveraging constrained ASN.1 types as defined in ITU recommendations X.680 and X.682 [2][3].

# Discussion

## Current approach

The current signaling approach uses ITU Packed Encoding Rules (PER), where the transfer syntax is schema-based [4]. This means encoders and decoders are generated from the abstract syntax, unlike Type-Length-Value (TLV) serialization methods, such as Basic Encoding Rules (BER) [5], where decoders can operate without the schema. A challenge arises because encoding constraints are described verbally in field descriptions, procedure texts, codes, conditioning, and comments. When compiling the schema into encoder and decoder implementations, these verbal instructions are ignored since compilers only process the formal schema. This is illustrated in Figure 1.

Figure 1: Abstract syntax defines how to construct encoders and decoders for a given schema.

Hence, the abstract syntax defines how to build encoders and decoders for the schema but leaves message content decisions to the designer. The actual content of messages and information elements are determined by interpreting verbal descriptions and comment lines. For example, an optionally present field restricts encoder behaviour only if the designer correctly understands the conditions and codes embedded in comment lines. While this may work for senders, receivers must be prepared to handle any valid encoding allowed by the syntax, including cases that deviate from normal expectations. For instance, an optional field might be missing when it should be present, or an integer might contain a value which falls within a syntactically correct value range but is not supposed to be used in signalling. Receivers must handle such cases, even if these values are not meant to be used.

So far, this issue has been addressed through elaborate error-handling mechanisms. For example, the radio interface standard permits declaring radio link failures, requesting connection re-establishments, and applying generic error handling, i.e., silently ignoring unrecognized or non-comprehended content like parts of a message or entire system information blocks. However, these measures are only reactive in the sense that they mitigate symptoms rather than addressing the root cause: the receiver must be able to handle all syntactically valid encodings.

## Constraint subtype approach

One proactive approach is using constraint subtypes in the ASN.1 schema to enforce restrictions on message content, including field presence, integer value ranges, and choice options. Based on ITU X.680 and X.682 recommendations and illustrated in Figure 2, this approach preserves the designer’s freedom to decide message content while ensuring the encoder implementation prevents invalid data serializations. By embedding encoding constraints directly in the schema, the process becomes independent of the designer’s interpretation of conditions and comments. This solution reduces failures and makes meaningless or invalid content syntactically impossible, thereby simplifying the receiver’s design.

Figure 2: Abstract syntax defines how to construct encoders and decoders for specific constraints on the content.

One use case involves defining an optional field and explicitly specifying its presence or absence through constraints when referenced. For example, a field may be defined as optional within an information element, but constraints can enforce when it must be present or absent. An example of this can be found in 36.331, where constraint subtype is used. The excerpt below illustrates a non-critical extension field defined as optional in a system information block,

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

yet constrained to be absent when referenced in AS-Config using keywords WITH COMPONENTS and ABSENT, as shown below,

AS-Config ::=				SEQUENCE {

sourceMeasConfig					MeasConfig,

sourceRadioResourceConfig			RadioResourceConfigDedicated,

sourceSecurityAlgorithmConfig		SecurityAlgorithmConfig,

sourceUE-Identity					C-RNTI,

sourceMasterInformationBlock		MasterInformationBlock,

sourceSystemInformationBlockType1	SystemInformationBlockType1(WITH COMPONENTS

{..., nonCriticalExtension ABSENT}),

sourceSystemInformationBlockType2	SystemInformationBlockType2,

Although a subtype is used herein primarily to prevent an error, this example demonstrates that the schema enforces the absence of the extension in the system information block. This correction prevents an empty sequence in the middle of the message from being present, which would otherwise cause the subsequent content after the system-information block reference to become padding bits. Importantly, this enforcement makes it impossible to build an encoder that creates such an error by serializing any non-critical extensions for the referenced system information block, since the presence of the extension sequence is constrained to be absent.

The concept of subtypes enables referencing fields within an information element using a variety of constraints beyond simply enforcing the presence or absence of optional content. Some other examples include:

- Constraining choice entries: Using the keyword WITH COMPONENTS, specific entries within a choice type can be limited when referencing an information element. For instance, if a choice contains eight entries, the allowed entries can be restricted to a subset by explicitly specifying which are permitted. This approach is clearer than describing exclusions in normative text and ensures that encoders cannot use disallowed entries.

- Constraining integer value ranges: When referencing integer fields, value ranges can be constrained to exclude certain values that are not permitted in specific contexts. This method provides a more precise and enforceable description than normative text alone, preventing encoders from using invalid values.

For example, restricting the value range of a field from 1–8 to 1–5, as in the example above, can be expressed using a subtype as follows:

AS-Config ::=				SEQUENCE {

sourceMeasConfig					MeasConfig,

sourceRadioResourceConfig			RadioResourceConfigDedicated,

sourceSecurityAlgorithmConfig		SecurityAlgorithmConfig,

sourceUE-Identity					C-RNTI,

sourceMasterInformationBlock		MasterInformationBlock,

sourceSystemInformationBlockType1	SystemInformationBlockType1(WITH COMPONENTS

{..., q-RxLevMinOffset (1..5),

nonCriticalExtension ABSENT}),

sourceSystemInformationBlockType2	SystemInformationBlockType2,

Additional examples of subtype usage are available here [9].

The concept of constraints raises important questions about where and how they should be applied, and a single example does not fully address these issues. Standards such as X.680 and X.682 recommend defining a “parent data type” and then creating constrained types or referencing the “parent data type” within different messages. Using 3GPP terminology, this concept is illustrated in Figure 3, where the “parent data type” corresponds to information elements. Although the example focuses on field optionality, other types of constraints can also be applied, as explained above. This approach allows fields to be defined without restrictions within information elements, with constraints introduced only when these information elements are referenced. Furthermore, information elements can be imported into other modules, enabling the application of module-specific constraints.

Figure 3: Illustration of how constraints are applied within messages.

Effectively, this approach can achieve the same outcome as delta signaling through appropriate use of constraints. However, a concern arises whether this would require creating as many messages as there are constraints. Prior experience shows that having numerous messages can be problematic. For example, one improvement in 36.331 and 38.331 over 25.331 was the consolidation of multiple reconfiguration messages into a single, comprehensive (but massive) message. A major issue with 25.331 was the duplication across multiple reconfiguration messages, which often shared the same procedure due to their similarity.

The key difference here is that constraints do not duplicate content; they all derive from the same base fields but apply different restrictions. As demonstrated in the example above, a constrained version of a system information block can be succinctly represented in just two lines.

The primary benefit of constraint subtypes is their ability to embed signaling restrictions directly within the schema, thereby making meaningless or invalid content syntactically impossible to serialize. Therefore, it is proposed to introduce constraint types in 6G signaling.

Proposal 1: Introduce constraint subtypes in 6G signaling.

An intriguing question arises concerning Need codes, which specify UE behavior based on the presence or absence of a field. If constraints explicitly define a field’s presence or absence, this information is a priori known, arguably rendering Need codes unnecessary. A possible simplification is that when a field is constraint to be present, its content is stored or overwrites any previously stored value, while when constraint to be absent, the content and associated configurations are released.

# Conclusion

This document discusses 6G signaling concepts. The proposals are summarized as follows:

Proposal 1: Introduce constraint subtypes in 6G signaling.

# Reference

- R2-2506957, “Control plane architecture and 6G RRC protocol design”, MediaTek Inc., 3GPP TSG-RAN WG2 Meeting #131bis, Prague, Czech Republic, 13-17 October 2025

- X.680: Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation

- X.682: Information technology - Abstract Syntax Notation One (ASN.1): Constraint specification

- X.691: Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)

- X.690: Information technology - ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)

- TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”

- TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”

- TS 25.331, “Radio Resource Control (RRC); Protocol specification”

- https://www.oss.com/asn1/resources/asn1-made-simple/advanced-constraints.html