3GPP TSG-RAN WG2 Meeting #132						R2-2508651

Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:		8.2.2

Source:			Lenovo

Title:		Discussion on MAC open issues

Document for:	 	Discussion and Decision

Introduction

There have following open issues according to Rapp’s suggestion:

- Issue 3-7: Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field.

- Issue 1-3: Paging ID length impact from new SA2 LS in S2-2507793.

- New issue 6-1: Whether there is RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision.

- New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

- New issue 6-3: Whether there is AS impact to support Temp ID, e.g., including upper layer ID type in paging message. Or, assume CT4 will address this based on SA2 LS S2-2509655.

In this paper, we discussed some of above open issues.

Discussion

Issue 3-7: new case for “no upper layer data available” other than long writing operation

In last RAN2 meeting, new case for ‘no upper layer data available’ other than delayed NAS response was discussed and agreement was reached that RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all. But one uncertain issue is for integrity check failure case that whether some information is needed to report, and an LS is sent to SA3/CT1. Although the issue needs to wait for SA3/CT1 reply LS, chairlady suggest company contact own SA3 delegate for the answer.

After some internal discussion, we understand SA3/CT1 has no intention to obtain the information that whether the no response is caused by integrity check failure or not. When this case happens, the device is do nothing. In the agreed CT1 LS in [1], there also not required to differentiate the ‘no response’ case. In this sense, we propose that for ‘no response’ case, no matter it is caused by any reason, could be indicated to the reader by the same bit. We support to set MDI as follows to indicate the corresponding case that proposed in last meeting:

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

Proposal 1: RAN2 is suggested to consider following indication for ‘no response’ case and ‘delayed NAS case’

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

New issue 6-1: RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision

In the agreed CT1 LS [2], CT1 concluded that there can be an attack scenario of false AIoT paging that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices. Thus, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

RAN2 has discussed parallel paging issue during Rel-19 work item phase, and reached following agreement：

Agreements on parallel service request - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

However, CT1 spotted collision case is caused by false paging which is not exactly same as what RAN2 discussed before. Thus, we would not like to re-open the RAN2 discussion on parallel service and would like to keep the RAN2 agreement in above.

Observation 1: CT1 observed problem is caused by false paging, while not because of parallel service request which was discussed in RAN2.

Proposal 2: RAN2 do not re-open the discussion for parallel service request and keep the agreement which was made before for parallel service request.

To cope with the CT1 spotted issue, we think we’d better to focus on how to cope with potential false paging. For example, paging without paging id, i.e. paging for all, is the most likely the potential false paging that used by an attacker during the discussion in SA3. SA3 expressed their concerns for paging all or paging a large group of AIoT devices are described in TS 33.369 as following:

NOTE 2: While a legitimate network is performing an inventory operation, an attacker may cause amplification of resource exhaustion at the legitimate network side by sending AIoT paging messages for all devices or to a large group of devices, which causes large number of devices sending D2R messages to the legitimate network that the legitimate network does not expect to receive. The security measure to such amplification of resource exhaustion attack is not specified in present document.

In this sense, the AIoT device could simply forbid to respond to paging message that has potential security problem e.g. paging for all if no security parameter is included.

Proposal 3: RAN2 discuss how to cope with potential false paging message e.g. the AIoT device does not respond to a paging message without paging identity information and security parameters.

New issue 6-3: Whether there is AS impact to support Temp ID

In the SA2 agreed CR [3] that indicated in the SA2 LS to CT4 [4], the following are defined for AIoT identification information

The AIoT Device Identification Information to be provided to NG-RAN contains either Filtering Information as defined in clause 5.8, an AIoT Device Permanent Identifier as defined in clause 5.7.2 or an AIoT Device Temporary Identifier as defined in clause 5.7.x. When the AIoT Identification Information includes an AIoT Device Temporary Identifier, the AIoT Device Temporary Identifier and related information is determined as specified in TS 33.369 [9]. <text omitted> If the received AIoT Identification Information contains: - Filtering Information, the AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8. - AIoT Device Temporary Identifier, the AIoT Device determines whether it matches the AIoT Identification Information, as described TS 33.369 [9]. - AIoT Device Permanent Identifier, the AIoT Device determines whether it matches the AIoT Identification Information by comparing it with the stored AIoT Device Permanent Identifier.

Since AIoT Device Identification Information includes either Filtering Information, an AIoT Device Permanent Identifier, or an AIoT Device Temporary Identifier, we believe CT4 will add corresponding ID type information so that the device can know how to cope with the received AIoT Device Identification. And for the T-ID information, SA3 has already defined T-ID type or whether to update etc. information in T-ID handling information, as in the following

- In step 2 and 3, the AIOTF includes the T-ID handling information. T-ID handling information includes: - T-ID type that can be either a concealed type or a stored type. If the T-ID type is the concealed type, the T-ID is generated based on the AIoT Device Permanent Identifier. If the T-ID is the stored type, the T-ID is generated based on the stored T-ID as specified in Annex B.1. - Whether the stored T-ID type is updated with a command via a Command procedure or without a command during step 4. NG-RAN includes the T-ID handling information in the paging message.

In this sense, AS layer does not need to further define any paging id type like indication in paging message, and expected that there has no AS impact to indicate ID type, or Temp ID type etc. And all these information could be included in NAS information.

Proposal 4: No AS impact to support different paging id type including Temp ID. RAN2 assume all information to support different paging id type including Temp ID is included in NAS information.

Conclusion

In this contribution we have discussed left open issues for Rel-19 Ambient IoT and made the following observations and proposals:

/* Issue 3-7: new case for “no upper layer data available” other than long writing operation */

Proposal 1: RAN2 is suggested to consider following indication for ‘no response’ case and ‘delayed NAS case’

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

/* New issue 6-1: RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision */

Observation 1: CT1 observed problem is caused by false paging, while not because of parallel service request which was discussed in RAN2

Proposal 2: RAN2 do not re-open the discussion for parallel service request and keep the agreement which was made before for parallel service request.

Proposal 3: RAN2 discuss how to cope with potential false paging message e.g. the AIoT device does not respond to a paging message without paging identity information and security parameters.

/* New issue 6-3: Whether there is AS impact to support Temp ID */

Proposal 4: No AS impact to support different paging id type including Temp ID. RAN2 assume all information to support different paging id type including Temp ID is included in NAS information.

Reference

- C1-256623, LS on no Command Response in NAS

- C1-256624, LS on handling of inventory and command collision

- S2-2509652, Update to Inventory procedure for use of Temporary Device ID, CR0106 to TS 23.369

- S2-2509655, Reply LS on Structure updates of AIoT Identifiers