3GPP TSG-RAN WG2 Meeting #132	R2-2508658

Dallas, USA, Nov. 17th – 21st  2025

Agenda item:	10.3.1.2

Source: 	Lenovo

Title: 	On 6G QoS Framework

Document for:	Approval

- 1. Introduction

In RAN2#131bis meeting, it was agreed that:

4. Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

This contribution discuss the 6G new traffic types and characterization of the new traffic types and possible study directions for RAN-level QoS framework to support service awareness.

- 2. Discussion

2.1	QoS framework and service awareness in 5G

The 5G QoS framework is built on QoS Flows and PDU session. Packets of each QoS flow are classified and marked using QoS Flow Identify (QFI). At NAS level, the QoS flow is thus the finest granularity of QoS differentiation in a PDU session. A QoS flow is characterised by a QoS profile provided by 5GC to NG-RAN and QoS rule(s) provided by 5GC to UE. The QoS profile is used by NG-RAN to determine the treatment on the radio interface while the QoS rules dictates the mapping between uplink User Plane traffic and QoS flows to the UE. The QoS profile contains QoS parameters, such as 5QI, ARP. GFBR, MFBR, MBR and etc. At AS level, DRB is used for transmission of data on Uu interface. For uplink, the UE performs QoS flows to DRB mapping in SDAP layer whereas this mapping is performed by the NG-RAN for downlink traffic.

Observation 1: In the initial phase of 5G, QoS flow is the finest granularity of QoS differentiation while DRB is used for data transmission on Uu interface.

In Release-18, to support more granular service awareness, XR-awareness was introduced. Besides the QFI, XR-Awareness further relies on PDU set, data burst and assistance information.

A PDU set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926). To support PDU set concept, PDU set QoS parameters are provided by control signalling from 5GC to NG-RAN to enable PDU set based QoS handling. The PDU set QoS parameters comprise PSDB, PSER and PSIHI. In addition, PDU set information are provided in the GTP-U header from UPF to NG-RAN to identify PDUs that belong to PDU Sets. The PDU set information includes PDU set sequence number, indication of end PDU, PDU sequence number within a PDU set, PDU set size, and PDU set importance (PSI). Besides the PDU set information, the indication of End of Data Burst is also provided in the GTP-U header. In the uplink, the UE identifies PDU Sets and Data Burst including PSI through its own implementation.

Another information is TSC Assistance Information, which includes UL and/or DL periodicity, and N6 Jitter information.

For uplink, to support more dynamic change of XR service, UE report uplink assistance information, such as jitter range, burst arrival time, UL data burst periodicity, of a Qos flow to RAN node by UE Assistance Information.

Observation 2: In Release-18, finer granularity of QoS differentiation than QoS flow was introduced, e.g., PDU set, data burst, TSCAI and uplink assistance information for XR services.

In Release-19, further enhancement to support multi-modality was introduced. A MMSID is used to associate different QoS flows of a multi-modal service, which is provided as part of QoS parameters of QoS flow.

For the Data Burst, the data burst size is provided in the GTP-U header of the first PDUs of the data burst in downlink. The Time To Next Burst is also provided in the GTP-U header which represents the interval between the transmission of the last PDU in the current data burst and the first PDU of the next data burst. Both information can be used by the gNB to assist radio resource management and scheduling.

Observation 3: In Release-19, MMSID was introduced to support multi-modal services and further enhancements made to data burst (including data burst size and time to next burst).

In current Release-20, RAN2 will study mobile AI service awareness, which aims to identify the new characteristics of mobile AI traffic, e.g., AI videos, audios, images, text and data and the differences from XR traffic, e.g., XR videos, and, if needed, develop enhancements to facilitate efficient and effective support for mobile AI applications and services.

Observation 4: In Release-20, RAN2 will study transmission characteristics and potential enhancements of uplink mobile AI services.

We assume the QoS flow concept continues to be used in 6G. It makes sense to support the Release-18 and Release-19 enhancements to support finer granularity of QoS differentiation than QoS flow at the 6G Day1.

Proposal 1:  RAN2 agrees to adopt XR Release-18 and Release-19 enhancements for finer-grained QoS differentiation as 6G Day 1 features, including PDU sets, data bursts, and MMSID awareness at the RAN.

2.2	Support of immersive service

SA1 has studied immersive gaming which enables single or multiple participants in various location(s) to interact in real time with digital representation of objects and their surrounding environments. The immersive gaming service can be classified into three traffic flows:

- Compute flows: traffic flows that related to immersion and AI related compute that might be split between the compute processing available across network entities.

- Conversional and game flows: traffic flows that are related to audio, video, hologram-based interaction between users.

- Streaming flows: traffic flows that are related to slower time scale streaming of information to certain users.

These immersive gaming services need tightly collaboration among the different traffic flows.

Furthermore, SA1 defined a critical immersive communication such as 3D remotely controlled repairs or surgery, as shown in the section 9.7.1 of TR 22.870:

The new synchronization requirement to support the user case is to support independent traffic flows of one or more applications, to be delivered to more than one devices (i.e. UE or tethered devices).

From above, it can be expected that 6G will impose stricter synchronization requirements. In 5G, only the MMSID is defined in the downlink to identify QoS flow dependencies for a single UE. However, RAN node does not know the synchronization transmission requirements of the multi-modal QoS flows. Furthermore, packet or PDU set based synchronization transmission is also not supported in 5G. To meet 6G’s new requirements, finer-grained multi-modal service awareness and dependent transmission (e.g. synchronization transmission requirements awareness, packet or PDU set based dependence) can assist RAN nodes in optimizing scheduling.

For example, UE can provide finer-grained assistance information such as per packet importance, inter packet or PDU set dependence information for uplink to the Network (e.g. gNB). Based on this information, RAN2 may need to study more flexible LCP procedure, inter-QoS dependent discard and etc.  However, before discussing enhancements in RAN2, SA4/SA2 need to first study potential enhancements required for the sufficient support of multi-modality services, e.g. for downlink.

Proposal 2: RAN2 should study enhancements to support finer-grained service awareness as a part of uplink assistance information, e.g. to support multi-modal QoS flows within a single UE and among multiple devices (UEs or tethered devices).

Proposal 3: RAN2 should coordinate with SA4/SA2 for finer-grained dependent transmission to satisfy the new requirements of multi-modal QoS flows.

Another use case is offloading rendering tasks to network or cloud from UEs. The network and cloud hosts sufficient computing power and it can also lower the weight and the power consumption of XR glasses and mobile phones. As shown in Figure 9.3.1-2 of TR 22.870, the offloading decision can be based on communication link status and computing resource availability in the cloud (operator managed data network, edge or central cloud) in order to maintain deterministic rendering task processing latency. There's an expectation for 6G systems to consider such dynamic rendering offload framework to adapt to the synergized multi-media service processing trend. In this use case, the QoS requirement of the traffic may be adapted dynamically since it may be adapted according to the exact communication link capability, the UE power consumption, and computing resource availability to support dynamic splitting/offloading.

Compared to 5G, 6G system should provide means to coordinate with a UE to make decisions on splitting/offloading rendering tasks dynamically based on the real time network status and the computing resource availability in the Service Hosting Environment, edge or central cloud.

Observation 5: Dynamic QoS adaptation based on radio condition, UE power consumption and computing resource availability is needed, e.g., to support dynamic splitting/offloading rendering tasks.

2.3	Support of AI services

The use case on GenAI services has been studied by SA1 as described in section 6.26 of TR 22.870. GenAI service offer services from simpler text-based chatting to more complex multi-modal services like audio and video generation. Examples of these new applications and services include real time translation, image or video recognition / visual assistant and real-life assistance (e.g. home doctor, cooking and game agent). Some of them are already commercialized and have the potential to be a main type of traffic in the future. SA1 gives the traffic characteristic of typical GenAI application as depicting in the following Figure:

As observed in the TR 22.870, the GenAI services have the following characteristics:

- burst traffic: high data rate requirement during a short time, while the application #1 mainly for uplink and application #2 for downlink;

- unpredictable data rate requirement: the GenAI application supports multi-modal communication, and could transmit text, audio or video randomly based on user request, and therefore has no periodic traffic pattern

- small packet length: Chatbot applications (Application #3) typically have very small packet length, including both data packets and ACK packets, which would lead to decrease of processing speed considering the current user plane design.

- short-lived connection: the application #1 has short lived connection. The image upload sets up a new connection each time.

SA1 also proposed to consider the following enhancements for 6G:

- UE and network coordination: UE could help network to identify the application and traffic type more accurately and quickly, and the network could help UE make better flow control;

- Better QoS assurance: to ensure the GenAI application’s performance and service reliability, the network need to be aware of the burst and provide assurance for the whole burst; to deal with short-lived connections, QoS configuration needs to be done in a shorter time period; provide end-to-end service assurance by choosing an optimized path to the AS; and make QoS adjustments according to payload type, real time network status and user feedback.

From RAN2 point of view, especially for uplink, it should be study to support dynamic QoS configuration and dynamic traffic awareness in RAN node.

Observation 6: Dynamic QoS adaptation to support GenAI service is needed.

In 5G even though alternative QoS is supported, but it is only for GBR QoS flow and cannot support randomly dynamic QoS adaptation. The 5G QoS adaptation needs end to end signalling via NAS and PDU session management related signalling. Then how to support dynamic QoS adaptation without involving many signalling exchange need to be studied. The

One of possible solution is to support application-optimized communication, e.g. semantics and context of the data should be considered for resource allocation given that different data packets of a service contribute differently to the user experience. Mechanisms should be introduced for the application layer to convey metadata to the RAN, e.g., packet priority, dependency, relevance, or semantic weight. The metadata should be available to both UE and gNB in the AS for uplink and downlink resource optimization.

Dynamic QoS adaptation may impact on the  layer 2 operation, for example, the priority of a QoS flow can be changed dynamically, the delay budget of packets in a QoS flow may also be changed which may impact switch from delay-critical to non-delay critical transmission, the reliability requirements may also be changed which may impact the retransmission protocol. We think that RAN2 should study the potential impact of an adaptive QoS mechanism to the layer 2 protocols.

Based on observation 5&6, it is proposed:

Proposal 4: RAN2 should study layer 2 impacts (e.g., adaptation of LCP /retransmission of packets of a DRB) caused by dynamic QoS adaptation e.g., to support dynamic splitting/offloading rendering tasks and GenAI service.

At RAN2#131bis, several contributions ([1], [2], [3]) propose a token-based service. Tokens - generated and processed by large AI/ML models - can be regarded as compact data carrying context information. As the basic unit of AI information processing and generation, tokens can be derived from various data modalities, including text, images, and videos.

Token-based services may exhibit the following characteristics: high error tolerance, varying importance across different tokens, and dependent transmission. We consider token-based services a new service requiring further study.

In last RAN1 meeting, RAN1 has already sent a LS to SA4 to ask the traffic mode of token-based communication [4]. It makes sense that RAN2 can start the study on whether and how token-based services impact Layer 2 (e.g., scheduling) according to the SA4’s response.

Proposal 5: RAN2 can start the study on whether and how token-based services impact Layer 2 (e.g., scheduling) once the SA4 provides its response.

- 3. Conclusion

This contribution discuss the 6G new traffic types and characterization of the new traffic types and possible study directions for RAN-level QoS framework to support service awareness.

Observation 1: In the initial phase of 5G, QoS flow is the finest granularity of QoS differentiation while DRB is used for data transmission on Uu interface.

Observation 2: In Release-18, finer granularity of QoS differentiation than QoS flow was introduced, e.g., PDU set, data burst, TSCAI and uplink assistance information for XR services.

Observation 3: In Release-19, MMSID was introduced to support multi-modal services and further enhancements made to data burst (including data burst size and time to next burst).

Observation 4: In Release-20, RAN2 will study transmission characteristics and potential enhancements of uplink mobile AI services.

Observation 5: Dynamic QoS adaptation based on radio condition, UE power consumption and computing resource availability is needed, e.g., to support dynamic splitting/offloading rendering tasks.

Observation 6: Dynamic QoS adaptation to support GenAI service is needed.

Proposal 1:  RAN2 agrees to adopt Release-18 and Release-19 enhancements for finer-grained QoS differentiation as 6G Day 1 features, including PDU sets, data bursts, and MMSID awareness at the RAN.

Proposal 2: RAN2 should study enhancements to support finer-grained service awareness as a part of uplink assistance information, e.g. to support multi-modal QoS flows within a single UE and among multiple devices (UEs or tethered devices).

Proposal 3: For downlink, RAN2 should coordinate with SA4/SA2 for finer-grained dependent transmission to satisfy the new requirements of multi-modal QoS flows.

Proposal 4: RAN2 should study layer 2 impact (e.g., adaptation of LCP /retransmission of packets of a DRB) caused by dynamic QoS adaptation e.g., to support dynamic splitting/offloading rendering tasks and GenAI service.

Proposal 5: RAN2 can start the study on whether and how token-based services impact Layer 2 (e.g., scheduling) once the SA4 provides its response.

- Reference

[1] R2-2506905	Consideration on 6GR User Plane 	CMCC

[2] R2-2506850	Discussion on the 6G user plane features 	OPPO

[3] R2-2506940	Discussion on 6GR User Plane functions and architecture	Huawei, HiSilicon

[4] R1-2508184	LS on traffic model study in RAN1. RAN1 to SA4 (Contact Huawei)