3GPP TSG-RAN WG2 Meeting #132	R2-2508659

Dallas, USA, Nov. 17th - 21st , 2025

Agenda item: 		10.2.2

Source:		Lenovo

Title:		Considerations on TN-NTN integration for 6GR

Document for:		Discussion

- Introduction

The latest RAN2#131bis meeting made the following agreement regarding TN-NTN integration for 6GR:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution we discuss requirements and functionalities for 6G NTN, including the essentials to be considered in Day 1 in the common design with TN, the important ones that depend on 6GR decision, and others that can develop NTN-specific designs.

- Discussion

In 4G/5G, the IoT/NR NTN deployment is considered as an extension of 4G/5G TN with specific protocol enhancements or optimizations, e.g., to cope with the NTN characteristics including large propagation delay and satellite movement. Due to the workload and progress of LTE-A/5G-A standardization, the study and enhancement for IoT/NR NTN have been focusing on the typical reference scenarios with the satellites as NTN payloads, which are the LEO satellites with the maximum payload velocity and Doppler shift, and the GEO satellites with the maximum propagation delay and differential propagation delay. Meanwhile, satellites on other orbits (MEO or HEO) and other NTN payloads (HAPS) are considered extendable from LEO and GEO scenarios, and ATG scenario is implicitly supported simply applying enhancements for NTN.

Under the above principle, the standardization of IoT/NR NTN causes many differences between 4G/5G TN and NTN with non-unified CP signaling, UP handling as well as corresponding UE behaviors. Even within the NTN itself, the characteristics of specific NTN payloads (e.g., HAPS) were not fully considered. On the other side, with the evolution of services and infrastructure some NTN characteristics including BS/TRP movement and temporary availability are also becoming applicable in TN.

Observation 1: The studies and designs of IoT/NR NTN in 4G/5G started as a supplement to TN, and all NTN standardizations were developed as enhancements based on TN scheme to cope with NTN characteristics.

Based on the lessons learned from IoT/NR NTN, and to achieve the target of unified TN-NTN design in 6GR, it is better to take NTN scenarios as well as the characteristics into consideration as much as possible in the beginning. However, considering the workload and timeline of 6GR, it is also reasonable to firstly identify the most essential and important requirement and functions for unified TN-NTN designs, while some other NTN-specific aspects can be considered later as optional extensions that minimize the UE and network complexity.

## 2.1 Essential requirements and functions for NTN in 6GR Day 1

In this part we discuss the essential requirements and functions that shall be considered for both TN and NTN with common designs in 6GR at the very beginning.

- UL synchronization

In IoT/NR NTN, UL synchronization in time domain and frequency domain is mandatory for NTN access, and to obtain and maintain UL synchronization, the UE is required to have valid GNSS position and ephemeris information (also common TA of the feeder link for transparent architecture). For the time domain, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters, and autonomously pre-compensate the NTA for the RTT between the UE and the UL synchronization reference point. For the frequency domain, the UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris.

For the TA derivation in legacy 4G/5G TN, UL frame number for transmission from the UE shall start NTA+NTA,offset⨯Tc before the start of the corresponding DL frame at the UE. To accommodate the propagation delay in NTN, several timing relationships are enhanced by a Common TA on feeder link and UE-specific TA on service link on the basis of TN scheme, making it as NTA+NTA,offset+NTA,adjcommon+NTA,adjUE⨯Tc..

Observation 2: For IoT/NR NTN in 4G/5G, the TA compensation at UE UL synchronization is designed separately as add-on to the TN scheme.

This legacy UL synchronization scheme for IoT/NR NTN has multiple disadvantages, including:

- UE complexity: The capabilities of GNSS positioning, ephemeris acquiring and time/frequency pre-compensation are mandatory to access NTN cells. The UE also needs to guarantee real-time calculation for satellite position, movement and propagation delay. There are also duplicated calculations at UE e.g., for service link propagation delay and distance between UE and reference location.

- Signalling overhead: The size of ephemeris information (ephemerisInfo) of one NTN payload (satellite or HAPS) is 200 bits (orbital parameters) or 168 bits (position & velocity vectors), and the number of ephemeris information is limited to 5 (1 serving cell and 4 neighbour cells). The size of common TA information is 60 bits, making the size of one satellite information approximately 300 bits. Besides, TA reporting is necessary to provide the service link RTT information to the gNB, and PDD reporting is necessary to provide the service link delay difference information to gNB.

- Service interruption: UE is required to have a valid serving cell ephemeris and acquire its update before expiry, or the UE will trigger UL synchronization loss that suspends all UL transmission and flush HARQ buffer until it is obtained. For normal HO and CHO without RACH-less HO configuration, UE needs to calculate and pre-compensate the TA to the target cell before initiating RACH. For RACH-less HO, the UE still needs to calculate and pre-compensate the TA to the target cell for UL transmission.

- Security risk: The ephemeris information, either in the form of orbital parameters or position & velocity vectors, reveals the current and future positions of the NTN payload (satellite or HAPS), making it vulnerable to planned security attacks. In particular for the regenerative architecture wherein the gNB is on the NTN payload, these information exposes the current and future positions of gNBs, which is a significant safety risk considered for TN.

Observation 3: For IoT/NR NTN in 4G/5G, the UL synchronization relies on UE position and ephemeris information as mandatory requirements, which leads to additional UE complexity, signaling overhead, service interruption and security risk.

This process was carried out based on the fact that the IoT/NR NTN deployment is considered as an extension of 4G/5G TN with specific protocol enhancements or optimizations. However, it also causes many differences between 4G/5G TN and NTN with non-unified signaling and UE behaviors for UL synchronization. In 6G to achieve the goal of harmonized TN-NTN design, a unified UL synchronization scheme (e.g., determination of TA to be compensated by UE) is expected, which may have the chance to reduce the UE complexity, signaling overhead, service interruption and security risk.

To facilitate the unified scheme of UL synchronization for both TN and NTN, unified CP signaling solution for UL synchronization is expected as well to minimize UE complexity, signaling overhead and service interruption. For example, common signaling for the common part of TA that needs to be compensated by UEs within a cell ranging from typical TN to typical NTN propagation delays, and dedicated signaling for individual adjustment to correct the differential TA at a specific UE. Based on the unified CP signaling, the corresponding UP reporting (TA reporting) and timer handling (timer extension or start offsetting with UE-BS RTT) can be reconsidered as well.

Proposal 1: The 6G RAN design shall aim for a unified scheme of UL synchronization (TA compensation and adjustment) for both TN and NTN, which may be decoupled from UE position information and/or ephemeris information.

- Mobility

Mobility has always been a fundamental function throughout the develop of mobile systems. In the latest RAN2#131bis meeting for 6GR, the following agreements were made for mobility mechanism:

Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition).

The study and standardization for IoT/NR NTN have introduced several enhancements for mobility considering NTN characteristics, including cell movement and availability, and fuzzy RSRP difference within an NTN cell. The mobility enhancements for IoT/NR NTN include distance-based measurement reporting, distance-based CHO in CONNECTED and distance-based neighbour cell measurement triggering in IDLE/INACTIVE, as well as time-based CHO in CONNECTED and time-based neighbour cell measurement triggering in IDLE/INACTIVE. However, for IoT/NR NTN it seems that only the general characteristics of NTN have been considered, while the differences within the NTN between TN and NTN (and also within the NTN on different orbits/altitudes) were merely touched.

Observation 4: For IoT/NR NTN in 4G/5G, the mobility enhancements focused on the general characteristics of NTN and the differences between TN and NTN were merely considered.

In the SA1 Rel-20 study on satellite access (Phase 4), several use cases regarding multi-orbit satellite deployments were proposed and discussed in 3GPP TR22.887, aiming to achieve service continuity and diversity over satellites on different orbits. The consolidated requirements for multi-orbit satellite deployment include mobility support, QoS guaranteeing and service interruption minimizing. As the multi-orbit satellite use cases and scenarios are not included in Rel-20 IoT/NR NTN, it is expected that in 6G this can be extended to include both TN and NTN with multi-orbit or multi-altitude deployment (HAPS/LEO/MEO/HEO/GEO) for mobility designs.

Proposal 2: The 6G RAN design shall aim to unified design to support seamless mobility between TN and multi-orbit NTN (HAPS/LEO/MEO/HEO/GEO).

- Energy saving

Energy saving is of great importance for environmental sustainability and operational cost savings. As 5G became pervasive across industries and geographical areas, solutions to improve energy savings were also developed, with both NES (Network Energy Saving) and UE power optimizations being considered. The NR NES features focus on reducing gNB energy consumption during low traffic by enabling mechanisms like on-demand SSB/SIB1 transmission, cell DTX/DRX, common channel adaptation, SSB-less transmission, while UE power saving is achieved through mechanisms including DRX for IDLE, CONNECTED and eDRX, low power WUS, and reduced/relaxed mobility monitoring. For 6GR the energy saving is still an essential requirement, and the following agreements were made in the latest RAN2#131bis meeting:

Design goal: Strive to develop solutions that consider network energy efficiency and UE power saving. For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

Energy saving and energy efficiency have been greater requirements in NTN than TN. From network side, due to the limited processing payload and bandwidth, a satellite with limited transmission power and limited processing bandwidth may only serve part of the potential satellite coverage area at the same time. As a result, there is a strong need to optimize energy usage to maximize the number of satellite beams that can be activated simultaneously, and to ensure that all UEs can be served across the satellite coverage through dynamic and efficient power control. In the beginning of Rel-19 for NR NTN, the NES techniques introduced for TN in Rel-18 was once considered as baselines, while only SSB periodicity extension was actually specified for Rel-19 NR NTN due to workload and compatibility. From UE side the nature of NTN (deployed in higher altitude than TN) leads to more UE power consumption for access than that in TN, and minimizing UE power consumption for NTN access would be a key success factor for NTN commercialization so that users may have the willingness to use NTN access without consuming much more energy than using TN.

Observation 5: NTN has greater requirements in energy saving than TN from both UE and network sides, and only a small part of the schemes developed in 5G NR for UE power saving and NES were adopted in NR NTN.

Therefore, energy saving techniques in 6GR designs shall toward a unified scheme for both TN and NTN, covering UE and network sides.

Proposal 3: The 6G RAN design shall aim for a unified scheme of energy saving at UE and network sides for both TN and NTN.

## 2.2 Important requirements and functions for NTN depending on 6GR decision

In this part we discuss the important requirements and functions that shall also be considered for both TN and NTN with common designs in 6GR, while the progress may depend on whether 6GR determines to support these functions in Day 1 e.g., positioning, sensing and broadcasting.

- Positioning

UE positioning has been an important function in 4G LTE and 5G NR TN that provides means of determining UE position that enable various services including piloting and navigation. For NTN the UE positioning plays a role more than services. NTN has the advantages of wide coverage that can provide positioning services especially in remote, underserved, and maritime areas where traditional terrestrial networks are not feasible or cost-effective. Meanwhile NTN itself also needs robust and high-accuracy positioning for commercial operation e.g., to fulfill regulatory requirements brough by extraterritorial coverage across country borders, or to guarantee UL synchronization if the legacy IoT/NR NTN scheme is to be inherited. For the extraterritorial coverage issue, there were studies in Rel-18 NR NTN called UE location verification wherein single satellite providing multi-RTT positioning was enhanced. And for the UL synchronization issue, GNSS resilient support is being studied in Rel-20 NR NTN, wherein both directions of close-loop adjustment and RAT-dependent positioning via NTN are on the table.

In 6G, NTN is expected to provide ubiquitous and robust coverage for global connectivity as well as positioning services, and a unified positioning scheme (including RAT-dependent positioning) applicable in both TN and NTN is necessary. Such scheme will not only support provisioning of more diverse and more precise positioning services to users, but also will guarantee compliant operations and extend commercial opportunities for NTN operators and vendors.

However, whether this function and corresponding designs will be considered at the beginning depends on the common decision for 6GR, i.e., whether positioning will be supported in Day 1 6GR.

Proposal 4: If positioning is supported, the 6G RAN design shall aim for a unified scheme of positioning for both TN and NTN, wherein NTN TRP for positioning is included.

- Sensing

For the sensing services, the current studies on ISAC have been mostly focusing on the scenarios, requirements and general procedures of providing sensing services alongside with communication leveraging TN infrastructures (e.g., TRPs and gNBs on the ground), while the NTN payloads (satellites and HAPS) also have the advantages in providing sensing services in both horizontal and vertical directions with their large coverage and high altitude deployment. The NTN payloads embarking sensing modules can serve as the sensing nodes (i.e., TRPs) to implement ISAC, while the NTN characteristics shall also be considered in sensing procedures, e.g. sensing capability exchange that can be related to satellite hardware limits, satellite beam management and ISL, and sensing service availability that can be related to satellite movement and three-dimensional coverage.

However, similar to the situation of positioning, whether this function and corresponding designs will be considered at the beginning depends on the common decision for 6GR, i.e., whether sensing (as an independent function or as part of positioning) will be supported in Day 1 6GR.

Proposal 5: If sensing is supported, the 6G RAN design shall aim for a unified scheme of sensing for both TN and NTN.

- Broadcast and multicast

The broadcast and multicast (including MBS and PWS) have been important services in 4G LTE and 5G NR to provide the same services to multiple UEs in an efficient way, both for commercial purposes and emergency situations. The NTN with large coverage is well suited to support broadcast and multicast in providing services to more UEs with higher resource efficiency compared to TN. For 5G NR the MBS broadcast scheme was initially designed for TN wherein cell coverage is typically smaller than MBS broadcast service area, and as a result the MBS broadcast service area is directly mapped to one or multiple cells for simplicity e.g., UE can simply derive whether it can receive an MBS broadcast service by its serving cell. This scheme encounters the issue that NTN cell coverage is much larger and the MBS broadcast service area can only be mapped to part of an NTN cell. Therefore in Rel-19 NR NTN the enhancements for MBS broadcast support were specified allowing geo-fencing indication for MBS broadcast services. The ETWS in PWS has the similar issues and thus geo-fencing was also introduced to ETWS reusing the CMAS scheme. Meanwhile there are also service continuity issues related to the geo-fencing and mapping issues between multicast HARQ feedback options and HARQ process feedback enabling/disabling. It is expected that in 6GR these issues shall be considered for both TN and NTN, as well as a unified scheme for providing broadcast and multicast services in TN and NTN.

However, whether this function and corresponding designs will be considered at the beginning depends on the common decision for 6GR, i.e., whether broadcast and multicast services will be supported in Day 1 6GR.

Proposal 6: If broadcast or multicast are supported (e.g., MBS, PWS), the 6G RAN design shall aim for a unified scheme of broadcast or multicast services for both TN and NTN, wherein the 5G-A enhancements for MBS broadcast, multicast and PWS can be used as baselines.

## 2.3 Other requirements and functions for NTN in further consideration

In this part we discuss the other requirements and functions that can develop NTN-specific designs, which may also be applicable to TN.

- Store and forward operation (including UE-satellite-UE communication)

For IoT NTN, satellite operators and vendors proposed to support store and forward (S&F) satellite operations in NTN for delay-tolerant, non-real-time services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas), and considered it as a key to facilitate cost-effective deployment of NTN services with sparse satellite constellations and reduced ground segment infrastructure.

Based on the S&F operation, the UE-satellite-UE communication was originated from the need of direct communication between UEs with large distance (e.g., in case of remote area, emergency or disaster rescue), wherein a satellite can provide large coverage for these UEs while the feeder link to connect the satellite with the ground network may not be always available.

The enhancements for S&F operation support were done in Rel-19 IoT NTN with specific indications for operation mode and cell barring introduced. While for UE-satellite-UE communication there is currently no RAN study to implement.

For 6G, S&F operation and UE-satellite-UE communication in NTN are expected to be supported (although maybe not Day 1) e.g. as identified by 6G-NTN project, with prospective in multiple scenarios including public safety, maritime, IoT, defense and automotive. Considering this NTN-specific scenario and operation mode as identified so far, the 6GR can consider support for this with NTN-specific designs if time allows. Meanwhile, the solutions may have chance to be applied in TN as well in case of new scenarios or service requirements emerging.

Proposal 7: If time allows, the 6G RAN design can consider support for store and forward operation (including UE-satellite-UE communication) in NTN, wherein the solutions may also be applicable for TN.

- Conclusion

In this contribution we discuss 6G NTN under the principle of unified TN-NTN design in 6G radio. It is observed that:

Observation 1: The studies and designs of IoT/NR NTN in 4G/5G started as a supplement to TN, and all NTN standardizations were developed as enhancements based on TN scheme to cope with NTN characteristics.

Observation 2: For IoT/NR NTN in 4G/5G, the TA compensation at UE UL synchronization is designed separately as add-on to the TN scheme.

Observation 3: For IoT/NR NTN in 4G/5G, the UL synchronization relies on UE position and ephemeris information as mandatory requirements, which leads to additional UE complexity, signaling overhead, service interruption and security risk.

Observation 4: For IoT/NR NTN in 4G/5G, the mobility enhancements focused on the general characteristics of NTN and the differences between TN and NTN were merely considered.

Observation 5: NTN has greater requirements in energy saving than TN from both UE and network sides, and only a small part of the schemes developed in 5G NR for UE power saving and NES were adopted in NR NTN.

And the following proposals are given:

Essential requirements and functions for NTN in 6GR Day 1

Proposal 1: The 6G RAN design shall aim for a unified scheme of UL synchronization (TA compensation and adjustment) for both TN and NTN, which may be decoupled from UE position information and/or ephemeris information.

Proposal 2: The 6G RAN design shall aim to unified design to support seamless mobility between TN and multi-orbit NTN (HAPS/LEO/MEO/HEO/GEO).

Proposal 3: The 6G RAN design shall aim for a unified scheme of energy saving at UE and network sides for both TN and NTN.

Important requirements and functions for NTN depending on 6GR decision

Proposal 4: If positioning is supported, the 6G RAN design shall aim for a unified scheme of positioning for both TN and NTN, wherein NTN TRP for positioning is included.

Proposal 5: If sensing is supported, the 6G RAN design shall aim for a unified scheme of sensing for both TN and NTN.

Proposal 6: If broadcast or multicast are supported (e.g., MBS, PWS), the 6G RAN design shall aim for a unified scheme of broadcast or multicast services for both TN and NTN, wherein the 5G-A enhancements for MBS broadcast, multicast and PWS can be used as baselines.

Other requirements and functions for NTN in further consideration

Proposal 7: If time allows, the 6G RAN design can consider support for store and forward operation (including UE-satellite-UE communication) in NTN, wherein the solutions may also be applicable for TN.