3GPP TSG RAN WG2 #132		     	R2-2508661
Dallas, U.S., Nov 17th – 21st, 2025

Agenda item:		10.3.1.1

Title:					Optimizing 6GR for small UL packets

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:	Discussion

# Introduction

5G NR deployments have become widespread on mid-band, opening the opportunity to use bandwidths of up to 100 MHz. Such bandwidths (as well existing larger bandwidths in FR2 and studied FR3 bands [1]) contain a very large number of Resources Blocks (RBs). The large number of RBs make the dynamic grant signaling relative inefficient for very small allocations. This is particularly prominent in UL which carries most small packets, namely uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs).

It is well-known that Internet traffic follows a bimodal distribution as highlighted in [2]. As cited from a reference in [2], 40-60% of Internet packets are rather small. However, as mobile communication systems have grown in bandwidth the allocation of small packets, especially UL packets, have become increasingly inefficient. This is described in rather detail in [2].

This contribution addresses this problem showing how 6G signaling overhead can be reduced significantly for the very common case of small UL packets.

# Problem description

Internet traffic is mostly composed of two types of packets:

- Large packets. Often, they follow the Ethernet limit of 1518 (without VLAN) or 1522 octets (for VLAN tagged traffic).

- Small packets which are mostly acknowledgments of the large packets. Common sizes are, for example, possible TCP ACK packet sizes: 58 (18 Ethernet, 20 IPv4 header, 20 TCP header), 70 (18 Ethernet, 20 IPv4 header, 32 TCP header with timestamp) or 90 octets (18 Ethernet, 40 IPv4 header, 32 TCP header with timestamp)

Observation 1: The (mobile) Internet traffic is bimodal, composed of small packets and large packets. Most small packets are UL data generated as acknowledgment of the DL data.

The generation of acknowledgments is mandatory for many Internet protocols, such as TCP [3] and QUIC [4]. For DL oriented traffic, as most eMBB traffic, the acknowledgments are generated by the UE, and they become UL traffic for RAN. The problem, as described in [2], is that existing dynamic grant signaling is very inefficient for small UL packets. In large bands, it is common to have more than hundreds of RBs. In this case just the frequency domain resource allocation can be rather large. In the resource allocation type 0 with 273 RBs the size of an RB group is 16 PRBs. 16 PRBs is a huge waste for allocating the transmission UL packets as small as 58, 70 or 90 octets.

Observation 2: The existing dynamic grant signaling is very inefficient for small UL packets.

With the allocation type 1 less RBs would be wasted on the traffic channel, but still a fully sized UL grant DCI transmitted on QPSK is a significant overhead for allocation such small amount of RBs.

# Proposed Solution – Fractional UL resources in Round-Trip Grants

As discussed thoroughly in our accompanying contribution R2-2508060 [1], Round-Trip Grants allow for a smooth flow of eMBB traffic, effectively eliminating most of SR/BSR/Tx cycles. But there is more to it. The introduction of Round-Trip Grants (RTG)s also allows to optimize the overhead of UL allocations because the UL allocation is done associated to the DL one.

For example, if it is defined that UL shall be allocated 1/8 of the resources of the DL allocation, the exact fraction assigned to a certain UE can be signaled with 3 bits, instead of the 16-18 bits on existing DCI formats (allocation Type 0 and allocation Type 1). Also, the usage of a fraction of resources solves the problem described in [2] about the efficiency of small UL packet transmission. The allocation of resources for DL can still use a bitmap allocation (NR allocation type 0) and the UL resources are simply a fraction of that. If the UE is assigned a single Resource Block Group on DL, it can be assigned a smaller fraction of that in UL – for example 1/8 of a RBG on deployment with 273 RBs (100 MHz bandwidth) would be simply 2 RBs. That is an allocation large enough for the transmission of any single TCP ACK in good radio conditions, even one with IPv6 header and TCP timestamps, or smaller TCP ACKs in worse radio conditions.

The concept of allocating a fraction of the same DL resources to UL is shown in Figure 1. This is an elegant solution to reduce the overhead to carry the most common type of small UL packet: uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs).

Proposal 1: Study and introduce Round-Trip Grants, allocating both downlink and uplink resources, where the uplink resources are a fraction of the downlink ones.

Figure 1 – By using a Round-Trip Grant (RTG) the overhead of small UL packet transmission can be significantly reduced. A 3-bit indication suffices to indicate that UL RBs are a certain 1/8 fraction of the DL resources.

# Conclusions

The following observations and conclusions have been made in this contribution:

Observation 1: The (mobile) Internet traffic is bimodal, composed of small packets and large packets. Most small packets are UL data generated as acknowledgment of the DL data.

Observation 2: The existing dynamic grant signaling is very inefficient for small UL packets.

Proposal 1: Study and introduce Round-Trip Grants, allocating both downlink and uplink resources, where the uplink resources are a fraction of the downlink ones.

# References

- R2-2508060, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #109 Beijing, China, September 15-18, 2025.

- 6GWS-250009, “Ten tiny little things… that could make 6G worth”, Bouygues, 3GPP 6G Workshop, March 2025, Incheon, KR.

- W. Eddy. 2022. RFC 9293: Transmission Control Protocol (TCP). RFC Editor, USA.

- J. Iyengar and M. Thomson. 2021. RFC 9000: QUIC: A UDP-Based Multiplexed and Secure Transport. RFC Editor, USA.