3GPP TSG-RAN WG2 #132	R2-2508668

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.2.1 – UE capability framework

Title:		Discussion on fundamentals of UE capability framework

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#131bis, RAN2 discussed the UE capability framework a lot, for example the problems in the legacy framework as well as possible improvements to the dynamic capability change. The following agreements were made on the aspects to be discussed in this meeting [1]. In this contribution, we discuss the problems in the legacy UE capability framework and the use case of dynamic capability change. We provide our views on those aspects.

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities. Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field. Possible ways to address them.

# 2. Discussion

2.1	Problems with legacy UE capability framework

Following the last meeting outcome, we discuss critical problems in the legacy UE capability framework. In the legacy systems, UE capability framework is introduced for the network to retrieve the UE capabilities when the network needs them. Normally the UE capability reporting will be initiated after the UE has performed initial access to the network (e.g., RRC setup and AS security activation).

The main drawback in this legacy basic framework is to send the huge size of UE capability signaling over the air. The main part of this problem is L1/RF related capabilities which are RAN1/4 responsibility. The other part is to send the UE capabilities which are not used by the network at least in the current serving cell, for example, due to lack of supporting corresponding feature or functions. In addition, the UE capability size problem is not only over Uu but also over network interfaces (e.g., NG, Xn). The stored UE capability in the CN will be retrieved by the RAN node upon mobility or when the UE performs the RRC setup again. As this basic framework is working well and it is very difficult to drastically change from this, improvements to this basic framework or new schemes on top of this framework will be necessary. From the network implementation perspective, the required memory size is also the problem.

Observation 1. The main problem with the legacy UE capability framework is the huge size of the UE capability signaling.

In the NR, some mechanisms are introduced to mitigate the UE capability size problem, for example:

- Feature Set, Feature Set Combination

- Filtering (e.g., band/band combination filter)

- RACS (ID-based capability retrieval)

- RRC segmentation for UE capability signaling

Firstly, in 5G NR, new concept of Feature Set and Feature Set Combination were introduced, which can avoid repeating the same capability set information in different places (e.g., each band, band combination) in the UE capability information. Also, a certain filtering (e.g., band/band combination filter) is supported. This is simple and gives clear gain to solve the UE capability size problem. Unfortunately, the size problem still remains due to large flexibility of L1 parameters.

Observation 2. Total UE capability size problem still remains after introducing new concept of Feature Set/Feature Set Combination and also the filtering (e.g., band, band combination filter).

In order to reduce the UE capability signaling size, the new concept of RACS was introduced, which is the ID-based UE capability retrieval mechanism. The ID is associated with either Manufacturer assigned ID or Network (PLMN) assigned ID. The UE capability retrieval based on these ID is done over NAS signaling and also the network signaling between the RAN and the CN. This mechanism is simple and was expected to be available in the commercial system. However, it seems not. It may be because this is not mandatory for UEs and thus the availability highly depends on the vendor implementations as well as operators’ plan. If we introduce this kind of ID based capability retrieval, it should be supported from Day1.

Observation 3. ID-based capability management could be beneficial, but it should be mandatory for the UEs.

Another mechanism to solve the huge size of UE capability signaling over the radio is to allow the segmentation for that. Although this does not solve the capability size problem itself, it could be considered also for 6G.

Observation 4. Segmentation of the UE capability signaling could be considered for 6G, regardless of the improvements to the UE capability framework.

Regarding the capability introduction guideline, we observed that in the NR, sometimes there are many UE capabilities for the same goal, i.e. a feature under the same WI in the same release. Segmented capabilities complicate the UE capability signalling and configuration signalling, and it also complicates the network implementation for configuration decision. We take the XR as example, which is not so big function but introduce many separate capabilities. For example, in Rel19 XR, to support XR service with low latency and high reliability requirement, we introduced ~10 capability bits. In extreme case, we have two independent capability bits for giving up out-dated packet retransmission at UE transmission operation and for giving up receiving an outdated packet at UE receiving side. These two functions as such, which highly relevant to the same target but could be enabled independently, would not well justify two independent capabilities. Therefore, it would be good to study a possibility that the UE capabilities for the same goal are not too segmented.

Observation 5. Another issue with the legacy UE capability framework is to have some segmented UE capabilities for the same goal, i.e. a function under the same WI in the same release.

Based on the discussion above, we suggest considering the following proposals.

Proposal 1: RAN2 confirm the following problems with the legacy UE capability framework.

- Huge size of the UE capability signaling,

- Segmented UE capabilities for the same goal, i.e. a function under the same WI in the same release.

Proposal 2: RAN2 consider the following aspects for 6G UE capability framework.

- ID-based capability management could be beneficial, if it can be mandatory for the UEs.

- Segmentation of the UE capability signaling could be considered, regardless of the improvements to the UE capability framework.

Proposal 3: RAN2 consider a guideline that if two (or more) functions could be configured/enabled independently but target for the same goal in the same WI, it should not motivate two (or more) capability bits.

2.2	Dynamic capability change

In the last meeting, RAN2 also discussed the dynamic capability change in Connected mode and agreed to firstly justify the use case for that and analyse the impacts to the network side. We assume the main use case is that the UE wants to request to relax (i.e. degrade) some parameters currently configured with the UE due to UE internal problems (e.g. overheating, low battery) or service continuity (e.g. MUSIM). Another use case may be to inform the latest situation at the UE side (i.e. UE internal functional update), for example, AI/ML model online re-training/fine-tuning could be studied in 6G which can impact AI/ML related capability and/or applicability reporting. The intended functionality is based on the basic/static UE capability, but its applicability could be changed time to time. This may happen more in 6G which could be AI-native system.

Proposal 4: RAN2 discuss if the following use cases are target of dynamic capability change:

- UE requests relaxing some configured parameters due to UE internal problems (e.g. overheating, low battery) or service continuity (e.g. MUSIM),

- UE informs the UE internal functional update to be applied (e.g., AI/ML model re-training/fine-tuning)

The motivation could be understood, while the essential discussion is whether the network should fully follow the UE request or update. For the latter use case (informing the UE internal functional update), it is reasonable for the network to respect it. For the former use case (requesting relaxation configured parameters), it actually impacts on the ongoing services, because the network and the UE should be synchronized with the current configurations each other. Therefore, the UE should not be able to update dynamically by itself without confirmation from the network side. On the other hand, we assume some improvements compared with 5G mechanism via the UE Assistance Information (UAI) could be studied for 6G.

Proposal 5: RAN2 confirm that the dynamic capability change should not lose the configuration synchronization between the UE and the network, while study some improvements from the UAI in 5G.

# 3. Conclusion

In this contribution we discussed the problems in the legacy UE capability framework and the use case of dynamic capability change. We made the following observations and proposals.

Problems with legacy UE capability framework:

Observation 1. The main problem with the legacy UE capability framework is the huge size of the UE capability signaling.

Observation 2. Total UE capability size problem still remains after introducing new concept of Feature Set/Feature Set Combination and also the filtering (e.g., band, band combination filter).

Observation 3. ID-based capability management could be beneficial, but it should be mandatory for the UEs.

Observation 4. Segmentation of the UE capability signaling could be considered for 6G, regardless of the improvements to the UE capability framework.

Observation 5. Another issue with the legacy UE capability framework is to have some segmented UE capabilities for the same goal, i.e. a function under the same WI in the same release.

Based on the observations above:

Proposal 1: RAN2 confirm the following problems with the legacy UE capability framework.

- Huge size of the UE capability signaling,

- Segmented UE capabilities for the same goal, i.e. a function under the same WI in the same release.

Proposal 2: RAN2 consider the following aspects for 6G UE capability framework.

- ID-based capability management could be beneficial, if it can be mandatory for the UEs.

- Segmentation of the UE capability signaling could be considered, regardless of the improvements to the UE capability framework.

Proposal 3: RAN2 consider a guideline that if two (or more) functions could be configured/enabled independently but target for the same goal in the same WI, it should not motivate two (or more) capability bits.

Dynamic capability change:

Proposal 4: RAN2 discuss if the following use cases are target of dynamic capability change:

- UE requests relaxing some configured parameters due to UE internal problems (e.g. overheating, low battery) or service continuity (e.g. MUSIM),

- UE informs the UE internal functional update to be applied (e.g., AI/ML model re-training/fine-tuning)

Proposal 5: RAN2 confirm that the dynamic capability change should not lose the configuration synchronization between the UE and the network, while study some improvements from the UAI in 5G.

# References

[1] RAN2#131bis Chair notes