3GPP TSG-RAN WG2 #132	R2-2508669

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.1.1 Functionality for User Plane and related functional requirements

Title:		Discussion on required functions and requirements for UP

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#131bis meeting, RAN2 discussed the User Plane (UP) functions and reached the following agreements [1]. In this contribution we discuss the required functionality and the requirement to be consider for the protocol design.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

# 2. Discussion

2.1	Required functionalities in 6G

The 5G NR Layer 2 protocol stack for the user plane (UP) is organized into SDAP (Service Data Adaptation Protocol), PDCP (Packet Data Convergence Protocol), RLC (Radio Link Control), and MAC (Medium Access Control) sublayers. Each sublayer contains and executes a set of distinct functions. Overall, in NR communication, the MAC layer ensures the basic requirements for UP data transmission and reception, while also supporting advanced functionalities. In comparison, the SDAP, PDCP and RLC sublayers handle more specialized functions.

In the last meeting, RAN2 agreed to firstly discuss and agree on the required functionalities for the protocol stack. The table 1 shows the L2 functions assuming the 5G NR. All these functions are considered as essential from overall system perspective. However, all of them are necessary for all possible services supported by the 6G. We further discuss this point.

- MAC:

In details, 5G MAC layer provides very basic function insurance of UP data reception and transmission via at least the following functionalities:

- Multiplexing/demultiplexing

- UL scheduling (e.g., SR, BSR, DSR, PHR, LCP)

- Random access (CBRA, CFRA)

Most of the functionalities ensures the minimum requirement for data transmission, while whether some of the functionalities are still needed pending further detailed 6G design

In addition, MAC layer also provides additional functionalities to further enhance on such as UE’s power saving and transmission reliability, which are listed in below:

- UE power saving (e.g., DRX)

- Beam management (BFD, BFR)

- Mapping between logical and transport channels

- Retransmission with FEC coding (HARQ)

- Support of packet duplication

Furthermore, in 5G SDAP, PDCP and RLC also provide most additional functionalities which mainly enhance the transmission and reception performance, instead of the basic functionalities. The additional functionalities are listed in below:

- SDAP:

- Mapping between QoS and radio bearer configuration

- Support of reflective QoS

In details, the mapping between QoS and radio bearer can be regarded as a basic function since through this function, a data packet can be given a way of transmission via AS configuration.

- PDCP:

- Header Compression (e.g., ROHC)

- PDCP duplication

- Ciphering /Deciphering

- Integrity protection

- Out-of-order delivery

- RLC:

- Segmentation, concatenation and re-assembling

- ARQ (automatic repeat request)

According to the conclusion and analysis for the functionalities of each protocol layer in 5G, a functionality table can be maintained to track for 6G protocol layer function discussion, which shall include at least the following aspects:

- Whether each function belongs to basic or additional category

- Whether the function needs to be reused in 6G

Additional functionalities which is not used in 5G, but might be useful in 6G can be further investigated and added into this table in the following discussion.

The discussion of the specific protocol layer to carry each function can be further discussed after the content of the table is consolidated.

Table 1. L2 functions in 5G NR and categorization

| Protocol layer in 5G | Essential functions | Category | Reused in 6G? |
| --- | --- | --- | --- |
| SDAP | Mapping of packet from upper layer (CN) to radio bearer (e.g. QoS flow to DRB mapping) | Basic | Yes |
|  | Support of reflective QoS flow | Additional | pending |
| PDCP | Encryption, decryption | Additional | Yes |
|  | Integrity protection, verification | Additional | Yes |
|  | Header compression (ROHC) | Additional | Yes |
|  | Out-of-order delivery | Additional | pending |
|  | Support of PDCP duplication | Additional | pending |
|  | Packet discarding (duplicated detection, timer-based) | Additional | Yes |
| RLC | Segmentation and reassembly | Basic | Yes |
|  | Retransmission (ARQ) for ensuring packet delivery | Additional | Pending |
| MAC | Multiplexing/demultiplexing | Basic | Yes |
|  | Retransmission with FEC coding (HARQ) | Basic | Yes |
|  | UL scheduling (e.g., SR, BSR, DSR, PHR, LCP) | Basic | Yes |
|  | Random access (CBRA, CFRA) | Basic | Yes |
|  | UE power saving (e.g., DRX) | Additional | Yes |
|  | Beam management (BFD, BFR) | Additional | Yes |
|  | Support of packet duplication | Additional | Yes |

Proposal 1: RAN2 adopt the following methodology to study the user plan functions

- Adopt the categorization in the table 1 to track the basic function and additional function defined for 5G user plane, and discuss the category for each function and whether it can be reused in 6G

- Add extra functions which specifically support for 6G use case into the table

- Discuss specific protocol layer to carry each function which is consolidated in the above table.

2.2	Requirements for User Plane

In the last meeting, it was confirmed that 6G UP should aim to reduce the radio and end-to-end latency for general services (including eMBB) and also that the UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. In the 5G NR, data packets are sequentially processed by each sublayer of L2 and their respective function blocks. This would mean to process in one sublayer (e.g., PDCP) followed by the next sublayer (e.g., RLC) and so on. This is one potential area to be improved from the latency perspective as well as hardware-processing friendly.

In general, we consider there can be three aspects for improving the UP design in this study, which are minimizing the inter-layer interaction, adaptability and being lean. Note that some of these may be co-exist, while some may be not.

Minimizing inter-layer interaction is generally considered in protocol stack design. In NR, however, interaction between layers has increased. For instance, the date packet discard timer is managed at PDCP layer but is also utilized for DSR reporting in MAC layer and for polling and/or ARQ retransmission in RLC layer. As a result, discard timer-related indications are exchanged among these sublayers with greater frequency in NR. In the context of 6G, reducing inter-layer interactions is a potential objective.

Looking ahead to 6G, adaptability is an important consideration for Layer 2 protocol stacks, which is vital to support a wide range of services and devices—ranging from very low to very high data rates, short to long delays, lossless to best-effort delivery, and from small battery to power connected devices. Addressing as many combinations of requirements as possible is desirable. For example, prior to Rel-19 NR, RLC UM supports services requiring low reliability and short latency, while RLC AM supports services demanding higher reliability and longer latency. With NR Rel-19, enhanced RLC AM is introduced to handle use cases such as XR, which require both high reliability and short latency. However, supporting these diverse requirements may increase UE complexity, especially when traditional traffic types (e.g., VoIP) coexist with high-demand services.

In addition to or alternatively to the adaptability, a lean protocol stack is also improving the UP design, e.g. reduce processing time/latency, simplified protocol stack. In the 5G (especially at later releases), some advanced features have been specified, such as two-step RACH, EDT, enhanced RLC AM. If these advanced features are popular/widely used, they could be included in 6G from the start. This could replace the original feature and reduce number of options. For example, RAN2 could discuss if it is possible to remove ARQ or shift ARQ down close to HARQ, move reordering up to PDCP, eventually remove RLC sublayer. Potentially a more powerful HARQ would be required. In example, RAN2 study a possible lean 6G User Plane protocol stack by:

- Avoiding duplicated function in different sublayers

- Adopting commonly applicable advanced features (e.g. two step RACH, small data transmission, enhanced RLC AM) from Day 1

- Considering remove/merge RLC sublayer into other sublayers.

Proposal 2: RAN2 UP design should aim to minimize the inter-layer interaction, adaptability and/or lean design.

# 3. Conclusion

In this contribution we discussed the required functionality and the requirement to be consider for the protocol design.

Proposal 1: RAN2 adopt the following methodology to study the user plan functions

- Adopt the categorization in the table 1 to track the basic function and additional function defined for 5G user plane, and discuss the category for each function and whether it can be reused in 6G

- Add extra functions which specifically support for 6G use case into the table

- Discuss specific protocol layer to carry each function which is consolidated in the above table.

Table 1. L2 functions in 5G NR and categorization

| Protocol layer in 5G | Essential functions | Category | Reused in 6G? |
| --- | --- | --- | --- |
| SDAP | Mapping of packet from upper layer (CN) to radio bearer (e.g. QoS flow to DRB mapping) | Basic | Yes |
|  | Support of reflective QoS flow | Additional | pending |
| PDCP | Encryption, decryption | Additional | Yes |
|  | Integrity protection, verification | Additional | Yes |
|  | Header compression (ROHC) | Additional | Yes |
|  | Out-of-order delivery | Additional | pending |
|  | Support of PDCP duplication | Additional | pending |
|  | Packet discarding (duplicated detection, timer-based) | Additional | Yes |
| RLC | Segmentation and reassembly | Basic | Yes |
|  | Retransmission (ARQ) for ensuring packet delivery | Additional | Pending |
| MAC | Multiplexing/demultiplexing | Basic | Yes |
|  | Retransmission with FEC coding (HARQ) | Basic | Yes |
|  | UL scheduling (e.g., SR, BSR, DSR, PHR, LCP) | Basic | Yes |
|  | Random access (CBRA, CFRA) | Basic | Yes |
|  | UE power saving (e.g., DRX) | Additional | Yes |
|  | Beam management (BFD, BFR) | Additional | Yes |
|  | Support of packet duplication | Additional | Yes |

Proposal 2: RAN2 UP design should aim to minimize the inter-layer interaction, adaptability and/or lean design.

# References

[1] RAN2#131bis Chair notes