3GPP TSG-RAN WG2 #132	R2-2508671

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.2.1 RRC Modelling and connection management

Title:		Discussion on RRC functionality and states

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#131bis, the RRC general framework was discussed including some functional specific aspects. For the RRC state modelling, the following agreements were made [1]. In this contribution, we discuss RRC functionality and the RRC state modelling and then provide our views.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality

# 2. Discussion

2.1	RRC functionality

There are multiple aspects to be studied from CP perspective including:

- Initial access procedure and System information delivery

- RRC connection management (establishment, reconfiguration and release)

- RRC connection failure and recovery (RLF, re-establishment)

- Paging framework

- Network energy saving and UE power saving

All these are not independent, but tightly related each other. For example, the initial access procedure involves the transmission of the essential information and signals required for cell access, which is largely related to the network energy saving from the lessons in 5G (e.g. NES). Another essential aspect is the RRC connection management including the study on RRC states to be supported. The network energy saving and the UE power saving are very important aspects regardless of the RRC states. Then for RRC_IDLE state, an efficient paging framework is required from Day1.

Observation 1: RAN2 study multiple CP aspects jointly to achieve system-wide optimization.

Most of these functionalities are discussed in the corresponding sub-agendas. In the following, we discuss the RRC connection management aspects.

2.2	RRC connection management

In 5G, there are three RRC states: RRC_CONNECTED (Connected), RRC_IDLE (Idle) and RRC_INACTIVE (Inactive). For 6G, it is under discussion whether the Inactive is supported or new sub-state is introduced. We firstly focus on Connected and Idle.

From the Idle to Connected transition, the UE performs the initial access for RRC connection setup based on the essential system information broadcasted in the serving cell. After security activation, the network may request the UE capability information for the UE (unless the core network provides it) and then provides the dedicated configurations for the UE to perform the intended services. This takes relatively longer period to complete, while it would be considered as baseline for 6G, too. Further details of each component (e.g. initial access, system information delivery, security) can be discussed separately (e.g. 10.3.2.3).

During the Connected state, the RRC reconfiguration is applied for new/additional configuration, modification of the current configuration and release of the current configuration. We assume this basic framework should be fine for 6G. In the other sub-agenda (10.3.2.2), the improvement of RRC reconfiguration is to be discussed for better efficiency, reliability, and/or removing ambiguity.

From the Connected to Idle transition, the network releases the RRC connection of the UE and may remove the UE context. As the RRC connection should be controlled by the network, there is no need for any optimization, for example the UE autonomous release, or the UE request for fast/early release. more efficient RRC state control should be studied under Inactive/sub-state discussions. The same framework would be considered as baseline for 6G also for transition to the Idle.

Proposal 1: RAN2 assume that the RRC state transition framework between the RRC_Idle and RRC_Connected in 5G can be baseline for 6G, where details of each component (e.g. initial access, system information) are studied further.

2.3	RRC state modelling

2.3.1	Requirements for Inactive or sub-state

In the last meeting, it was confirmed to support at least the Connected and Idle. In addition, Inactive or sub-state (which intends sub-state of Connected) were discussed. It was concluded that RAN2 should study to support at least the following functions:

- UE based mobility (e.g. cell selection/reselection)

- UE context and identification

- Energy efficient for both UE and NW (while UE is in Inactive or new sub-state)

Then, to decide whether to support one of two additional modes (or maybe none of two as one option), the following aspects need to be studied further:

- Fast transitions

- Data transfer

- Improvements to UE identification and RNA management

- Paging/wake-up simplifications and/or enhancements.

UE based mobility

This basically intends the UE mobility is similar or the same as that in Idle mode. This could avoid signaling between the UE and the network upon mobility and also reduce the UE power consumption while there is no data to be transmitted. Regardless of the selection of Inactive or new sub-state, the design goal should be that the network does not need to know exactly which cell the UE is moving and camping on while the UE is in Inactive or sub-state. The UE location can be managed by a RAN area like the RNA. From the UE based mobility framework perspective, we do not see big differences between the Inactive and the sub-state. One potential aspect is that when the sub-state is defined, there may be another way to efficiently manage the UE mobility. We wonder if this possibility is precluded from the beginning.

For the UE based mobility, one issue (lessons) from the 5G Inactive is the paging monitoring. Even though the UE is in Inactive, the UE needs to monitor paging from both RAN node via RAN paging and CN via CN paging like Idle mode. The latter is to ensure the fallback from the unexpected case where RAN paging is not reached and the recovery via CN paging is needed. This would cause the additional restriction. It is necessary to study how we can simplify the paging reception in Inactive or new sub-state. Merging or unifying the paging framework for both types of paging may be one solution. For example, a certain simple signaling can be defined like PEI or LP-WUS in 5G. It triggers the actual paging monitoring, which depends on UE’s state and/or maybe explicit indication in the signaling. From the paging framework perspective, there may be some differences between two approaches, because it is not very clear whether the UE in the sub-state needs to monitor CN paging for fallback.

UE context and identification

Upon UE based mobility, the UE may update its location periodically or the UE may move to the Connected for data transmission or reception. As long as the UE is located in the configured RNA, the mechanism can work well from the specification perspective. The other aspect related to the UE identification is the fast transition to the Connected for some purposes (e.g. bigger data transmission, responding to paging for MT data/call). In the NR, within the configured RAN notification area (RNA), the UE can quickly resume its RRC connection with proper configurations by using the I-RNTI as the anchor information.

However, other issue (lessons) from the 5G is that due to some reasons (e.g., Xn connection unavailability), the UE context retrieval and its identification may fail. Of course, if the RAN area is reasonably small (e.g. only cells under one CU), it should work well. There is the trade-off between the benefit (e.g. UE based mobility within wider area) and the drawback (e.g. UE context retrieval failure). It should be clarified how big this problem is and also if there is any other problem.

Energy efficient for both UE and NW

The main motivation to introduce the Inactive or new sub-state is to save the UE power consumption by avoiding the location updates at every cell change occasion and performing the longer cycle of the DRX. For some aspects (e.g., signaling upon UE mobility), the network energy saving can be also expected. We assume there would be no big difference between two approached (Inactive or sub-state) with respect to this aspect.

The other motivation is to save the UE power consumption for UL data transmission by supporting the data transmission from the Inactive or sub-state like the SDT in the NR. This basically targets small data transmission, as the data size is bigger, it is much more suitable for the UE to move to the Connected. In the case of the sub-state, it is not sure if the UE should keep staying in the sub-state, given that the UE can move to full Connected easier than the case of Inactive. This aspect should be studied further.

Proposal 2: RAN2 discuss the following aspects related to support of Inactive or new sub-state:

- UE based mobility:

  - whether to assume the same requirement as NR Inactive? E.g., no need of network visibility for UE mobility within the configured RAN area, need to support monitoring both RAN paging and CN paging.

- UE context and identification:

  - whether the Xn connection loss/unavailability problem should be solved by functional modelling?

- Energy efficiency:

  - whether data transmission without (sub-)state transition is not the requirement but one way to achieve the fast data transmission especially for Inactive case? E.g., if transition from sub-state to Connected is so fast and simple, it is possible that data transmission can be in Connected.

2.3.2	Comparison of Inactive and sub-state

In this section, we discuss each requirement for Inactive and sub-state based on the discussions in the 2.3.1 and show our understanding on comparison of Inactive and sub-state in the Table-1 below. In high-level, the same or similar mechanism could be applied for both Inactive and sub-state, if we take the same requirement for both. On the other hand, there could be some differences in the sub-state compared with the Inactive.

Table-1: Comparison of Inactive and Sub-state

|  | Inactive | Sub-state |
| --- | --- | --- |
| Mobility | Cell reselection without NW awareness | Similar to Cell reselection without NW awareness, FFS for other possibility depending on exact requirement |
| UE context | UE AS context stored in anchor 6G NB | UE AS context stored in serving 6G NB, FFS for other possibility depending on exact requirement for mobility |
| UE identification | Using I-RNTI as anchor information for context retrieve and resume, FFS for other possibility depending on exact requirement | Using UE dedicated identifier common for a configured area, FFS for other possibility depending on exact requirement for mobility |
| Energy efficiency | The main power consumption comes from: - RRM measurement - Paging monitoring | The main power consumption may come from: - RRM measurement. FFS for Paging/WUS monitoring depending on exact requirement for paging/WUS |
| Fast transition | RRC Resume with the stored UE AS context, FFS for more efficient air interface signalling exchange and context retrieve procedure | No RRC Resume-like procedure is expected, Depending on the modelling, lower layer signaling (e.g. MAC) may be considered. |
| Data transfer | Small DL/UL data transmission (SDT) without entering Connected mode, FFS for more efficient data transmission mechanism | Depending on exact requirement, may support small DL/UL data transmission during sub-state, or in CONNECTED after sub-state transition |
| RNA management | RNA consists of Cell list, TAC and optionally RAN area code. FFS for improvements considering practical deployment demand, e.g., impact on interface between 6G NBs | Depending on exact requirement for mobility (e.g. NW awareness), may support RNA-like management |
| Paging/wake-up signal | UE monitors both RAN Paging and CN Paging (for fallback), FFS for need of improvement considering energy efficiency | UE monitors a certain WUS, depending on the modelling, e.g., Low power radio signaling |

Proposal 3: RAN2 to take the table-1 for comparison of Inactive and Sub-state into account for further study.

# 3. Conclusion

In this contribution we discussed RRC functionality and the RRC state modelling. We made the following proposals.

Proposal 1: RAN2 assume that the RRC state transition framework between the RRC_Idle and RRC_Connected in 5G can be baseline for 6G, where details of each component (e.g. initial access, system information) are studied further.

Proposal 2: RAN2 discuss the following aspects related to support of Inactive or new sub-state:

- UE based mobility:

  - whether to assume the same requirement as NR Inactive? E.g., no need of network visibility for UE mobility within the configured RAN area, need to support monitoring both RAN paging and CN paging.

- UE context and identification:

  - whether the Xn connection loss/unavailability problem should be solved by functional modelling?

- Energy efficiency:

  - whether data transmission without (sub-)state transition is not the requirement but one way to achieve the fast data transmission especially for Inactive case? E.g., if transition from sub-state to Connected is so fast and simple, it is possible that data transmission can be in Connected.

Proposal 3: RAN2 to take the table-1 for comparison of Inactive and Sub-state into account for further study.

Table-1: Comparison of Inactive and Sub-state

|  | Inactive | Sub-state |
| --- | --- | --- |
| Mobility | Cell reselection without NW awareness | Similar to Cell reselection without NW awareness, FFS for other possibility depending on exact requirement |
| UE context | UE AS context stored in anchor 6G NB | UE AS context stored in serving 6G NB, FFS for other possibility depending on exact requirement for mobility |
| UE identification | Using I-RNTI as anchor information for context retrieve and resume, FFS for other possibility depending on exact requirement | Using UE dedicated identifier common for a configured area, FFS for other possibility depending on exact requirement for mobility |
| Energy efficiency | The main power consumption comes from: - RRM measurement - Paging monitoring | The main power consumption may come from: - RRM measurement. FFS for Paging/WUS monitoring depending on exact requirement for paging/WUS |
| Fast transition | RRC Resume with the stored UE AS context, FFS for more efficient air interface signalling exchange and context retrieve procedure | No RRC Resume-like procedure is expected, Depending on the modelling, lower layer signaling (e.g. MAC) may be considered. |
| Data transfer | Small DL/UL data transmission (SDT) without entering Connected mode, FFS for more efficient data transmission mechanism | Depending on exact requirement, may support small DL/UL data transmission during sub-state, or in CONNECTED after sub-state transition |
| RNA management | RNA consists of Cell list, TAC and optionally RAN area code. FFS for improvements considering practical deployment demand, e.g., impact on interface between 6G NBs | Depending on exact requirement for mobility (e.g. NW awareness), may support RNA-like management |
| Paging/wake-up signal | UE monitors both RAN Paging and CN Paging (for fallback), FFS for need of improvement considering energy efficiency | UE monitors a certain WUS, depending on the modelling, e.g., Low power radio signaling |

# References

[1] RAN2#131bis Chair notes