3GPP TSG-RAN WG2 #132	R2-2508673

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.2.3 Initial and System Access and Others

Title:		Initial thoughts on aggregation of multiple pieces of spectrum

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#131bis, RAN2 discussed the spectrum aggregation a little bit with the general agreements below [1]. In this contribution, we discuss some aspects of spectrum aggregation and provide our views.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

# 2. Discussion

2.1	Support of carrier aggregation and BWP

One of the most important functions in the legacy system, e.g. 5G NR is the carrier aggregation (CA). The new concept of more flexible frequency resource utilization is the Bandwidth Part (BWP). Both are essential in the NR. For 6G, to support high data rate, or to adapt to moderate data rate of UE or and to save UE power consumption, it is necessary to continue support spectrum aggregation and bandwidth part (or the like).

In the NR, spectrum resource is modelled into carrier, and each carrier form a cell with or without SSB signal. For a given UE, it could be configured with multiple carriers, i.e. carrier aggregation, or configured to work on a smaller BWP for power saving. Both carrier aggregation and BWP operation are visible only up to MAC layer, invisible to SDAP/PDCP/RLC.

In 6G, it depends on whether (discontinuous or continuous) spectrum resource owned by a 6G NB will be identified as carrier/cell, or just purely used as schedulable resource in frequency domain. In the latter case, it may be invisible to MAC layer as well. Modelling of frequency resource is to be discussed in the subsection 2.2 below. Regardless of the support of the latter case, the former case just like the legacy should be supported at least for consecutive frequency resources of one band and another consecutive frequency resources of the other band by the CA with the BWP within a cell.

Proposal 1: RAN2 assume to support the CA as baseline for aggregating consecutive frequency resources of one band (as one cell) and another consecutive frequency resources of the other band (as the other cell) and study its improvements.

Proposal 2: RAN2 assume to support the BWP as baseline at least for consecutive frequency resources within a cell and study its improvements.

2.2	Spectrum aggregation of multiple pieces of spectrum

In the last meeting, it was agreed to study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. As RAN1 will also study this but do the detail discussions from Feb. meeting, RAN2 should start discussions from higher layer perspective.

Looking at the companies’ views in the last meeting, there are many companies who consider such the multiple pieces of spectrum to be aggregated as one cell. The main motivation is to reduce signaling overhead to support the CA. Also, it is to avoid unnecessary complicated mechanisms which mainly exist in L1 functions, e.g. scheduling of multiple cells, PUCCH/PUSCH transmission.

We discuss the issues that need to be addressed in the legacy CA from higher layer perspective. The biggest problem is that there are a lot of configurations necessary per cell. For example, the network has to configure, per cell, L1 parameters (e.g., PDCCH, PDSCH, PUCCH and PUSCH configurations) and CSI measurement configurations. If more than one BWPs are supported, then the above configuration should be per BWP and also BWP related L2 parameters should be also configured. Other issues are the complicated UE behaviour during the CA or SCell activation delay. For example, when the DRX is configured, the DRX operation (e.g. DRX Active Time, short or long cycle decision) is common among the aggregated cells or the DRX group (if secondary DRX is configured).

Proposal 3: RAN2 discuss whether the following issues observed in the legacy CA need to be addressed:

- “per cell” configuration overhead (e.g., L1 parameters per cell, CSI measurement configurations)

- L2 UE behaviour (e.g., common DRX among the aggregated cells, SCell activation delay)

- L1 functions (e.g., multiple-cell scheduling, PUCCH/PUSCH transmissions)

Given the above issues need to be addressed, it is worth studying to aggregate multiple pieces of spectrum in 6G. The simplest way is to consider those spectrums are a part of a cell. In other words, a cell may consist of multiple pieces of spectrums. If such the aggregated spectrums can be managed as if they are different frequency resources within a cell, per cell configuration can be reduced a lot. Scheduling for those spectrums could be also simplified compared with the per cell scheduling or cross carrier scheduling in 5G NR.

Another way is to consider each part of the aggregated spectrums as a kind of BWP. Each BWP may be used as the single active BWP, or more than one BWPs may be used together, i.e. BWP aggregation, where more flexible resource usage may be expected without undesirable delay like SCell activation delay for CSI report.

We see some potential benefits in having aggregation of the multiple pieces of spectrums. On the other hand, key aspects would be RAN1 and/or RAN4 scope rather than RAN2. It would be good to study in RAN2 from higher layer perspective but should be coordinated with RAN1 and/or RAN4 study in 6G.

Proposal 4: RAN2 study potential usage of aggregation of multiple pieces of spectrums with coordinating with RAN1 and/or RAN4, where examples are:

- consider each part of the aggregated spectrums as a kind of BWP, and use single active BWP at a time,

- consider each part of the aggregated spectrums as a kind of BWP, and use more than one BWPs as BWP aggregation.

# 3. Conclusion

In this contribution we discussed support of the CA and the BWP, and also Spectrum aggregation of multiple pieces of spectrum. The following proposals are made:

Carrier aggregation and BWP:

Proposal 1: RAN2 assume to support the CA as baseline for aggregating consecutive frequency resources of one band (as one cell) and another consecutive frequency resources of the other band (as the other cell) and study its improvements.

Proposal 2: RAN2 assume to support the BWP as baseline at least for consecutive frequency resources within a cell and study its improvements.

Spectrum aggregation of multiple pieces of spectrum:

Proposal 3: RAN2 discuss whether the following issues observed in the legacy CA need to be addressed:

- “per cell” configuration overhead (e.g., L1 parameters per cell, CSI measurement configurations)

- L2 UE behaviour (e.g., common DRX among the aggregated cells, SCell activation delay)

- L1 functions (e.g., multiple-cell scheduling, PUCCH/PUSCH transmissions)

Proposal 4: RAN2 study potential usage of aggregation of multiple pieces of spectrums with coordinating with RAN1 and/or RAN4, where examples are:

- consider each part of the aggregated spectrums as a kind of BWP, and use single active BWP at a time,

- consider each part of the aggregated spectrums as a kind of BWP, and use more than one BWPs as BWP aggregation.

# References

[1] RAN2#131bis Chair notes