3GPP TSG-RAN2 Meeting #132	R2-2508678

Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Remaining issues of Rel-19 Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #131 made the following agreement

- Agreements

- RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

- For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication).

- For integrity failure, for now RAN2 assumes that there is no AS response to the reader.  Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

In this paper, we will discuss some remaining issues of Rel-19 Ambient IoT (A-IoT).

# Discussion

RAN2 has confirmed it is valid that there are some cases A-IoT NAS doesn’t provide a response in addition to the delayed response. It means that the NAS response is not required by AIOTF, and for Reader, it is ‘no-NAS response expected’. However, Reader still should be indicated the correct expectation to differentiate whether this is the case of ‘delayed NAS response’ or ‘no-NAS response expected’. Otherwise, Reader may continue sending R2D messages to schedule the D2R Layer Data Transfer message from the device even when A-IoT device has no NAS response to send.

CT1 #157 has agreed that for the A-IoT NAS message ‘integrity failure’ and ‘message too short’ case, A-IoT device should discard the NAS message, and NAS may provide an indication to lower layers. So, the A-IoT AS should be aware of the status of A-IoT NAS message, i.e., delayed NAS response, no-NAS response expected or regular NAS response.

In the D2R Upper Layer Data Transfer message format, the ’More Data Indication’ field (MDI) is specified as the indication whether there is more upper layer data in the segmentation scenario. Note that RAN2 has agreed that MDI should be set to 0 in the ‘delayed NAS response’ case which is aligned with the existing principle.

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

We prefer not to misuse the MDI field to the other cases, i.e., ‘no-NAS response expected’. Instead, a new field is introduced to indicate ‘no-NAS response expected’ so that all cases can be correctly indicated.

Proposal 1: (Issue 3-7) Besides 0 SDU in the D2R Layer Data Transfer message, A-IoT device uses a new field (instead of MDI field) to indicate ‘no-NAS response expected’ in order to differentiate with the ‘delayed NAS response’ case.

# Conclusion

Based on the discussion and observations, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: (Issue 3-7) Besides 0 SDU in the D2R Layer Data Transfer message, A-IoT device uses a new field (instead of MDI field) to indicate ‘no-NAS response expected’ in order to differentiate with the ‘delayed NAS response’ case.

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.