3GPP TSG-RAN WG2 Meeting #132							       	 R2-2508702

Dallas, USA, 17 - 21 November, 2025

Title: 	Discussion on RRM measurement event prediction

Source: 	Huawei, HiSilicon

Agenda item:	9.3.3

Document for:	Discussion/Decision

# Introduction

In RAN2#131 meeting, agreements related to functionality management for RRM measurement event prediction were reached as follows [1]:

Agreements - Aim for a single framework for event prediction and performance monitoring. From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring. Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction. - For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters) - For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) - For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

In accordance with the WID, this document discusses functionality management for RRM measurement event prediction, covering aspects such as UE capability, applicability, and inference configuration/reporting. Furthermore, as aligned with the agreements from RAN2#131, with the lack of consensus on the feasibility of performance monitoring for direct event prediction, this document proceeds with discussing the functional management for indirect measurement event prediction only.

# Discussion

## Procedure for applicability reporting

RAN2 #130 made the agreement as following:

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). - No prohibit timer is introduced.

As mentioned above, indirect measurement event prediction is based on measurement prediction for different use cases, such as frequency domain, intra-frequency temporal domain case A, temporal domain case B or spatial domain, as captured in [1]. The applicability reporting procedure for the indirect measurement event prediction can thus follow exactly the same procedure as the applicability reporting for RRM measurement prediction.

Observation 1: Since indirect measurement event prediction is based on measurement prediction, applicability reporting procedure for the indirect measurement event prediction can follow the same procedure as applicability reporting for the RRM measurement prediction.

Based on this observation and RAN2 agreement to follow the beam management agreements for both RRM measurement prediction and RRM measurement event prediction, the applicability reporting procedure for RRM measurement event prediction shall align with that of RRM measurement prediction. Furthermore, the parameters for the applicability check can be reused from those defined for RRM measurement prediction.

Proposal 1: The applicability configuration and report procedure for RRM measurement event prediction is the same as for RRM measurement prediction, including the set of inference related parameters.

## Inference and reporting configuration

In RAN2 #130 meeting, we agreed the following to be included in the inference configuration for RRM measurement event prediction:

- The length of PW for the associated AIML model

- Event-related information (event type and event-related parameters)

TR 38.744 has clarified that the inference configuration for RRM measurement event prediction, except for the event type, will be defined within RRM measurement prediction. As for the event related information (event type and the event-related parameters), considering that the existing RRM measurement framework is reused for RRM measurement prediction, the contents of event-related information (e.g., A1-A6 thresholds, timeToTrigger, etc.) can be based on the legacy design.

Proposal 2: The same inference and reporting configuration (e.g., the length of PW, and the inclusion of the event-related information into the reporting configuration) is used for both RRM measurement event prediction and RRM measurement prediction.

## Event reporting aspects

The handover model was defined to facilitate SLS, where measurement event is predicted based on either intra-frequency temporal domain case A or intra-frequency temporal domain case B. For both cases, network starts with 40ms handover preparation once a predicted measurement event is received. A handover command will be transmitted after preparation is completed at the earliest. After handover command is sent, another 40ms is assumed for HO execution duration. Based on the modelling described in TR 38.744, if a measurement event is predicted based on intra-frequency temporal domain case A, there are two options for deciding the time point to transmit handover command:

Option 1: Relying on legacy measurement event. E.g., at t0 a measurement event is predicted at some point of time in future. Network performs HO preparation but will not transmit a handover command until a real measurement event is reported for the same neighboring cell.

Option 2: Relying on predicted measurement event. E.g., at t0, measurement event is predicted at future time t1. Network performs HO preparation and transmits a handover command when an entry condition of the predicted measurement event is met based on actual measurement result at t2, unless t2 is earlier than handover preparation phase end time. In the latter case, network transmits a handover command immediately after handover preparation phase. In this way, not only handover preparation time can be saved but also a handover can be executed earlier.

In general, the handover modelling option can be left to NW implementation. It means that it should be possible for the network to configure both predicted event reporting and actual event reporting at the same time, for the same measurement object. Then, the network may decide when to execute the handover preparation and send the handover command.

Observation 2: It should be possible for the network to configure both predicted event reporting and actual event reporting at the same time, for the same measurement object.

Furthermore, the network should be able to distinguish whether the event report is for a legacy event or a predicted event. This distinction enables the network to decide when to initiate the transmission of the handover command based on either option 1 or option 2. Consequently, an indication is required to identify whether the reported event is actual or predicted.

Proposal 3: An indication is needed to indicate whether the measurement event report is triggered based on an actual or predicted event to help the network decide the time point to transmit a handover command.

## UE capability for RRM measurement event prediction

Indirect measurement event prediction for the use cases of intra-frequency temporal domain case A and B and frequency domain is based on the measurement result(s) predicted by a RRM measurement prediction model [1]. The support of indirect measurement event prediction for a use case will hence depend on whether UE also supports the RRM measurement prediction for the corresponding use case.

Observation 3: The support of indirect measurement event prediction for a use case depends on whether UE also supports the RRM measurement prediction for the corresponding use case.

On whether the UE needs to additionally report separate capabilities for different event types A1~A6, as long as the UE supports the RRM measurement prediction for a use case, the UE should be able to support all the event types in our view. Hence there is no need to indicate which event types are supported in the UE capability signaling for a use case. As long as the legacy events A1-A6 are supported (as in eventA-MeasAndReport), it should also be applicable to the indirect measurement event prediction (i.e. a pre-requisite can be added to the capability for each use case that it can only be set if the legacy eventA-MeasAndReport is supported).

Observation 4: There is no need to separately indicate the support of the different event types A1-A6 that can be predicted and reported. As long as the legacy events A1-A6 are supported (as in eventA-MeasAndReport), it should also be applicable to the indirect measurement event prediction.

Based on the observation 3 and 4, it is clear that there is no need for an explicit capability signaling for indirect measurement event prediction. Hence it is proposed:

Proposal 4: No explicit capability signaling is required for indirect measurement event prediction. It will be a component in the RRM measurement prediction capability signaling.

Whether RRM measurement prediction capability signalling is per use case or sub-use case is discussed in our RRM measurement prediction Tdoc [3].

# Conclusion

Applicability reporting

Observation 1: Since indirect measurement event prediction is based on measurement prediction, applicability reporting procedure for the indirect measurement event prediction can follow the same procedure as applicability reporting for the RRM measurement prediction.

Proposal 1: The applicability configuration and report procedure for RRM measurement event prediction is the same as for RRM measurement prediction, including the set of inference related parameters.

Inference and reporting configuration

Proposal 2: The same inference and reporting configuration (e.g., the length of PW, and the inclusion of the event-related information into the reporting configuration) is used for both RRM measurement event prediction and RRM measurement prediction.

Event reporting aspects

Observation 2: It should be possible for the network to configure both predicted event reporting and actual event reporting at the same time, for the same measurement object.

Proposal 3: An indication is needed to indicate whether the measurement event report is triggered based on an actual or predicted event to help the network decide the time point to transmit a handover command.

UE capabilities

Observation 3: The support of indirect measurement event prediction for a use case depends on whether UE also supports the RRM measurement prediction for the corresponding use case.

Observation 4: There is no need to separately indicate the support of the different event types A1-A6 that can be predicted and reported. As long as the legacy events A1-A6 are supported (as in eventA-MeasAndReport), it should also be applicable to the indirect measurement event prediction.

Proposal 4: No explicit capability signaling is required for indirect measurement event prediction. It will be a component in the RRM measurement prediction capability signaling.

# Reference

[1] 3GPP TR 38.744 v1.2.0

[2] 3GPP TSG-RAN WG2 Meeting #131 chair note of AI/ML for mobility