3GPP TSG-WG2 Meeting #132	R2-2508732

Dallas, USA, November 17 – 21, 2025

Agenda Item:	10.2.1

Source:	Ericsson

Title:	Improvements on UE capabilities

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#131-bis, the following was agreed:

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

This contribution discusses issues for UE capabilities and which aspects to prioritize for 6G.

# 2	Discussion

## 2.1	Capability filtering

It has been mentioned that better network filtering can help the UE to provide tailored UE capabilities and reduce the signalling size of the UE capability information message.

NR’s band filtering is an effective mechanism since it allows UEs to report only band combinations with bands which are actually used in the current PLMN. But NR offers also filters for specific features e.g. uplinkTxSwitchRequest-r16, codebookTypeRequest-r16 and srs-SwitchingTimeRequest, which turned out to be problematic in NR.

Taking one of the NR cases of per feature filtering as example (srs-SwitchingTimeRequest), the UE will only include in its UE capability information message SRS switching capabilities if requested by the network. This was adopted since this feature was defined per band per band combination, which is heavy in terms of signalling overhead. The srs-SwitchingTimeRequest filter avoids that overhead if the UE supports the features while the network does not. However, once any gNB in that PLMN intends to use the feature, all gNBs should enquire those capabilities and hence all UEs that support the feature will anyway have to include it in their UE capability information message.

In general, filtering UE capabilities is only beneficial for functionality that meets all the following conditions:

- Capability signalling is heavy in size and processing complexity,

- Many UEs implement the feature,

- No gNB in a PLMN supports/uses that feature.

All the conditions must hold for a per feature filtering to have some value. For features which are implemented and used, per feature filtering just adds complexity since the filter logic must be implemented in both the UE and the network, and both UE and the network will include the capabilities most of the time anyway.

And for features which no UE implements, capability filtering is also not required.

UE capability filtering is only useful if the UE implements a feature which no gNB in a given PLMN implements or uses. For operating bands this is often the case; for other features typically not.

Furthermore, filtering tends to increase the number of UE capability re-enquiries if some gNBs in a PLMN intend to use a feature for which other gNBs in the same PLMN did not enquire the corresponding capabilities.

Using per feature filters tend to increase UE capability re-enquiries.

We should thus find ways to introduce a slim signalling from the beginning rather than relying on filtering that can hinder the use of a feature once implemented and increase the number of UE capability re-enquiries.

Introduce capability filters only for functionality that a) adds considerable signalling overhead and b) is likely supported by many UEs but c) used only by a subset of PLMNs (e.g. frequency bands).

## 2.2	Fixing problems in the field

Several companies mentioned challenges related to inter-operability device testing and suggested to seek solutions overcoming those.

Despite all IODT we discover UEs that don’t support a feature correctly despite setting the corresponding capability signalling. Failures can be different in nature but are often fatal for the UE’s performance. Since other UEs that set the same capability work properly, it would not be acceptable to disable the feature for all UEs. But configuring the faulty UE repeatedly with the problematic functionality is also not a viable approach. In these cases the RAN is usually required to apply a work-around for the problematic UEs.

If a feature does not work properly with a UE in the field, networks need to identify the affected UEs and to omit the configuration of the affected functionality.

It is cumbersome but possible to alleviate the error with an affected UE. Via the OAM system the operators could then configure their RAN nodes to disable the feature for all UEs with the IMEI SV that was found in the affected UE. In addition, the operator would need to configure the CN (AMF) so that it passes the IMEI SVs of all UEs to the RAN (usually not the default). However, we notice that such errors are usually not tied to a single UE model but rather to all UEs that use the same chipset hard- and software version. In some cases, it affected hundreds of IMEI SVs which would need to be identified one-by-one and which all operators would then need list in their OAM systems.

Problems are typically common for all UE models that are based on the same chipset version. Those UE models can correspond to hundreds of IMEI SVs which are hard to discover and use for work-arounds.

Furthermore, we discovered cases where chipset vendors updated their software and where UE vendors pushed the new firmware to their UEs to fix the problem. However, in some cases UE vendors did not increment the IMEI SV. Hence, IMEI SV based filtering would classify this UE anyway as faulty and omit the functionality even though the updated UE supports it correctly now.

To overcome the above-mentioned problems, we propose that UEs signal an identifier for the contained chipset type covering both the hard- and software version. We would like to emphasize that such identifier is not meant to replace standardized UE capabilities. Those are required for full inter-operability in a global market. But the chipset ID would allow the gNB to address problems that were identified with a specific chipset and with all the UEs that use this chipset. Since the gNB would use the chipset ID to adjust the UEs’ configuration, it would be natural to include it in the UE capability container, too.

Furthermore, we noticed a different but related problem related to IODT opportunities, which was also raised in the previous meeting. Traditionally, chipset vendors were encouraged to perform IODT with at least two large network vendors before setting the corresponding bits in their chipset’s capabilities. In some cases that leads to the problem that features cannot be rolled out since chipset vendors lack IODT opportunities with a second NW vendor. Even if the risk of identifying problems with the secondary network’s may be low, the risk and consequences are considered too high. As explained above, a UE that sets a capability bit but does not support the feature correctly, risks bad performance or may require all NWs to disable the feature for all UEs. However, the chipset ID would allow networks to cater for those UEs in the unlikely event that the chipset that had been tested with only one or two networks runs into problem with a later NW vendor’s implementation.

A chipset ID would make it easier to cater for those UEs in the unlikely event that the chipset that had been tested with only one or two networks runs into problem with a later NW vendor’s implementation.

Based on the discussion above, we propose the following:

Study how a chipset ID that identifies the chipset type and its hardware- and software version can help to overcome problems in the field.

## 2.3	RACS ID

It has been mentioned that RACS ID should be a feature to support from day one in 6G, and early coordination with SA2 is needed to enable this. While RACS ID can be beneficial to avoid certain UE capability enquiries, most of UE capability enquiries are already avoided by having the UE capabilities stored in the core network (AMF). In NR, RACS ID can optionally be used to have a further reduction in the number of UE capability enquiries. Such feature can also be considered for 6G to reduce the number of UE capability enquiries. But it should not be only mechanism since appropriate filtering design (as discussed in 2.1.1), having UE capabilities stored in the network and appropriate UE capability signaling are key components to reduce the number of enquiries.

Thus, while we think it is important to aim for reducing the number of UE capability enquiries, we do not think RACS is the main feature to achieve this (even though it is a possible option).

RACS ID can reduce the number of UE capability enquires, but other aspects/solutions can have a higher impact on such reduction and should be prioritized.

## 2.4	Flexibility and band combination design

The UE capability framework in NR has signalling flexibility on multiple granularities (per UE, per band, per band combination, per band-per band combination). While the ‘per UE’ and ‘per band’ signalling allow for flexibility with reasonable signalling size, the ‘per band combination’ and ‘per band-per band combination' signaling has a huge impact on the overall UE capability size and is the reason for NR’s problematically complexity and large UE capability signalling size.

Some of the issues with the current band combination lists are:

- Unnecessary flexibility: many UEs report certain features with the same value across band combinations even though a feature can be supported differently for each band combination. For instance, simultaneousCSI-ReportsAllCC is a per band combination parameter which indicates the number of CSI report(s) which the UE can simultaneously process across all CCs within a band combination. But there are UEs that support the exactly same value for simultaneousCSI-ReportsAllCC for all the reported band combinations, even though such UEs support multiple band combinations with FDD and TDD bands. This is a hint that this feature could have been defined with lower granularity without limiting the UE implementation in practice.

- Multiple band combination lists: there are different band combination lists used e.g. for regular CA and for UL Tx switching, that have to be maintained on every specification update since the parameters defined for regular CA have to be instantiated also for UL Tx switching, while the fallback rules between the two lists are not clear;

- Inefficient band combination entry structure: The fallback band combination rules in NR can help to reduce the band combination list size. However, there are still cases where a UE that wishes to indicate the support of a few bands must generate dozens of band combinations. For instance, a UE that supports n25, n46, n66 and n71 with up to 3 carriers simultaneously, generates 50 band combination entries to list all possible permutations including different positions of the UL carriers and with contiguous and non-contiguous constellations.

- Use of featureSetCombinationId: NR allows the UE to reuse a FeatureSetCombination (FSC) for several band combinations with the aim to reduce the signalling. But such reuse is not widely applied in practice. Many UEs generate almost as many FeatureSetCombinations as BandCombinations, i.e., hardly any two BandCombinations refer to the same FeatureSetCombination. And even though FSC:s comprise only of IDs that point to FeatureSetDownlink/Uplink:s (FSD/FSU), the list of FSC:s tends to become one of the main size-contributors. Interestingly, we also observed that many UEs create relatively few FSDs and FSUs and refer to those from many different FSCs. Hence, some optimizations in the capability design seem beneficial while others appear premature and counter-productive.

Overall, the band combination framework in NR has multiple mechanisms that introduce complexity but not all of them are largely used or can considerably reduce the signalling size.

Band combination framework in NR offers a lot of flexibility to the UEs which results in a complex structure and large capability size. However, the offered flexibility is often not used.

Hence, while multiple solutions can be discussed to improve UE capabilities, it is important to have a band combination structure that can effectively reduce the size of UE capabilities but still provide a reasonable flexibility on what can be signalled and without adding unnecessary complexity. This structure should be the baseline for further improvements. We think this is the main aspect to be focused on since ultimately the UE needs to generate a band combination list based on the multiple features the UE can support and the network needs to handle such band combination list, regardless of additional features discussed for the UE capability framework.

Study a simplified UE capability reporting for 6G with better trade-off between size and flexibility compared to 5G. RAN WGs should also be more mindful about the granularities and the corresponding complexity/size when adding capability signalling.

As examples of simplifications to the UE capability framework, taking into account the issues raised above, are:

- Identify which parameters can have lower granularity: Many capability parameters that NR UEs signal per band combination (or higher granularity) could likely be signalled with coarser granularity (per band or per UE). As an example, it could be that certain UE envelope limitations such as codebookParametersPerBC-r16 could be defined per UE.

- Account for multiple features within a single band combination list: to this end, UL Tx switching for instance must be considered since the first release of 6G e.g. it could be assumed no simultaneous UL transmission is supported when more than one UL is reported for a band combination while an additional capability can be introduced to indicate which cases the UE can support simultaneous UL.

- Allow reporting of superset band combinations: if the UE could report a band combination with more band entries than a RAN4 defined band combination, it could reduce the need for the UE to report multiple similar flavours of band combinations (this was discussed in the past for NR but it was not possible to adopt such scheme since e.g. the bandwidthCombinationSet is tied to a specific band combination).

- Removal of FeatureSetCombinationId: since UEs don’t re-use FeatureSetCombinations in several BCs they just add overhead and processing complexity.

- Reduction of featureSet IDs: The FeatureSetIds (UL/DL) are in the range of [0..1024], thus each reference to an FSD/U consumes 11 bit. In practice, UEs define only few FSDs/FSUs and refer to them often. Since IDs contribute significantly to the total size of UE capabilities, their range should be chosen with care.

Study solutions to simplify the band combination list, including: identifying parameters which can have lower granularity, how to avoid multiple band combination lists, how to reduce the number of reported band combinations, usage of FeatureSets and FeatureSetCombinations.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	UE capability filtering is only useful if the UE implements a feature which no gNB in a given PLMN implements or uses. For operating bands this is often the case; for other features typically not.

Observation 2	Using per feature filters tend to increase UE capability re-enquiries.

Observation 3	If a feature does not work properly with a UE in the field, networks need to identify the affected UEs and to omit the configuration of the affected functionality.

Observation 4	Problems are typically common for all UE models that are based on the same chipset version. Those UE models can correspond to hundreds of IMEI SVs which are hard to discover and use for work-arounds.

Observation 5	A chipset ID would make it easier to cater for those UEs in the unlikely event that the chipset that had been tested with only one or two networks runs into problem with a later NW vendor’s implementation.

Observation 6	RACS ID can reduce the number of UE capability enquires, but other aspects/solutions can have a higher impact on such reduction and should be prioritized.

Observation 7	Band combination framework in NR offers a lot of flexibility to the UEs which results in a complex structure and large capability size. However, the offered flexibility is often not used.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce capability filters only for functionality that a) adds considerable signalling overhead and b) is likely supported by many UEs but c) used only by a subset of PLMNs (e.g. frequency bands).

Proposal 2	Study how a chipset ID that identifies the chipset type and its hardware- and software version can help to overcome problems in the field.

Proposal 3	Study a simplified UE capability reporting for 6G with better trade-off between size and flexibility compared to 5G. RAN WGs should also be more mindful about the granularities and the corresponding complexity/size when adding capability signalling.

Proposal 4	Study solutions to simplify the band combination list, including: identifying parameters which can have lower granularity, how to avoid multiple band combination lists, how to reduce the number of reported band combinations, usage of FeatureSets and FeatureSetCombinations.